| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0443 |
| --- | --- |

| Title: | Restriction for handling long-term reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Sachin Deshpande Ye-Kui Wang Jonatan Samuelsson | Tel: Email: | - hendry.hendry@lge.com sdeshpande@sharplabs.com yekuiw@qti.qualcomm.com jonatan.samuelsson@ericsson.com |
| Source: | LG Electronics, Sharp, Qualcomm, Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes two restriction options to address the issue of handling long-term reference pictures described in contribution JCTVC-L0254. The first restriction option is for Reference Picture Set (RPS) and the second restriction option is for the value of delta_poc_msb_present_flag[ i ].

# Proposal

Restriction option 1

The following highlighted restriction should be added for Reference Picture Set (RPS):

When long_term_ref_pics_present_flag is equal to 1, and one or more of the following conditions are true:

- The current picture currPic is a sub-layer non-reference picture or a RASL picture

- The current picture currPic has TemporalId greater than 0

for each reference picture picX in the DPB, when there is one or more than one other reference picture picY in the DPB for which PicOrderCnt( picY ) & ( MaxPicOrderCntLsb − 1 ) is equal to PicOrderCnt( picX ) & ( MaxPicOrderCntLsb − 1 ), or when PicOrderCnt( currPic ) & ( MaxPicOrderCntLsb − 1 ) is equal to PicOrderCnt( picX ) & ( MaxPicOrderCntLsb − 1 ), picX shall be included in RefPicSetStCurrBefore, RefPicSetStCurrAfter, RefPicSetStFoll, RefPicSetLtCurr, or RefPicSetLtFoll of the current picture.

The above restriction works by keeping reference pictures in the buffer to disambiguate the long-term reference pictures signalled.

It is assessed that the advantage and disadvantages of this version of restriction is as follow:

- Advantage: there is no need for encoders to keep track of the pictures that have ever been included in the RPS of all pictures starting from prevTid0Pic.

- Disadvantages:

  - When the above conditions are true, it requires to keep reference pictures that share the same POC LSB in DPB until the next picture in decoding order that has TemporalId equals 0 and nal_unit_type not equals to one of TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, RASL_R, RSV_VCL_N10, RSV_VCL_N12, and RSV_VCL_N14.

  - More bits are needed for signaling compared to only signaling POC MSB cycle.

Restriction option 2

The following highlighted text should be added for the semantic of delta_poc_msb_present_flag[ i ] :

Let prevTid0Pic be the previous picture in decoding order that is neither a sub-layer non-reference picture nor a RASL picture, and that has TemporalId equal to 0.

Let setOfPrevPicsPoc be the set consisting of:

- the PicOrderCntVal of prevTid0Pic,

- the PicOrderCntVal of each picture in the RPS of prevTid0Pic,

- – the PicOrderCntVal of each picture that follows prevTid0Pic in decoding order and precedes the current picture in decoding order.

delta_poc_msb_present_flag[ i ] shall be equal to 1 when there is more than one value in the setOfPrevPicsPoc that modulo MaxPicOrderCntLsb is equal to PocLsbLt[ i ].

The above restriction works by signaling delta_poc_msb_present_flag[i] equal to 1 for long-term reference pictures from when there is more than one picture in setOfPrevPicsPoc with picture order count modulo MaxPicOrderCntLsb equal to PocLsbLt[ i ] till the next TemporalId 0 picture which is neither a sub-layer non-reference picture nor a RASL picture.

It is assessed that the advantages and disadvantage of this version of restriction is as follow:

- Advantages:

  - When the above conditions are true, it does not require to keep reference pictures that share the same POC LSB in DPB until the next picture in decoding order that has TemporalId equals 0 and which is neither a sub-layer non-reference picture nor a RASL picture.

  - Use less bit compared to option 1. It only requires signaling POC MSB cycle for long-term reference pictures that have same POC LSB with other reference pictures in setOfPrevPicsPoc.

- Disadvantages: Encoders need to keep track of the picture order count that has been used in pictures since prevTid0Pic.

# Reference

- Hendry and B. Jeon, “AHG9: On RPS derivation and marking process for long-term reference pictures,” Document JCTVC-L0254, the 12th JCTVC meeting, Geneva, CH, January 2013.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).