| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0447 |
| --- | --- |

| Title: | Cross-check report of Temporal motion vector prediction hook for efficient merge mode in MV-HEVC (JCTVC-L0257) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution is a cross check report of JCTVC-L0257.

# Cross check

The code of JCTVC-L0257 v3 [2] is implemented on top of both HM9.1 and HTM5.1 with Ticket #925 fix.

Cross checker confirmed that the implementation is consistent with the text provided in v3 of [2] regarding to the macro MOTION_HOOK_L0257 for HM9.1 and QC_TMVP_MRG_IDX and SECOND_OPTION for HTM5.1.

# Results

The simulation is conducted under JCT-3V CTC [1] using the modified HTM5.1 with the following settings. The results are shown in Table 1 and 2. The result by cross checker is identical to the result provided by the proponents.

Common setting:

#define BUGFIX_925   1   //x-checker comment: Ticket #925 fix in JCT-VC trac

Anchor setting:

#define            QC_TMVP_MRG_IDX		0

#define            SECOND_OPTION          0

Tested setting:

#define            QC_TMVP_MRG_IDX		1

#define            SECOND_OPTION          1

Table 1: Results on MV-HEVC condition (3 view test case)

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -3.0% | -2.9% | -1.4% |
| Kendo | 0.0% | -2.4% | -2.2% | -1.0% |
| Newspapercc | 0.0% | -1.4% | -1.1% | -0.5% |
| GhostTownFly | 0.0% | -5.6% | -5.4% | -1.5% |
| PoznanHall2 | 0.0% | -1.8% | -0.9% | -0.6% |
| PoznanStreet | 0.0% | -2.1% | -2.1% | -0.7% |
| UndoDancer | 0.0% | -1.8% | -1.9% | -0.7% |
| 1024x768 | 0.0% | -2.3% | -2.1% | -1.0% |
| 1920x1088 | 0.0% | -2.9% | -2.6% | -0.9% |
| average | 0.0% | -2.6% | -2.4% | -0.9% |

Table 2: Results on MV-HEVC condition (2 view test case)

|  | video 0 | video 1 |  | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -3.0% |  | -0.9% |
| Kendo | 0.0% | -2.4% |  | -0.7% |
| Newspapercc | 0.0% | -1.4% |  | -0.4% |
| GhostTownFly | 0.0% | -5.6% |  | -0.9% |
| PoznanHall2 | 0.0% | -1.8% |  | -0.5% |
| PoznanStreet | 0.0% | -2.1% |  | -0.4% |
| UndoDancer | 0.0% | -1.8% |  | -0.4% |
| 1024x768 | 0.0% | -2.3% |  | -0.6% |
| 1920x1088 | 0.0% | -2.9% |  | -0.6% |
| average | 0.0% | -2.6% |  | -0.6% |

# References

- JCT3V-B1100, “Common Test Conditions of 3DV Core Experiments”

- JCTVC-L0257, “Temporal motion vector prediction hook for efficient merge mode in MV-HEVC”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).