| Joint Collaborative Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Geneva, CH, 17-23 Jan. 2013 |  |  |  | Document: JCTVC-L0448r1 |
| --- | --- | --- | --- | --- |
| Title: | BoG report on SHVC |  |  |  |
| Status: | Report Document |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Jill Boyce |  | Email: jill@vidyo.com |  |
| Source: | BoG |  |  |  |

# Abstract

The BoG on SHVC met on 20 January to discuss high-level syntax topics.  The lack of a working draft meant that most input contributions could not describe the syntax and semantics associated with their contributions, making it difficult to precisely understand what was being proposed.

The BoG recommends that an AHG mandate be created to further study signaling of inter-layer prediction constraints.  The AHG is suggested to use as a starting point JCTVC-L0449, edited by Hendry and M. Hannuksela, which combines the relevant contributions on the topic.

The r1 version of this document reflects the meeting on the morning of 21 January, in which the scope of the BoG was extended to SHVC general topics, in the following areas

- SHVC high-level syntax

- Other not previously presented SHVC contributions

- SHVC common test conditions

- SHVC core experiments: to be discussed

- Recommendations

The BoG recommends adoption of the following:

- JCTVC-L1xxx as an output document: SHVC common test conditions

- JCTVC-L0178: Use cropped output picture for inter-layer texture prediction, used with padding

- Meeting notes

## SHVC high-level syntax contributions

JCTVC-L0071 AHG9: Inter-layer prediction flag and inter-layer syntax prediction flag [T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

In this contribution, two high-level syntax changes are proposed. One proposed syntax change is to signal a no_inter_layer_pred_flag in video parameter set (VPS) extension to specify whether inter-layer prediction may be used for enhancement layer (EL) coding. The other proposed syntax change is to signal a no_inter_layer_syntax_pred_flag is in picture parameter set (PPS) extension or slice segment header extension in order to support a base layer (BL) format different from HEVC with no_inter_layer_syntax_pred_flag set to 1.

No inter_layer_pred_flag in the VPS, the functionality can already be accomplished with the existing design.  May make sense to have a flag of this type in the PPS or slice header.

no_inter_layer_syntax_pred_flag in PPS.  Conceptually makes sense, but what conditional syntax to use must be considered.  or possibly in the slice header.

No CU syntax provided.

Has some similarities with to JCTVC-L0278.

Further study encouraged after a working draft is available, including proposed modifications to low level syntax and decoder process.

JCTVC-L0111 Lightweight single-loop scalability with SHVC [H. Roodaki, K. Ugur, M. M. Hannuksela (Nokia)]

Experimental results show that constructing a scalable bitstream where scalable coding tools are used only when base-layer pictures are RAP pictures improve the enhancement layer coding performance more than 11% compared to simulcast. It is argued that, this encoding configuration is a lightweight option for many encoders as no multi-loop is necessary and inter-picture related syntax elements need not to be decoded. This contribution proposes indication of such restriction in the scalable bitstream.

Inter-layer prediction would be performed only for RAP.  Inter-layer prediction would be disabled for non-RAP pictures in the enhancement layer.

Proposes a flag in the VPS indicating that inter-layer prediction flag is disallowed for non-RAP pictures but allowed for RAP pictures.

Not proposing any syntax change if the flag indicates no inter-loop prediction.

Suggest an AHG mandate to specify inter-layer prediction constraint signaling.

JCTVC-L0119 Adaptive resolution change with SHVC [K. Ugur, H. Roodaki, M. M. Hannuksela (Nokia)]

Adaptive Resolution Change (ARC) refers to dynamically changing the resolution within the video sequence without using IDR pictures. As SHVC inherently supports resolution change and inter-layer prediction, it is argued that it can effectively be used to with ARC use-cases. This contribution proposes sequence level flags for SHVC to indicate the ARC operation. Experimental results are also provided demonstrating the bitrate reduction of using SHVC and inter-layer prediction for resolution change, compared to using IDR pictures.

Refers to ARC proposed in JCTVC-F158.

Use scalability tools with some restrictions to perform ARC.  Profiling of the restrictions is an issue.

Potential applications include video conferencing and trick mode support.

Further study encouraged within AHG related to inter-layer prediction constraints.

JCTVC-L0171 SHVC HLS: support for unequal BL and EL GOP lengths [M. M. Hannuksela, A. Hallapuro, K. Ugur (Nokia)]

The following changes are proposed in this contribution:

- To specify an access unit to contain VCL NAL units of a single value of nuh_layer_id only.

  - Consequently, to require that nuh_layer_id of the access unit delimiter NAL unit (if present) shall be equal to nuh_layer_id of the VCL NAL units contained in the access unit.

- To allow a decoding/bitstream order of coded pictures with a certain value of nuh_layer_id to differ from the decoding/bitstream order of coded pictures with another value of nuh_layer_id.

The second change is asserted to enable SHVC to have a different prediction structure of the enhancement layer (EL) from that of the base layer (BL). The claimed benefits of this are:

- Higher coding efficiency for EL can be achieved by having a longer GOP length than base layer. For example, a dBR of -2.7% was reached for EL luma when the EL GOP length is twice of the BL GOP length for 2X spatial scalability and random access common conditions when the inter-layer motion hook was turned on.

- Lower complexity and decoded picture buffer (DPB) memory use for EL could be achieved by using a shorter EL GOP length than the BL GOP length.

It is asserted that all proposals of this contribution apply to a reference index based (a.k.a. high-level syntax based) SHVC design and an SHVC design where low-level changes are made.

Consequence that the coding order differs for the BL and the EL. Could have DPB implications, as EL and BL need to be treated separately.  Temporal_id values would also be different for different layers, although this could be avoided.

Reported gains are for EL only.  Some of the gains may be lost if the base layer followed the GOP structure of the EL, in part because of QP offsets.

Some support expressed for providing that flexibility, especially for hybrid codec scalability.

Specification changes would be required for marking picture process.

Further study after draft specification is available to clearly understand the specification impacts. Additional experimental results are also requested.

JCTVC-L0231 Inter-layer slice header syntax element prediction in SHVC [D.-K. Kwon, M. Budagavi, M. Zhou (TI)]

In many scalable video coding applications, all spatial and SNR layers are coded using the same GOP structure. It is usual for all layer pictures at the same temporal position to have the same slice type and the same temporal references. Therefore, in this contribution, it is proposed to signal inter_layer_rps_prediction_flag in enhancement-layer slice header (SH) so that reference picture set (RPS) and reference picture list modification syntax elements can be inferred from those of a collocated reference-layer SH. When the pictures at the same temporal position are coded using the identical reference picture lists, their weighted prediction (WP) parameters are likely to be identical to each other. Therefore, it is also proposed to signal inter_layer_wp_prediction_flag so that the WP parameters for all reference indices can be inferred from those of a collocated reference-layer SH. It is asserted that the slice headers of enhancement layer can be signaled more efficiently with the proposed flags.

It is asserted that no parsing dependencies are introduced.

RPS and WP parameters are proposed to be inferred from a lower layer.

Not clear what would happen if there are multiple layers.

3D-HEVC has something similar already.

Further study encouraged after a working draft is available, including proposed modifications to low level syntax and decoder process, and consideration of harmonization with 3D-HEVC.

JCTVC-L0258 Inter layer prediction indication flags and pictures management [Hendry, B. Jeon (LG), D. Kwon, M. Budagavi, M. Zhou (TI)]

This contribution proposes two flags so called ilp_flag and ilp_ref_flag to be added in slice header. The ilp_flag indicates whether or not the current picture is needed for inter-layer prediction whereas the ilp_ref_lag indicates whether or not the current picture is needed for inter prediction by any picture whose ilp_flag equals 1. It is claimed that the two flags helps to determine whether a picture should be decoded or not given a target decoding layer which will lighten decoding burden. In addition, even though the pictures whose ilp_flag equal 1 are discarded, the two flags guarantee the reliable decoding when they are used for reference picture list construction process in the non-target layer.

The revision, the following items are added:

- Signalling of ilp_flag in sub layer layers

- Combination of signalling ilp_flag in sub layers and in slice header.

Propose inter layer prediction flag in slice segment header.

Propose VPS extension flag to indicate which temporal sub-layers are allowed to use inter-layer prediction

Similar to proposal in L0278.

How would the constraint be imposed?

Suggest an AHG mandate to specify inter-layer prediction constraint signaling, and how constraints would be reflected in syntax, semantics, and decoding process.

BoG recommends that a document be prepared that combines several of the relevant contributions to be used as a starting point for the AHG study.  Hendry and M. Hannuksela to edit.

JCTVC-L0042 Content Adaptive Complexity Reduction Scheme For Quality/Fidelity Scalable HEVC [H.-R. Tohidypour, M.T. Pourazad, P. Nasiopoulos (UBC & TELUS)]

This contribution proposes a method to reduce the complexity of the SNR/Quality/Fidelity scalable HEVC. This proposes method is asserted to use the correlation between the enhancement layers and the base layer to minimize redundant computations while encoding the enhancement layer. This is achieved by adaptively adjusting the motion search range in the enhancement layer based on the motion vector information of the base layer and implementing an adaptive, early-termination approach for inter and intra prediction mode search in the enhancement layer.  It is asserted that the method reduces HEVC/SVC’s coding complexity by up to 61.66%, while maintaining the overall bitrate.

Informative. Noted.

JCTVC-L0178 Legacy base layer codec support in SHVC [J. Boyce, D. Hong, W. Jang (Vidyo)]

Discussion chaired by A. Segall.

It is asserted that a legacy base layer codec outputs a cropped output picture rather than the full size decoded picture, when cropping is used.  So when a legacy base layer codec is used, the scalable HEVC enhancement layer codec would not have access to the coded samples outside the cropping window of the base layer resolution picture.  It is proposed that in the SHVC test model the enhancement layer codec predict from the cropping window output base layer picture rather than the decoded base layer picture.  It is also proposed to apply padding to the cropped base layer picture when there is no available sample in the scaled reference layer corresponding to the enhancement layer CU sample, so that a corresponding reference layer sample can always be considered to be available.

Asserted that a legacy baselayer codec could be combined with the current SMuC design, which is multi-loop.

Proponent has identified a problem when output cropping is used by the baselayer codec.

Proposing to use the cropped output for inter-layer prediction and pad the (cropped) output layer for upsampling/inter-layer prediction.

Comment that for some implementations, the non-cropped version may be used.

Proponent requests that the image data available for inter-layer prediction be the cropped data.

Comment that there may be some impact on implementation

Comment that with field pictures in HEVC the cropping multiple maybe 16.

Decision: Agree to use cropped output from the baselayer for enhancement layer texture prediction in the Scalable Test Model (SM) [with adjustment of padding process]

## Other SHVC contributions

JCTVC-L0042 Content Adaptive Complexity Reduction Scheme For Quality/Fidelity Scalable HEVC [H.-R. Tohidypour, M.T. Pourazad, P. Nasiopoulos (UBC & TELUS)]

This contribution proposes a method to reduce the complexity of the SNR/Quality/Fidelity scalable HEVC. This proposes method is asserted to use the correlation between the enhancement layers and the base layer to minimize redundant computations while encoding the enhancement layer. This is achieved by adaptively adjusting the motion search range in the enhancement layer based on the motion vector information of the base layer and implementing an adaptive, early-termination approach for inter and intra prediction mode search in the enhancement layer.  It is asserted that the method reduces HEVC/SVC’s coding complexity by up to 61.66%, while maintaining the overall bitrate.

Informative. Noted.

## SHVC common test conditions

JCTVC-L1xxx SHVC Common Test Conditions [X. Li, P. Onno, Y. Ye, J. Boyce]

Will register an output document, JCTVC-L1xxx

Modified JSVM to output AVC metadata will be made available in SM SVN repository

Agreed to change the AVC base layer QPs to match the HEVC base layer QPs.  AVC base layer bitstreams not yet available because of change of QP values,  but will be provided soon.

Suggest to rename the SMuC the SM.

Updated SM software will be made available.

Should we switch to HM10?   or should CEs just use HM8.1?  Suggestion that SM version used in the CEs and technical contributions should be based on HM8.1.

Agreed to make SM1.0 based upon HM8.1, and have SM1.0 used for all CE/non-CE technical contributions to next meeting.

Agreed that AVC base layer metadata file support will be integrated into SM1.1, for both IntraBL and RefIdx approaches, as a hook.

Ask software coordinators to have an HM10 aligned version of SM as input to the next meeting, not to be used for technical contributions to the next meeting.

3 options for QP conditions:

- Keep same conditions as for this meeting.  2 base QPs, 4 QP deltas.

- Use the CfP conditions.  Double the number of test points.  4 base QPs, 4 QP deltas

- 4 base QPs, 2 QP deltas.  Calculate BD-rate for a particular delta QP.

Some discussion about whether or not to report EL only numbers (in addition to EL+BL).  May be particularly of interest for comparing performance of a particular tool for an HEVC base layer with an AVC base layer.  Some concern that EL only calculations are somewhat unstable.  However, EL only information is useful in some cases.  Suggest that EL could be optionally provided as additional information.

Agreed to add EL reporting to the template but give it a less prominent position in the reporting, with exact formatting left to the discretion of the template provider (X. Li)

Agreed to switch to option 3.

Options for base QP values:

- HM uses 22, 27, 32, 37

- CfP used  22, 26, 30, 34

Suggestion to have different base QP range for SNR scalability

Agreed to use option 2, CfP base layers, and middle two EL QP deltas.  Spatial (0, 2) and SNR (-6, -4)

Question raised about changing the number of reference pictures in the test conditions.  No change.

## SHVC core experiments

- JCTVC-L1101 Description of Core Experiment 1: Upsampling filter for SHVC [A. Segall, J. Chen, J. Dong, E. Alshina (TE Coordinators)]

- JCTVC-L1102 Description of Core Experiment 2: Inter-layer Texture Prediction Signaling in SHVC [L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang (TE Coordinators)]

- JCTVC-L1103 Description of Core Experiment 3: Combined Prediction in SHVC [X. Li, E. Francois, P. Lai, D. Kwon, A. Saxena (TE coordinators)]

- JCTVC-L1104 Description of Core Experiment 4: Inter-layer filtering in SHVC [J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park (TE coordinators)]

- JCTVC-L1105 Description of Core Experiment 5: Inter-layer syntax prediction using HEVC base layer [V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang (TE coordinators)]

- JCTVC-L1106 Description of Core Experiment 6: Intra residual and mode prediction [A. Tabatabai (TE coordinators)]