| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0449 |
| --- | --- |

| Title: | Design Considered for Signalling Inter-layer Prediction Indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Miska Hannuksela Hendry Do-Kyoung Kwon | Email: | miska.hannuksela@nokia.com hendry.hendry@lge.com d-kwon@ti.com |
| Source: | Nokia, LG Electronics, TI |  |  |

_____________________________

# Abstract

This document includes the specifications of the “straw-man” design for signaling of inter-layer prediction indication at sequence level that was discussed during the 12th JCT-VC meeting.

# Straw-man text

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i<= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| for( j = 0; j <i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| for( i = 0; i<= vps_max_layers_minus1; i++ ) { |  |
| max_sublayer_for_ilp_plus1[ i ] | u(3) |
| } |  |
| } |  |

max_sublayer_for_ilp_plus1[ i ] equal to 0 indicates that non-RAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction. max_sublayer_for_ilp_plus1[ i ] greater than 0 indicates that pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and with TemporalId greater than max_sublayer_for_ilp_plus1[ i ] – 1 are not used as reference for inter-layer prediction. When not present, max_sublayer_for_ilp_plus1[ i ] is unspecified.

# References

- K. Ugur, et. al, “Lightweight single-loop scalability with SHVC,” Document JCTVC-L0111, the 12th JCTVC meeting, Geneva, CH, January 2013.

- Hendry, et. al, “Inter layer prediction indication flags and pictures management,” Document JCTVC-L0258_r1, the 12th JCTVC meeting, Geneva, CH, January 2013.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).