| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0451 WG11 Number: m28341 |
| --- | --- |

| Title: | Request for developing a lossless coding profile for graphics and text with motion |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Haoping Yu (Huawei) Robert Cohen (MERL) Ajay Luthra (Motorola Mobility) Tao Lin (Tongji University) Pankaj Topiwala (FastVDO) Edouard François (Canon Research Centre France) Wade Wan (Broadcom) Dzung Hoang (Zenverge) Andrew Segall (SHARP Laboratories of America, Inc.) Yi-Jen Chiu (Intel) Madhukar Budagavi (Texas Instruments, Inc.) | Email: | haoping.yu@huawei.com cohen@merl.com aluthra@motorola.com lintao@tongji.edu.cn pankajtva@gmail.com edouard.francois@crf.canon.fr wwan@broadcom.com Dzung.Hoang@Zenverge.com asegall@sharplabs.com yi-jen.chiu@intel.com madhukar@ti.com |
| Source: | Huawei, MERL, Motorola Mobility, Tongji University, FastVDO, Canon, Broadcom, Zenverge, SHARP, Intel, Texas Instruments |  |  |

_____________________________

# Abstract

This document requests developing/creating a lossless 4:4:4 video profile, targeted exclusively at video content containing only graphics and text, with motion, for a future extension of HEVC. The need for such a profile, along with evidence of improvements in performance is presented.

# Motivation

With rapid and continuous advancements made in semiconductors, networking, communications, displays, computers and devices such as tablets and smartphones, screen-sharing has become a common and highly desirable feature in many applications. For example, both Microsoft and Apple have a screen-sharing feature, i.e. Remote Desktop in Windows and Screen Sharing in MAC OS X, as part of their operating systems. Most web collaboration and conferencing software, e.g. Skype, GoToMeeting, and WebEx Meeting support the screen sharing feature as well. Today, most screen-sharing applications are using proprietary approaches, and there is no interoperability among products from different companies. Another emerging application is cloud-zero-client computing, where "cloud" can be "enterprise cloud" or "private cloud" within a big or small company. In this computing platform, all computing tasks including screen rendering are done on the cloud and the client device is simply a (compressed) screen content decoder plus a mouse-keyboard set or a touchscreen. There are products in this area but they all use proprietary approaches today. Therefore, a standard approach/solution is highly demanded. The requirements for coding this kind of content are further emphasized in Section 3.5 of ISO/IEC JTC1/SC29/WG11/N11872, “Vision, Applications and Requirements for High Efficiency Video Coding (HEVC)” [2].

Potential applications of compressing and delivering screen-content include remote desktop/screen sharing, cloud computing, web-based collaboration, and wireless display, to name a few.  To make an acceptable and competitive compression solution for the above applications, lossless coding with high compression efficiency for the 4:4:4 color sampling format is required because:

- HD quality displays with high resolutions such as 326 pixels per inch have become a trend for smartphones, tablets, and computers. Using lossy compression, any blur occurring in text and graphic edges will become rather visible and annoying, especially on portable devices.

- Today’s GPUs are capable of rendering pixel-sharp graphics and text while switching among different display resolutions, while also changing the content size within an application, e.g. zoom-in and zoom-out.

- Network bandwidth is still quite limited for many web-based applications. Therefore, high compression efficiency is still one of the essential requirements for many screen-content coding applications. In addition, low latency for both encoding and decoding is a basic requirement.

# Why lossless coding only for graphics and text

In general, screen content coming from today’s typical software applications may consist of a mix of graphics/texts/animations together with video and images. The main frame buffer (FB) in a typical GPU design stores the graphics, text, and animation (2D/3D) pixel information generated by the main GPU processor. In the case where applications also include video and image content, usually compressed versions of that material are decoded and processed separately either by CPU, by a dedicated decoder processor in the GPU, or by a combination of both. The decoded visual content may also be saved in the FB. However, in many GPU implementations, this content will be added on the fly to the FB pixel data using an overlay process to generate the final video signal output to the display monitor. Sometimes, this kind of content with a mix of natural video, images, and computer-generated graphics and text is also called compound video.

In the case where the compound video contains previously compressed images or video, re-encoding them along with the computer-generated graphics is a waste of computational resources. Re-encoding may also yield poor compression performance. This occurs mainly because screen-sharing applications often require real-time processing and encoding. The video encoder on a general-purpose host computer often has rather limited computational resources, as compared with hosts with dedicated video encoders. Because video and image content is typically compressed in web-based and other popular applications, re-encoding them for screen sharing is a suboptimal and inefficient engineering solution.

In the case where the video and image content in compound video has not been previously compressed, separating them from the computer-generated graphics and text and encoding them separately will give us the flexibility of using more appropriate and efficient codecs when necessary.

In the case where compound video cannot be separated into graphics and non-graphics regions, a lossless coding scheme is still beneficial, as it will not degrade the quality of the pre-compressed material, and it will provide significant compression gains over the existing HEVC main profile when compressing text/graphics portions of the video.

In summary, having an efficient lossless video coding solution exclusively targeting the graphics and text content seems be rather necessary, even when blank areas, representing video and image content, are present.

# Feasibility and evidence

Several lossless coding tools have been proposed for HEVC and have been studied in JCT-VC. In particular, JCTVC-L0302 [1] reports the latest test results from lossless coding the 4:4:4 screen content sequences available to JCTVC.

In the tables shown below, we show one example from JCTVC-L0302 where the comparison was made between 7-Zip and a modified version of the HM range-extension (HM-RE) software. For all cases, the reference data was generated using the unmodified HM-RE software. For the modified HM-RE case, the main partition and coding structure was not changed. The only modifications were done for CU intra-coding. From this example, it is quite appropriate to say that using the main-coding and partitioning architecture of HEVC in combination with custom-designed lossless coding modes will make HEVC a truly competitive compression solution for important applications such as remote desktop/screen sharing, cloud computing, web-based collaboration, and wireless display connection.

Table 1. Results for AI-main with Modified HM-RE-Software vs. HM-RE-Software

|  | Reference | Tested | Bit-rate Saving |
| --- | --- | --- | --- |
|  | kbps | kbps |  |
| Cadwaveform | 76493 | 13753 | -82.02% |
| PcbLayout | 63345 | 10038 | -84.15% |
| PptDocXls | 109307 | 28632 | -73.81% |
| VcDocSharing | 92707 | 32231 | -65.23% |
| webBrowsing | 99647 | 41492 | -58.36% |
| TwistTunnel | 100256 | 33277 | -66.81% |
| Programming | 303736 | 190909 | -37.15% |
| WordEditing | 258931 | 116638 | -54.95% |
| Map | 304293 | 204416 | -32.82% |
| VenueVu | 544385 | 701507 | 28.86% |
| All |  |  | -52.64% |

Table 2 Results of AI-main with 7-Zip vs. HM-RE-Software

|  | Reference | Tested | Bit-rate Saving |
| --- | --- | --- | --- |
|  | kbps | kbps |  |
| Cadwaveform | 76493 | 17267 | -77.43% |
| PcbLayout | 63345 | 15804 | -75.05% |
| PptDocXls | 109307 | 36926 | -66.22% |
| VcDocSharing | 92707 | 37483 | -59.57% |
| webBrowsing | 99647 | 42757 | -57.09% |
| TwistTunnel | 100256 | 42157 | -57.95% |
| Programming | 303736 | 185784 | -38.83% |
| WordEditing | 258931 | 102429 | -60.44% |
| Map | 304293 | 267575 | -12.07% |
| VenueVu | 544385 | 603478 | 10.85% |
| All |  |  | -49.38% |

Table 3. Results for LP-main with Modified HM-RE-Software vs. HM-RE-Software

|  | Reference | Tested | Bit-rate Saving |
| --- | --- | --- | --- |
|  | kbps | kbps |  |
| Cadwaveform | 981 | 249 | -74.58% |
| PcbLayout | 2647 | 740 | -72.05% |
| PptDocXls | 989 | 419 | -57.61% |
| VcDocSharing | 4253 | 3311 | -22.15% |
| webBrowsing | 2465 | 1262 | -48.78% |
| TwistTunnel | 75989 | 32184 | -57.65% |
| Programming | 77492 | 76831 | -0.85% |
| WordEditing | 14279 | 8835 | -38.12% |
| Map | 15946 | 14549 | -8.76% |
| VenueVu | 450142 | 468439 | 4.06% |
| All |  |  | -37.65% |

Table 3. Results of LP-main with 7-Zip vs. HM-RE-Software

|  | Reference | Tested | Bit-rate Saving |
| --- | --- | --- | --- |
|  | kbps | kbps |  |
| Cadwaveform | 981 | 468 | -52.35% |
| PcbLayout | 2647 | 1133 | -57.19% |
| PptDocXls | 989 | 574 | -41.97% |
| VcDocSharing | 4253 | 4291 | 0.91% |
| webBrowsing | 2465 | 1362 | -44.72% |
| TwistTunnel | 75989 | 40222 | -47.07% |
| Programming | 77492 | 81575 | 5.27% |
| WordEditing | 14279 | 6806 | -52.33% |
| Map | 15946 | 19074 | 19.61% |
| VenueVu | 450142 | 584811 | 29.92% |
| All |  |  | -23.99% |

# Timeline

The timeline for this work can be discussed in a JCT-VC plenary or in a joint Requirements meeting. In principle, the schedule for this activity can be separate from those of the existing HEVC extension work.

# Conclusions

Existing JCT-VC contributions show that potential improvements in compression efficiency of over 80% are achievable using lossless-coding tools in the HEVC framework when coding computer graphics and text. The authors of this document request developing/creating a lossless 4:4:4 video profile targeted exclusively at video content with only graphics and text, with motion, for a future extension of HEVC. This request is consistent with Section 3.5 of MPEG N11872 “Vision, Applications and Requirements for High Efficiency Video Coding (HEVC)” [2].

# References

[1]. H. Yu, W. Gao, J. Ye, Y. Cao, and X. Wang, “AHG8: Results from lossless coding the 4:4:4 screen content sequences”, JCT-VC Document, JCTVC-L0302, Geneva, Switzerland, Jan. 2013.

[2]. Video and Requirements Subgroups and JCT-VC, “Vision, Applications and Requirements for High Efficiency Video Coding (HEVC),” N11872, ISO/IEC JTC1/SC29/WG11, Daegu, KR, January 2011.

# Patent rights declaration(s)

Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Broadcom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP Laboratories of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Zenverge may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).