| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0455 |
| --- | --- |

| Title: | Cross-segment decoding of HEVC bitstreams with SAD/PSNR information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiangtao Wen, Shunyao Li and Spencer Cheng | Tel: Email: | +86-15101679256 jtwen@tsinghua.edu.cn |
| Source: | Tsinghua University and Nanjing Yuyan Information Technology Ltd. |  |  |

_____________________________

# Abstract

This document proposes an algorithm for significantly improving the quality of HEVC decoding by using information that is available to the decoder from the prior independently encoded segment but not taken into account in the encoding of the current independently encoded segment with very minor changes to the decoder. Performance gains of up to 2dB has been observed in our experiments to this point.

# Background

Video encoding and communications systems have traditionally been designed under the assumption that the encoding system has a much higher processing power and much larger storage than the decoder system. With the ever-widening popularity of mobile multimedia applications, especially with user-generated content, this assumption is no longer valid. Many widely watched clips on YouTube were captured on mobile phones, but are played back not only on mobile phones or tablet computers but also on smart TVs, smart set-top-boxes, as well as laptop and desktop computers, all of which may possess much more computational and storage resources than the cell phone on which the video clip was originally captured and encoded.

Due to the high complexity associated with implementing a state-of-the-art video encoding system using the H.264/AVC and especially the up-coming HEVC standards, encoders found in mobile devices, although compliant to the standard syntaxes, were often designed with sub-optimal implementations of only small subsets of the numerous encoding tools supported by the standard. Typical video encoding tools that are not fully implemented include sub-pel motion estimation (ME), many Inter/Intra partition sizes and/or Intra prediction directions, and etc. The number of reference frames used in the ME is also often limited.

Aside of the video encoding tools that are not supported completely, the algorithms for selecting the tools that supported, conducting rate-distortion optimized ME, quantization, macroblock (MB) mode decision, frame prediction type decision and group of picture (GOP) reference structures are also drastically simplified in many encoding systems. Various implementation constrains (e.g. cache sizes) as well as application requirements (e.g. video conferencing or live streaming) make it very difficult to perform bit rate allocation optimized globally for the entire clip.

Furthermore, when the bitstreams generated by such encoders are streamed over the network, in response to network bandwidth variations, the streaming servers will often adjust the rate at which the clip is encoded downward so as to prevent the playback on the receiving device from stalling.

All of the above factors contribute to loss in coding efficiency as well as visual quality variations in the bitstream that the playback device receives. Although it is possible to deploy cloud-based systems that transcode such low quality (in terms of encoding performance) bitstreams to higher quality bitstreams, cloud based systems are not viable generically due to scalability issues and privacy concerns. In addition, cloud-based transcoding of a low compression efficiency bitstream to a bitstream of higher compression efficiency could only lead to bandwidth savings, not visual quality improvements. The visual quality is determined by the output of the first encoder with which the bitstream was produced.

It is desirable, therefore, to come up with a system that can improve the quality of experience (QoE) of the end user given all the limitations, constraints and variations present in the content encoding, transcoding, streaming and playback processes, and taking into account the reality that many decoders today have higher computational and storage capability that many encoders. The process for such enhancement should also standardized (if not all algorithm parameters), similar to the standardization of video encoding, where video encoding tools are defined while leaving the selection (e.g. mode decision) of the tools and their parameters (e.g. motion search range) are left for the implementations.

# Algorithm description

In this proposal, we described an improved algorithm for decoding live streamed or pre-encoded video with time-varying quality. The algorithm utilizes information received by the decoder in a segment of the clip that 1) has already been received and decoded, but 2) was encoded independently from the current segment, and 3) has a higher visual quality than the current segment. By extracting information contained in such a segment that is available to the decoder but was not taken advantaged by the encoder, the proposed decoder is capable of significantly improve the QoE of the user without incurring significant overhead to the storage and computational complexities of both the encoder and the decoder.

The algorithm is based on an algorithm we submitted to the MPEG meeting in Shanghai (m27205). The difference is, in m27205, we ran the core algorithm multiple times with different parameters such as patch sizes, different values for the thresholds for determining patch matching and etc., and reported the best results for the all the combinations of parameters we have tested. Therefore, instead of proposing a practical algorithm, m27205 was intended to show the potential room for improvement given by a simple way of utilizing a more sophisticated decoder (as compared to conventional decoders as well as the encoder with which the bitstream to be decoded was produced) for improving visual quality.

The algorithm we are describing here on the other hand, is a practical algorithm requesting only minimal changes to the video coding standard.

Similar to m27205, we define the last frame (in playback order) in a self-contained high quality/bitrate segment immediately preceding the current one the “good frame” (GF), the first Intra frame in the current segment the “start frame” (SF), and the output from the current algorithm the “fresh start” (FS).

As described previously, the goal of the enhancement algorithm is to use information contained in the GF to improve the quality of the decoded SF to get an improved reference frame FS for subsequent frames in the low quality segment. To this end, we segment the SF into non-overlapping 16x16 patches, and calculate the Sum of Squared Differences (SSD) between a patch in the SF and the corresponding patch in the GF. If the SSD is smaller than a threshold Thres, we replace the patch in the SF with the patch in the GF. A flow chart of the algorithm can be found in Figure 1.

Figure 1  Flow Chart of the Proposed Patch Enhancement Algorithm

Obviously, the performance of the proposed algorithm depends on the value of Thres. In our experiments, we experimented all integer values of Thres between 10 and 600, and found the threshold (ThresOpt) value that provided the largest average PSNR gain over the frames after the SF.

We plotted the relationship between the values of ThresOpt and the average (with regard to the number of MVs in the bitstream) motion estimation (ME) cost of the MVs between the GF and the SF if the SF was encoded as an Inter frame as opposed to an Intra frame, as well as the PSNR of the SF after encoding, as in Figures 2 and 3. Here, MECost is defined as Distortion + lambda*Bits(mv).

We then fitted the data in Figures 2 and 3 using a power and a Laplacian function respectively. The best fitting is found to be:

(1)

for the power function and

(2)

for the Laplacian.

Figure 2   Thres-PSNR

Figure 3  Thres-MECost

# Experimental Results

We ran the algorithm in Figure 1 again with the Thres set to the larger of the two thresholds calculated in equations (1) and (2). The results are given in Table 1. In the table, we ran the encoder using the HM8 HEVC encoder with the low delay configuration. We divided each test sequence into two segments, namely the first 32 frames in the clip, and the remaining frames. The segments were encoded independent from each other using the low delay configuration, with the QPs for the 1st frame in the 1st segment being 5 levels lower than the 1st frame in the 2nd segment.

In Table 1, the QP values are the QP for the 1st frame of the second segment, “std” standards for the average PSNR for all frames in the second segment when decoded using the standard HEVC decoder. “%-cost” and “%-PSNR” are the percentages of the patches in the SF that were determined to be a match for the corresponding patch in the GF when the threshold was calculated using (1) and (2) respectively. The corresponding PSNR gain averaged for all frames in the second segment was given in “gain-cost” and “gain-PSNR”, whereas the best average PSNR gain found by swapping the integer values of Thres between 10 and 600. In Table 1, the red shaded cells correspond to the larger of the two percentages of “%-cost” and “%-PSNR”, which also correspond to the larger of the two thresholds as calculated by (1) or (2). The corresponding PSNR gain from the algorithm is shown in the blue shaded cells in the Table, which is summarized in Table 3, the corresponding RD curves are given in Figure 4. It should be noted that because for frames in the second segment after the SF, the residual contained in the bitstream no longer matches the residual between the reference and the new, enhanced reference, there exists drift in the decoding process, making the RD curves for the enhanced decoder non-convex (but still monotonic). Some detailed visual comparisons can be found in Figure 5.

Similarly, if we calculate the ME cost not between the GF and the SF, but SF and the next frame (“SF+1”), then the results are given in Table 2. The advantage of using ME Cost (SF, SF+1) instead of ME Cost (GF, SF) is that because the motion vectors and QPs are already in the bitstream, the cost can be easily calculated from the SAD by the decoder, if the SAD is included in the bitstream.

From the Tables, it is easy to see, that, given the capability of using information contained in the GF, significant gain in PSNR can be achieved with even a simple algorithm such as in Figure 1.

|  | QP | PSNR-standard decoder (dB) | %-cost | %-psnr | Gain-cost (dB) | Gain-PSNR (dB) | Gain-Exhaust Search (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vidyo1 | 34 | 37.51 | 96.28 | 92.22 | 0.24 | 0.09 | 0.23 |
|  | 35 | 36.96 | 91.86 | 93.17 | 0.31 | 0.20 | 0.24 |
|  | 36 | 36.40 | 94.94 | 94.31 | 0.30 | -0.14 | 0.39 |
|  | 38 | 35.22 | 87.92 | 95.78 | 0.26 | 0.18 | 0.44 |
|  | 39 | 34.67 | 90.56 | 96.39 | 0.29 | 0.47 | 0.51 |
|  | 40 | 34.10 | 83.50 | 96.83 | 0.24 | 0.46 | 0.51 |
| Vidyo3 | 34 | 37.29 | 86.78 | 85.47 | 0.17 | 0.18 | 0.21 |
|  | 35 | 36.73 | 85.14 | 86.14 | 0.22 | 0.25 | 0.26 |
|  | 36 | 36.14 | 86.72 | 86.53 | 0.31 | 0.32 | 0.33 |
|  | 38 | 34.92 | 83.78 | 87.69 | 0.45 | 0.44 | 0.44 |
|  | 39 | 34.27 | 83.50 | 88.42 | 0.52 | 0.52 | 0.53 |
|  | 40 | 33.67 | 79.22 | 89.00 | 0.51 | 0.50 | 0.50 |
| Vidyo4 | 34 | 37.13 | 88.67 | 83.78 | 0.03 | 0.05 | 0.06 |
|  | 35 | 36.61 | 88.81 | 85.03 | 0.17 | 0.16 | 0.16 |
|  | 36 | 36.06 | 88.78 | 85.72 | 0.20 | -1.91 | 0.14 |
|  | 38 | 34.95 | 88.33 | 87.50 | 0.25 | 0.26 | 0.24 |
|  | 39 | 34.41 | 88.64 | 88.53 | 0.25 | 0.28 | 0.25 |
|  | 40 | 33.84 | 88.00 | 89.39 | 0.27 | 0.31 | 0.30 |
| Johnny | 34 | 38.12 | 99.44 | 93.75 | 0.10 | 0.04 | 0.11 |
|  | 35 | 37.63 | 99.47 | 94.50 | 0.02 | -0.02 | 0.03 |
|  | 36 | 37.12 | 99.56 | 95.64 | 0.20 | 0.20 | 0.26 |
|  | 38 | 36.03 | 99.53 | 97.28 | 0.25 | 0.37 | 0.38 |
|  | 39 | 35.52 | 99.42 | 97.83 | 0.19 | 0.36 | 0.40 |
|  | 40 | 34.93 | 99.39 | 98.33 | 0.19 | 0.46 | 0.48 |
| SE | 34 | 36.22 | 84.00 | 87.22 | 1.40 | 1.34 | 1.40 |
|  | 35 | 35.13 | 63.78 | 87.44 | 1.54 | 1.58 | 1.62 |
|  | 36 | 34.34 | 74.67 | 87.56 | 1.54 | 1.54 | 1.56 |
|  | 38 | 32.28 | 50.06 | 87.83 | 1.78 | 1.77 | 1.80 |
|  | 39 | 31.35 | 52.69 | 88.03 | 1.85 | 1.79 | 1.87 |
|  | 40 | 30.40 | 42.42 | 88.19 | 1.96 | 1.87 | 1.97 |
| K&S | 34 | 37.59 | 96.17 | 90.72 | 0.42 | 0.24 | 0.39 |
|  | 35 | 37.05 | 92.44 | 91.67 | 0.47 | 0.29 | 0.45 |
|  | 36 | 36.50 | 94.64 | 92.92 | 0.48 | 0.38 | 0.49 |
|  | 38 | 35.37 | 88.08 | 95.47 | 0.36 | 0.15 | 0.62 |
|  | 39 | 34.79 | 89.14 | 96.36 | 0.34 | 0.51 | 0.64 |
|  | 40 | 34.23 | 83.67 | 96.92 | 0.32 | 0.62 | 0.81 |
| 4People | 35 | 35.76 | 81.11 | 95.22 | 0.50 | 0.52 | 0.51 |
|  | 38 | 33.94 | 72.78 | 96.89 | 0.67 | 0.67 | 0.66 |
|  | 39 | 33.33 | 80.33 | 97.22 | 0.63 | 0.80 | 0.81 |
|  | 40 | 32.69 | 67.08 | 97.83 | 0.70 | 0.97 | 0.94 |
| CHSpeed | 34 | 33.88 | 72.43 | 78.68 | 0.14 | 0.23 | -0.14 |
|  | 35 | 29.18 | 64.55 | 80.92 | 0.04 | 0.18 | 1.66 |
|  | 36 | 32.62 | 70.28 | 82.62 | 0.16 | 0.25 | 0.06 |
|  | 38 | 31.34 | 64.13 | 85.64 | 0.33 | 0.36 | 0.29 |
|  | 39 | 30.70 | 65.66 | 87.24 | 0.40 | 0.40 | 0.36 |
|  | 40 | 30.06 | 62.99 | 88.02 | 0.46 | 0.47 | 0.34 |

Table 1 PSNR Improvement By the Proposed Algorithm with ME Cost (GF, SF) and PSNR (SF)

|  | QP | PSNR-std(dB) | Gain-cost(dB) | Gain-PSNR(dB) | Gain-Exhaust Search(dB) |
| --- | --- | --- | --- | --- | --- |
| Vidyo1 | 34 | 37.51 | 0.23 | 0.09 | 0.23 |
|  | 35 | 36.96 | 0.24 | 0.2 | 0.24 |
|  | 36 | 36.4 | 0.34 | -0.14 | 0.39 |
|  | 38 | 35.22 | 0.3 | 0.18 | 0.44 |
|  | 39 | 34.67 | 0.41 | 0.47 | 0.51 |
|  | 40 | 34.1 | 0.26 | 0.46 | 0.51 |
| Vidyo3 | 34 | 37.29 | 0.22 | 0.18 | 0.21 |
|  | 35 | 36.73 | 0.26 | 0.25 | 0.26 |
|  | 36 | 36.14 | 0.31 | 0.32 | 0.33 |
|  | 38 | 34.92 | 0.43 | 0.44 | 0.44 |
|  | 39 | 34.27 | 0.52 | 0.52 | 0.53 |
|  | 40 | 33.67 | 0.42 | 0.5 | 0.5 |
| Vidyo4 | 34 | 37.13 | 0.04 | 0.05 | 0.06 |
|  | 35 | 36.61 | 0.16 | 0.16 | 0.16 |
|  | 36 | 36.06 | 0.07 | -1.91 | 0.14 |
|  | 38 | 34.95 | 0.26 | 0.26 | 0.24 |
|  | 39 | 34.41 | 0.28 | 0.28 | 0.25 |
|  | 40 | 33.84 | 0.29 | 0.31 | 0.3 |
| Johnny | 34 | 38.12 | 0.11 | 0.04 | 0.11 |
|  | 35 | 37.63 | 0.02 | -0.02 | 0.03 |
|  | 36 | 37.12 | 0.26 | 0.2 | 0.26 |
|  | 38 | 36.03 | 0.38 | 0.37 | 0.38 |
|  | 39 | 35.52 | 0.37 | 0.36 | 0.4 |
|  | 40 | 34.93 | 0.3 | 0.46 | 0.48 |
| SE | 34 | 36.22 | 1.4 | 1.34 | 1.4 |
|  | 35 | 35.13 | 1.58 | 1.58 | 1.62 |
|  | 36 | 34.34 | 1.55 | 1.54 | 1.56 |
|  | 38 | 32.28 | 1.71 | 1.77 | 1.8 |
|  | 39 | 31.35 | 1.74 | 1.79 | 1.87 |
|  | 40 | 30.4 | 1.61 | 1.87 | 1.97 |
| K&S | 34 | 37.59 | 0.4 | 0.24 | 0.39 |
|  | 35 | 37.05 | 0.33 | 0.29 | 0.45 |
|  | 36 | 36.5 | 0.48 | 0.38 | 0.49 |
|  | 38 | 35.37 | 0.49 | 0.15 | 0.62 |
|  | 39 | 34.79 | 0.42 | 0.51 | 0.64 |
|  | 40 | 34.23 | 0.27 | 0.62 | 0.81 |
| 4People | 35 | 35.76 | 0.5 | 0.52 | 0.51 |
|  | 38 | 33.94 | 0.62 | 0.67 | 0.66 |
|  | 39 | 33.33 | 0.63 | 0.8 | 0.81 |
|  | 40 | 32.69 | 0.67 | 0.97 | 0.94 |
| CHSpeed | 34 | 33.88 | 0.16 | 0.23 | -0.14 |
|  | 35 | 29.18 | 1.66 | 0.18 | 1.66 |
|  | 36 | 32.62 | 0.21 | 0.25 | 0.06 |
|  | 38 | 31.34 | 0.35 | 0.36 | 0.29 |
|  | 39 | 30.7 | 0.39 | 0.4 | 0.36 |
|  | 40 | 30.06 | 0.34 | 0.47 | 0.34 |

Table 2 PSNR Improvement with ME Cost (SF, SF+1) and PSNR(SF)

|  | QP | PSNR-Y | PSNR-U | PSNR-V |  | QP | PSNR-Y | PSNR-U | PSNR-V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (dB) | (dB) | (dB) |  |  | (dB) | (dB) | (dB) |
| Vidyo1 | 34 | 0.24 | 1.1 | -0.25 | SE | 34 | 1.4 | 1.92 | 1.3 |
|  | 35 | 0.31 | 1.07 | 0.26 |  | 35 | 1.54 | 1.73 | 1.04 |
|  | 36 | 0.3 | 1.13 | -0.26 |  | 36 | 1.54 | 1.43 | 0.86 |
|  | 38 | 0.26 | 1.37 | 0.02 |  | 38 | 1.77 | 0.94 | 0.67 |
|  | 39 | 0.47 | 1.39 | 0.41 |  | 39 | 1.79 | 1.35 | 0.56 |
|  | 40 | 0.46 | 0.94 | 0.11 |  | 40 | 1.87 | 0.12 | -0.06 |
| Vidyo3 | 34 | 0.17 | 1.36 | 0.19 | K&S | 34 | 0.42 | 1.18 | 0.73 |
|  | 35 | 0.22 | 1.46 | 0.11 |  | 35 | 0.47 | 1.56 | 0.75 |
|  | 36 | 0.31 | 1.61 | -0.07 |  | 36 | 0.48 | 1.67 | 0.59 |
|  | 38 | 0.44 | 1.9 | 0.04 |  | 38 | 0.36 | 1.45 | 0.7 |
|  | 39 | 0.52 | 1.87 | 0 |  | 39 | 0.51 | 1.31 | 0.67 |
|  | 40 | 0.5 | 1.44 | 0.03 |  | 40 | 0.62 | 1.87 | 0.62 |
| Vidyo4 | 34 | 0.03 | 0.98 | 0.69 | 4People | 35 | 0.5 | 1.65 | 0.32 |
|  | 35 | 0.17 | 1.25 | 0.7 |  | 38 | 0.67 | 1.53 | 0.6 |
|  | 36 | 0.2 | 1.32 | 0.7 |  | 39 | 0.8 | 1.52 | 0.25 |
|  | 38 | 0.26 | 1.52 | 0.74 |  | 40 | 0.97 | 1.21 | 0.44 |
|  | 39 | 0.28 | 1.08 | 0.94 |  |  |  |  |  |
|  | 40 | 0.31 | 0.63 | 0.2 |  |  |  |  |  |
| Johnny | 34 | 0.1 | 1.53 | 1.27 | CHSpeed | 34 | 0.23 | 0.32 | -0.6 |
|  | 35 | 0.02 | 1.33 | 1.08 |  | 35 | 0.18 | 0.16 | -0.36 |
|  | 36 | 0.2 | 1.24 | 1.11 |  | 36 | 0.25 | 0.23 | -0.38 |
|  | 38 | 0.25 | 1.79 | 1.29 |  | 38 | 0.36 | 0.23 | -0.26 |
|  | 39 | 0.19 | 1.99 | 1.36 |  | 39 | 0.4 | 0.27 | -0.32 |
|  | 40 | 0.46 | 1.96 | 1.09 |  | 40 | 0.47 | 0.32 | -0.37 |

Table 3 Overall average PSNR improve by enhanced decoder

Figure 4 RD Curve of Standard Decoder and Enhanced Decoder

# Conclusions

In this proposal, we describe an algorithm for significantly improving the quality of HEVC decoding by using information that is available to the decoder but not taken into account in the encoding of the bitstream.

The algorithm requires that the encoder send ME Cost and PSNR information in the bitstream for the decoder to select the threshold with which a match between patches in the GF and SF can be determined. Although, in our experiments, we always used the threshold calculated either with ME Cost of PSNR that produces a higher percentage of patch matches, and therefore theoretically, only one of either ME Cost or PSNR needs to be sent along with a 1-bit flag indicating which formula is used, we believe a well-designed decoder or streamer could make use of the ME Cost and PSNR in other intelligent ways (e.g. for adaptive streaming), and therefore propose that both be included in the bitstream.

We also believe that other means of improving decoding quality using ideas similar to that presented in this proposal exist, and therefore, propose that the MPEG body issue a call for evidence for such techniques.

# Patent rights declaration(s)

Tsinghua University and Nanjing Yuyan Information Technology Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

| Figure 5.1 (a) Standard Decoder (FourPeople, QP=40) Figure 5.1 (b) Enhanced Decoder (FourPeople, QP=40) Figure 5.1 (c) Details Comparison (FourPeople, QP=40) Figure 5.2 (a) Standard Decoder (SlideEditing, QP=40) Figure 5.2 (b) Enhanced Decoder (SlideEditing, QP=40) Figure 5.2 (c) Details Comparison (SlideEditing, QP=40) Figure 5.3 (a) Standard Decoder (ChinaSpeed, QP=40) Figure 5.3 (b) Enhanced Decoder (ChinaSpeed, QP=40) Figure 5.3 (c) Details Comparison (ChinaSpeed, QP=40) |  |
| --- | --- |