| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L1006 |
| --- | --- |

| Title: | Common test conditions and software reference configurations for HEVC range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | David Flynn, Chris Rosewarne | Email: | dflynn@rim.com, chris.rosewarne@cisra.canon.com.au |
| Source: | JCT-VC |  |  |

_____________________________

# Abstract

This document defines common test conditions and software reference configurations to be used in the development of range extensions conducted between the 12th and the 13th JCT-VC meetings.  These common test conditions are also recommended for use in technical contributions to the 13th JCT-VC meeting, if applicable.

# Introduction

Common test conditions are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments.

This document defines three test conditions, three encoding modes and two QP ranges that reflect a variety of interesting application areas.

| Test conditions | - Intra, - Random access, - Low delay (B) |
| --- | --- |
| Encoding modes | - R’G’B’ 4:4:4, - Y’CbCr 4:4:4, - Y’CbCr 4:2:2 |
| QP ranges | - Main tier (MT), - High tier (HT) - Super-high tier (SHT) |

A subset of these conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

Experiments investigating R’G’B’ coding tools should also provide results using Y’CbCr 4:4:4.

Revision r3369 (HM10.0_RExt2.0) of the common software (based upon HM-10.0) with range extension support is expected to be used for most experiments.  More recent versions are encouraged where applicable. Availability of the software was announced on the JCT-VC email reflector on 19th February 2013.

The following sections define test sequences, quantization parameter values, encoder configuration files, and compile-time options to be used.

People bringing input contributions should provide a set of results as complete as possible that apply to the proposal. Results should be reported using the attached Excel sheets.

# Test sequences

Table 1 defines the set of test sequences to be used for intra, random-access, and low-delay conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences, test conditions, QP ranges and encoding modes as described below (see Section 4 for definitions of configuration files).

Test sequences are available on ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/ (please contact the JCT-VC chairs for login information).

Table 1 Test sequences

| Sequence name | Frame count | Frame rate (Hz) | Bit depth |  | RGB 4:4:4 | YCbCr 4:4:4 | YCbCr 4:2:2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Input | Internal |  |  |  |
| Traffic | 150 | 30 | 12 | 12 | All |  |  |
| Traffic | 150 | 30 | 10 | 10 |  | All | All |
| Kimono | 240 | 24 | 10 | 10 | All | All | All |
| EBULupoCandlelight (down conv.) | 500* | 50 | 10 | 10 | All | All |  |
| EBURainFruits (down conv.) | 500* | 50 | 10 | 10 | All | All |  |
| EBUHorse | 500 | 50 | 10 | 10 |  |  | All |
| EBUGraphics | 500 | 50 | 10 | 10 |  |  | All |
| EBUWaterRocksClose | 500 | 50 | 10 | 10 |  |  | All |
| EBUKidsSoccer | 500 | 50 | 10 | 10 |  |  | All |
| VenueVu | 300 | 30 | 8 | 10 | All |  |  |
| VenueVu | 300 | 30 | 10 | 10 |  | All |  |
| DucksAndLegs | 300 | 30 | 10 | 10 | All |  |  |
| BirdsInCage | 600 | 60 | 10 | 10 |  | All |  |
| OldTownCross | 500 | 50 | 10 | 10 | All |  |  |
| CrowdRun | 500 | 50 | 10 | 10 |  | All |  |
| Seeking | 500 | 50 | 10 | 10 |  |  | All |
| ParkScene | 240 | 24 | 10 | 10 | All |  |  |

*600 frames present, but only the first 500 frames used.

Note: When the encoder operates in 10-bit mode (InternalBitDepth=10) for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behavior is built into the reference encoder and no external conversion program is required.

(Although not applicable with the current test conditions, when the encoder operates in 8-bit mode (InternalBitDepth=8) for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range.)

# Quantization parameter values

For each video sequence four quantization parameter values are to be used depending upon the configuration.  These values define the QP values used for the I-frames in a sequence (configuration files further define QP values used for other frames):

| Configuration | QP |
| --- | --- |
| MT: | 22, 27, 32, 37 |
| HT: | 17, 22, 27, 32 |
| SHT | 12, 17, 22, 27 |

# Configuration files

The following sections define encoder configuration files to be used for each test case. Parameters to be changed for each test point are:

- InputFile to reflect the location of the source video sequence on the test system.

- FrameRate to reflect the frame rate of a given sequence as per Table 1.

- SourceWidth to reflect the width of the source video sequence.

- SourceHeight to reflect the height of the source video sequence.

- FramesToBeEncoded to reflect the frame count of a given sequence as per Table 1.

- IntraPeriod to reflect the intra refresh period in the random access test cases. The intra refresh period is dependent on the frame rate of the source: a value 16 shall be used for sequences with a frame rate equal to 20fps, 24 for 24fps, 32 for 30fps, 48 for 50fps, and 64 for 60fps.

- QP to reflect the quantization parameter values defined in section 3.

- InputBitDepth and InternalBitDepth to reflect the bit depth of a given sequence as per Table 1.

- InputChromaFormat to indicate the chroma format of the source file. Use 420 for 4:2:0, 422 for 4:2:2, 444 for 4:4:4 chroma formats.

- InputColourSpaceConvert (for RGB4:4:4 only) should be set to ‘RGBtoGBR’.

- SNRInternalColourSpace (for RGB4:4:4 only) should be set to 1.

- OutputInternalColourSpace (for RGB4:4:4 only) should be set to 0.

For RGB4:4:4, the decoder should be configured with:

- OutputColourSpaceConvert (RGB4:4:4 decoder only) should be set to ‘GBRtoRGB’.

Configuration files are provided in the cfg/ folder of the common software package (available at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-range-extensions). There are three encoder configurations provided as follows:

- “All Intra” (AI-MT, AI-HT, AI-SHT): encoder_intra_main10.cfg

- “Random access” (RA-MT, RA-HT): encoder_randomaccess_main10.cfg

- “Low-delay B” (LB-MT, LB-HT): encoder_lowdelay_main10.cfg

There are additional configurations that support the other test configurations used with the HM version 1 4:2:0 model, but these are not required to be used for the Range Extension test conditions.

Sequence-specific parameters are to be found in the cfg/per-sequence/ folder, which can be used to configure the InputFile, FrameRate, SourceWidth, SourceHeight, and FramesToBeEncoded and the encoder RGB4:4:4 options.

A combination of an encoder configuration, a sequence configuration and the required QP (and IntraPeriod for the “Random access” case) is necessary to supply the required coding parameters to the software encoder model. In addition, the RGB version of Traffic needs to be configured to use an InternalBitDepth of 12. Finally, the SEIDecodedPictureHash must also be configured to use MD5 hashes (set to 1).

# Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/TLibCommon folder of the common software. The default settings provided in the source code should be used.