| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L1009 |
| --- | --- |

| Title: | Common Test Conditions and Software Reference Configurations for the Scalable Test Model |  |  |
| --- | --- | --- | --- |
| Status: | Output Document Approved by JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiang Li Jill Boyce Patrice Onno Yan Ye | Tel: Email: | lxiang@qti.qualcomm.com jill@vidyo.com patrice.onno@crf.canon.fr yan.ye@interdigital.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document defines common test conditions and software reference configurations to be used in the context of SHVC core experiments (CE) conducted between the 12th and the 13th JCT-VC meetings. These common test conditions are also recommended for use in technical contributions to the 13th JCT-VC meeting, if applicable.

# Testing configurations

Common test conditions are desirable to conduct experiments in a well-defined environment and ease the comparison of the outcome of experiments.

This document defines 11 test conditions, reflecting several versions for spatial scalability (2x and 1.5x), and SNR scalability, and of intra-only, random-access, and low-delay settings. The following Table 1 summarizes the mandatory and optional configurations for the case where the base layer is AVC base layer or HEVC base layer

| Mandatory tests for the Common Test conditions |  |
| --- | --- |
| AVC base layer case | HEVC base layer case |
| AVC conditions are mandatory only if a contribution is specifically directed at an AVC base layer. | - Intra, spatial 2x - Intra, spatial 1.5x - Random access, spatial 2x - Random access, spatial 1.5x - Low delay, P slices only, spatial 2x - Low delay, P slices only, spatial 1.5x |

| Optional configurations for the Common Test conditions |  |
| --- | --- |
| AVC base layer / HEVC enhancement layer | HEVC base layer / HEVC enhancement layer |
| - Random access, spatial 2x - Random access, spatial 1.5x - Low delay, P slices only, spatial 2x - Low delay, P slices only, spatial 1.5x - Random access, SNR - Low delay, P slices only, spatial SNR - Low delay, B slices, spatial 2x - Low delay, B slices, spatial 1.5x - Low delay, B slices, spatial SNR | - Random access, SNR - Low delay, P slices only, spatial SNR - Low delay, B slices, spatial 2x - Low delay, B slices, spatial 1.5x - Low delay, B slices, spatial SNR |

Table 1: Configuration tests

A subset of these test conditions might be used for a particular experiment. For example, when testing an intra coding tool, only intra configurations might be used.

Version 1.0 of the SHVC common software (SHM 1.0) is expected to be used for most experiments. SHM 1.0 will be based on HM8.1 and will be made available two weeks after the end of the 12th JCT-VC meeting.

The following sections define test sequences, quantization parameter values, encoder configuration files, and compile-time options to be used.

People bringing input contributions should provide a set of results as complete as possible that apply to the proposal. Results should be reported using the attached Excel sheets.

# Test sequences and AVC bitstreams

Table 1 defines the set of test sequences (and AVC bitstreams) to be used for intra, random-access, and low-delay conditions. All frames (as defined by frame count in the table) shall be encoded for all sequences and test cases described below (see Section 4 for definitions of spatial 2x, spatial 1.5x and SNR).

Original test sequences are available on ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/ and ftp://hevc@ftp.tnt.uni-hannover.de/scalable/sequences/

AVC base bitstreams are available on the ftp://hevc@ftp.tnt.uni-hannover.de/scalable/bitstreams/ (please contact the JCT-VC chairs for login information).

(Note the AVC base layer for the specified bitstreams do not yet exist for the QP=22, QP=26, QP=30, QP=34 in the above FTP site and will be made available three weeks after the end of 12th JCT-VC meeting)

| Class | Sequence name | Frame count | Frame rate | Base Layer Resolution | Enhancement Layer Resolution | Intra | Random access | Low-delay P or B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 150 | 30fps | 1280x800 2560x1600 | 2560x1600 2560x1600 | Spatial 2x | Spatial 2x SNR | Spatial 2x SNR |
| A | PeopleOnStreet | 150 | 30fps | 1280x800 2560x1600 | 2560x1600 2560x1600 | Spatial 2x | Spatial 2x SNR | Spatial 2x SNR |
| B | Kimono | 240 | 24fps | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | ParkScene | 240 | 24fps | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | Cactus | 500 | 50fps | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | BasketballDrive | 500 | 50fps | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |
| B | BQTerrace | 600 | 60fps | 960x540 1280x720 1920x1080 | 1920x1080 1920x1080 1920x1080 | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR | Spatial 2x Spatial 1.5x SNR |

Table 2: Test sequences

## Metafile creation for AVC base layers.

A modified version of the JSVM decoder is available at the following address: https://hevc.hhi.fraunhofer.de/svn/svn_SHVCMetadataExtractor/. This modified JSVM decoder enables to produce a metafile from an AVC bitstream. This metafile contains the needed motion information of the base layer to be used for the prediction of the HEVC enhancement layer.

For each 4x4 block in a raster scan order, motion information included in the metafile is represented on 10 bytes. The details of the 10-byte motion information is as follows: [RefIdxL0(1byte), RefIdxL1(1byte), MVxL0(2bytes), MVyL0(2bytes), MVxL1(2bytes), MVyL1(2bytes)].

The Version 1.1 of the SHVC common software (SHM 1.1) will be able to read this metafile for inter-layer prediction to create the HEVC enhancement layer. SHM 1.1 will be made available three weeks after the end of the 12th JCT-VC meeting.

# Quantization parameter values

Table 3 defines the quantization parameter values used for the I-frames in base and enhancement layers of a sequence (configuration files further define QP values used for other frames) for the AVC and HEVC base layers cases. They are currently identical.

| Quantization parameter values |  |
| --- | --- |
| AVC base layer | HEVC base layer |
| Scalability; QP of AVC Base Layer; Delta QP of Enhancement layer. Spatial 2x; 22, 26, 30, 34; 0, 2. Spatial 1.5x; 22, 26, 30, 34; 0, 2. SNR; 26, 30, 34, 38; -6, -4. | Scalability; QP of HEVC Base Layer; Delta QP of Enhancement layer. Spatial 2x; 22, 26, 30, 34; 0, 2. Spatial 1.5x; 22, 26, 30, 34; 0, 2. SNR; 26, 30, 34, 38; -6, -4. |

Table 3: Quantization parameter values

# Configuration files

The following sections define encoder configuration files to be used for each test configurations. Only two layers are considered for the common testing conditions. Parameters to be changed for each test point are the following (where N=0 for the Base Layer and N=1 for the Enhancement Layer):

- InputFileN to reflect the location of the source video sequence for layer N on the test system

- FrameRateN to reflect the frame rate of a given sequence for layer N as per Table 1

- SourceWidthN to reflect the width of the source video sequence for layer N

- SourceHeightN to reflect the height of the source video sequence for layer N

- FramesToBeEncoded to reflect the frame count of a given sequence as per Table 1

- IntraPeriodN to reflect the intra refresh period for layer N in the random access test cases. The intra refresh period is dependent on the frame rate of the source: a value 16 shall be used for sequences with a frame rate equal to 20fps, 24 for 24fps, 32 for 30fps, 48 for 50fps, and 64 for 60fps.

- QPN to reflect the quantization parameter values for layer N defined in section 3.

These configuration files are provided in the cfg/ folder of version 1.0 of the common SHVC software (SHM 1.0) package (available at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/). There are 4 configurations provided as follows:

- “All Intra – Main” (AI-Main): encoder_intra_main.cfg

- “Random access – Main” (RA-Main): encoder_randomaccess_main.cfg

- “Low-delay B – Main” (LB-Main): encoder_lowdelay_main.cfg

- “Low-delay P – Main” (LP-Main, optional): encoder_lowdelay_P_main.cfg

Sequence-specific parameters are to be found in the cfg/per-sequence-svc/ folder.

# Results

The results of a given test will be reported into one of the two Excel files attached to this document.  These two documents are named “JCTVC-L1009-SHVC-Template-HEVCBase.xls” and “JCTVC-L1009-SHVC-Template-AVCBase.xls”. The first one is using the HEVC base layer while the second one contains the AVC base layer.

For both documents, the “Test” sheet represented in Figure 1 is the one to be filled where the different columns are represented below. The mandatory data to be filled correspond to red columns while green columns are optional.

Figure 1: Excel sheet description.

# Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/TLibCommon folder of the common software. The default settings provided in the source code should be used.

# References

- F. Bossen, “Common test conditions and software reference configurations JCTVC-K1100”, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012