| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L1101 |
| --- | --- |

| Title: | Core Experiment 1: Intra Prediction Improvements in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Ali Tabatabai Krishna Rapaka Ankur Saxena Shan Liu | Tel: Email: | ali.tabatabai@am.sony.com krapaka@qti.qualcomm.com asaxena@sta.samsung.com shan.liu@meditek.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This output document provides a list of the tools being tested in Core Experiment 1 (CE1) for the 13th JCTVC meeting related to Intra Prediction improvements in SHVC.

# Introduction

During the 12th JCT-VC meeting in Geneva, multiple proposals related to Inter-layer Intra Texture and Syntax Prediction for the SHVC design were proposed. It was then decided to coordinate for further study and testing of these methods in a core experiment.

# Participants

| Company | Name & Email |
| --- | --- |
| Qualcomm | K. Rapaka (krapaka@qti.qualcomm.com), J. Chen (cjianle@qti.qualcomm.com), X. Li (lxiang@qti.qualcomm.com), M. Karczewicz (martak@qti.qualcomm.com |
| Sony | C. Auyeung (cheung.auyeung@am.sony.com), A. Tabatabai (ali.tabatabai@am.sony.com), Jun Xu (jun.xu@am.sony.com) |
| Samsung | E. Alshina (elena_a.alshina@samsung.com), C. Kim (mailtodionism@samsung.com) |
| Nokia | J. Lainema (jani.lainema@nokia.com), K. Ugur (kemal.ugur@nokia.com) D. Bugdayci (ext-done.bugdayci@nokia.com) |
| LG Electronics | C. Kim (chulkeun.kim@lge.com) J. Park (jy.park@lge.com) J. Kim (jungsun.kim@lge.com) |
| Vidyo | W. Jang (wonkap@vidyo.com ), J. Boyce (jill@vidyo.com), A. Abbas (adeel@vidyo.com) |
| MediaTek | M. Guo (mei.guo@mediatek.com), S. Liu (shan.liu@mediatek.com) |

# Crosschecking assignments

| Tests | Technique | Proponent | Cross-checkers |
| --- | --- | --- | --- |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The test conditions defined in CTC shall be used for this CE. The CE will use the Class A and Class B sequences that were considered in the SHVC Call for Proposals. All frames shall be processed, and the lower resolution representations of the sequences shall correspond to the same lower resolution representations as considered in the SHVC Call for Proposals.

The CE shall test following cases.

|  | AI | RA |
| --- | --- | --- |
| Scalability | 2x, 1.5x | 2x, 1.5x, and SNR |

Since this CE deals with only Intra tools, All Intra and RA configurations are required. LD-P configurations are not mandatory.

[Coordinators] is RA optional or mandatory?

  - Software

Reference software SHM1.0 will be used as the anchor and base software in this CE. SHM1.0 will be available on February 7, 2013.

- Feb 7: SHM Available; and CE1 description finalized

- Feb 12: Release Category 1 tools in CE1 on top of SHM1.0 software i.e., SHM + Nokia (L-0036)+ LG (L-0099).

- Feb 18: Release Category 2 tools based on the combination of listed tools in this category on top of software in (B) (Volunteer/organizer: Qualcomm/LG/Sony). The tool should be Lowest Common Denominator of all the tools in Category 2.

- Feb 22: Release Category 3 tools based on the combination of the listed tools in this category on top of software (C) (Volunteer/organizer: Mediatek/LG/Qualcomm/Sony). The tool should be Lowest Common Denominator of all the tools in Category 3.

- Mar 1: Proponents from all categories release their tools on top of software in (D)

- Mar 15: Cross-Check Finished.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, and complexity. Complexity assessment is defined in 2.3.1.

### Complexity assessment

Both hardware and software complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding should be performed with or without output of decoded YUV data for both the anchors and tested tool. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented. Timelines

2013-Feb -7:	CE1 description finalized and uploaded.

2013-Feb -7:	Release of reference software from AHG ‘SHVC software’

2013-Feb -14:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators.

2013-Feb -28:	Cross-verifiers report results to CE participants (2 weeks from cross-check start)

2013-xxx -x:	Contribution documents uploaded, the document number and title are reported to coordinators.

2013-xxx -x:	Summary report uploaded

# Description of Core Experiment

Intra prediction improvement core experiments are divided into 3 categories

- CE1-1.1: Intra prediction based on reconstructed base layer

- CE1-1.2: Intra Prediction Based on Differential Picture

- CE1-1.3: Inter -Layer Intra Mode Prediction

The following experiments will be conducted and evaluated in CE6.

| Test | Technique | Proponent | Cross-checker |
| --- | --- | --- | --- |
| CE1-1.1 | Intra Prediction Based on Reconstructed Base Layer |  |  |
|  | JCTVC-L0036 | Nokia |  |
|  | JCTVC-L0099 | LG |  |
| CE1-1.2 | Intra Prediction Based on Differential Picture |  |  |
|  | JCTVC-L0037 | Nokia |  |
|  | JCTVC-L0135 | LG |  |
|  | JCTVC-L0140 | LG |  |
|  | JCTVC-L0183 | Vidyo/Samsung |  |
|  | JCTVC-L0215 | Sony |  |
|  | JCTVC-L0222 | Qualcomm |  |
|  | JCTVC-L0294 | Qualcomm |  |
| CE1-1.3 | Intra Layer Intra Mode Prediction |  |  |
|  | JCTVC-L0220 | Sony |  |
|  | JCTVC-L0239 | LG |  |
|  | JCTVC-L0224 | Qualcomm |  |
|  | JCTVC-L0260 (1) | MediaTek |  |
|  | JCTVC-L0260 (2) | LG |  |
|  | JCTVC-L0031 | Nokia |  |
|  | JCTVC-L0156 | Leibniz Universitaet Hannover |  |

## Intra prediction based on reconstructed base layer

### Intra DC Correction (JCTVC-L0036, Nokia)

DC correction is performed for all intra PUs which have a size of 8x8 or larger and are not IntraBL or planar coded. This mode adds a constant value to each sample of the enhancement layer block so that the DC of the enhancement layer prediction equals to the DC of the co-located base layer block. In other words, a dc_delta is calculated by taking the difference between enhancement layer block’s average sample value and corresponding upsampled base layer block’s average sample value, and added to all the samples of the enhancement layer block:

P(x,y) = clip ( P’(x,y) + dc_delta )

where:

P(x,y) denotes the sample to be predicted in the enhancement layer picture.

P’(x,y) denotes a predicted enhancement layer sample after HEVC intra prediction process

dc_delta denotes the difference of average sample values in P’(x,y) and corresponding upsampled base layer block.

The method is to be tested alone and together with selected other intra coding tools in this CE.

### filling unavailable reference samples in intra prediction at enhancement layer(JCTVC-L0099, LG)

If there is any unavailable reference sample during intra prediction EL, the corresponding sample at BL will replace it. For example, in below figure, left-bottom reference samples are unavailable and then are replaced with corresponding samples at BL. The method is to be tested alone and together with selected other intra coding tools in this CE.

Figure 1:.Filling unavailable reference samples in intra prediction at enhancement layer

## Intra prediction based on differential picture

### Gradient based intra prediction (JCTVC-L0037, Nokia)

For directly horizontal and vertical intra prediction modes, the prediction is enhanced by measuring how the base layer pixel values change in the corresponding direction and applying the generated differential values when reconstructing prediction samples for the enhancement layer video. This process is illustrated in Figure 2 below for a 4x4 block using vertical prediction.

Figure 2: Gradient based enhancement of intra prediction for vertical prediction

There is no additional syntax or signalling associated with the proposed technology. Instead, the intra prediction process at enhancement layer is modified as follows for the mentioned intra prediction directions:

P(x,y) = clip ( PR(xR,yR) + B(x,y) - BR(xR,yR) )

Where:

P(x,y) denotes the sample to be predicted in the enhancement layer picture.

PR(xR,yR) denotes the enhancement layer reference sample in the enhancement layer picture based on the directionality of the intra prediction mode

B(x,y) in the base layer picture with location corresponding to the enhancement layer sample P(x,y)

BR(xR,yR) denotes the base layer reference sample in the base layer picture based on the directionality of the intra prediction mode

The method is to be tested alone and together with selected other intra coding tools in this CE.

### Difference Domain Intra Prediction (JCTVC-L0222, Qualcomm)

In the Difference Intra Prediction (DIP) method, as shown in Figure 3, the difference between the pixels of current neighbors and those of collocated BL neighbors are used to generate a difference prediction based on the intra prediction mode. The generated difference prediction signal is added to the collocated BL block signal to form the final prediction.

Figure 3: Difference Intra prediction

The intra prediction method for difference signal remains unchanged with respect to HEVC except for the planar mode. After intra prediction is performed to get the difference prediction signal, the bottom-right portion of the difference prediction for the planar mode is set to zero (that is each pixel(x, y) satisfying the condition (x + y) >= N-1, where N is the width of the current block is set to zero). Typically farther the current pixel from its reference pixels, higher is the prediction error. To reduce this error, in this method the farther pixels do not predict from reference pixels but only from collocated Base layer pixels.

Due to the high frequency nature of the difference signals, HEVC mode dependent intra smoothing (MDIS) process is disabled in difference domain intra prediction mode. As a result of no MDIS filtering, the latency and complexity are reduced with gain in BD-Rate.

The difference domain Prediction mode is indicated by a flag intra_resi_pred_flag at CU level.

Following two combinations are to be tested

1) Difference Domain Intra Prediction + planar Modification + MDIS Disabled

2) Difference Domain Intra Prediction + MDIS Disabled

The method is to be tested alone and together with selected other intra coding tools in this CE.

### Simplified Two Mode Difference Domain Intra Prediction w (JCTVC-L0294, Qualcomm)

In this method, only two intra prediction modes, namely, horizontal and vertical are enabled for Difference Intra Prediction (DIP). The best modes of these two are signaled using a flag at PU level. In Addition, MDIS and pixel filtering for horizontal and vertical modes are disabled for DIP mode. As a result, the DIP process at the enhancement layer is shown as follows,

Pf(x,y) = B(x,y) + (Pe(xR,yR) - Pb(xR,yR))

where,

Pf(x,y) : Final Prediction Pixels for the enhancement layer block

B(x,y) : Collocated Base layer pixels (INTRA-BL Prediction)

Pe(xR,yR) :  Spatial reference pixels for the enhancement layer block based on intra prediction direction

Pb(xR,yR) :  Corresponding spatial reference pixels for the collocated base layer block based on intra prediction direction

Since MDIS and Intra Filtering for horizontal and vertical mode are disabled, the intra prediction pixel generation for Pe(xR,yR) and Pb(xR,yR) can be by-passed in DIP.

Additionally, the Chroma Intra Mode is always derived from Luma Intra Mode.

The method is to be tested alone and together with selected other intra coding tools in this CE.

### Residual intra planar prediction (JCTVC-L0215, Sony) (JCTVC-L0140, LG)

In JCTVC-L0215 and JCTVC-L0140, Sony and LG independently proposed to modify the intra prediction method for the planar prediction from SMuC-0.1.1 in Figure 4 for the intra prediction of differential picture.

Figure 4: Intra Planar Prediction in SMuC-0.1.1.

In particular for SMuC-0.1.1, the intra planar prediction of each pixel Px,y at co-ordinate 1≤ x ≤ N and 1 ≤ y ≤ N  is the following,

PVx,y  = (N – y)·Rx,0  +  y·R0,N+1

PHx,y  = (N – x)·R0,y  +  x·RN+1,0

Px,y    = (PVx,y + PHx,y  + N)  >>  (log2(N) + 1)

where R0,N+1 is the bottom left pixel, and RN+1,0 is the top right pixel.

In this proposed method, the the residual planar intra prediction is obtained by setting the value of the bottom left pixel R0,N+1  and the value of the top right pixel RN+1,0 to zero in the calculation of the planar prediction of differential picture, i.e.,

PVx,y  = (N – y)·Rx,0  +  0

PHx,y  = (N – x)·R0,y  +  0

Px,y    = (PVx,y + PHx,y  + N)  >>  (log2(N) + 1)

The method is to be tested alone and together with other intra coding tools in this CE.

### Residual intraBL and DC prediction (JCTVC-L0140, LG)

In JCTVC-L140, modifications on intraBL and DC prediction are proposed.

For both predictions, the prediction block is obtained by intraBL plus boundary process using difference reference samples. The boundary process is as follows.

Where,  represents the final prediction values,  represents the values of intraBL block, and  represents the difference reference sample values. Boundary process is applied only to luma component.

Different values for shift_bits and offsets are to be tested. And the methods are to be tested alone and together with other intra coding tools in this CE.

### Intra coding in difference domain (JCTVC-L0135, LG)

Figure 5 shows the reconstruction process of a block to which the proposed technique is applied. In order to perform intra prediction in difference domain, we need reference samples in difference domain. They can be obtained by subtracting EL reconstruction samples from up-sampled BL reconstruction samples. After intra prediction with reference samples in difference domain, we get reconstruction block in difference domain by adding residuals. And then, we obtain the final reconstruction block by adding collocated block from BL.

In the proposed method, reference, prediction and reconstruction values in difference domain are clipped from [-255,255] into [-128, 127]. By doing it, we can keep the range size the same with the original domain.

If the block which the reference samples belong to is also coded with the proposed technique, reference samples are not calculated. Instead, reconstruction samples in difference domain are directly employed.

Figure 5: Reconstruction process of proposed technique

The method is to be tested alone and together with other intra coding tools in this CE.

## Inter-Layer Intra Mode prediction

### Inter-layer Intra Mode Prediction (JCTVC-L0260(1), MediaTek)

In this subset, an inter-layer intra mode prediction method will be tested. Three most probable modes (denoted as MPM[ x ] with x=0…2) are used at enhancement layer(EL), so there are no changes to EL parsing. Three most probable modes are derived for each luma PU from the intra mode of the corresponding unit at base layer (denoted as modeBL), the intra mode of neighboring PU on the left of the current PU (denoted as modeLeft) and the one of the neighboring PU on top of the current PU  (denoted as modeTop).

For each of modeBL, modeLeft and modeTop, if it is available and is not DC/Planar, it is put into MPM according to the priority (i.e., modeBL>modeLeft>modeTop) with the repetition removed. When three modes are available in MPM, {modeBL, modeLeft, modeTop} are directly used to predict the current intra mode at enhancement layer; otherwise, MPM is further generated as follows. Denote the number of modes available in MPM as numAvai.

- numAvai = 2, MPM = {MPM[0], MPM[1], DC}

- numAvai = 1, MPM = {MPM[0], MPM[0]-1, MPM[0]+1}

- numAvai = 0, MPM = {DC, VER, HOR}

The combination with coding tools in other subsets (e.g., subset 6.2) will also be tested.

### Inter-layer Intra Mode Prediction (JCTVC-L0260(2), LG)

In this subset, an inter-layer intra mode prediction method will be tested. If the intra prediction mode of the corresponding block in base layer is available, the most probable mode, MPM for the enhancement layer is {modeBL, modeBL-1, modeBL+1}. Otherwise MPM is derived just as in the base layer.

It also will be tested as the combination with coding tools in other subsets (e.g., subset 6.2).

### Inter-layer Intra Mode Prediction (JCTVC-L0224, Qualcomm)

In this method of Inter-Layer Intra Mode prediction, only the setting of the inputs to the MPM generation process namely intraPredModeA (the intra mode of neighboring PU on the left of the current PU) and intraPredModeB (neighboring PU on top of the current PU) is modified. The MPM list generation process is kept unchanged.

The proposed MPM list generation procedure is follows,

If the co-located BL block is available and intra coded, both intraPredModeA and intraPredModeB is set equal to co-located BL Intra Mode (iColBaseDir).

Else, intraPredModeA and intraPredModeB is derived from left and above block based on HEVC process.

Then HEVC MPM list generation process is used without any modifications and also there are no changes to EL parsing.

### Inter-layer Intra Mode Prediction (JCTVC-L0031, Nokia)

Intra prediction mode of enhancement layer is indicated by coding the difference with respect to the mode of base layer. Since the base layer intra prediction mode information is already present in the decoder side, the actual enhancement layer intra prediction mode information can be reconstructed by adding this difference to the base layer intra prediction mode. Probability distribution of the difference values can be exploited during binarization in order to improve the coding efficiency as follows:

- First bin indicates whether the difference is equal to zero, i.e. base layer intra prediction mode is equal to enhancement layer intra prediction mode or not.

- Second bin indicates the sign of the difference (either positive or negative)

- Third bin indicates whether the absolute value of the difference is equal to 1 or not.

- Fourth bin indicates whether the absolute value of the difference is equal to 2 or not.

- Since the difference values ranges from -34 to 34 and values 0, +-1 and +-2 are covered by the previous bins and the sign of the difference value is indicated by the second bin, there are a total of 32 absolute values that the difference can take. The remaining values can be coded by assigning absolute value of the difference minus 3 to 5 bins with equal probability (i.e. in bypass mode).

### Inter-layer Intra MPM derivation for EL in SHVC (JCTVC-L0220, Sony)

JCTVC-L0220 proposed an inter-layer intra mode prediction algorithm consisting of two parts.

First part is enabling SVC_BL_CAND_INTRA in SMuC0.1.1.

With the macro SVC_BL_CAND_INTRA in SMuC0.1.1 enabled, co-located BL intra prediction mode is used for MPM derivation if co-located BL CU is intra coded. Otherwise, the MPM derivation stays the same as HEVC. The MPM derivation is as follows.

- If co-located BL intra prediction mode is different from left and above neighbours in EL, co-located BL intra prediction mode is set to be the first MPM followed by intra modes of left and above neighbours.

- Otherwise, co-located BL intra prediction mode is equal to either left or above neighbour of EL. HEVC MPM derivation is applied.

Second part changes MPM derivation about neighbouring modes as follows.

- If the left or above neighbour is IntraBL mode and the co-located BL CU of left or above neighbour is intra coded, the intra mode of the co-located BL CU of left or above neighbour is used as left or above mode for MPM derivation.

- Otherwise, left or above mode is set to DC.

The second part can be applied to other inter-layer intra mode prediction algorithms.

The method is to be tested alone and together with other intra coding tools in this CE.

### Inter-layer Intra Mode assignment for EL in SHVC (JCTVC-L0239, LG)

In this subset, modes of neighbouring blocks for MPM derivation in the enhancement layer are inferred as the intra prediction mode of the corresponding blocks in the base layer.

If a neighbouring block is intraBL block and its corresponding block in the base layer is intra block, the intra prediction mode of the corresponding block is used as neighbouring mode(left/above) for MPM derivation. If its corresponding block in the base layer is not intra block or is not available, the mode is set to DC.

It also will be tested as the combination with coding tools in other subsets or will be tested as a part of other coding tool (e.g. 6.3.5)

### Inter-layer intra mode prediction (JCTVC-L0156, Leibniz Universitat Hannover)

Intra prediction mode of enhancement layer is explicitly signaled whether the intra prediction mode of enhancement layer is equal to that of the co-located unit in base layer. In case intra prediction mode of enhancement layer is derived from that of co-located unit in base layer, MPM process in single layer is bypassed. There is no change to the MPM derivation process.

# References

- J. Lainema, K. Ugur (Nokia), “TE3: Results of test 4.1.2 on intra DC correction”, JCTVC-L0036, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013

- J. Lainema, K. Ugur (Nokia), “TE3: Results of test 4.2.1 on gradient based intra prediction”, JCTVC-L0037, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013

- K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm), “TE3 : Results of Test 4.2.2 on Intra prediction based on differential picture”, JCTVC-L0222, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013

- K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm), “Non-TE3 : Simplification of Difference Intra Prediction In SHVC”, JCTVC-L0294, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013

- M. Guo, S. Liu, S. Lei (MediaTek), J. Park, J. Kim, B. Jeon, “Non-TE5.1: Inter-layer Intra mode prediction”, JCTVC-L0260, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013

- K. Rapaka, J. Chen, V. Seregin, M. Karczewicz (Qualcomm), “TE5 : Results of Test 5.1.2 on Inter-Layer Intra Mode Prediction”, JCTVC-L0224, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013

- C. Auyeung and A. Tabatabai (Sony) “Non-TE3: Cross-checking and simplification of intra residual planar prediction in TE3-4.2.2”, JCTVC-L0224, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013.

- J. Park and B. Jeon (LG), “Non-TE3 : Modifications on inter-layer texture, DC, planar predictions”, JCTVC-L0140, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013.

- J. Xu, A. Tabatabai, “Non-TE5: Inter-layer Intra MPM derivation for EL in SHVC”, JCTVC-L0220, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013.

- C. Kim, B. Jeon (LG). “TE 3: Results of test 4.1.3 on filling unavailable reference samples in intra prediction at enhancement layer”, JCTVC-L0099, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013.

- J. Park, B. Jeon (LG), “TE3-4.2.3: Intra prediction based on difference picture”, JCTVC-L0135, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013.

- J. Kim, J. Park, B. Jeon (LG), S. Liu, M. Gui, S. Lei (MediaTek), “Assigning intra prediction mode to inter layer intra predicted blocks in enhance layer”, JCTVC-L0239, 12th JCT-VC Meeting, Geneva, CH, Jan. 2013.