| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-1103r2 |
| --- | --- |

| Title: | Description of Core Experiment SCE 3: Combined Inter and Inter-Layer Prediction in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | X. Li E. François P. Lai D. Kwon A. Saxena | Tel: Email: | lxiang@qti.qualcomm.com edouard.francois@crf.canon.fr Polin.Lai@mediatek.com d-kwon@ti.com asaxena@sta.samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of Core Experiment 3 on Combined Inter and Inter-layer Prediction in SHVC.

# Introduction

This document defines Core Experiment 3 (CE3) on combined prediction in SHVC, to be performed for the 13th JCT-VC meeting. The CE consists of proposals which use combined base layer and motion compensated enhancement layer information to predict enhancement layer. Intra coding is not addressed in this CE.

# Participants

| Company | Name & Email |
| --- | --- |
| Canon | E. François (edouard.francois@crf.canon.fr), C. Gisquet (Christophe.gisquet@crf.canon.fr), G. Laroche (guillaume.laroche@crf.canon.fr), P. Onno (patrice.onno@crf.canon.fr) |
| Qualcomm | X. Li (lxiang@qualcomm.com), V. Seregin (vseregin@qti.qualcomm.com), K. Rapaka (krapaka@qti.qualcomm.com), J. Chen (cjianle@qti.qualcomm.com) |
| MediaTek | P. Lai (Polin.Lai@mediatek.com), T.-D. Chuang (peter.chuang@mediatek.com), Y.-W. Huang (yuwen.huang@mediatek.com), S. Liu (shan.liu@mediatek.com), S.-T. Hsiang (shih-ta.hsiang@mediatek.com), X. Zhang (Ximin.zhang@mediatek.com), Z. Chen (Zhenzhong.chen@mediatek.com), Shih-Ta Hsiang (shih-ta.hsiang@mediatek.com) |
| TI | D. Kwon (d-kwon@ti.com) M. Budagavi (madhukar@ti.com) |
| Samsung | A. Saxena (asaxena@sta.samsung.com), E. Alshina (elena_a.alshina@samsung.com), T. Lee (tammy.lee@samsung.com); C. Kim (dionism@samsung.com), A.Alshin (alexander_b.alshin@samsung.com) |
| Sharp | T. Yamamoto (yamamoto.tomoyuki@sharp.co.jp) |
| Intel | Y.-J. Chiu (yi-jen.chiu@intel.com), W. Zhang (wenhao.zhang@intel.com), X. Cai (xiaoxia.cai@intel.com) |
| Nokia | J.Lainema (Jani.Lainema@nokia.com), K. Ugur (kemal.ugur@nokia.com), A. Aminlou (ext-alireza.aminlou@nokia.com) |
| LG | J. Park (jy.park@lge.com) |
| Vidyo | W. Jang (wonkapjang@gmail.com) |
| ETRI | Jung Won Kang (jungwon@etri.re.kr), Jiho Lee (jinosoul@etri.re.kr), Hahyun Lee (hanilee@etri.re.kr) |
| Gachon University | - Woo-Jin Han (hurumi@gmail.com) |
| Huawei | - W. Gao (wen.gao@huawei.com), X. Wei (weixianghui@huawei.com), J. Zan (jinwen.zan@huawei.com), J. Song (songjianbin@huawei.com) |
| University of Hannover | - Z. Zhao (zhao@tnt.uni-hannover.de); Y. Liu (liuyiqun@tnt.uni-hannover.de) |
| RWTA-Aachen Univ. | M. Wien (wien@ient.rwth-aachen.de), C. Feldmann (feldmann@ient.rwth-aachen.de) |

# Crosschecking assignments

| Tests | Technique | Proponent | Cross-checkers |
| --- | --- | --- | --- |
| 3.1 | Adaptive predictor compensation | T.-D. Chuang (peter.chuang@mediatek.com) | A. Saxena (asaxena@sta.samsung.com) |
| 3.2 | Combined inter mode | V. Seregin (vseregin@qti.qualcomm.com) | H. Lee (hanilee@etri.re.kr) |
| 3.3 | Generalized residual prediction | X. Li (lxiang@qti.qualcomm.com), J.Lainema (Jani.Lainema@nokia.com) E. François (edouard.francois@crf.canon.fr) | W. Zhang (wenhao.zhang@intel.com) E. Alshina (elena_a.alshina@samsung.com) M. Wien (wien@ient.rwth-aachen.de) X. Wei (weixianghui@huawei.com) |
| 3.4 | Generalized combined and residue prediction | PoLin Lai (polin.lai@mediatek.com) T.-D. Chuang (peter.chuang@mediatek.com) | J. Park (jy.park@lge.com) |
| 3.5 | Generalized residual prediction with MC at BL | E. François (edouard.francois@crf.canon.fr) | T. Tsukuba (tsukuba.takeshi@sharp.co.jp) |
| 3.6 | Generalized residual prediction with shorter MC filter | T. Tsukuba (tsukuba.takeshi@sharp.co.jp) | K. Sato (Kazushi.Sato@jp.sony.com) |
| 3.7 | Difference domain inter prediction | J. Park (jy.park@lge.com) W. Jang (wonkap@vidyo.com) | T.-D. Chuang (peter.chuang@mediatek.com) |
| 3.8 | RefIdx based differential coding | J.Lainema (Jani.Lainema@nokia.com) | E. François (edouard.francois@crf.canon.fr) X. Li (lxiang@qti.qualcomm.com) |
| 3.9 | Base Mode with Residual Prediction | E. François (edouard.francois@crf.canon.fr) | X. Li (lxiang@qti.qualcomm.com) |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The SHVC common test conditions defined in JCTVC-L1009 [2] shall be used for this CE.

The CE shall test following cases.

|  | RA | LD-P |
| --- | --- | --- |
| Scalability | 2x, 1.5x, and SNR | 2x, 1.5x, and SNR |

Since the CE only addresses changes to inter pictures, only RA and LD-P tests are required.

  - Software

Reference software SM-1.0 will be used as the anchor and base software in this CE. The anchor depends of the SM1.0 framework (CU-level or Ref-idx) used by the proponent. The proponent must use as anchor the SM1.0 framework in which his proposal is implemented.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, and complexity. Complexity assessment is defined in 4.3.1.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed either with or without output of decoded YUV data for both the Anchors and the Test. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

To evaluate memory bandwidth requirement and computational complexity related to up-sampling and MC interpolation filter, methodologies defined in JCTVC-L0440 [1] shall be used.

# Tools to test

## Mixed Inter-Inter-layer Prediction

### Test 3.1 Adaptive predictor compensation by using reconstructed BL texture (JCTVC-0072, MediaTek)

JCTVC-L0072 [3] proposes an adaptive predictor compensation (APC) that uses reconstructed base layer (BL) texture to refine enhancement layer (EL) sample predictors. It only applied on inter-coded CU. A CU-level enabling flag is signaled for APC. If APC is applied, the EL predictor will be as follow.

PredEL’ = (PredEL + RecBL_cur) >> 1

The complexity and bandwidth requirement data will be provided. The results (include BD-rate, complexity, and bandwidth requirement) of combining general residual prediction will be provided as well.

### Test 3.2 Combined Inter Mode (JCTVC-L0206, Qualcomm)

The combined prediction mode [4] is applied to inter predicted CUs. The prediction in this mode is defined as an average of two predictions which are done in intra-BL mode and enhancement layer inter prediction mode. Let Pb denote intra-BL prediction and Pe denote enhancement layer inter prediction. The final prediction P for the CU is defined as

This method is applied only to the luma component. The enhancement layer inter prediction is used for chroma components when the combined prediction mode is enabled. The combined_mode_flag is used to indicate the usage of the mode at CU level.

Bandwidth requirement and complexity of the method will be studied, some parameters can be adjusted to address the complexity issues.

## Residual Prediction

### Test 3.3 Generalized Residual Prediction (JCTVC-L0078, JCTVC-L0190, JCTVC-L0038, JCTVC-L0100)

In this technique [5][6][7][8], inter predicted residue of the collocated BL block is used to predict that of the current block. This method is applied to inter CUs, and skip mode CUs. The framework of this method is shown in Figure 1Error: Reference source not found where the case of uni-prediction is illustrated.

Let Be and Bb denote the current block in the EL picture and its collocated BL block, respectively. Let Pe0 denote the temporal prediction for the block Be obtained by using the motion vector MVe0 (where sub-index 0 refers to reference list 0). Similarly, Pb0 represents the temporal prediction for the block Bb obtained by using the same motion vector MVe0 in the BL reference picture.

Then, the inter predicted residue of the BL block  is obtained as

Considering the temporal prediction Pe0 for block Be, the final uni-prediction P for block Be is

,

where w is a the weighting factor, which takes the values 0, 0.5, or 1.

When extending this method to the bi-predictional case, the final prediction P for Be is formed as

where sub-index 1 indicates reference list 1.

Moreover, another weighting mode can be applied for uni-prediction as follows

When extending to bi-predictional case, the final prediction P for Be is formed as

To distinguish the two weighting 0.5, the first one is called res-0.5 while the second is diff-0.5. The weighting factor w is signaled at CU level as a weighting index. The weighting index 0, 1, 2 and 3 are used to indicate the weighting factors 0, res-0.5, 1 and diff-0.5, respectively. A truncated unary code in regular CABAC mode is used for the signaling.

To reduce requirement of memory bandwidth, bilinear MC interpolation filter, 4-tap up-sampling filter, and block size constrained GRP mode will apply. In addition, the performance of GRP only on luma component will be investigated.

Figure 1 Inter residual prediction for uni-prediction

### Test 3.4 Generalized combined and residue prediction (JCTVC-L0074, MediaTek)

JCTVC-L0074 [9] evaluates the results for the proposed generalized combined and residual prediction (GCRP) method. In this method, the EL predictor for GCRP is as the following:

PredEL’= (1-a) * PredEL + a* RecBL_cur + b * (RecBL_cur – RecBL_ref)

The factor a adjusts the weighting between EL temporal prediction PredEL and the collocated reconstructed BL RecBL_cur, while the factor b adjusts the weighting of inter predicted residue of the collocated BL block (RecBL_cur – RecBL_ref). The factors a and b can take the possible values of 0, 0.25, 0.5, 1. The weighting factor a and b are signaled at CU level as weighting indices. Truncated unary codes in regular CABAC mode are used for the signaling.

In addition, to reduce memory bandwidth, shorter MC interpolation filter, combined up-sampling MC interpolation filter, constraining block size in GCRP mode, constraining GCRP to uni-prediction, will be tested.

### Test 3.5 Generalized Residual Prediction with MC at BL resolution (JCTVC-L0104, Canon)

JCTVC-L0104 [10] proposes a simplification of the Generalized Residual Prediction which consists in directly accessing the temporal reference BL data instead of the upsampled temporal reference BL data to generate the second order residual, as illustrated in the next figure.

Figure 2 Generalized Residual Prediction with MC at BL resolution

As this approach only makes sense for spatial scalability and not for SNR scalability, the proposal will be implemented and tested only for spatial scalability with ratios 2x and 1.5x. In order to reduce requirement of memory bandwidth, three adaptations will be tested:

- limitation of the PU block sizes;

- limitation to Uni-Prediction;

- reduction of the size of the combined upsampling and motion compensation filter.

The optimal settings in terms of trade-off complexity/performance will be provided.

### Test 3.6 Generalized Residual Prediction with shorter-tap MC filter (JCTVC-L0265, Sharp)

Generalized Residual Prediction, such as JCTVC-L0038, JCTVC-L0100 or JCTVC-L0078, is simplified by applying shorter-tap MC filter to create motion-compensated up-sampled base-layer reconstruction [11]. As the shorter-tap MC filter for luma, bilinear filter is used for B_SLICE and DCT-IF based 4-tap filter is used for P_SLICE. As the shorter-tap MC filter for chroma, bilinear filter is used irrespective of slice type.

## Diff inter prediction

### Test 3.7 Inter prediction based on difference picture (JCTVC-L0136, JCTVC-L0184, LG/Vidyo)

JCTVC-L0136 and JCTVC-L0184 [12][13] proposed inter coding in differential domain. When inter difference mode is applied, the reconstruction samples can be represented as below.

Rec = Up(Rec_BL) + MC(Diff, MV) + Residual, where Diff = Ref_EL-Up(Ref_BL)

In the proposed method, prediction and reconstruction values in D-domain are clipped to keep the range the same as the normal pixel values. To reduce the memory access bandwidth and computational complexity, bi-linear MC filter for differential domain was proposed in JCTVC-L0184. And the 0.5-weight to the difference signals has been introduced in the crosscheck document (JCTVC-L0081) of JCTVC-L0136. We plan to test 4 variants of inter prediction based on difference picture.

- weight = 1 && DCT-IF as MC interpolation filter

- weight = 1 && bi-linear IF as MC interpolation filter

- weight = 0.5 && DCT-IF as MC interpolation filter

- weight = 0.5 && bi-linear IF as MC interpolation filter

### Test 3.8 RefIdx based differential coding (JCTVC-L0213, Nokia)

JCTVC-L0213 [14] proposes a differential coding approach based on RefIdx signaling. The method creates new reference pictures using a differential process, includes those together with a base reference to reference picture lists and applies HEVC version 1 weighted prediction to perform motion compensated prediction utilizing the new reference pictures. Picture below illustrates the process.

Figure 3 RefIdx based differential coding

This test will evaluate coding performance and complexity of the approach proposed in JCTVC-L0213. In addition, results for different configurations based on different weighting and placement of reference pictures in the reference picture lists may be provided.

## Base mode with residual prediction

### Test 3.9 Base Mode with Residual Prediction (JCTVC-L0103, Canon)

JCTVC-L0103  proposes a Base Mode, applied at the CU-level and consisting in an 8x8-based Base Layer syntax inheriting, combined with Generalized Residual Prediction. The MC filters used for BL and EL motion compensation are bilinear.

Test 3.4 will be based on this design and will evaluate the usage of Generalized Residual prediction combined with the Base Mode. In particular the goal of this test is to identify if it is beneficial to adaptively enable or disable GRP for Base Mode.

As for test 3.5, in order to reduce requirement of memory bandwidth, three adaptations will be tested:

- limitation of the PU block sizes;

- limitation to Uni-Prediction;

- reduction of the size of the combined upsampling and motion compensation filter.

The optimal settings in terms of trade-off complexity/performance will be provided.

# Time-line

2013-Feb-06:	CE-3 description finalized and uploaded. Release of the Excel template for Complexity evaluation

2013-Feb-06:	Release of reference software SM1.0 from AHG ‘SHVC software’

2013-Feb-20:	Release of reference software SM1.0 with complexity assessment modules (from Samsung/InterDigital)

2013-Mar-06:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators

2013-Mar-13:	Proponents provide complexity analysis results to cross-verifiers(s) and CE coordinators

2013-Mar-27:	Cross-verifiers report results to CE participants

2013-Apr-08:	Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Apr –15:	Summary report uploaded

# References

- E. François, A. Tabatabai, E. Alshina “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-L1009, Geneva, Switzerland, 14–23 Jan. 2013

- T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei, “TE3-4.3.1: Adaptive predictor compensation by using reconstructed BL texture”, JCTVC-L0072, Geneva, Switzerland, 14–23 Jan. 2013.

- V. Seregin, J. Chen, M. Karczewicz, “TE3: Combined inter mode (test 4.6.2.2)”, JCTVC-L0206, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0078, Geneva, Switzerland, 14–23 Jan. 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczewicz, “Non-TE3: Extension of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0190, Geneva, Switzerland, 14–23 Jan. 2013.

- J. Lainema, K. Ugur, “TE3: Results of test 4.6.3 on base enhanced motion compensated prediction”, JCTVC-L0038, Geneva, Switzerland, 14–23 Jan. 2013.

- C. Gisquet, F. Le Léannec, J. Taquet, E. François, G. Laroche, P. Onno, “TE3: Results of test 4.6.1 on the Generalized Residual Inter-Layer Prediction (GRILP)”, JCTVC-L0100, Geneva, Switzerland, 14–23 Jan. 2013.

- T.-D. Chuang, S. Liu, M. Guo, Y.-W. Huang, S. Lei, “Non-TE3: Adaptive predictor compensation with generalized residual prediction”, JCTVC-L0074, Geneva, Switzerland, 14–23 Jan. 2013.

- E. François, J. Taquet, C. Gisquet, G. Laroche, P. Onno, “Non-TE3: Simplification of Generalized Residual Inter-Layer Prediction (GRILP) in SHVC”, JCTVC-L0104, Geneva, Switzerland, 14–23 Jan. 2013.

- T. Tsukuba, T. Yamamoto, T. Ikai, “On interpolation filter for Generalized Residual Prediction”, JCTVC-L0265, Geneva, Switzerland, 14–23 Jan. 2013.

- J. Park, B. Jeon, “TE3-4.4.1: Inter prediction based on difference picture”, JCTVC-L0136, Geneva, Switzerland, 14–23 Jan. 2013.

- W. Jang, J. Boyce, A. Abbas (Vidyo), E. Alshina, C. Kim, “TE3: Results of test 4.4.2 on inter prediction based on difference coding”, JCTVC-L0184, Geneva, Switzerland, 14–23 Jan. 2013.

- A. Aminlou, J. Lainema, K. Ugur, M. Hannuksela, “Differential coding for RefIdx based scalability”, JCTVC-L0213, Geneva, Switzerland, 14–23 Jan. 2013.

- C. Gisquet, E. François, G. Laroche, P. Onno, “Non-TE5.3: Inter-layer inferred prediction mode and motion compensation restrictions in SHVC”, JCTVC-L0103, Geneva, Switzerland, 14–23 Jan. 2013.