| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L1105 |
| --- | --- |

| Title: | Description of Core Experiment SCE5: Inter-layer syntax prediction using HEVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Vadim Seregin Patrice Onno Shan Liu Elena Alshina Chulkeun Kim Haitao Yang | Email: | vseregin@qti.qualcomm.com patrice.onno@crf.canon.fr shan.liu@mediatek.com elena_a.alshina@samsung.com chulkeun.kim@lge.com haitao.yang@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 5 (SCE5) on inter-layer inter syntax prediction using HEVC as a base layer in SHVC.

# Introduction

During the 12th JCT-VC meeting in Geneva, multiple proposals using inter-layer inter syntax prediction for the SHVC design were proposed. It was then decided to coordinate for further study and testing of these methods in a core experiment.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| Qualcomm | V. Seregin K. Rapaka L. Guo J. Chen X. Li M. Karczewicz | vseregin@qti.qualcomm.com krapaka@qti.qualcomm.com liweig@qti.qualcomm.com cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com martak@qti.qualcomm.com | X | X |
| Canon | P. Onno G. Laroche E. Francois | patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr Edouard.francois@crf.canon.fr | X | X |
| MediaTek | P. Chuang Y.W. Huang S. Liu M. Guo Shih-Ta Hsiang | Peter.Chuang@mediatek.com yuwen.huang@mediatek.com shan.liu@mediatek.com mei.guo@mediatek.com Shih-ta.Hsiang@mediatek.com | X | X |
| Samsung | A. Saxena Z. Ma E. Alshina | asaxena@sta.samsung.com zhan.ma@sta.samsung.com elena_a.alshina@samsung.com | X |  |
| LG | C. Kim J. Park | chulkeun.kim@lge.com jy.park@lge.com | X | X |
| Huawei | H. Yang X. Wei J. Zan H. Yu W. Gao | haitao.yang@huawei.com weixianghui@huawei.com jinwen.zan@huawei.com haoping.yu@huawei.com wen.gao@huawei.com |  |  |
| HHI | H. Laksman | haricharan.lakshman@hhi.fraunhofer.de |  |  |
| InterDigital | Y. He Y. Ye J. Dong X. Xiu | Yong.He@InterDigital.com Yan.Ye@InterDigital.com Jie.Dong@InterDigital.com Xiaoyu.Xiu@interDigital.com |  |  |
| KDDI | K. Kawamura | kei@kddilabs.jp | X | X |
| Intel | W. Zhang Y.-J. Chiu Z. Deng | wenhao.zhang@intel.com yi-jen.chiu@intel.com zhipin.deng@intel.com | X | X |
| TI | M. Budagavi D.-K. Kwon | madhukar@ti.com d-kwon@ti.com |  |  |
| ETRI | J. Lee J. Kang H. Lee | jinosoul@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr | X |  |
| Vidyo | W. K. Jang | wonkap@vidyo.com |  |  |
| Nokia | D. Bugdayci K. Ugur J. Lainema | ext-done.bugdayci@nokia.com Kemal.Ugur@nokia.com Jani.Lainema@nokia.com | X |  |
| Sony | K. Sato C. Auyeung J. Xu S. Lu | Kazushi.Sato@jp.sony.com Cheung.Auyeung@am.sony.com Jun.Xu@am.sony.com Shuo.Lu@jp.sony.com | X | X |
| Sharp | A. Segall K. Misra S.-H. Kim T. Ikai T. Yamamoto T. Tsukuba | asegall@sharplabs.com misrak@sharplabs.com kimse@sharplabs.com ikai.tomohiro@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp tsukuba.takeshi@sharp.co.jp |  |  |
| I2R | Y. H. Tan | yhtan@i2r.a-star.edu.sg | X | X |
| JVC Kenwood | H. Nakamura | nakamura.hiroya@jvckenwood.com |  | X |
| Gachon University | W.-J. Han | hurumi@gmail.com |  |  |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The CE will use the common test condition[1] provided by AhG10 with additionally included SNR test case.

IntraBL framework is used as an anchor.

  - Software

SHVC test model SHM1.0 software and available on February 06, 2013.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and complexity. Complexity assessment is performed on the base of decoding complexity measurement.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

# Description of Experiments

The tools in this CE are classified into two categories:

- 5.1  Inter-layer motion information prediction

  - Using base layer MV as a candidate in Merge and AMVP modes.

  - Inter-layer inferred prediction mode, where the enhancement layer is divided into sub-blocks and motion information associated with each sub-block is derived from the corresponding block in the base layer.

Every change comparing to the anchor such as increase number of candidates, additional base layer MV candidates, different derivation of the base layer MV, or similar should be studied separately.

Compressed base layer motion field should be used for the tests in this category, and uncompressed base layer field can be used for supplementary data.

- 5.2  Motion data compression and memory reduction (memory measurement)

In this section, motion data compression of the base layer in relation to the base layer motion vector derivation is studied as well as a memory reduction associated with not using temporal MV candidate.

Amount of memory required for storing compressed base layer motion field should be used as a comparison point for the tests where compression of the base layer is changed.

The following experiments will be conducted and evaluated in SCE5.

| Test | Technique | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 5.1 | Inter-layer motion information prediction |  |  |
| 5.1.1 | JCTVC-L0205 | Qualcomm | MediaTek |
| 5.1.2 | JCTVC-L0266 | Qualcomm | LG |
| 5.1.3 | JCTVC-L0101 | Canon | I2R |
| 5.1.4 | JCTVC-L0376 | KDDI | Sony |
| 5.1.5 | JCTVC-L0070 | MediaTek | Sony |
| 5.1.6 | JCTVC-L0025 | Nokia | Qualcomm |
| 5.1.7 | JCTVC-L0291 | Intel | KDDI |
| 5.1.8 | JCTVC-L0070 | LG | Qualcomm |
| 5.1.9 | JCTVC-L0115 | Samsung | Intel |
| 5.1.10 | JCTVC-L0065 | ETRI | Canon |
| 5.2 | Motion data compression and memory reduction |  |  |
| 5.2.1 | JCTVC-L0073 JCTVC-L0221 | I2R Sony | JVC Kenwood |
| 5.2.2 | JCTVC-L0279 JCTVC-L0070 | Sony LG | MediaTek |

  - Inter-layer motion information prediction

### JCTVC-L0205 (Qualcomm)

BL MV candidates can be included into the Merge/AMVP candidate modifying the process of candidate list construction or be included into the already completed candidate list.

Compressed and uncompressed base layer motion field can be used to derive BL MV.

Also, combination with the base layer motion compression or memory reduction tools from the category 5.2 will be studied.

### JCTVC-L0266 (Qualcomm)

An inferred base layer mode is introduced and applied to enhancement layer coding. In this mode, prediction information for a current enhancement layer CU is inferred from its co-located base layer blocks.

For a CU with this mode, it is divided into a number of basic units, and for each basic unit, its motion information is derived from its co-located base layer block. Adaptive basic unit size derivation will be investigated.

Also, combination with the base layer motion compression tools from the category 5.2 will be studied. The interaction of inferred mode and the tools for motion vector prediction from category 5.1 will be investigated.

### JCTVC-L0101 (Canon)

The contribution JCTVC-L0101 brings first results about the Merge list modification only and second results in combination with the Base Mode.

The first experiment to be tested here is a particular motion vector prediction scheme for the Merge list where the current positions of the motion vector predictors are changed. In particular, the order of candidates and possibly the number of candidates are modified according to the description of JCTVC-L0101.

In a second experiment, the position of the Base Layer motion vector and possibly the number of candidates are changed in the Merge candidate list when the Base Mode is enabled.

### JCTVC-L0376 (KDDI)

### In this contribution, an inter-layer motion-vector prediction is proposed for both the merge list and the AMVP. The maximum number of merge list candidates becomes six. A motion vector is derived from that of collocated and neighboring blocks with some refinement process which is similar to the SVC manner. The motion vector is not just copied from the collocated block in the derivation process. The predicted motion vector is added to merge lists as a third candidate. Furthermore, the maximum number of candidates of AMVP is three. Derivation process of motion vector is similar to that for merge mode. The predicted motion vector is added to the AMVP as a third candidate.

### JCTVC-L0070 (MediaTek)

In this test, in addition to the center BL motion candidate in TM, an addition bottom-right BL motion candidate is added in merging/AMVP candidate list. The simplified pruning process is also investigated to reduce the complexity. The compressed BL MV is used for BL motion candidate derivation.

### JCTVC-L0025 (Nokia)

Two simplifications are proposed in this experiment

- Scaling of the base layer motion vector is done in integer arithmetic using the motion vector scaling method in TMVP.

- Merge list construction is modified as follows to include BMVP in the merge list:

- Place the base layer MVP in the beginning of the list

- Select maximally 3 spatial MVP candidates.

- Place the temporal MVP to the list.

- Base layer MVP does not participate in redundancy check process to reduce complexity

### JCTVC-L0291 (Intel)

This test will study the performance of inter-layer motion data inferring for enhancement layer inter coding. The 2Nx2N PU is partitioned into four NxN sub-blocks. Each sub-block derives its collocated base layer MV. IntraBL will be used if the base layer MV is unavailable. The minimum sub-block size is limited to be 8x8.

### JCTVC-L0070 (LG)

In this test, BL MV can be included in the AMVP candidate list. The BL MV candidate is placed in the first place in the AMVP candidate list only when its reference list and reference picture are the same as those of the current PU in EL. The number of AMVP candidates is unchanged, which is 2 in this contribution. If a spatial candidate is same with BL MV candidate, pruning is performed.

### JCTVC-L0115 (Samsung)

So-called InterBL mode is applied to enhancement layer coding. This additional mode is signaled on CU level. In this mode, prediction information for a current enhancement layer CU is inferred from its co-located base layer blocks.

For a CU with this mode, it is divided into a number of basic units, and for each basic unit, its motion information is derived from its co-located base layer block. An interaction between InterBL mode and other tools utilizing motion vector prediction from base layer will be investigated.

### JCTVC-L0065 (ETRI)

In this test, the motion information of the colBase is added as the first merge candidate, and a duplication check between colBase and spatial candidate A1 is added when deriving A1.

The motion information of the colBase is added as the first AMVP candidate, and the motion vector of the colBase is scaled when the POC of reference frame of the colBase is different from the POC of reference frame of the current PU in enhancement layer. In addition, duplication check and pruning process are performed when the number of candidate is larger than two after deriving spatial candidates.

  - Motion data compression and memory reduction

### JCTVC-L0073 (I2R) and JCTVC-L0221 (Sony)

This test will study the effectiveness of the temporal motion vector prediction (TMVP) candidates in the enhancement layers when base layer motion vectors are available as prediction candidates.

Change in syntax will be discussed with the result as:

- temporal_mvp_enabling_flag be totally disabled at the enhancement layer

- Having a flag to control inter-layer MV prediction on/off. When inter-layer MV is on, TMV for EL is off.

- Having both temporal_mvp_enabling_flag and colblmv_enabling_flag separately.

Combination of motion compression as described in 5.2.2 will also be tested.

### JCTVC-L0279 (Sony) and JCTVC-L0070 (LG)

In this test impact of motion data compression on coding efficiency and complexity including memory reqirement will be studied.

Fig. 1 (a) is the scheme implemented in the anchor: motion compression be executed right after encoding/decoding of the base layer. The scheme as shown in Fig. 1 (b) will be tested: motion compression be postponed after encoding/decoding of the enhancement layer. As intermediate solution the scheme as shown in Fig. 1 (c) will also be tested: motion compression by 2:1 be executed right after encoding/decoding of the base layer, and another compression by 2:1 be executed after encoding/decoding of the enhancement layer.

Fig. 1

# Time-line and Responsibilities

2013-Feb-06:       CE5 description finalized and uploaded.

2013-Feb-06:       Software SHM1.0 distributed by AhG on SHVC Software

2013-Feb-06:     Configuration files and anchor performance will be distributed

2013-Feb-20:      Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators.

2013-Mar-06:       Cross-verifiers report results to CE participants (2 weeks from cross-check start)

2013-Apr-08:       Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Apr-16:      Summary report uploaded

# Reference

[1] X. Li, J. Boyce, P. Onno, and Y.Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model”, JCTVC-L1109, Jan. 2013, Geneva, Switzerland.