| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L1121 |
| --- | --- |

| Title: | HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | J. Sole K. Kawamura J. Kim | Email: | joels@qti.qualcomm.com ki-kawamura@kddi.com jungsun.kim@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Range Extensions Core Experiment 1 on inter component decorrelation methods.

# Introduction

Several proposals have been presented in the 12th JCT-VC meeting aiming at improving the HEVC Range Extensions performance by using methods that reduce the correlation across the three components. The goal of this CE is to study the performance and complexity associated to each of these proposals for 4:20, 4:2:2 and 4:4:4 chroma formats.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| BBC | M. Mrak R. Weerakkody | Marta.Mrak@bbc.co.uk Rajitha.Weerakkody@bbc.co.uk |  |  |
| Canon | C. Rosewarne | chris.rosewarne@cisra.canon.com.au |  | x |
| FastVDO | P. Topiwala | pankajtva@gmail.com | x |  |
| Huawei | W. Gao | wen.gao@huawei.com |  |  |
| JVC Kenwood | H. Nakamura | nakamura.hiroya@jvckenwood.com |  |  |
| KDDI | K. Kawamura | ki-kawamura@kddi.com | x |  |
| LG | J. Kim | jungsun.kim@lge.com | x |  |
| Mediatek | C.-W. Hsu C.-Y. Chen C.-Y. Tsai Y.-W. Huang | cw.hsu@mediatek.com chingyeh.chen@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com |  | x |
| Qualcomm | J. Sole | joels@qti.qualcomm.com |  | x |
| RIM | D. Flynn | dflynn@rim.com |  |  |
| Samsung | S. Lee | sunil.lee@samsung.com |  | x |
| Sony | N. Saunders K. Sharman | nick.saunders@eu.sony.com karl.sharman@eu.sony.com |  |  |

# Experiments

## Tests

Methods to be tested in the CE:

### Test 1.1

JCTVC-L0240, AHG7: The performance of extended chroma mode for non 4:2:0 format

  - Two methods for chroma prediction which predicts chroma samples using linear combination of luma samples (LM mode)

    - LM as was proposed in HEVC version 1, extended to non-4:2:0

    - LM where parameter calculation is performed on a CU basis rather than per-TU

  - Proponent: LG

  - Cross-checker: Samsung

### Test 1.2

JCTVC-L0370, AHG7: Inter-plane intra coding of residual signals for range extensions

  - Intra coding approach based on inter-plane prediction of intra residual signals by using a linear model. Prediction parameter of the model is derived on the encoder. Resulting parameter is predicted from neighbouring parameters.

  - Proponent: KDDI

  - Cross-checker: Samsung

### Test 2.1

JCTVC-L0371, AHG7: In-loop color-space transformation of residual signals for range extensions

  - This method transforms prediction error signals into those in a adaptively computed colour space. A transformation matrix is derived from pixel domain for each coding unit. The colour-space transformation is applied to prediction error of both intra and inter mode. No additional signalling is necessary.

  - Proponent: KDDI

  - Cross-checker: Canon

### Test 2.2

JCTVC-L0175, AHG7: A Consumer 4:4:4 Coding Method Based On A 4:2:0 Codec

  - Four integer colour spaces:

    - YCbCr	= [.213, .715 .072; -.117, -.394, .511; .511, -.464, -.047] (Rec. 709)

    - YCoCg	= [.25 .5 .25; 1/2 0 -1/2;-.25 .5 -.25]

    - YFbFr	= [5/16 3/8 5/16; -1/2 1 -1/2;1 0 -1]

    - YFbFr2	= [3/16 5/8 3/16; -1/2 1 -1/2;1 0 -1]

  - Conversion at the input and output of the codec, with SNRs generated relative to the input GBR data

  - Proponents: FastVDO

  - Cross-checker: Mediatek

### Test 3

Combination of proposals

  - In order to study the interaction of the four proposals, performance of combinations of them will be tested:

    - JCTVC-L0240 (method 1) + JCTVC-L0370

    - JCTVC-L0240 (method 1) + JCTVC-L0370 + JCTVC-L0175 (YFbFr2)

    - JCTVC-L0240 (method 1) + JCTVC-L0370 + JCTVC-L0175 (YCbCr & YFbFr2) + JCTVC-L0371

  - Proponents: LG (provides results for 3.1), KDDI (for 3.2), FastVDO (for 3.3)

  - Cross-checkers: Qualcomm (3.1), Qualcomm (3.2), Canon (3.3)

  - Results for other combinations can be provided

  - Software

HM10.0_RExt2.0 shall be used.

  - Test Coding Conditions

The recommended sequences, configurations and test cases described in JCTVC-L1006 will be used. Results for all mandatory test cases shall be provided.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and complexity. Complexity assessment is performed on the base of hardware and encoding/decoding complexity.

### Complexity assessment

To measure software run time, HM10.0_RExt2.0 software with the anchor configuration and the proposal implemented on the same software shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged, specially the following points were brought up in the discussions:

- Dependencies between luma and chroma channel processing

- Computational cost of the derivation of the parameters (e.g., linear model parameters, SVD coefficients)

# Timeline

- 04-Feb-2013: HM-10.0 available.

- 06-Feb-2013: CE description finalized and uploaded.

- 11-Feb-2013: HM10.0_RExt2.0 available.

- 25-Feb-2013: HM10.0_RExt2.0 with each method integrated available.

- 4-Mar-2013: HM10.0_RExt2.0 with the four methods combined available.

- 11-Mar-2013: Cross-check begins: Proponents provide software, draft of contribution text and results to CE participants.

- 05-Apr-2013: Input document and summary report uploaded.