| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0002 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC Draft and Test Model editing (AHG 2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross (Fraunhofer HHI) Ken McCann (Samsung/ZetaCast) Woo-Jin Han (Gachon University) Il-Koo Kim (Samsung) Jens-Rainer Ohm (RWTH Aachen) Kazuo Sugimoto (Mitsubishi) Gary J. Sullivan (Microsoft) Thomas Wiegand (Fraunhofer HHI) | Email: | benjamin.bross@hhi.fraunhofer.de ken@zetacast.com hurumi@gmail.com ilkoo.kim@samsung.com ohm@ient.rwth-aachen.de sugimoto@merl.com garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC ad hoc group on Draft and Test Model editing (AHG2) between the 12th meeting in Geneva (14–23 January 2013) and the 13th meeting in Incheon (18-26 April 2013).

# Mandate

| Title and Email Reflector | Chairs | Meeting |
| --- | --- | --- |
| HEVC Draft and Test Model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-L1002 HEVC Test Model 10 (HM 10) Encoder Description. - Produce and finalize JCTVC-L1003 HEVC text specification Draft 10 / FDIS & Consent text. - Gather and address comments for refinement of these documents. - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text. | B. Bross K. McCann (co-chairs) W.-J. Han I.-K. Kim J.-R. Ohm K. Sugimoto G. J. Sullivan T. Wiegand (vice chairs) | No |

# Background

The tenth High Efficiency Video Coding (HEVC) test model (HM10) was developed from the ninth HEVC test model (HM9), following the decisions taken at the 12th JCT-VC meeting in Geneva.

# Method of Working

Two editorial teams were formed to work on the two documents that were to be produced:

- JCTVC-L1002 HEVC Test Model 10 (HM 10) Encoder Description [1]

  - Il-Koo Kim

  - Ken McCann

  - Kazuo Sugimoto

  - Benjamin Bross

  - Woo-Jin Han

- JCTVC- L1003 HEVC text specification Draft 10 / FDIS & Consent text [2]

  - Benjamin Bross

  - Woo-Jin Han

  - Jens-Rainer Ohm

  - Gary J. Sullivan

  - Thomas Wiegand

Editing JCTVC-L1003 was assigned a higher priority than editing JCTVC-L1002.  The text of the final draft of JCTVC-K1003 (version 34) was submitted to ISO/IEC JTC1/SC29 for Final Draft International Standard ballot and to ITU-T SG16 for Consent.

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of issues on the text of both documents.

# Status

Three successive versions of JCTVC-L1002 and 34 successive versions of JCTVC-L1003 were published by the Editing AHG following the 12th JCT-VC meeting.

The main changes in JCTVC-L1003, relative to the previous JCTVC-K1003, were as follows (with issue tracker number, where appropriate):

- Made various cleanup changes, including an addition that a lower value of general_level_idc indicates a lower level

- Fixed syntax and semantics of short_term_ref_pic_set_idx and lt_idx_sps[ i ] (#930)

- Fixed the typo of DiffPicOrderCnt( currPic, pic ) -> DiffPicOrderCnt( pic, currPic ) (#910)

- Integrated some editorial changes noted under JCTVC-K0214 and JCTVC-K0142

- Fixed several issues, including missing bottom picture boundary check in bottom right collocated motion vector derivation (#924), a cross-tile cabac dependency (#912) and several other issues (#921, #926)

- Fixed a typo in the semantics of hrd_op_set_idx[ i ] (#936)

- Fixed several issues, including WPP availability check for the second CTB in a row when this CTB is incomplete (#948) and several other issues (#950, #941, #932, #805)

- Fixed sao_merge_left/up parsing for dependent slices (#900)

- Adding missing condition on zero POC difference in MVP candidate A scaling (#927)

- Fixed deblocking for 4x4 chroma coding blocks to unly filter edges lying on 8x8 chroma grid (#882)

- Fixed several issues (#937, #938, #907, #909)

- Integrated the following decisions noted under JCTVC-L0363:

  - Modified check for equal reference pictures to be always based on POC difference

  - Increase the length of general_reserved_zero_16bits to 48 bits (and change its name accordingly)

  - Integrated byte alignment in the profile_tier_level( ) syntax structure

  - Correct the description of (forward increment only) modulo wrapping calculation for CPB removal delay

  - Allow the post-filter hint, user data registered, user data unregistered, progressive refinement segment end, and filler payload SEI messages to be suffix SEI messages

  - Allow encoder to send a separate bit rate for sub-picture HRD operation

- Fixed SAO luma and chroma sample locations (#879)

- Fixed an issue of slice_disable_deblocking_filter_flag controlling the deblocking of a whole picture (#881)

- Fixed several minor issues in the deblocking filter process (#883)

- Took care of some editor's notes.

- Fixed several minor issues (#955, #698, #956, #894, #964, #957, #944, #966)

- Increased bit expansion limit ratio from 4/3 to 5/3 (regardless of profile) according to the decision noted under JCTVC-L0292.

- Conditioned the presence of the syntax elements low_delay_hrd_flag[ i ] and cpb_cnt_minus1[ i ] on fixed_pic_rate_within_cvs_flag[ i ] and low_delay_hrd_flag[ i ], respectively, according to JCTVC-L0372

- Corrected the condition for the "memorization" process for context variables when dependent slice segments are supported, and changed the syntax element entry_point_offset[ i ] to entry_point_offset_minus1[ i ], according to the decisions noted under JCTVC-L0116.

- Removed profilePresenceFlag from the profile_tier_level( ) syntax structure according to the decision noted under JCTVC-L0227.

- Integrated the following decisions noted under JCTVC-L0043:

  - Classify NAL unit types 48–55 (the first 8 of the 16 NAL unit types that have "unspecified" usage) as "prefixes" (i.e. may start a new access unit)

  - Remove bit_rate_pic_rate_info( ) from VPS

  - Change the coding of the min_spatial_segmentation_idc syntax element from u(8) to ue(v), and specify the allowed value range as 0..4095

  - Require that the TemporalId of any non-VCL NAL unit shall not be less than the TemporalId of the access unit containing the NAL unit (which implies that a VPS, SPS or PPS NAL unit is disallowed to be present in access units with TemporalId greater the TemporalId of the VPS, SPS or PPS NAL unit)

  - Signal clock tick value outside of HRD parameters (so that it will be available for use with indication of POC proportional to timing) and make the syntax structure for this the same in the VPS as in the SPS (nesting HRD parameters within the timing presence if statements, and adjusting semantics such that when POC is indicated to be proportional to timing in the VPS, this shall also be indicated in the SPS)

- Removed frame_packing_arrangement_type = 0, 1, 2 (interleaved types) and 7 ("tiled" frame packing) without renumbering the remaining one, according to the decision noted under JCTVC-L0444.

- Updated list of HEVC applications based on the HEVC requirements N11096.

- Moved output_flag_present_flag and num_extra_slice_header_bits up to immediately after dependent_slice_segments_enabled_flag in the PPS according to the decision noted under JCTVC-L0255.

- Integrated updated text of the SOP description SEI message (JCTVC-L0208).

- Requirement to set delta_poc_msb_present_flag[ i ] equal to 1 in a problematic corner case (option 2 of JCTVC-L0443 w/ later cleanup).

- Integrated changes for num_ref_idx_lX_(default_)active_minus1 and *_max_dec_pic_buffering and according to the decision noted under (JCTVC-L0323).

- Copied scene information SEI message syntax and semantics from AVC.

- Copied film grain characteristics SEI message syntax and semantics from AVC.

- Integrated changes to the scene information SEI message to limit its scope to be within a CVS (JCTVC-L0431).

- Integrated the following decisions noted under JCTVC-L0328 (w/ later cleanup, and further cleanup):

  - Restrict buffering period SEI messages to be sent only for pictures with temporal ID equal to 0 that are not RASL, RADL or sub-layer non-reference pictures.

  - Concatenation flag and associated CPB removal delay delta for bitstream splicing.

- Integrated the following decisions noted under JCTVC-L0044:

  - Addition of the cpb_delay_offset and dpb_delay_offset syntax elements in the buffering period SEI message, and their use in derivation of CPB removal times and DPB output times, respectively.

  - Modified CPB removal time calculation to remove dependence of AU removal time on DU CPB parameters while maintaining alignment of nominal CPB removal time of AU and last DU (w/ later cleanup).

- Integrated the following decisions noted under JCTVC-L0045:

  - When non-nested, the BP, PT, DU SEI message would apply to the whole bitstream. The version 1 decoder would ignore all NAL units with NR6 > 0. The encoder for version 1 would be required to set NR6 = 0. The sub-bitstream extraction process would remove SEI NAL units with NR6 = 0 that contain non-nested BP, PT, DU SEI messages when the actual max layer ID = 0 and the target temporal ID is less than the highest actual temporal ID in the bitstream.

  - Restrictions would apply on nesting of SEI messages (listed in contribution, as modified in discussion).

  - The APS SEI message, when present, is required to be in the first SEI NAL unit and to be alone in the NAL unit and cannot be nested.

  - Non-nested BP, PT, DU SEI cannot be in the same NAL unit as any other SEI message. Their order immediately follows APS SEI (if present) and precedes all others. Between these the order is required to be BP, then PT, then DU (when each is present).

  - Nested BP, PT, DU SEI must immediately follow the non-nested ones (when present) and shall not be in the same NAL unit as any SEI message other than these three.

  - An SEI message of a given type cannot be used as both a prefix and suffix SEI message in the same AU (w/ later cleanup).

  - Allow suffix SEI NAL units between VCL NAL units of an AU (in general).

  - If it's a prefix with whole-picture or higher scope, repetitions may be present between, but they must be repetitions. Similarly, if it's a suffix … it may be preceded by repetitions between VCL NAL units.

  - Improved semantics for recovery point SEI – adopted (needs some editorial refinement).

  - Improved semantics for region refresh information SEI – adopted (needs some editorial refinement).

  - Modified syntax and semantics of progressive refinement segment start SEI message (w/ later cleanup).

  - For all five SEI messages with this type of persistence, convert the "repetition period" into a persistence flag, such that 0 and 1 are supported and >1 is not.

  - Copied syntax and semantics of the post-filter hint SEI message from the AVC spec, and clarified the persistency scope of the post-filter hint SEI message to be just the picture that contains the SEI message.

  - Picture timing SEI, condition the presence of some syntax elements on CpbDpbDelaysPresentFlag.

- Integrated bounding redundant SEI messages (JCTVC-L0325).

- Integrated indication of parameter sets properties in active parameter sets SEI (JCTVC-L0047, w/ later cleanup).

- Fixed matrix_coefficients semantics.

- Removed frame_packing_arrangement_type = 6 according to the decision noted under JCTVC-L0454.

- Integrated the following decisions noted under JCTVC-L0046 (w/ later cleanup, and further cleanup):

  - Renamed progressive_source_idc to source_scan_type

  - In profile/tier/level syntax structure (general_ and sub_layer_ prefixed), specify progressive_source_flag and interlaced_source_flag (these two go first among the four), as follows:

    - both zero indicates that the source scan type is unknown or unspecified

    - both equal to 1 indicates that the source type is indicated at picture level in source_scan_type

  - These two bits are followed by the following two bits (general_ and sub_layer_ prefixed):

    - non_packed_constraint_flag equal to 1 specifies that FPA SEI, if present, has frame_packing_type equal to 6. non_packed_constraint_flag equal to 0 indicates that this constraint is not applied.

    - frame_only_constraint_flag equal to 1 specifies that field_seq_flag is equal to 0. frame_only_constraint_flag equal to 0 indicates that field_seq_flag may or may not be equal to 0.

- Integrated the following decisions noted under JCTVC-L0219 (w/ later cleanup): In bumping HRD operation, the bumping process should be invoked until there are no detected constraint violations rather than just until there is an empty frame buffer, and the checked conditions should checking immediately after decoding and should include checking both maximum picture reordering and maximum picture latency.

- Copied syntax and semantics of the filler payload SEI message, the user data registered by Rec. ITU-T T.35 SEI message, the user data unregistered SEI message, the full-frame snapshot SEI message, and the progressive refinement segment end SEI message from the AVC spec.

- Fixed the following:

  - Dropped the extension flag in the syntaxes of the post-filter hint SEI message and the display orientation SEI message, and fixed the syntax of the post-filter hint SEI message related to chroma_format_idc equal to 0.

  - Updated the list of normative references.

- Fixed editorial issues from NB comments JP15, JP27 and JP23 (#875).

- Fixed several issues (#797, #914, #971, #975, #978).

- Moved the bumping process one level up.

- Made the following changes for resolving some editor's notes:

  - Moved the specific processes for output order DPB operations one level down, and aligned the conditions for invocation of the "bumping" process.

  - Simplified the LTRP derivation process in subclause Error: Reference source not found and made another RPS related cleanup.

  - Made some NAL unit types related cleanups, including fixes of the definitions of coded slice segment NAL unit and RAP picture, not to set the value of the nal_unit_type syntax element when present but rather use a variable nal_unit_type and set the value of the variable, and the NAL unit types that are not covered in Error: Reference source not found. Later reverted the nal_unit_type variable introduction and applied an alternative approach to the "handle CRA as BLA" case (with later cleanups).

  - Made cpbBrVclFactor and cpbBrNalFactor global variables (i.e. renamed to CpbBrVclFactor and CpbBrNalFactor, respectively).

  - Cleaned-up the semantics of the syntax elements active_video_parameter_set_id, num_sps_ids_minus1, and active_seq_parameter_set_id[ i ] in the active parameter sets SEI message.

  - Removed and resolved some other editor's notes.

- Made the following cleanups:

  - Changed "to " (search "to^s") to "to ", "of " to "of ", and "or " to "or ", when non-separable spaces are not needed.

  - Removed all instance of "pseudocode", "pseudo code" and "pseudo-code".

  - Consistently use "set equal to" or "set to be" instead of "set to".

  - Editorially improved the definition of "associated VCL NAL unit".

  - Aligned the wording for specifying the presence of BP, PT and DUI SEI messages, and fixed the presence condition for DUI SEI message.

  - Some other editorial cleanups, including syntax and semantics of structure of picture description SEI message.

  - Cleaned-up semantics of reserved syntax elements or reserved values of syntax elements, partiularly regarding how decoders ignore the reserved syntax elements or the or reserved values of syntax elements.

  - Clarified restrictions for APS, BP, PT and DUI SEI messages that are between VCL NAL units in within an AU are specified, similarly as those for the same SEI messages that precede the first VCL NAL unit within an AU.

- Defined the abbrevation of CVS for coded video sequence and used it throughout the spec.

- Used the abbreviations of VPS, SPS and PPS throughout the spec.

- Changed the term random access point (RAP) to intra random access point (IRAP).

- Made the following cleanups:

  - In SEI message semantics, when a syntax element indicates a value of another syntax element of VPS, SPS, PPS or VCL NAL unit, use the wording "indicates and shall be equal to".

  - Replaced operation point set with layer set.

- Clarification of temporal sub-layer zero SEI message syntax and semantics.

- Removal of little-used RapPicFlag and IdrPicFlag variables.

- Variable naming convention cleanup for improved camel-case usage consistency and elimination of variable names that were substrings of other variable names.

- Made the following cleanups in Annex C:

  - Established AU-level CPB and DPB timing values when the HRD operates at sub-picure level, for the purpose of bitstream conformance checking

  - Changed variableNames( n ) to variableNames[ n ] for variables for the CPB and DPB timing values

  - Avoided directly using syntax element names as variables like as syntax_element_a[ n ]

  - For a particular variable, instead of using v( n ) for the value of the AU and v( m ) for the value of DU, use different variable names

- Updated figures.

- Improved definitions.

- Some editor's notes addressed and general cleanup.

- Fixed weighted prediction semantics (#982).

- Made the following cleanups:

  - On bitstream extraction: to align with the bitstream conformance tests as specified in Annex C, it is specified that each any sub-bitstream corresponds to a paricular value of TemporalId and a layer set specified in the active video parameter set shall be conforming to the requirements of bitstream conformance.

  - On syntax and semantics of deblocking filter controlling parameters in the PPS and the slice header.

  - Corrected the value ranges of vps_num_ticks_poc_diff_one_minus1 and vui_num_ticks_poc_diff_one_minus1 from 0..232 − 1 to 0..232 − 2.

  - Corrected the decoding order restriction on nested BP, PT, and DUI SEI messages.

  - Aligned the style for all syntax tables

- Fixed usage of sps_max_latency_increase (#999) including correcting zero-value handling for sps_max_latency_increase (including application of the "_plus1" coding convention as had been intended for handling the zero-valued case), usage of MaxLatencyPictures versus sps_max_latency_increase, and hierarchical relationship between VPS-level and SPS-level latency limits.

- Fixed the RPS related issues on an invalide case (#997) and on variable names (#1000).

- Made some general cleanups, including the issue reported in tickets #1001, #1003, #1004, #1005, and #1006.

- Made various cleanups, including the following:

  - The sample adaptive offset semantics

  - The coding unit semantics

  - Issues reported in tickets #1008, #1009, and #1010

- CABAC cleanup including:

  - Removed redundancy in initialization, binarization and ctx derivation tables.

  - Added all CABAC coded syntax elements in the ctx derivation table with indication for each bin whether it is ctx, bypass coded, using decode terminate or never occurs.

  - Renamed variables in order to match the naming convention and updated the figures accordingly.

  - Replaced truncated unary binarization by truncated rice (TR) with rice parameter equal to 0 and integrated unary binarization in the TR process because it is only used in there.

  - Added abbreviations for TR, FL, EG, LPS and MPS.

- Fixed several issues (#903, #980, #1001, #1002, #1016).

- Integrated the following editorial adoptions noted under JCTVC-L0363:

  - Decision (Ed.): Add a note that a single ID value is conceptually the same message regardless whether it is a prefix or suffix SEI NAL unit.

  - Decision (Ed.): Add a note to indicate that when slice_temporal_mvp_enable_flag is equal to 1 in an I slice, it has no impact on normative decoding process but merely expresses a constraint. Also include in the note that decoders may use the flag for emptying “motion vector storage” and for error resilience.

  - Decision (Ed.): The semantics of general_profile_idc should be modified to say something like this: "When the coded video sequence conforms to multiple profiles, general_profile_idc should indicate the profile that provides the preferred decoded result or the preferred bitstream identification, as determined by the encoder (in a manner not specified in this Specification)." It was noted that this seems to be essentially what Note 2 already says in 7.4.3.

  - Decision (Ed.): Add a note that filtering across slice boundaries can be enabled but filtering across tile boundaries can be disabled and vice versa (to point out that decoders should be prepared for such operation).

- Made various cleanup changes, including:

  - Fixed issues reported in tickets #1017, #1019, and #1020

  - Some cleanups on the deblocking process, the intra decoding process, the inter decoding process and the parsing process

  - Fixed the semantics of the structure of pictures information SEI message

  - Fixed some invalid cross-references to clause 9 (appeared as clause 0), and used the language “the decoding process specified in clauses 2 through 10” instead of “the decoding process specified in clauses 2 through 9”

  - Changed “except for” to “except” for consistency

- Made various cleanup changes, including:

  - Consistently applied the Note 1 style for all informative NOTEs in the spec

  - Changed all definitions to be one-sentence

  - Changed sub_pic_cpb_params_present_flag, SubPicCpbPreferredFlag, and SubPicCpbFlag to sub_pic_hrd_params_present_flag, SubPicHrdPreferredFlag, and SubPicHrdFlag, respectively

  - Removed empty lines after figure captions for consistency

- Made various cleanup changes, including:

  - Fixed issues reported in #1014, #1015, #1021, #1022, #1023, #1029, #1025, #1035, and #1036.

  - Addressed some editors notes.

- Made various cleanup changes, including:

  - Fixed some issues with tone mapping SEI message.

  - Fixed some broken links, phrasing inconsistencies, typos, etc.

- Fixed the following issues:

  - Context derivation for coeff_abs_level_greater1_flag (#904).

  - Scan order array initialization for minimum transform sizes greater than 8x8 (#1037).

- Made various cleanup changes, including:

  - Renaming the only syntax element that truncated the word "value" as "val".

  - Clarifying the description and specification of partitioning relationships.

  - Clarification of no_output_of_prior_pics_flag handling.

- Fixed the following issues:

  - CABAC init decoding engine after parsing pcm_flag equal to 1 (#1043).

  - Temporal merge candidate when current slice is P and collocated slice is B (#1044).

# Recommendations

The recommendations of the HEVC Draft and Test Model Editing AHG are to:

- Approve the edited JCTVC-L1002 and JCTVC-L1003 documents as JCT-VC outputs

- Encourage the use of the issue tracker to facilitate the reporting of issues with the text of either document

- Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text.

# References

- JCT-VC, “HM10: High Efficiency Video Coding (HEVC) Test Model 10 Encoder Description”, JCTVC-L1002, 12th JCT-VC meeting, Geneva, January 2013

- JCT-VC, “HEVC text specification Draft 10 / FDIS & Consent text”, JCTVC-L1003, 12th JCT-VC meeting, Geneva, January 2013