| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0005 |
| --- | --- |

| Title: | JCT-VC AHG report: HEVC range extensions development (AHG5) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
| Source: | AhG |  |  |

_____________________________

# Abstract

This report summarises the HEVC range extensions development Ad hoc (AHG5) between the 12th JCT-VC meeting in Geneva, CH (January 2013) and the 13th JCT-VC meeting in Incheon, KR (April 2013).

# Mandate

The mandate of the HEVC range extensions development Ah hoc (AHG5) are as follows:

- Study aspects of the technical design and develop software relating to the support of non-4:2:0 chroma formats and bit depths beyond 8 bits.

- Discuss and propose test conditions and test material for the development of the range extensions.

- Study techniques for colour conversion and resampling and their relationship to non-4:2:0 chroma coding.

# Activity and list of related input contributions

## Reflector

The e-mail reflector is [jct-vc@lists.rwth-aachen.de].  To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG5] in the subject line for easy grouping.

After kick-off, there were no emails on the reflector regarding AHG5.

## Test conditions

Contribution JCTVC-L1006 was uploaded, defining the test conditions as agreed at the 12th JCT-VC meeting for Range extensions development.

# Related contributions

- JCTVC-M0072 AHG7: Modification of in-loop colour-space transformation [S. Matsuo, M. Matsumura, S. Takamura, H. Fujii, A. Shimizu (NTT)]

  - Cross-check: JCTVC-M0407

- JCTVC-M0080 AHG7: Post filtering for colour-space transformation [M. Matsumura, S. Matsuo, S. Takamura, H. Fujii, A. Shimizu (NTT)]

  - Cross-check: JCTVC-M0408

- JCTVC-M0098 AHG5: Independent chroma transform depth from luma transform depth for non 4:2:0 format [J. Kim (LGE)]

  - Cross-check: JCTVC-M0369 [miss]

- JCTVC-M0099 AHG5: using zero-depth chroma transform unit for 4:2:2 format [J. Kim (LGE)]

  - Cross-check: JCTVC-M0386

- JCTVC-M0107 AHG5: Chroma intra prediction for extended color formats [J. Min, S. Jeong, S. Lee, C. Kim (Samsung)]

  - Cross-check: JCTVC-M0379

- JCTVC-M0111 AHG5: On chroma QP for HEVC RExt [E. François, C. Gisquet, G. Laroche, P. Onno (Canon)]

  - Cross-check: JCTVC-M0420

- JCTVC-M0127 AHG5: Unification of chroma intra prediction angles between 4:2:2 chroma format [H. Nakamura (JVC Kenwood)]

  - Cross-check: JCTVC-M0372, JCTVC-M0383

- JCTVC-M0136 AHG5: 32x32 transform in chroma for 4:2:2 [V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)]

  - Cross-check: JCTVC-M0378

- JCTVC-M0137 AHG5: Square transform deblocking for 4:2:2 [C. Rosewarne, M. Maeda (Canon)]

  - Cross-check: JCTVC-M0440

- JCTVC-M0138 AHG5: On chroma intra prediction for 4:2:2 [C. Rosewarne, M. Maeda (Canon)]

  - Cross-check: JCTVC-M0424

- JCTVC-M0156 AHG5: Simplification of chroma intra prediction process for YUV4:2:2 coding [A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

  - Cross-check: JCTVC-M0409

- JCTVC-M0178 AHG5: Range Extensions and High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

  - Cross-check: [miss]

- JCTVC-M0229 AHG5: Backwards compatible enhancement of chroma format [K. Ugur, D. Bugdayci, M. M. Hannuksela (Nokia)]

  - Cross-check: [miss]

- JCTVC-M0230 Non-RCE1/Non-RCE2/AHG5/AHG8: Adaptive Inter-Plane Prediction for RGB Content [T. Nguyen, A. Khairat, D. Marpe (Fraunhofer HHI]

  - Cross-check: JCTVC-M0418 [miss], JCTVC-M0435 [miss]

- JCTVC-M0232 AHG5:Intra Prediction Mode-Dependent Coefficient Scanning for 4:2:2 Chroma Extended Format [Z. Chen, S. Liu, S. Lei (MediaTek)]

  - Cross-check: JCTVC-M0368

- JCTVC-M0255 Decoding a 10-bit sequence using an 8-bit decoder [D. Flynn, G. Martin-Cocher, D. He (RIM)]

  - Cross-check: JCTVC-M0443 [miss]

- JCTVC-M0298 Bandwidth reduction for range extension [V. Seregin, X. Wang, J. Sole, M. Karczewicz (Qualcomm)]

  - Cross-check: [miss]

- JCTVC-M0335 AhG5: Offset Scaling in SAO for High Bit-depth Video Coding [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

  - Cross-check: JCTVC-M0405

Other related contributions:

- JCTVC-M0116 Non-RCE1: Multiple LM chroma modes [C.-Y. Chen, C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

  - Cross-check: JCTVC-M0377

- JCTVC-M0304 Non-RCE1/Non-RCE2: Simplified level coding for Transform skip and lossless coding [S.H. Kim, A. Segall (Sharp)]

# Proposals

## Residual quad-tree

JCTVC-M0098 AHG5: Independent chroma transform depth from luma transform depth for non 4:2:0 format [J. Kim (LGE)]

In this contribution, chroma transform depth could be decided independently from luma transform depth for a CU. To indicate chroma transform depth, transform split flags for chroma components are signalled. It shows 0.0%, -0.6%, -0.7% in the case of AIMT,  0.0%, -0.6%, -0.5%	in the case of AIHT and -0.1%, -0.5%, -0.4% in the case of AISH. This contribution also limits the depth of chroma transform depending on luma transform depth. The depth of chroma transform should be less than or equal to luma transform depth. It means that the size of chroma transform should be larger than or equal to luma transform. The limitation brings -0.1%, -0.4%, -0.6% for AIMT, -0.1%, -0.4%, -0.4% for AIHT, -0.1%, -0.3%, -0.3% for AIST in average.

JCTVC-M0099 AHG5: using zero-depth chroma transform unit for 4:2:2 format [J. Kim (LGE)]

In this contribution, chroma transform depth could be decided as zero or same as luma transform depth by RD costs. To indicate the chroma transform depth, one flag is signalled when it is required. It prevents overly small transform size for chroma components. It shows gains of 0.0%, -1.3%, -1.6% for AIMT, 0.0%, -0.8%, -0.9% for AIHT, and 0.0%, -0.6%, -0.5% for AIST in average.

JCTVC-M0136 AHG5: 32x32 transform in chroma for 4:2:2 [V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)]

This contribution presents an approach to the residual quad-tree for the 4:2:2 chroma format that allows a 32x32 transform to be used into the chroma channel.  At present, the 32x32 transform is used in chroma channels for the 4:4:4 chroma format.  It is asserted that reusing the transform logic required for the 4:4:4 chroma format in the 4:2:2 chroma format is beneficial.  Simulation results show a gains of 0.0%, -0.3% and 0.3% in luma, chroma for AI Main tier in 4:2:2, 0.0%, -0.4%, -0.4% in luma and chroma for RA Main and 0.0%, -0.7%, -0.9% in luma and chroma for LD Main.

## Chroma intra-prediction

JCTVC-M0107 AHG5: Chroma intra prediction for extended color formats [J. Min, S. Jeong, S. Lee, C. Kim (Samsung)]

In extended chroma formats, effect of prediction accuracy of chroma components is large for both coding efficiency and subjective visual quality. In this proposal, new designs of chroma intra prediction optimized for extended color formats are proposed. In the common software for range extension support, only five intra prediction modes are used for chroma intra prediction. In the proposed methods, 7 and 11 modes were tested with appropriate changes in mode coding methods. With 7 intra prediction modes, 0.0%,-0.1%,-0.2% gains were obtained where gains of 0.0%,-0.4%,-0.6% were observed with 11 modes for each color component. Additional tests were performed with chroma reference sample filtering method[2]. The proposed methods with combination of chroma reference sample filtering approach provide higher gains which are -0.1%, -0.4%,-0.5% and -0.1%, -0.7%, -0.9% for 7 and 11 prediction modes.

JCTVC-M0127 AHG5: Unification of chroma intra prediction angles between 4:2:2 chroma and the others” [H. Nakamura (JVC Kenwood)]

In the current draft of HEVC Range Extensions and the test model HM-10.0-RExt-2.0, some additional intra prediction angles for 4:2:2 chroma are calculated from intra prediction angle for luma and 4:2:0/4:4:4 chroma.

This contribution proposes the unification of chroma intra prediction angles between 4:2:2 chroma and the others without adding any intra prediction angles for 4:2:2 chroma.

JCTVC-M0138 AHG5: On chroma intra prediction for 4:2:2 [C. Rosewarne, M. Maeda (Canon)]

Chroma intra-prediction in the 4:2:2 chroma format is performed on rectangular blocks and thus an adjustment to the intra-prediction angle is used so that the resulting angle corresponds to the angle used for luma blocks.  The adjustment results in new intra-prediction angles being used (effectively new modes), that are not used in luma or in non-4:2:2 chroma formats.  This contribution proposes to alter the mapping between intra-prediction modes and intra-prediction angles so that no new modes are introduced.

JCTVC-M0156 AHG5: Simplification of chroma intra prediction process for YUV4:2:2 coding [A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

In this contribution, a simplified scheme of YUV4:2:2 intra chroma prediction is proposed. The proposed scheme simplifies the current WD process for YUV4:2:2 chroma intra prediction by introducing a mode index conversion table for intra DM mode. The coding performance of the proposed scheme is comparable to that of the current HEVC Range Extension anchor.

JCTVC-M0229 AHG5: Backwards compatible enhancement of chroma format [K. Ugur, D. Bugdayci, M. M. Hannuksela (Nokia)]

This contribution proposes a backwards compatible chroma format enhancement method, where base layer codes the 4:2:0 with HEVC version 1 and enhancement layer codes 4:4:4 U,V colour planes separately, using the functionality indicated with separate_colour_plane_flag in HEVC. The enhancement layer colour components can optionally be predicted from the base layer components for improved coding efficiency. It is argued that the proposed method includes minimal changes to HEVC as it reuses the existing mechanisms already present in HEVC version 1. Several benefits of this approach over using SEI messages are discussed in the contribution.

## In-loop colour transformation

JCTVC-M0072 AHG7: Modification of in-loop colour-space transformation [S. Matsuo, M. Matsumura, S. Takamura, H. Fujii, A. Shimizu (NTT)]

This document introduces a modification of in-loop colour-space transformation of residual signals. To reduce the redundancy among the Y-Cb-Cr/R-G-B components, a colour-space transformation method was proposed in the contribution of JCTVC-L0371. In this contribution, to prevent the deterioration of the coding performance of L0371 and to speed up the run-times, the transformation is adaptively applied to residual signals using an eigenvalue derived by the calculation process of transformation matrices. The overall average coding gains against the anchor (HM10.0 RExt2.0) were 9.8% / 8.1% / 11.5% (AI High-tier), 11.2% / 7.5% / 12.7% (RA High-tier), and 11.1% / 5.8% / 12.7% (LD High-tier). The average run-times of encoding and decoding were 103.5% and 106.5%, respectively.

JCTVC-M0080 AHG7: Post filtering for colour-space transformation [M. Matsumura, S. Matsuo, S. Takamura, H. Fujii, A. Shimizu (NTT)]

To reduce the redundancy among Y-Cb-Cr/R-G-B components, an in-loop color-space transformation method was proposed in JCTVC-L0371. In this contribution, a post filter which replaces the in-loop colour-space transformation with post processing is proposed. The overall average coding gains based on GBR BD-rate compared to the anchor (HM10.0 RExt. 2.0) of  23.8% / 13.5% / 7.5% (AI main-tier / high-tier / super-high-tier), 33.2% / 12.7% (RA main-tier / high-tier), and 33.4% / 18.5% (LD-B main-tier / high-tier) were confirmed. The average run-times of encoding and decoding (including pre/post filtering) were 86.0% and 90.3%, respectively.

JCTVC-M0230 Non-RCE1/Non-RCE2/AHG5/AHG8: Adaptive Inter-Plane Prediction for RGB Content [T. Nguyen, A. Khairat, D. Marpe (Fraunhofer HHI]

This document describes a low-cost adaptive inter-plane prediction scheme suitable for RGB materials, especially for screen content. The proposed technique operates in a closed in-loop on the prediction residuals. In order to achieve the low-cost property, the proposed scheme involves a class of predictors consist of one addition and one shift operation. Experimental results presented at the end of this document show the advance of the proposed scheme relative to other fixed color transformation schemes, especially when considering screen content and higher bit rates operation points.

## Uncategorised

JCTVC-M0111 AHG5: On chroma QP for HEVC RExt [E. François, C. Gisquet, G. Laroche, P. Onno (Canon)]

This contribution proposes a slight modification related to the chroma QP in the Range Extension of HEVC (HEVC RExt). In the current HEVC RExt Draft, one chroma QP table, linking the chroma QP (QPC) to the luma QP (QPY), is specified for each one of the three chroma formats 4:2:0, 4:2:2 and 4:4:4. In this contribution, two changes are proposed. The first change consists in removing one of the three tables, namely the 4:2:2 table. This change reportedly simplifies the design, since only the 4:2:0 table, already present in the HEVC V1 specification, and the 4:4:4 table, which is actually based on a straightforward QPC derivation, are kept. The second change consists in adding one syntax element, preferably specified in the SPS, that indicates which table must be used in case of 4:2:2 or 4:4:4 content.

The coding efficiency impact of removing the 4:2:2 table is reportedly small, mainly resulting in a slightly different balance between the luma and chroma quality. The proposed added syntax element is reported to provide a simple mechanism to easily and flexibly control this repartition.

JCTVC-M0137 AHG5: Square transform deblocking for 4:2:2 [C. Rosewarne, M. Maeda (Canon)]

At the 12th JCT-VC meeting, square transforms were adopted for the rectangular blocks present in the chroma channels when the 4:2:2 chroma format is used.  This adoption results in boundaries between the pairs of square transforms that are not deblocked.  We assert that having transform boundaries in the design that are not deblocked is inconsistent with the 4:2:0 design and hence could lead to deblocking artefacts.  This contribution introduces deblocking to the boundaries between the square transforms in chroma when the 4:2:2 chroma format is in use.  The simulation results show 0.0%, -0.2% and -0.2% gains in luma and chroma for AI Main tier, 0.0%, -0.1% and -0.2% gains in luma and chroma for RA Main tier and 0.0%, -0.1% and -0.1% gains in luma and chroma for LB Main tier.

JCTVC-M0178 AHG5: Range Extensions and High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

The document presents the current capabilities of HEVC Range Extensions when operating at low (including negative) QPs, and proposes that such low QPs will be needed if the codec is to truly support high bit depths, as required by the mandate of the Range Extensions amendment. Possible sources of errors that may be caused by internal accuracy limitations currently present in HEVC are explored. It is claimed that some changes to those accuracies can mitigate the errors and thereby extend the operating range of HEVC. In addition two independent changes to the entropy coding are presented, which are stated to benefit both efficiency and practicality. These changes have been included in this proposal rather than in two separate proposals due to their applicability to high operating points.

JCTVC-M0232 AHG5:Intra Prediction Mode-Dependent Coefficient Scanning for 4:2:2 Chroma Extended Format [Z. Chen, S. Liu, S. Lei (MediaTek)]

HEVC adopts intra prediction mode-dependent coefficient scanning (MDCS) for 4x4 and 8x8 transform block, in which the scan order of transform coefficients is determined by the intra prediction mode. For intra prediction angular mode 6-14, vertical scanning is applied and for intra prediction angular mode 22-30, horizontal scanning is applied. For the rest modes, diagonal scanning is applied.

For HEVC range extension, different color formats such as YCbCr 4:2:2 are supported, in addition to YCbCr4:2:0. For YCbCr4:2:2, when a luma block size 2Nx2N (width x height), the chroma block size is Nx2N (width x height). To support square transform, an Nx2N chroma block is split into 2 NxN blocks for square transform [1]. As derived mode (DM), in which the prediction is performed by using the corresponding luma PU mode, is used for chroma intra prediction, the angle of prediction in chroma DM mode is modified accordingly.

JCTVC-M0255 Decoding a 10-bit sequence using an 8-bit decoder [D. Flynn, G. Martin-Cocher, D. He (RIM)]

HEVC, like other well-known codecs, contains profiles that accommodate the decoding of non-8-bit video sequences. In particular, HEVC contains a 10-bit profile that permits the carriage of 10-bit video sequences. However, for some implementations, the extra resources required for conformance to a higher bit-depth operating point can be prohibitive, yet there exist circumstances where a decoder may wish to offer best-effort decoding of a bitstream. Two techniques are presented that facilitate the decoding of 10-bit video bitstreams using an 8-bit decoder. The first uses the normal 8-bit reconstruction process, while the second involves addition of a rounding process during the reconstruction process. PSNR losses using low-delay main-10 bitstreams average at a 6 dB and 2.5 dB respectively. Further results are presented and  specification text is provided for a possible annex.

JCTVC-M0298 Bandwidth reduction for range extension [V. Seregin, X. Wang, J. Sole, M. Karczewicz (Qualcomm)]

HEVC Range Extensions 4:4:4 potential applications include consumer applications, so the complexity increase due to the 4:4:4 processing has to be taken into account. The scope of this contribution is the bandwidth increase for 4:4:4 motion estimation and compensation, which is around 37% more than the 4:2:0 case for 4×2 memory block. Specifically, in this contribution, restriction of bi-directional prediction for 8×8 chroma components is studied, while luma component still can have bi-directional prediction. This modification increases bandwidth by 9%, as opposed to 37%. The impact on coding performance is about 0.2% and 0.4% luma BD rate loss for Main-tier RA and LB, and High-tier RA and LB respectively in for both YUV422 and YUV444.

JCTVC-M0335 AhG5: Offset Scaling in SAO for High Bit-depth Video Coding [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

In the last meeting deficiency of SAO was reported when bit-depth is larger than 10. In this contribution offset scaling methods in SAO are proposed when bit-depth is larger than 10 to improve coding efficiency. In the first method, different scale factor is used for luma and chroma offsets with linear scaling according to bit-depth. In case of luma offset is left-shifted by (bitDepth – 11) and in case of chroma offset is left shifted by (bitDepth – 10). In the second method, it is proposed to apply either linear or non-linear offset scaling for luma and chroma separately with independent scale factors. These parameters can be signaled in the slice header so that offset scaling can be adaptive to input sequence characteristics. In both methods, no change is applied for entropy coding/decoding part.

Experimental results reportedly show coding efficiency improvements by x.x%/x.x%/x.x% for Y/Cb/Cr in case of All-Intra-HE-Main-Tier with internal bit-depth increase by 2 (IBDI 2), and x.x%/x.x%/x.x% with internal bit-depth increase by 4 (IBDI 4).

## Other related contributions

JCTVC-M0116 Non-RCE1: Multiple LM chroma modes [C.-Y. Chen, C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

In this contribution, multiple LM chroma modes are implemented on top of JCTVC-M0097, where the prior LM chroma mode is extended for non-420 video formats. Three LM chroma modes are proposed. One is the prior LM chroma mode using above and left neighboring reconstructed samples to derive parameters of the LM linear model, and the other two modes only use left or above neighboring reconstructed samples. Simulation result shows that the average BD-rates of the proposed method for the three color components are -8.5%, -11.6%, and -11.7% for AI-Main-tier, -6.9%, -8.3%, and -9.4% for AI-High-tier, and -5.4%, -5.9%, and -6.8% for AI-SuperHigh-tier. For intra test cases, the encoding time is increased by 21%, and the decoding time is roughly unchanged.

JCTVC-M0304 Non-RCE1/Non-RCE2: Simplified level coding for Transform skip and lossless coding [S.H. Kim, A. Segall (Sharp)]

In this document, a simplified level coding for a transform skip block is presented. Except for removing greather_than_1 coding and greather_than_2 coding, there is no change to the level coding in the proposed simplified level coding. The simplified level coding is employed/activated when transform skip flag is equal to 1. Simulation result shows that the average BD-rates of the proposed method for the three color components are 0.0%, 0.0%, and 0.0% for AI-Main-tier, 0.0%, 0.0%, and -0.1% for RA-Main-tier, and 0.0%, 0.1%, and 0.0% for LD-Main-tier.

The simplified level coding method is always activated in lossless coding because transform and quantization process is not used. Hence, worst case context coded bins per pixel has been reduced from 1.56 bins/pixel (which has been mentioned/discussed in JCTVC-H0728) to 1 bin/pixel. It is reported that the overall coding gain of the simplified level coding is 0.8% for AI, 1.8% for RA, and 1.8% for LD, respectively. To improve lossless coding performance, remaining coefficient coding process has been additionally modified on top of the simplified level coding by signaling rice-parameter for each sub-block level and removing the rice-parameter update process. When it comes to computational complexity reduction, worst case context coded bins per pixel has been reduced from 1.56 bins/pixel to 1.06 bin/pixel because 1 context coded bin is added for rice parameter coding. With the reduction of the worst cased context coded bin, simulation result shows that the average BD-rates of the proposed method are -3.5% for Al, -1.7% for RA, and -1.3% for LD.

# Recommendations

The AHG recommends:

- Review all related contributions.

- Include appropriate methods into HEVC range extensions WD and HM range extensions software.