| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0017 |
| --- | --- |

| Title: | JCT-VC AHG report: SHVC complexity assessment (AHG17) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Budagavi, E. Alshina, E. Francois, M. Karczewicz, A. Tabatabai, Y. Ye | Email: | madhukar@ti.com, elena_a.alshina@samsung.com, edouard.francois@crf.canon.fr, martak@qti.qualcomm.com, Ali.Tabatabai@am.sony.com, Yan.Ye@interdigital.com |
| Source: | AHG17 |  |  |

_____________________________

# Abstract

This document reports on the work of the JCT-VC AHG on SHVC complexity assessment (AHG17) between the 12th JCT-VC meeting in Geneva, Switzerland (14–23 Jan. 2013) and 13th JCT-VC meeting in Incheon, KR, (18–26 Apr. 2013), and lists the related input documents.

# Mandate

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| SHVC complexity assessment (AHG17) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth, memory usage and computational complexity of scalable tools and methodologies to evaluate them - Study restrictions to reduce memory bandwidth and complexity in scalable tools (e.g. restrictions on PU size, subpel accuracy, 1D vs. 2D interpolation filters, use of bi-predictive PUs, reduction in the number of EL reference pictures etc.) - Evaluate the impact of such restrictions on coding efficiency. | M. Budagavi (chair), E. Alshina, E. Francois, M. Karczewicz, A. Tabatabai, Y. Ye. (vice-chairs) | N |

# Summary of activities related to mandates

With regards to the first mandate, a complexity assessment module and spreadsheet was developed for use in SCE3 and SCE4 activity. Reference [1] provides the relevant patch and spreadsheet and also a brief description. The default complexity assessment template has been recently modified to correctly handle GRP.

With regards to second and third mandate, there are several non-SCE contributions that address complexity reduction and coding efficiency impact of the simplifications.

There were four email exchanges on JCTVC related to this AHG activity. These mainly dealt with issues that were discussed during the development of complexity assessment module (Please see next section).

# Open issues related to complexity assessment

During the development of the complexity assessment module, some issues were extensively discussed. These are highlighted below.

- Should up-sampling and MC operations/memory bandwidth be reported separately or should they be added and reported as a single number?

- What would be the scaler hardware architecture for SNR scalability in the current SHVC test model? (1) Would the scaler block be reused assuming scaling factor of 1 (i.e. integer pel positions in SNR case would be filtered with [ ... 0 0 1 0 0 ...] filter) or (2) Would BL picture pointer is used?

- What is the implication of JCTVC-L0178 cropping and padding adoption at last meeting on memory bandwidth for SNR scalability for RefIdx picture based processing? Can BL picture pointer be directly reused by EL decoder or will picture data copy of BL picture be needed to create reference picture for EL?

- Memory bandwidth for RefIdx with picture based processing is computed in the complexity assessment module assuming LCU by LCU up-sampling with LCU size = 64. What would be the optimal block size to use? Would it need to be matched with min LCU size? Currently, average complexity for RefIdx picture based approach is calculated using 64x64 block, whereas the worst case is calculated using 16x16 block.

- What scaling ratio do we consider for the worst spatial scalability case? Currently only ratios 2x and 1.5x are supported in SHM-1.0. But in the template, ratio 1x is used to compute the worst case numbers of spatial scalability (based on the hypothesis that in the future, SHVC will support any spatial ratio).

- Do we consider BL reference pictures as additional pictures to be stored in the EL DPB?

With regards to Issue 2, the following opinions were expressed by experts on the JCTVC email reflector:

- A scalar architecture would have input and output buffer plus interpolation logic. If EL and BL decoding is synchronized at frame-level in which BL reconstructed frame is normally in off-chip memory,  then the DMA could directly write the BL into the output buffer of the scaler in the SNR case. If such a capability does not exist for certain architecture, then a bypass mode should be added to the scaler logic so that the BL data (which is DMAed in to the scaler input buffer) can be copied from the input buffer to the output buffer of the scaler. If EL and BL decoding is synchronized at group of blocks level to reduce the bandwidth and latency for the EL decoding, the BL decoder should have capability to put the data directly in the input buffer of the scaler (avoiding going through off-chip memory). In the SNR case, the BL decoder could also write data directly into the output buffer of the scaler to bypass the scaler, or if such a capability does not exist, then a bypass mode should be added to the scaler logic to enable data copy for the input to the output buffer of the scaler. There is no memory bandwidth difference between the two options and whether a bypass mode is needed in the scaler purely depends on architecture choice.

- For SNR scalability, in EL one would not actually filter BL picture using a [0 0 1 0 0 ] filter.

- Frame buffer copy from BL to EL decoder would not be required. BL picture pointer can be passed instead.

# List of related input documents

The following documents are related to AHG17.

- JCTVC-M0086: AHG-17: complexity and performance analysis of SHM1.0 compare to HM8.1 simulcast.

- JCTVC-M0088: AHG-13,17: complexity and performance analysis of different length up-sampling filters in SHM1.0.

- JCTVC-M0182: AHG17: complexity analysis of SHM1.0.

- There are several non-SCE related documents that are related to complexity reduction and are not listed here.

- Documents JCTVC-M0180 and JCTVC-M0274 address Issue 3 of Section 3.

# Recommendations

The SHVC complexity assessment AHG recommends the JCT-VC to:

- Discuss open issues listed in section 3.

- Review all input documents listed in Section 4.

# References

- JCTVC-L0440, E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC.