| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0022 |
| --- | --- |

| Title: | SCE2: Summary Report of SHVC Core Experiment on Inter-layer Texture Prediction Signaling and Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core experiment report |  |  |
| Author(s) or Contact(s): | Liwei Guo Yong He Do-Kyoung Kwon Jinwen Zan Jung Won Kang | Email: | liweig@qti.qualcomm.com Yong.He@InterDigital.com d-kwon@ti.com JINWEN.ZAN@huawei.com jungwon@etri.re.kr |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The purpose of SHVC Core Experiment 2 (SCE 2) is to evaluate inter-layer texture prediction signaling and deblocking. This document provides a summary of activities and results for this core experiment.

# Introduction

SHVC Experiment 2 (SCE 2) includes two subtests which are related to inter-layer texture prediction. In subtest1, methods related to inter-layer texture prediction flag signaling were investigated. In subtest 2, a method for BS setting of inter-layer texture prediction was evaluated. This report provides a summary of activities and results for this core experiment.

# Participants

| No. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Liwei Guo Vadim Seregin Jianle Chen Geert Van Der Auwera Krishna Rapaka Marta Karczewicz | Qualcomm | liweig@qti.qualcomm.com vseregin@qti.qualcomm.com cjianle@qti.qualcomm.com geertv@qti.qualcomm.com krapaka@qti.qualcomm.com martak@qti.qualcomm.com | Proponent |
| 2 | Yong He Yan Ye Xiaoyu Xiu | InterDigital | Yong.He@InterDigital.com Yan.Ye@InterDigital.com Xiaoyu.Xiu@InterDigital.com |  |
| 3 | Do-Kyoung Kwon Madhukar Budagavi | Texas Instruments | d-kwon@ti.com madhukar@ti.com | Proponent |
| 4 | Jinwen Zan Xianghui Wei Haoping Yu Haitao Yang Dong Jiang Wen Gao Jiang Dong | Huawei | JINWEN.ZAN@huawei.com weixianghui@huawei.com haoping.yu@huawei.com haitao.yang@huawei.com dong.jiang@huawei.com wen.gao@huawei.com dong.jiang@huawei.com |  |
| 5 | Haricharan Lakshman | HHI | haricharan.lakshman@hhi.fraunhofer.de |  |
| 6 | Jung Won Kang Jinho Lee Hahyun Lee | ETRI | jungwon@etri.re.kr jinosoul@etri.re.kr hanilee@etri.re.kr |  |
| 7 | Kawamura Kei | KDDI | kei@kddilabs.jp |  |
| 8 | Tmoyuki Yamamoto Takeshi Tsukuba Andrew Segall Kiran Misra Jane Zhao Ikai Tohomiro | Sharp | yamamoto.tomoyuki@sharp.co.jp tsukuba.takeshi@sharp.co.jp asegall@sharplabs.com misrak@sharplabs.com jzhao@sharplabs.com ikai.tomohiro@sharp.co.jp |  |
| 9 | Wenhao Zhang | Intel | wenhao.zhang@intel.com |  |
| 10 | Edouard Francois Fabrice Le Léannec Patrice Onno | Canon | Edouard.francois@crf.canon.fr fabrice.leleannec@gmail.com Patrice.onno@crf.canon.fr |  |
| 11 | Tzu-Der Chuang Yu-Wen Huang Shan Liu Polin Lai Shih-Ta Hsiang | MediaTek | Peter.chuang@mediatek.com yuwen.huang@mediatek.com shan.liu@mediatek.com polin.lai@mediatek.com shih-ta.hsiang@mediatek.com |  |
| 12 | Chulkeun Kim Joonyoung Park Jungsun Kim | LG | chulkeun.kim@lge.com jy.park@lge.com jungsun.kim@lge.com | Proponent |
| 13 | D. Bugdayci K. Ugur J. Lainema | Nokia | ext-done.bugdayci@nokia.com Kemal.Ugur@nokia.com Jani.Lainema@nokia.com |  |
| 14 | Kazushi Sato J. Xu S. Lu Cheung Auyeung Suzuki Teruhiko | Sony | Kazushi.Sato@jp.sony.com jun.xu@am.sony.com Shuo.Lu@jp.sony.com Cheung.Auyeung@am.sony.com teruhikos@jp.sony.com |  |
| 15 | Ankur Saxena Zhan Ma Tammy Lee Elena Alshina | Samsung | asaxena@sta.samsung.com zhan.ma@sta.samsung.com tammy.lee@samsung.com elena_a.alshina@samsung.com | CrossChecker |
| 16 | Y. H. Tan C. H. Yeo | I2R | yhtan@i2r.a-star.edu.sg chyeo@i2r.a-star.edu.sg |  |
| 17 | Andrey Norkin | Ericsson | andrey.norkin@ericsson.com | CrossChecker |
| 18 | Woo-Jin Han | Gachon University | hurumi@gmail.com |  |

# Summary of Proposals and Results

3.1 Subset 1 Signaling for inter-layer texture prediction

Table 1 Documents list for Subtest 1

| Test | Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- | --- |
| 1.1 (TI) | JCTVC-M0216 | SCE2: Results of test 1.1 on skipped inter-layer texture prediction (ILTP) block signaling in Inter slice | Samsung (JCTVC-M0341) |
| 1.2 (TI) | JCTVC-M0217 | SCE2: Results of test 1.2 and test 1.3 on inter-layer texture prediction | Samsung (JCTVC-M0054) |
| 1.3 (TI) | JCTVC-M0217 | SCE2: Results of test 1.2 and test 1.3 on inter-layer texture prediction | Samsung (JCTVC-M0054) |

Table 2 Summary of technical details for tools in Subset 1

| Test | Changes in Intra-BL flag coding |
| --- | --- |
| 1.1 (TI) | - In P and B slice, cu_skip_flag =1 with IntraBL = 1is used to indicate zero residue for IntraBL |
| 1.2 (TI) | - PU-level signaling - Conditionally adding Intra-BL prediction as a candidate in the merge list for non-2Nx2N PU for 2Nx2Nlly enabled as a candidate in merge list IntraBL |
| 1.3 (TI) | - Combination of 1.1 and 1.2 |

Table 2 Summary of results for tools in Subtest 1 (Anchor SHM1.0 IntraBL)

|  | Avg. LD-P/RA 2x |  |  | Avg. LD-P/RA 1.5x |  |  | Avg. LD-P/RA SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V | Y | U | V |
| 1.1 | -0.1% | 0.1% | 0.1% | -0.2% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| 1.2 | -0.3% | 0.0% | 0.1% | -0.3% | -0.2% | 0.0% | -0.2% | 0. 1% | 0.3% |
| 1.3 | -0.4% | 0.1% | 0.2% | -0.4% | 0.0% | 0.1% | -0.3% | 0.1% | 0.3% |

Optional LD-B test

|  | LD-B 2X |  |  | LD-B 1.5x |  |  | LD-B SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V | Y | U | V |
| 1.1 | -0.1% | 0.1% | 0.1% | -0.3% | 0.3% | 0.3% | -0.1% | 0.0% | 0.0% |
| 1.2 | -0.3% | -0.1% | -0.1% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | 0.1% |
| 1.3 | -0.4% | 0.0% | 0.0% | -0.6% | -0.2% | 0.0% | -0.3% | -0.2% | 0.1% |

3.2 Subset 2 Deblocking for inter-layer texture prediction

Table 4 Documents list for Subtest 2

| Test | Proposal No. | Proposal Title | Cross-checker |
| --- | --- | --- | --- |
| 2.1 (LG && Qualcomm) | JCTVC-M0093 | SCE2.2: Deblocking boundary strength modification for Intra BL | Ericsson (JCTVC-M0174) |
| 2.2 (Qualcomm) | Merged with 2.1 |  |  |

Table 5 Summary of technical details for tools in Subtest 2

| Test |  |
| --- | --- |
| 2.1 (LG && Qualcomm) | - BS =1 for Intra-BL CU - Deblocking chroma for both Intra and Intra-BL |

Table 6 Summary of technical details for tools in Subtest 2

|  | AI 2X |  |  | AI 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V |  |  |  |
| 2.1 | -0.1% | -0.2% | -0.2% | -0.1% | -0.3% | -0.1% |  |  |  |
|  | Avg. LD-P/RA 2x |  |  | Avg. LD-P/RA 1.5x |  |  | Avg. LD-P/RA SNR |  |  |
| Test | Y | U | V | Y | U | V | Y | U | V |
| 2.1 | 0.0% | -0.2% | -0.2% | -0.1% | -0.3% | -0.4% | 0.0% | -0.4% | -0.4% |

# Conclusions

In this CE, a number of tools related to inter-layer texture prediction signaling and deblocking were studied. This report summarizes the technical features and the test results of these tools.