| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0024 |
| --- | --- |

| Title: | SCE4: Summary Report of SHVC Core Experiment on inter-layer filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Summary |  |  |
| Author(s) or Contact(s): | Jianle Chen (primary) Andrew Segall Elena Alshina Shan Liu Jie Dong Joonyoung Park | Email: | cjianle@qti.qualcomm.com asegall@sharplabs.com elena_a.alshina@samsung.com shan.liu@mediatek.com jie.dong@interdigital.com jy.park@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCE4 on inter-layer filtering. Eight tools have been evaluated on the test conditions defined in document JCTVC-L1104.

# Introduction

During the 12th JCT-VC meeting in Geneva, multiple proposals for SHVC inter-layer filters design were proposed. It was then decided to coordinate for further study and testing of these filters in this Core Experiment (CE).

# Participants List

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Krishna Rapaka Liwei Guo Wei Pu Vadim Seregin Jianle Chen Marta Karczewicz Xiang Li | Qualcomm | krapaka@qti.qualcomm.com liweig@qti.qualcomm.com wpu@qti.qualcomm.com v.seregin@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com lxiang@qti.qualcomm.com | P, C |
| 2 | Andrew Segall Tomoyuki Yamamoto Kiran Misra Jie Zhao | Sharp | asegall@sharplabs.com yamamoto.tomoyuki@sharp.co.jp misrak@sharplabs.com jzhao@sharplabs.com | P, C |
| 3 | Elena Alshina Ankur Saxena Felix Fernandes Zhan Ma | Samsung | elena.alshina@gmail.com elena_a.alshina@samsung.com asaxena@sta.samsung.com ffernandes@sta.samsung.com zhan.ma@sta.samsung.com | P, C |
| 4 | Shang Liu Mei Guo Ximin Zhang Polin Lai Shih-Ta Hsiang Tzu-Der (Peter) Chuang Yu-Wen Huang CW Hsu ChingYeh Chen Chiayang Tsai Zhenzhong Chen ChihMing Fu | MediaTek | shan.liu@mediatek.com mei.guo@mediatek.com ximin.zhang@mediatek.com polin.lai@mediatek.com shih-ta.hsiang@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com chiayang.tsai@mediatek.com zhenzhong.chen@mediatek.com chihming.fu@mediatek.com | P, C |
| 5 | Jie Dong Yuwen He Yan Ye | Interdigital | jie.dong@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com | P, C |
| 6 | Le Leannec Fabrice Francois Edouard Onno Patrice LAROCHE Guillaume | Canon | fabrice.leleannec@crf.canon.fr edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr Guillaume.LAROCHE@crf.canon.fr |  |
| 7 | Madhukar Budagavi Do-Kyoung Kwon | TI | madhukar@ti.com d-kwon@ti.com |  |
| 8 | Chulkeun Kim Joonyoung Park | LGE | chulkeun.kim@lge.com jy.park@lge.com |  |
| 9 | Yi-jen Chiu Wenhao Zhang | Intel | yi-jen.chiu@intel.com wenhao.zhang@intel.com | P, C |
| 10 | Jinwen Zan Haitao Yang Haoping Yu Wen Gao | Huawei | jinwen.zan@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com wen.gao@huawei.com |  |
| 11 | Lu, Shuo Ikeda, Masaru Auyeung, Cheung Kazushi SATO | Sony | shuo.lu@jp.sony.com Masaru.Ikeda@am.sony.com Cheung.Auyeung@am.sony.com Kazushi.Sato@jp.sony.com |  |
| 12 | Tan Yih Han | I2R | yhtan@i2r.a-star.edu.sg |  |
| 13 | Jung Won Kang | ETRI | jungwon@etri.re.kr |  |
| 14 | Woo-Jin Han | Gachon University | hurumi@gmail.com |  |

# List of Contributions

Eight tools listed in the table below were evaluated.

| Test | Proposal documents | Proposal Title | Cross-checking documents |
| --- | --- | --- | --- |
| SCE4.1 | ## TE4.1 fixed filter on integer pixel for SNR scalability |  |  |
| 4.1.1 | JCTVC-M0087 | De-noising filter for SNR scalability | JCTVC-M0185 |
| 4.1.2 | JCTVC-M0058 | Inter-layer fixed refining filter | JCTVC-M0246 |
| SCE4.2 | Interlayer adaptive filter |  |  |
| 4.2.1 | JCTVC-M0265 | Results of Test 4.2.1 on High Frequency Pass Inter Layer Sample Adaptive Offset Filter | JCTVC-M0337 |
| 4.2.2 | JCTVC-M0267 | Results of Test 4.2.2 on Adaptive Re-Sampling Filter | JCTVC-M0278 |
| 4.2.3 | JCTVC-M0195 | Inter-Layer Adaptive Filter on Reconstructed Base Layer Pixels | JCTVC-M0061 |
| 4.2.4 | JCTVC-M0183 | Results of test 4.2.4 on chroma enhancement for inter layer prediction | JCTVC-M0238/ JCTVC-M0338 |
| 4.2.5 | JCTVC-M0055 | Switchable De-ringing Filter for Inter-layer Prediction | JCTVC-M0225 |
| 4.2.6 | JCTVC-M0213 | Adaptive up-sampling of base layer picture using bilateral filters | JCTVC-M0239 |

# Summary of Techniques

The main technical features of each test are summarized in the table below.

| Test | Technique Summary |
| --- | --- |
| 4.1.1 JCTVC-M0087 | - Low pass smoothing filter on integer luma samples Note: 5 taps filter is used instead of originally proposed 7 tap - Only apply to SNR scalability - TextureRL framework: CU level on/off - RefIdx framework: always enabled |
| 4.1.2 JCTVC-M0058 | - Fixed 5x5 cross with a 3x3 square 2D non-separable filter - Low pass smoothing filter on integer luma samples - TextureRL framework: CU level on/off - Only apply to SNR scalability |
| 4.2.1 JCTVC-M0265 | - Picture level SAO - SAO type is derived from the high frequency component of the reconstructed samples - Offset of two SAO types are signaled and added to the reconstructed pixels simultaneously - SAO parameters are coded in the slice header |
| 4.2.2 JCTVC-M0267 | - The adaptive up-sampling filter has same length, coefficient accuracy with the existing up-sampling filter - Filtering process is also applied to integer pixel position - Filter parameters are signaled at picture level and switchable at picture level, |
| 4.2.3 JCTVC-M0195 | - Adaptive 5x5 cross with a 3x3 square 2D non-separable filter - Apply to reconstructed base layer picture before up-sampling process - Filter parameters are signaled at slice header - TextureRL framework: CU level on/off |
| 4.2.4 JCTVC-M0183 | - Enhance the chroma samples by using the surrounding luma samples - Apply to up-sampled base layer picture - Adding a offset to the chroma sample, the offset is obtained by using an adaptive high pass 4x3 filter with luma samples as input - Filter coefficients are derived at encoder side for each chroma plane of a picture - Picture level on/off, filter parameters are signaled at slice header, |
| 4.2.5 JCTVC-M0055 | - 3x3 bilateral filter applies to the up-sampled base layer picture - Each filter coefficient ( i = 0, 8) is derived as follows - a weight w1[i] determined by the spatial position and derived by lookup table, - a weight w2[i] derived by lookup table by using the sample value absolute difference between the current sample and the supporting sample - w[i] = w1[i]*w2[i] - Different table is trained for difference spatial scalability - Division is used as the final normalization, - A lookup table method is additionally proposed to replace the final division operation |
| 4.2.6 JCTVC-M0213 | - 5x5 bilateral filter applies to the upsampled base layer picture - Each filter coefficient ( i = 0, 24) is derived as follows - a weight w1[i] determined by the spatial position and derived by lookup table, - a weight w2[i] derived by lookup table by using the sample value absolute difference between the current sample and the supporting sample - w[i] = w1[i]*w2[i] - Division is used as the final normalization, |

# Summary of Results

The test condition for this CE was specified in JCTVC-L1104, where All Intra, Random Access and Lowdelay-P are mandatory to be tested. The BD-rate is calculated based on EL+BL bit rates with anchor as SHM 1.0. The results of all tests are summarized in the table below. The results containing complexity analysis regarding average bandwidth and computational operation are provided in the attached excel sheets.

| Test |  | All Intra (2x, 1.5x) |  |  |  | RA, LD-P (2x, 1.5x) |  |  |  | RA, LD-P (SNR) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | Cr&Cb | EncT | DecT | Y | Cr&Cb | EncT | DecT | Y | Cr&Cb | EncT | DecT |
| 4.1.1. | IBL |  |  |  |  |  |  |  |  | -2.1% | 0.0% | 102% | 103% |
|  | RefIdx |  |  |  |  |  |  |  |  | -1.4% | -0.7% | 100% | 127% |
| 4.1.2. | IBL |  |  |  |  |  |  |  |  | -2.1% | -0.2% | 103% | 114% |
|  | RefIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.1. | IBL | -0.2% | -0.5% | 99% | 113% | -0.6% | -1.1% | 99% | 129% | -1.8% | -1.2% | 99% | 129% |
|  | RefIdx | -0.2% | -0.4% | 101% | 116% | -0.6% | -0.9% | 100% | 131% | -1.8% | -1.5% | 100% | 130% |
| 4.2.2. | IBL | -0.2% | -0.6% | 115% | 107% | -0.5% | -0.6% | 106% | 102% | -2.3% | -0.9% | 106% | 129% |
|  | RefIdx | -0.2% | -0.5% | 102% | 104% | -0.5% | -0.6% | 103% | 105% | -2.3% | -1.3% | 100% | 133% |
| 4.2.3. | IBL | -0.6% | -0.2% | 110% | 99% | -0.5% | 0.1% | 103% | 81% | -2.4% | 0.1% | 103% | 101% |
|  | RefIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.4. | IBL | -0.8% | -7.6% | 102% | 105% | -0.2% | -8.2% | 101% | 108% | -0.3% | -6.2% | 100% | 109% |
|  | RefIdx | -0.8% | -8.3% | 104% | 105% | -0.3% | -8.7% | 101% | 109% | -0.3% | -6.8% | 101% | 109% |
| 4.2.5. | IBL | -0.5% | -0.9% | 112% | 117% | -0.6% | -0.7% | 104% | 107% | -0.7% | -0.6% | 103% | 107% |
|  | RefIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.6. | IBL | -1.3% | -1.1% | 121% | 155% | -0.9% | -0.8% | 106% | 124% | -0.8% | -0.8% | 109% | 131% |
|  | RefIdx |  |  |  |  |  |  |  |  |  |  |  |  |

# List of non-CE proposals

Nine related non-CE proposals are listed in the table below.

| Proposal documents | Proposal Title | Proponents |
| --- | --- | --- |
| JCTVC-M0089 | Non SCE4: simplified design of cross-color inter-layer filter (test 4.2.4) | E.Alshina, A.Alshin, Y.Cho, (Samsung) |
| JCTVC-M0114 | Non-SCE4: On Interlayer SAO in SHVC | G. Laroche, P. Onno, J. Taquet, C. Gisquet, E. François (Canon |
| JCTVC-M0215 | Non-SCE4: Adaptive up-sampling of base layer picture using Simplified Separable bilateral filters | Zhao, K. Misra, A. Segall (Sharp) |
| JCTVC-M0223 | Non-SCE4.1: Fixed Inter-layer Filter for SNR Scalability with Only One Non-unity Coefficient | X. Zhang, P. Lai, S. Liu, S. Lei (MediaTek), |
| JCTVC-M0224 | Non-SCE4.2: Inter-layer Fixed Directional Filtering | P. Lai, S. Liu, S. Lei (MediaTek) |
| JCTVC-M0233 | Non-SCE4: Inter-layer switchable upsampling filters for spatial scalability | Z. Chen, P. Lai, X. Zhang, S. Liu, S. Lei (MediaTek) |
| JCTVC-M0253 | Non-SCE4: Simplification of chroma enhancement for inter layer reference picture generation | X. Li, J. Chen, W. Pu, M. Karczewicz (Qualcomm), |
| JCTVC-M0271 | Non-SCE4.2.2: 6-Tap Adaptive Re-Sampling Filter | W. Pu, J. Chen, X. Li, M. Karczewicz (Qualcomm) |
| JCTVC-M0273 | Non-SCE4: Switchable Filter on Integer Position | W. Pu, V. Seregin, J. Chen, X. Li, M. Karczewicz (Qualcomm), E. Alshina, Alexander Alshin, Yongjin Cho (Samsung) |