| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0025 |
| --- | --- |

| Title: | SCE5: Summary report of core experiment on inter-layer syntax prediction using HEVC base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Patrice Onno Shan Liu Elena Alshina Chulkeun Kim Haitao Yang | Email: | vseregin@qti.qualcomm.com patrice.onno@crf.canon.fr shan.liu@mediatek.com elena_a.alshina@samsung.com chulkeun.kim@lge.com haitao.yang@huawei.com |
| Source: | SCE coordinators |  |  |

_____________________________

# Introduction

This contribution summarizes the activities and test results performed in SCE5 on inter-layer syntax prediction using HEVC base layer. The tools in SCE5 were classified into four categories:

- 5.1 Inter-layer motion information prediction

  - Using base layer MV as a candidate in Merge and AMVP modes.

  - Inter-layer inferred prediction mode, where the enhancement layer is divided into sub-blocks and motion information associated with each sub-block is derived from the corresponding block in the base layer.

- 5.2  Motion data compression and memory reduction (memory measurement)

In this category, motion data compression of the base layer in relation to the base layer motion vector derivation is studied as well as a memory reduction associated with not using temporal MV candidate.

# Document list

| Test | Proposal | Cross-checking documents |
| --- | --- | --- |
| SCE5.1 | ## Inter-layer motion information prediction |  |
| 5.1.1 | JCTVC-M0295 (Qualcomm) “SCE5: Simplified base layer MV candidate for merge mode (test 5.1.1)” | JCTVC-M0395 (MediaTek) |
| 5.1.2 | JCTVC-M0308 (Qualcomm) “SCE5: Results of Test 5.1.2 on Inter-layer Inferred Prediction Mode” | JCTVC-M0302 (LG) |
| 5.1.3 | withdrawn |  |
| 5.1.4 | JCTVC-M0413 (KDDI) “SCE5: Results of test 5.1.4 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer” | JCTVC-M0147 (Sony) |
| 5.1.5 | JCTVC-M0124 (MediaTek) “SCE5.1.5: Inter-layer motion information prediction” | JCTVC-M0149 (Sony) |
| 5.1.6 | JCTVC-M0160 (Nokia) “SCE5 – Results of 5.1.6 on inter-layer motion vector prediction” | JCTVC-M0300 (Qualcomm) |
| 5.1.7 | JCTVC-M0059 (Intel) “SCE5.1.7: Inter-layer motion data inheritance” | JCTVC-M0406 (KDDI) |
| 5.1.8 | JCTVC-M0289 (LG) “SCE5 : Inter-layer motion vector prediction in AMVP” | JCTVC-M0355 (Qualcomm) |
| 5.1.9 | withdrawn |  |
| 5.1.10 | JCTVC-M0047 (ETRI) “SCE5: Results of test 5.1.10 on inter-layer motion information prediction” | JCTVC-M0359 (Canon) |
| SCE5.2 | Motion data compression and memory reduction |  |
| 5.2.1 | JCTVC-M0283 (Sony, I2R) “SCE5 5.2.1 on Effectiveness of temporal motion vector prediction (TMVP)” | JCTVC-M0128 (JVC Kenwood) |
| 5.2.2 | JCTVC-M0141 (Sony, LG) “SCE5: Result of SCE5.2.2” | JCTVC-M0392 (MediaTek) |
|  | JCTVC-M0142 (Sony) “SCE5: Buffer Size Estimation of SCE5.2.x” |  |

# Summary

- SCE5.1 Inter-layer motion information prediction

In this category, motion vector derived from the base layer picture is used as an additional candidate in Merge and AMVP candidates list or new prediction mode is introduced, where base layer motion field is derived based on sub-blocks. In one test, second base layer motion vector is used as motion vector candidate for the merge mode.

More detailed description of the proposed methods can be found in the next table and the corresponding contribution documents.

| Test | Changes in Merge and AMVP modes |
| --- | --- |
| 5.1.1 (Qualcomm) | Merge modification: - The merge candidate list is constructed as in HEVC v1. - All the MV candidates in the merge list are shifted down by one position and BL MV candidate is placed in the first position. |
| 5.1.2 (Qualcomm) | - CU based inferred mode - Motion information is derived based on 8x8 blocks |
| 5.1.3 (Canon) | withdrawn |
| 5.1.4 (KDDI) | Merge modification: - BL MV candidate is moved to the third position in the merge list - Pruning between BL MV and above right, bottom left and left candidates - Optional base MV refinement |
| 5.1.5 (MediaTek) | Merge modification: - Additional bottom right BL MV candidate placed after above candidate - Additional pruning between two BL MV candidates - Reduced pruning for BL MV candidate (only with left and above candidates) |
| 5.1.6 (Nokia) | - BL MV scaling by using TMVP scaling process Merge modification: - No pruning between BL MV and spatial candidates |
| 5.1.7 (Intel) | - CU based inferred mode - Motion information is derived based on NxN partitioning |
| 5.1.8 (LG) | AMVP modification: - Center BL MV candidate added as the first MV candidate - Pruning with BL MV candidate |
| 5.1.9 (Samsung) | withdrawn |
| 5.1.10 (ETRI) | Merge modification: - Reduced pruning for BL MV candidate (only with left candidate) AMVP modification: - Center BL MV candidate added as the first MV candidate - Pruning with BL MV candidate |

- SCE5.2 Motion data compression and memory reduction

In this category, uncompressed or 8x8 compressed base layer motion filed is used to derive base layer motion vector candidates. Additionally, temporal motion vector candidate can be disabled for enhancement layer coding.

| Test | Changes in Merge and AMVP modes |
| --- | --- |
| 5.2.1 (I2R, Sony) | - Disable TMVP for EL - In addition, use uncompressed BL motion field to derive BL MV candidate |
| 5.2.2 (Sony, LG) | - Use uncompressed BL motion field to derive BL MV candidate - Alternatively, use 8x8 compressed BL motion field to derive BL MV candidate |

The tests results for both categories are summarised in the next table. Last column shows the difference of the MV comparison numbers in pruning process comparing to SHM1.0.

| Test | Short description | RA-2x | RA-1.5x | RA SNR | LP-2x | LP-1.5x | LP SNR | LB-2x | LB-1.5x | LB SNR | Enc | Dec | Comp.num. diff |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.1.1 (Qualcomm) | insert BL MV into the completed list | 0.25 | 0.19 | 0.29 | 0.26 | 0.21 | 0.32 |  |  |  | 100% | 100% | -5 |
| 5.1.2 (Qualcomm) | Inferred mode based on 8x8 sub-block | 0.03 | 0.00 | 0.03 | 0.01 | 0.01 | 0.02 |  |  |  | 101% | 103% | 0 |
| 5.1.3 |  | Withdrawn |  |  |  |  |  |  |  |  |  |  |  |
| 5.1.4 (KDDI) | 1. BL MV as a 3rd candidate | -0.03 | 0.17 |  | -0.22 | -0.06 |  |  |  |  | 100% | 100% | -2 |
|  | 2. MV refinement | -0.05 | 0.30 |  | -0.23 | -0.02 |  |  |  |  | 100% | 106% | 0 |
|  | 3. Test 1 + uncompressed MV | -0.84 | -0.19 |  | -0.89 | -0.35 |  |  |  |  | 100% | 100% | -2 |
| 5.1.5 (MediaTek) | Simplified pruning | 0.03 | 0.01 | 0.02 | 0.01 | 0.03 | 0.03 |  |  |  | 100% | 100% | -3 |
|  | 2 BL MVs with simplified pruning | -0.61 | -0.23 | -0.61 | -0.40 | -0.16 | -0.49 |  |  |  | 100% | 100% | -2 |
| 5.1.6 (Nokia) | 1. use TMVP scaling | -0.03 | 0.00 | 0.00 | -0.03 | 0.00 | 0.00 |  |  |  | - | - | -5 |
|  | 2. w/o BL MV pruning | 0.30 | 0.21 | 0.32 | 0.29 | 0.23 | 0.33 |  |  |  | - | - | -5 |
|  | 3. Test 1 + Test 2 | 0.27 | 0.21 | 0.32 | 0.27 | 0.23 | 0.33 |  |  |  | - | - | -5 |
| 5.1.7(Intel) | Inferred mode based on NxN | 0.09 | 0.08 | 0.12 | 0.09 | 0.08 | 0.11 | 0.09 | 0.06 | 0.10 | 103% | 98% | 0 |
| 5.1.8 (LG) | 1. BL MV for AMVP | -0.04 | -0.10 | -0.11 | -0.03 | -0.04 | -0.05 |  |  |  | 100% | 100% | +3 |
|  | 2. Test 1 + uncompressed MV | -1.26 | -0.64 | -0.63 | -1.07 | -0.47 | -0.69 |  |  |  | 100% | 100% | +3 |
| 5.1.9 |  | Withdrawn |  |  |  |  |  |  |  |  |  |  |  |
| 5.1.10 (ETRI) | 1. Simplified BL MV pruning | 0.08 | 0.05 | 0.07 | 0.09 | 0.07 | 0.08 |  |  |  | 100% | 100% | -4 |
|  | 2. BL MV for AMVP | 0.02 | -0.10 | -0.12 | 0.09 | 0.01 | -0.02 |  |  |  | 100% | 100% | +1 |
| 5.2.1 (I2R, Sony) | 1. disabling TMVP | 0.76 | 0.34 | 0.38 | 0.92 | 0.57 | 0.48 | 0.81 | 0.46 | 0.44 | 100% | 98% | 0 |
|  | 2. Test 1 + uncompressed MV | -0.49 | -0.20 | -0.17 | -0.27 | 0.06 | -0.25 |  |  |  | - | - | 0 |
|  | 3. Test 1 + 8x8 compressed MV | -0.15 | -0.12 | -0.11 | 0.13 | 0.15 | -0.14 |  |  |  | 101% | 100% | 0 |
| 5.2.2 (Sony, LG) | 1. uncompressed MV | -1.10 | -0.50 | -0.50 | -0.94 | -0.41 | -0.61 |  |  |  | 99% | 99% | 0 |
|  | 2. 8x8 compressed MV | -0.79 | -0.42 | -0.45 | -0.61 | -0.34 | -0.52 |  |  |  | 100% | 101% | 0 |

Next graph shows the difference in number of MV comparisons comparing to SHM1.0 along the x axis versus the average performance along the y axis.