| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0027r1 |
| --- | --- |

| Title: | RCE2: Summary report of HEVC Range Extensions Core Experiment 2 on Intra Prediction for Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wen Gao Minhua Zhou Peter Amon Sunil Lee | Email: | wen.gao@harmonicinc.com p.amon@siemens.com sunil.lee@samsung.com |
| Source: | RCE2 coordinators |  |  |

_____________________________

# Introduction

A summary of RCE2 on Intra prediction for lossless coding for HEVC Range Extensions is reported. Four methods and their combinations have been evaluated based on the CE description in JCTVC-L1122.

# Document List

## CE contributions and corresponding cross-checks

### Test 1: Residual DPCM for horizontal/vertical intra prediction directions

- JCTVC-M0079 (JCTVC-L0117), “RCE2: Test 1 - Residual DPCM for HEVC lossless coding”, S. Lee, I.-K. Kim, C. Kim (Samsung)

  - Cross-check: JCTVC-M0317 (Huawei)

### Test 2: Template based sample-adaptive weighted prediction

- JCTVC-M0052 (JCTVC-L0161), “RCE2: Sample-based weighted intra prediction for lossless coding”, P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)

  - Cross-check: JCTVC-M0082(Samsung)

### Test 3: Sample-based angular intra prediction (SAP) in all 33 angular prediction direction

- JCTVC-M0056 (JCTVC-L0176), “RCE2: Experimental results on Test 3 and Test 4”, M. Zhou, M. Budagavi (TI)

  - Cross-check: JCTVC-M0083(Samsung)

### Test 4: SAP in only vertical and horizontal direction: a variant of Test 3

- Document is the same as JCTVC-M0056 for Test 3

  - Cross-check: JCTVC-M0069 (I2R)

### Test 5: Residual sample-based prediction

- JCTVC-M0067 (JCTVC-K0157), “RCE2: Experimental Results for Test 5”, Y. H. Tan, C. Yeo (I2R)

  - Cross-check: JCTVC-M0318 (Huawei)

### Test 6: Combination of residual DPCM for horizontal/vertical intra prediction directions and template based sample-adaptive weighted prediction

- JCTVC-M0053 (JCTVC-L0117 + JCTVC-L0161), “RCE2: Experimental results for Test 6 – combination of RDPCM and SWP for HEVC lossless coding”, E. Wige (Universität Erlangen-Nürnberg), P. Amon (Siemens), S. Lee, I.-K. Kim, C. Kim (Samsung)

  - Cross-check: JCTVC-M0057 (TI)

### Test 7: Combination of SAP and template based sample-adaptive weighted prediction

- No Separate Document registered (JCTVC-L0161 + JCTVC-L0176). Simulation results are provided by E. Wige (Universität Erlangen-Nürnberg), P. Amon (Siemens) in JCTVC-M0052.

  - Cross-check: JCTVC-M0349(Qualcomm)

## Related non-CE contributions

- JCTVC-M0068, “Non-RCE2: TU-level flag for DPCM”, Y. H. Tan, C. Yeo (I2R)

- JCTVC-M0193, “Non-RCE2: Improved sample-based weighted intra prediction for lossless coding”, P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)

- JCTVC-M0288, “Non-RCE2: Extension of residual DPCM for lossless coding”, R. Joshi, J. Sole, M. Karczewicz (Qualcomm)

- JCTVC-M0291, “Non-RCE2: Modification of DC intra prediction mode for lossless coding”, R. Joshi, J. Sole, M. Karczewicz (Qualcomm)

- JCTVC-M0439, “Complexity reduction for residual DPCM for HEVC lossless coding”, M. Naccari, M. Mrak, A. Gabriellini (BBC) [late]

- JCTVC-M0442, “Inter-prediction residual DPCM”, M. Naccari, M. Mrak, A. Gabriellini (BBC), S. Blasi, E. Izquierdo (QMUL) [late]

- JCTVC-M0350, M. Budagavi, D.-K. Kwon (TI), AHG8: Video coding using Intra motion compensation

# Summary of RCE2 tests

A summary of the status of all the RCE2 tests including the related software, WD texts, cross-check results, proposal and complexity analysis are provided in the following table.

| TEST | Software | Results | WD text | Crosscheck Results | Proposal | Complexity Analysis |
| --- | --- | --- | --- | --- | --- | --- |
|  | Provided | Provided | Provided | Provided | Provided | Provided |
| 1 | Samsung | Samsung | Samsung | Huawei | M0079 | Provided |
| 2 | Siemens | Siemens | Siemens | Samsung | M0052 | Provided |
| 3 | TI | TI | TI | Samsung | M0056 | Provided |
| 4 | TI | TI | TI | I2R | M0056 | Provided |
| 5 | I2R | I2R | I2R | Huawei | M0067 | Missing |
| 6 | Siemens/Samsung | Siemens/Samsung | Siemens/Samsung | TI | M0053 | Provided |
| 7 | Siemens/TI | Siemens | Siemens/TI* | Qualcomm | M0052+M0056 | Provided * |

Notes:

- * The individual WD text and complexity analysis for SAP and Template based sample-adaptive weighted prediction are provided in JCTVC-M0052 and JCTVC-M0056.

- Cross-check simulation results all match those in the respective proposals.

# Tested methods

HM10.0 range extension software with SEQUENCE_LEVEL_LOSSLESS macro turned on and QP set to 0 in all lossless coding simulations. The mandatory test cases are AI-Main, RA-Main and LB-Main configurations.

The test sequences used are all four Class F sequences, Class B sequence 1080p Kimono1 and ParkScene, range extension 1080p 4:2:2 sequence EBUHorse and EBUWaterRocksClose (10-bit, 500 frames), range extension 1080p 4:4:4 sequence EBURainFruits and BirdsInCage (10-bit, 600 frames), and the ten screen content 4:4:4 RGB sequences as listed as following: VenueVue, cad_waveform, cg_twist_tunnel, pcb_layout, ppt_doc_xls, video_conferencing_doc_sharing, web_browsing, sc_map, sc_programming, and sc_wordEditing.

Please refer to JCTVC-L1122 for details.

# Summary of results

The results are provided in Table 1. Cross-checkers confirm the results.

Table 1 - Results for all tests in RCE2

|  | Test 1 (M0079) | Test 2 (M0052) | Test 3 (M0056) | Test 4 (M0056) | Test 5 (M0067) | Test 6 (Test 1+2) | Test 7 (Test2+3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI-main |  |  |  |  |  |  |
|  | Bit-rate saving |  |  |  |  |  |  |
| Class F | -9.27% | -7.80% | -11.60% | -10.10% | -6.90% | -11.40% | -12.60% |
| Class B | -4.52% | -6.80% | -5.80% | -4.40% | -6.20% | -7.20% | -7.50% |
| SC(GBR) | -11.08% | -7.20% | -10.40% | -12.40% | -3.50% | -12.90% | -11.80% |
| RangeExt | -2.79% | -4.70% | -4.10% | -2.90% | -0.90% | -4.90% | -5.30% |
| Overall (w/o SC) | -5.73% | -6.40% | -7.40% | -6.00% | -4.40% | -8.00% | -8.70% |
| Overall (w/ SC) | -8.40% | -6.80% | -8.90% | -9.20% | -4.00% | -10.50% | -10.30% |
| Enc Time[%] | 101% | 102% | 102% | 97% | 109% | 105% | 103% |
| Dec Time[%] | 98% | 98% | 97% | 95% | 117% | 100% | 97% |
|  | Random Access Main |  |  |  |  |  |  |
|  | Bit-rate saving |  |  |  |  |  |  |
| Class F | -5.66% | -3.90% | -6.70% | -6.10% | -4.20% | -6.60% | -7.30% |
| Class B | -0.87% | -1.80% | -1.20% | -0.90% | -1.00% | -1.90% | -2.00% |
| SC(GBR) | -7.66% | -5.20% | -6.90% | -8.80% | -2.20% | -9.40% | -8.50% |
| RangeExt | -0.68% | -1.20% | -1.00% | -0.70% | -0.40% | -1.30% | -1.40% |
| Overall (w/o SC) | -2.71% | -2.40% | -3.30% | -2.90% | -2.00% | -3.60% | -3.90% |
| Overall (w/ SC) | -5.18% | -3.80% | -5.10% | -5.80% | -2.10% | -6.50% | -6.20% |
| Enc Time[%] | 100% | 101% | 100% | 101% | 109% | 101% | 101% |
| Dec Time[%] | 102% | 98% | 97% | 100% | 107% | 99% | 98% |
|  | Low delay B Main |  |  |  |  |  |  |
|  | Bit-rate saving |  |  |  |  |  |  |
| Class F | -4.57% | -3.00% | -5.40% | -4.90% | -3.50% | -5.40% | -5.90% |
| Class B | -0.62% | -1.50% | -0.90% | -0.60% | -0.70% | -1.60% | -1.60% |
| SC(GBR) | -7.11% | -5.00% | -6.40% | -8.20% | -2.70% | -9.00% | -7.80% |
| RangeExt | -0.59% | -1.10% | -0.90% | -0.60% | -0.30% | -1.20% | -1.30% |
| Overall (w/o SC) | -2.19% | -1.90% | -2.70% | -2.30% | -1.70% | -2.90% | -3.20% |
| Overall (w/ SC) | -4.65% | -3.50% | -4.60% | -5.30% | -2.20% | -6.00% | -5.50% |
| Enc Time[%] | 100% | 100% | 100% | 99% | 107% | 101% | 101% |
| Dec Time[%] | 98% | 101% | 97% | 97% | 108% | 101% | 100% |

# Complexity Analysis

The summary of the complexity analysis is shown in Table 2.

Table 2 - Results for complexity analysis for Test 1-5 in RCE2

| Complexity Analysis | Test 1 (M0079) | Test 2 (M0052) | Test 3 (M0056) | Test 4 (M0056) | Test 5 (M0067) |
| --- | --- | --- | --- | --- | --- |
| Number of operation/Sample (typical case) | (nT+1)/2 additions for fully parallel decoder* 1 addition when decode one row/column in parallel | 4 multiplications 19 adds 1 shift 1 division 4 table look-ups 1 comparison 16 subtractions | Not Provided | (nT+1)/2 additions for fully parallel decoder* 1 addition when decode one row/column in parallel | Not Provided |
| Number of samples coded in parallel | Fully parallel at encoder and decoder | Fully parallel at encoder one row of a PU at decoder | Fully parallel at encoder one row/column of a PU at decoder | Fully parallel at encoder and decoder | Not Provided |

Note:

- * nT is the size of the PU

- The only difference between Test 1 and Test 4 is that edge filtering as in HEVC is used Test 1 while no edge filtering is used in Test 4

- No complexity analysis is provided for Test 5.

# Discussion

- JCTVC-M0079 also provides non-normative encoder only technique, which can further improve the coding efficiency in Test 1. Note that the same technique can be applied to Test 4 as well.

- JCTVC-M0052 provides additional results after fixing a bug in the JCTVC-L0161. The bugfix has little impact on coding efficiency. In addition, an improved version of the template based adaptive weighted prediction scheme is also provided with additional simulation results in JCTVC-M0052.

# References

- D. Flynn, K. Sharman “Common test conditions and software reference configurations for HEVC range extensions” Joint Collaborative Team on Video Coding, JCTVC-L1006, Geneva, January 2013.

- W. Gao, M. Zhou, P. Amon and S. Sunil, “HEVC Range Extensions Core Experiment 2 (RCE2) Intra Prediction for Lossless Coding”, Joint Collaborative Team on Video Coding, JCTVC-L1122, Geneva, January 2013.