| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0031 |
| --- | --- |

| Title: | SCE1: Results of test 1.1 on intra DC correction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Tel: Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports that adjusting average sample values of intra predicted blocks in enhancement layer to match the average sample values of corresponding base layer blocks provides objective bitrate reductions of -0.2 % and -0.1 % in AI 2x and AI 1.5x configurations, respectively.

# Introduction

Scalability Core Experiment 1 / Test 1.1 studied performance of adjusting the average (“DC”) sample values of the enhancement layer intra prediction blocks to the corresponding upsampled base layer block’s average sample value. The method is motivated by the assumption that typically the DC term of an enhancement layer sample block is close the DC term of the corresponding block in the base layer. However, the spatial intra prediction methods are not always able to predict that term as those rely on the boundary samples of the enhancement layer blocks. Introducing a DC correction process in the intra prediction is thus believed to help the coding of the residual DC coefficient (and potentially also reducing the amount of intra blocks requiring residual coding).

# Results of the test

The tables below summarize performance of the tested solution. Simulation runtime information was generated by disabling YUV output in order to minimize external impacts to the runtimes. Full results are provided in the attached Excel sheets.

Table 1: Performance of SCE1 Test 1.1 with respect to SHM 1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | 0.0% |  |  |  |
| Class B | -0.2% | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% |
| Overall (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% |
| Overall (Test vs single layer) | 12.2% | 13.5% | 13.3% | 10.2% | 10.2% | 9.6% |
| EL only (Test vs Ref) | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103.7% |  |  | 103.2% |  |  |
| Dec Time[%] | 99.4% |  |  | 100.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Algorithm description

DC correction is performed for all intra PUs which have a size of 8x8 or larger and are not utilizing IntraBL or planar prediction. DC correction adds a constant value to each sample of the enhancement layer block so that the DC of the enhancement layer prediction equals to the DC of the co-located base layer block. In other words, a dc_delta is calculated by taking the difference between enhancement layer block’s average sample value and corresponding upsampled base layer block’s average sample value, and added to all the samples of the enhancement layer block:

P(x,y) = P’(x,y) + dc_delta

where:

P(x,y) denotes the sample to be predicted in the enhancement layer picture.

P’(x,y) denotes a predicted enhancement layer sample after HEVC intra prediction process

dc_delta denotes the difference of average sample values in P’(x,y) and corresponding upsampled base layer block.

# Proposed specification text

8.4.4.2.6 Specification of Intra_DC (1) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	upsampled base layer samples b[ x ][ y ], with x, y = 0..nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

…

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as the following ordered steps:

- When decoding base layer representation, variable dcVal is derived as:

dcVal = x'=0nS−1p[x'][−1]+y'=0nS−1p[−1][ y']+nS>>(k+1)
where k=Log2(nT)

When decoding enhancement layer representation, variable dcVal is derived as:

dcVal = y'=0nS−1x'=0nS−1b[x'][y']+(1<<(2⋅k−1))>>2⋅k
where k=Log2(nT)

8.4.4.2.6 Specification of Intra_Angular (2..34) prediction mode

Inputs to this process are:

–	intra prediction mode intraPredMode,

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	upsampled base layer samples b[ x ][ y ], with x, y = −1..nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

[Insert between steps 2.b and 2.c:]

c.    The following applies to the enhancement layer decoding,

- Value of dcDelta is calculated as follows:

dcDelta = ( sum( b[ x ][ y ] ) - sum( predSamples[ x ][ y ] ) ) / (nT*nT)

and the values of predSamples[ x ][ y ] are updated as follows:

predSamples[ x ][ y ] = Clip ( predSamples[ x ][ y ] + dcDelta )

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).