| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0032 |
| --- | --- |

| Title: | SCE1: Results of test 2.1 on gradient based intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Tel: Email: | jani.lainema@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports that utilizing gradient based inter-layer intra prediction for enhancement layer provides objective bitrate reductions of -0.2 % and -0.2 % in AI 2x and AI 1.5x configurations, respectively. The method enhances the directly horizontal and vertical prediction process by measuring how the base layer pixel values change in the corresponding direction and applying the generated differential values when reconstructing prediction samples for the enhancement layer video. Also supplemental results are reported indicating that the claimed gains in coding efficiency are additive with those provided by SCE1 Test 1.1 (DC correction). The combined tools are reported to provide objective bitrate reductions of -0.4 % and -0.2 % in AI 2x and AI 1.5x configurations, respectively.

# Introduction

Scalability Core Experiment 1 / Test 2.1 studied performance of gradient based inter-layer intra prediction. The method is motivated by the assumption that already coded base layer texture can be utilized to improve the enhancement layer intra prediction. This is proposed to be done by modifying the directly horizontal and vertical intra prediction process by measuring how the base layer pixel values change in the corresponding direction and applying the generated differential values when reconstructing prediction samples for the enhancement layer video.

# Results of the test

The tables below summarize performance of the tested solution. Simulation runtime information was generated by disabling YUV output in order to minimize external impacts to the runtimes. Full results are provided in the attached Excel sheets.

Table 1: Performance of SCE1 Test 2.1 with respect to SHM 1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | -0.2% |  |  |  |
| Class B | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% |
| Overall (Test vs Ref) | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% |
| Overall (Test vs single layer) | 12.2% | 13.4% | 13.1% | 10.1% | 10.1% | 9.6% |
| EL only (Test vs Ref) | -0.3% | -0.2% | -0.3% | -0.4% | -0.3% | -0.3% |
| Enc Time[%] | 101.4% |  |  | 99.4% |  |  |
| Dec Time[%] | 99.3% |  |  | 100.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

The supplemental results below show the combined performance of this test with the SCE1 Test 1.1 (DC correction for enhancement layer intra prediction).

Table 2: Performance of combination of SCE1 Test 1.1 and 2.1 with respect to SHM 1.0 anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.2% |  |  |  |
| Class B | -0.4% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% |
| Overall (Test vs Ref) | -0.4% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% |
| Overall (Test vs single layer) | 12.0% | 13.3% | 13.0% | 10.0% | 10.1% | 9.5% |
| EL only (Test vs Ref) | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 104.7% |  |  | 102.2% |  |  |
| Dec Time[%] | 100.2% |  |  | 101.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Algorithm description

For directly horizontal and vertical intra prediction modes, the prediction is enhanced by measuring how the base layer pixel values change in the corresponding direction and applying the generated differential values when reconstructing prediction samples for the enhancement layer video. This process is illustrated in Figure 1 below for a 4x4 block using vertical prediction.

Figure 1: Gradient based enhancement of intra prediction for vertical prediction

There is no additional syntax or signalling associated with the proposed technology. Instead, the intra prediction process at enhancement layer is modified as follows for the mentioned intra prediction directions:

P(x,y) = PR(xR,yR) + B(x,y) - BR(xR,yR)

Where:

P(x,y) denotes the sample to be predicted in the enhancement layer picture.

PR(xR,yR) denotes the enhancement layer reference sample in the enhancement layer picture based on the directionality of the intra prediction mode

B(x,y) in the base layer picture with location corresponding to the enhancement layer sample P(x,y)

BR(xR,yR) denotes the base layer reference sample in the base layer picture based on the directionality of the intra prediction mode

# Proposed specification text

8.4.4.2.6 Specification of Intra_Angular (2..34) prediction mode

Inputs to this process are:

–	intra prediction mode intraPredMode,

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	upsampled base layer samples b[ x ][ y ], with x, y = −1..nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

[Insert between steps 2.b and 2.c:]

c.    The following applies to the enhancement layer decoding, depending on the value of intraPredMode.

- If intraPredMode is equal to 10, the predSamples[ x ][ y ]  are derived by

predSamples[ x ][ y ] = Clip ( ref[ y+1 ] + b[ x ][ y ] – b[ x ][ -1 ] )

- Otherwise if intraPredMode is equal to 26, the predSamples[ x ][ y ]  are derived by

predSamples[ x ][ y ] = Clip ( ref[ x+1 ] + b[ x ][ y ] – b[ -1 ][ y ] )

# Patent rights declaration(s)

Nokia may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).