| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0035 |
| --- | --- |

| Title: | AHG13: Slice based upsampling filter for improved error resiliency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Miska M. Hannuksela | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

In spatial scalability, base layer samples are upsampled to enhancement layer resolution and used as reference. If the base layer picture is coded with slices, sub-pixel samples close to the border of the slice are calculated using the integer samples from another slice. Similar to filtering across slice-boundaries, it is asserted that this increases error propagation and reduces the error resiliency when SHVC is used within error-prone environments. Similar to restricting filtering across slice boundaries, this contribution proposes an optional restriction on upsampling, so that sub-pixel samples never use integer samples from another slice. It is noted that the proposed functionality is already present SVC using the constrained_intra_resampling_flag syntax element.

# Introduction

In spatial scalability, base layer samples are upsampled to enhancement layer resolution and used as reference. If the base layer picture is coded with slices, sub-pixel samples close to the border of the slice are calculated using the integer samples from another slice. This is illustrated in the below example, where the half-pixel sample at the slice boundary utilizes four integer samples belonging to a different slice.

This phenomenon is actually well known, and H.264/AVC, SVC and HEVC include mechanisms to avoid filtering across slice boundaries to reduce error propagation. We propose a similar functionality also for upsampling so that sub-pixel samples do not use integer samples from another slice, but instead use padded pixels from the current slice. This is illustrated below, where the half-pixel sample is calculated not using integer samples belonging to a different slice but using the padded pixels (shown with dashed squares). It is noted that the proposed functionality is already present SVC using the constrained_intra_resampling_flag syntax element.

# Specification text

A flag is inserted to sequence parameter set extension to indicate the slice based upsampling process.

| if( sps_extension_flag ) |  |
| --- | --- |
| sps_inter_layer_mfm_enable_flag | u(1) |
| resampling_filter_across_slices_enabled_flag | u(1) |

If this flag is 0, resampling process is modified as shown as below with change marks.

The sample value tempArray[ n] with n = 0..7, is derived as follows.

yPosRL = Clip3( nRT, nRB – 1, yRef + n – 1 ) 	(8-xx)

nRL is the horizontal position of the sample that is the leftmost sample of the slice that the base layer sample positioned at ( xRef + m, yRef + n – 1 ) belongs to among the row of samples located at vertical position yRef + n – 1.

nRR is the horizontal position of the sample that is the rightmost sample of the slice that the base layer sample positioned at ( xRef + m, yRef + n – 1 ) belongs to among the row of samples located at vertical position yRef + n – 1.

nRT is the vertical position of the sample that is the topmost sample of the slice that the base layer sample positioned at ( xRef + m, yRef + n – 1 ) belongs to among the column of samples located at horizontal position xRef + m.

nRB is the vertical position of the sample that is the bottommost sample of the slice that the base layer sample positioned at ( xRef + m, yRef + n – 1 ) belongs to among the column of samples located at horizontal position xRef + m.

tempArray[n] = fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( nRL, nRR – 1, xRef – 3), yPosRL] +
                          fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( nRL, nRR – 1, xRef – 2), yPosRL] +
                          fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( nRL, nRR – 1, xRef – 1), yPosRL] +
                          fL[ xPhase, 3 ] * rlPicSampleL[ Clip3(  nRL, nRR – 1, xRef      ), yPosRL] +
                          fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( nRL, nRR – 1, xRef + 1), yPosRL] +	(8-xx)
                          fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( nRL, nRR – 1, xRef + 2), yPosRL] +
                          fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( nRL, nRR – 1, xRef + 3), yPosRL] +
                          fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( nRL, nRR – 1, xRef + 4), yPosRL]

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).