| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0039 |
| --- | --- |

| Title: | On lossless coding with SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Hoda Roodaki-Lavasani | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Coding a picture / video in two layers, where enhancement layer is coded in a lossless manner, is asserted to be useful for many applications (for example efficient lossless coding tools can be used in enhancement layer to improve coding efficiency in a backwards compatible way). HEVC and its draft scalable extension include mechanisms to achieve this operation, thanks to bypassing the transform and quantization mode indicated with transquant_bypass_flag. However, it is asserted that this operation cannot be easily used in because this operation is known to decoder after decoding the entire enhancement layer CUs and parsing the corresponding transquant_bypass_cu_flag syntax elements. We propose high level features indicating that enhancement layer is used to achieve lossless coding operation so that SHVC could more efficiently support the aforementioned applications. We also propose consideration of efficient lossless coding methods developed in range extensions work also for SHVC.

# Introduction

Lossless coding is seen as an important topic and there have been many proposals on developing optimized tools to achieve lossless operation and potentially develop a dedicated profile for lossless coding (JCTVC-L0451). It was also noted that lossless coding has important application areas for still picture coding (e.g. medical and satellite imaging).

For still pictures, coding a picture in two layers using SHVC, where enhancement layer is coded lossless, is useful as follows:

- Backwards compatible extension: Efficient tools for lossless coding can be used for enhancement layer to improve lossless coding efficiency. This means HEVC version 1 decoders not implementing the range extension tools could still decode and display a picture. This kind of setup would help the deployment of these applications as it is expected that HEVC version 1 decoders will be more widely available at least in the early days after range extensions is standardized) and backwards compatibility with HEVC version 1 would allow gradual deployment.

- Serving low-bitrate thumbnails for fast browsing: If lossless coded image needs to be transmitted, the receiver could request first the thumbnail (which could be a lossy coded version) to save bandwidth and only after the thumbnail is analyzed, request the full lossless version.

In addition, coding the video in such a scalable fashion allows:

- Dynamically adjusting the bitrate: Most of the use-cases for efficient lossless coding involve transmission of the video over a best effort network such as internet, hence it is useful to be able adapt the changing network conditions. As there is a single RD operation point for lossless coding, it is not possible to dynamically change the bitrate. If scalable coding is used, the encoder / router could switch to lossy coded base layer whenever needed.

- Serving multiple clients: Some of the use-cases listed above, like web based collaboration, involve connection of multiple clients. This means the bandwidth available to some of the clients might be lower than others and cannot cope with receiving lossless coded video. For those cases, it would be beneficial to code the video in a layered fashion so that the lossy coded base layer could be served to clients with low bandwidth and lossless enhancement layer could additionally be served to clients with higher bandwidth.

# Proposal

Two proposals are made in this contribution:

- To efficiently enable the use-cases described above, consider adding scalability functionality to the range extensions (i.e. enable coding enhancement layer using novel lossless coding tools). It should be noted that, high-level syntax only scalability (HLS-only) approach means that this could be enabled with very small changes only.

- Add a high-level syntax that indicates EL is used for achieving lossless operation (i.e. all the transquant_bypass_enable_flag of enhancement layer equals to 1). Alternatively this can be indicated as an SEI message.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| lossless_enhancement_flag | u(1) |
| … |  |
| } |  |

lossless_enhancement_flag equal to 1 indicates that all the coding units within any picture with nuh_layer_id greater than 1 have cu_transquant_bypass_flag set to 1 and if not present inferred as 1.

# Experimental results

In order to demonstrate the benefits of utilizing SHVC to achieve lossless operation for still pictures, we performed several experiments.

We encoded first picture of Class-F sequences with three different configurations:

- Simulcast: For each picture, both lossless and lossy versions (QP = 22, 27, 32, 37) are coded separately.

- Single-layer: Each picture is coded losslessly.

- Scalable: For each picture, base layer is coded with different QPs (QP = 22, 27, 32, 37) and enhancement layer is coded lossless.

The complete results are shown in the attached document, but following conclusions could be made:

- Compared to single layer bitstream, the additional bitrate scalable bitstream uses is very small (on average 2.7% additional bitrate is needed). This means the use-cases mentioned in Section 1 can be supported with very small penalty (at low QPs, the additional penalty is actually very low, around 0.3% additional bitrate)

- Compared to simulcast, the proposed method improves the coding efficiency more than 15%.

We then implemented an example lossless coding tool (Sample based Angular Prediction) on SHVC test model and run an additional experiment. This experiment shows that high coding efficiency for lossless coding is achieved (more than 5% improvement is achieved) while keeping backwards compatibility with existing version 1 HEVC decoders.

# Patent rights declaration(s)

To authors best knowledge Nokia does not have any current or pending patent rights relating to the technology described in this contribution.