| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0040 |
| --- | --- |

| Title: | AHG9: Using SHVC for adaptive resolution change and efficient trick mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Hoda Roodaki-Lavasani Miska M. Hannuksela | Tel: Email: | +358 50 486 0857 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

SHVC inherently supports resolution change and inter-layer prediction; therefore it can effectively be used for traditional single-layer use-cases, where resolution change could be desired. One such use-case is Adaptive Resolution Change (ARC), which refers to dynamically changing the resolution within the video sequence without using IDR pictures. Another use-case is application of SHVC for trick mode so that downsampled pictures are used for trick mode compared to full resolution pictures to achieve reduced bitrate for trick mode. This contribution proposes high-level syntax for SHVC, so that these use-cases can be more efficiently supported

Revision 1 provides an updated specification text where among other things the definition of access unit as agreed by JCT-VC on 20th April 2013 was taken into use.

# Introduction

At the Geneva meeting, JCTVC-L0119 proposed some inter-layer prediction restrictions so that SHVC could be efficiently used for ARC. It was requested to evaluate the proposal after the SHVC working draft text was available, to better judge the impact of proposal on specification. It was also remarked that, in addition to ARC, the proposal could also be useful for supporting trick mode where downsampled pictures are used for trick mode.

## Using SHVC for ARC

ARC was earlier proposed in JCTVC-F158 and it basically refers to dynamically changing the resolution within the video sequence, without utilizing IDR pictures, mainly for video-conferencing. Some of the use-cases for ARC were listed in JCTVC-F158 and JCTVC-G264:

- Better network adaptation and error resilience: For better adaptation to changing network requirements for different content, it is desired to be able to change both the temporal / spatial resolution in addition to quality.

- Fast start: The start-up time of the session could be increased by first sending a low resolution frame and then increasing the resolution:

- Conference compose: When the person starts speaking, his/her corresponding resolution is increased. Doing this with an IDR frame causes a “blip” in the quality as IDR frames need to be coded at a relatively low quality so the delay is not increased.

As SHVC inherently includes mechanisms for resolution change, we think ARC could efficiently be supported as indicated in the following Figure.

Figure 1 Illustration of using scalable video coding for ARC

In the above figure, switching happens at picture 3 and the decoder receives the bitstream with following pictures: BL0-BL1-BL2-BL3-EL3-EL4-EL6-EL6…

Full experimental results were provided in JCTVC-L0119, where the test-set was encoded in IPPP settings and switching from low resolution to high resolution was simulated for different QP settings. Consider the example in Figure 2, where switching from 720p to 1080p is simulated at the middle of the Parkscene sequence and the number of bits used at switching point for ARC and anchor are shown. As expected, the bitrate at switching point is reduced significantly using ARC.

Figure 2 Number of bits used for ARC (switching happens using SHVC) and anchor (switching happens using an IDR picture)

## Using SHVC for trick mode with downsampled pictures

It is well known that the coding efficiency of HEVC is improved more for P- and B- pictures compared to that of I- pictures. It was argued earlier that this phenomenon poses a problem for streaming trick mode pictures as the bitrate for trick mode will be relatively higher in HEVC compared to that of earlier standards. JCTVC-J0293 indicated that one way to solve this problem is “Explore a simplified SVC extension of HEVC where the layers below the top layer consist only of occasional lower-resolution I-frames, while the main bitstream contains a near-constant sequence of P and B-frames at full resolution”.

The benefit of using SHVC is illustrated using the following figure. In Figure 3.a, red pictures show intra frames which are also used for achieving trick mode operation. As mentioned above, streaming trick mode would result in a relatively higher bitrate for HEVC compared to that of earlier standards. On way to alleviate this problem is to store a separate trick mode track, as shown in Figure 3.b. (pictures TR0, TR4 and TR8), but this would result in a higher file size. However, SHVC could be used so that IRAP pictures of high resolution video track can be predicted from base layer low resolution pictures as shown in Figure 3.c. This solution would achieve lower bitrate for trick mode without increasing the file size.

...

...

0

1

2

3

4

5

6

7

8

# (a)

...

...

0

1

2

3

4

5

6

7

8

TR0

TR4

TR8

# (b)

...

...

0

1

2

3

4

5

6

7

8

TR0

TR4

TR8

(c)

(c)

Figure 3 Different options for supporting trick mode. Traditional trick mode is illustrated in (a) where the intra pictures of the video track are used for trick mode. In (b) a separate downsampled trick mode track is used and (c) illustrates using SHVC that allows prediction from downsampled pictures.

The benefit of using SHVC is illustrated as follows. Let’s consider the Parkscene sequence coded with random access configuration at QP=32. The video track bitrate at full resolution is 1.5 Mbps and the trick mode bitrate is 3.6 Mbps (assuming trick mode pictures are streamed at 5 fps). If the same bitrate is desired for trick mode, then the viewers would experience a slower preview of the video (i.e. trick mode would need to be streamed at a lower frame rate). However, if downsampled pictures are used for trick mode, the trick mode bitrate is reduced to 1.3 Mbps so the trick mode doesn’t consume higher bandwidth compared to video track. Using SHVC would allow achieving this without increasing the file size.

# Proposal

We propose the following high level features so that SHVC could be used with the use-cases described above:

- A coded video sequence constrained to perform the ARC operation is signaled in VPS (single_layer_for_non_irap_flag), specifically indicating:

  - except for IRAP pictures in enhancement layer, each AU in the sequence contains a single picture from a single layer (which may or may not be BL picture)

  - Access units containing IRAP picture of EL include pictures from two layers and inter-layer scalability tools may be used.

Indicating these restrictions in VPS allows the decoder to know that scalability is not used except for achieving resolution change, so that the decoder resources can be allocated accordingly upon session start. In addition, an SHVC decoder needs to know that some access units do not necessarily contain BL pictures. This is useful for the following case. Let’s assume an SHVC decoder has the capability to decode a lower resolution bitstream (i.e. 720p). If ARC is used and switching happens from 720p to 1080p, there should be a mechanism to indicate the decoder that it won’t be able to decode and display all the BL pictures.

- Specification text

The additions in the vps_extension syntax are indicated in red:

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| single_layer_for_non_irap_flag | u(1) |
| } |  |

single_layer_for_non_irap_flag equal to 1 indicates either that all the VCL NAL units of an access unit have the same nuh_layer_id value or that two nuh_layer_id values are used by the VCL NAL units of an access unit and the picture with the greater nuh_layer_id value is an IRAP picture. single_layer_for_non_irap_flag equal to 0 indicates that nuh_layer_id values may or may not be constrained beyond constraints specified in other parts of this Recommendation | International Standard.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).