| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR 18-26 Apr. 2013 | Document: JCTVC-M0042 |
| --- | --- |

| Title: | No display SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang 433 Hackensack Ave Hackensack, NJ 07726 USA | Tel: Email: | {jill,danny,wonkap}@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

A “no display” SEI message is proposed, which indicates that a coded picture be decoded but not displayed.  JCTVC-L0179 proposed the same SEI message as one of several options to enable a middle box to make a change to a coded bitstream to indicate that a particular picture not be displayed.  During the Geneva meeting, discussion on several SEI messages was postponed until after the technical freeze of HEVC version 1.  The meeting notes for the review of JCTVC-L0179 note at the end, “Tentative plan is to consider creating an SEI message in a future version.”  The proposed SEI message is suggested for adoption for the next HEVC release.

# Problem

In SVC, output_flag is included in the NAL unit header SVC extension.  When its value is 0, the decoder will decode the coded picture, but not output it.

In the current HEVC draft specification, pic_output_flag is conditionally located in the slice segment header, dependent on a presence flag in the PPS.  The pic_output_flag location follows variable length coded syntax elements in the slice segment header.  This location places a higher burden on the middle box to change the value of the output_flag than was the case in SVC, and reduces loss robustness.  The middle box must parse all of the preceding syntax elements in the slice segment header, including VLC-coded syntax elements, parse the stored PPS to determine if pic_output_flag is present, and then change the value of the pic_output_flag.  If any packet errors occurred in the path from the encoder to the middle box, and the middle box misses a PPS, the middle box could accidentally inject an error into a coded packet when attempting to change the value of a pic_output_flag syntax element that wasn’t present in the packet.

The current HEVC draft specification uses a PicOutputFlag variable, which is either assigned the value of pic_output_flag or its value is inferred for some picture coding orders associated with RAPs.  The PicOutputFlag variable is used in the HRD operation.

From the HM9 draft specification:

PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and the previous RAP picture in decoding order is a BLA picture or is a CRA picture that is the first coded picture in the bitstream, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

For applications which require that the middle box change the display/no display characteristic of individual pictures of an already coded bitstream, the existing output_flag syntax element is not an appropriate solution.

# Proposal

A no display SEI message is proposed, which may be associated with individual coded pictures.  A middle box may easily insert this SEI message to indicate that a particular coded picture is intended to not be displayed.

The proposed SEI message has no payload data.  It applies only to the current access unit, and does not have persistence to other access units.

The proposed SEI message may be repeated in the access unit, and is a prefix SEI message.  When the no display SEI message is present, the current coded picture is not displayed.

## No display SEI message syntax and semantics.

| no_display( payloadSize ) { | Descriptor |
| --- | --- |
| } |  |

- D.3.25  No display SEI message semantics

The no display SEI message indicates that the current picture should not affect the content of the display.

Table D-1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| .. |  |
| No display | - The access unit containing the SEI message |

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| … |  |
| else if( payloadType = = 134 ) |  |
| region_refresh_info( payloadSize ) |  |
| else if( payloadType = = 135 ) |  |
| no_display( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) |  |
| … |  |

# Patent rights declaration(s)

Vidyo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).