| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR 18-26 Apr. 2013 | Document: JCTVC-M0043 |
| --- | --- |

| Title: | Layers present SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang 433 Hackensack Ave Hackensack, NJ 07726 USA | Tel: Email: | {jill,danny,wonkap}@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

A layers present SEI message is proposed for SHVC and MV-HEVC.  Its function is similar to the layers not present SEI message in SVC, but with modified syntax.  Its purpose is to enable a middle box to communicate to a decoder when layers are removed during sub-bitstream extraction.  The proposed SEI message uses only fixed length coding, which is convenient for use by cascaded middle boxes.   For MV-HEVC, layer set present flag variable values may be derived using the syntax in the proposed SEI message, providing similar functionality to MVC’s operation point not present SEI message.

# Proposal

A middle box may perform sub-bitstream extraction on a coded bitstream, removing layers.  It is useful for a middle box to be able to communicate to a decoder when individual layers are removed.  SVC includes a layers not present SEI message which indicates layers that have been removed.  MVC includes an operation point not present SEI message.  The proposed SEI message syntax can be used for similar functions for SHVC and MV-HEVC.

The proposed layers present SEI message uses a layer mask approach to indicate for each possible layer whether or not the layer may be present in the coded bitstream, similar the layer mask approach used in the VPS for layer sets and layer dependency relationships.  Only fixed length coding is used, to enable easy modification by cascaded middle boxes.  For example a first middle box may remove one or more layers and insert a layers present SEI message.   A second middle box may remove one or more additional layers, and easily modify the contents of the existing layers present SEI message.  The proposed SEI message is a prefix SEI message.

The proposed semantics language requires that the layers present SEI message be included in a NAL unit with a nuh_layer_id which corresponds to a layer that is present in the bitstream.  The proposed semantics language also requires that all direct dependent reference layers of a present layer must also be present in the bitstream.  For example, if layer 1 is a direct dependent reference layer for inter-layer prediction of layer 2, a conforming bitstream in which layer 2 is present must also have layer 1 present.

For MV-HEVC,  LayerSetPresentFlag[ i ] variable values are derived from the syntax elements in the proposed SEI message and the VPS, which indicate which layer sets are present, similar to operation point presence information provided from MVC’s operation point not present SEI message.

## Proposed syntax and semantics

| layers_present( payloadSize ) { |  |
| --- | --- |
| lp_sei_active_vps_id | u(4) |
| for( i = 0; i <= vps_max_layers_minus1 ; i++ ) |  |
| layer_present_flag[ i ] | u(1) |
| } |  |

- Layers present SEI message semantics

The layers present SEI message provides a mechanism for signalling that NAL units of particular layers indicated by the video parameter set extension are present or not present in a particular set of access units.

When present, the layers present SEI message applies to the target access unit set that consists of the current access unit and all the subsequent access units, in decoding order, until the next layers present change SEI message or the end of the CVS, whichever is earlier in decoding order.

A layers present SEI message shall not be included in a scalable nesting SEI message.

A layers present SEI message shall not be included in a NAL unit with TemporalId > 0.

lp_sei_active_vps_id identifies an active VPS that contains the information about the layers in the CVS. The value of lp_sei_active_vps_id shall be equal to the value of video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

layer_present_flag[ i ] equal to 1 specifies that there may or may not be NAL units in the target access units with nuh_layer_id  equal to layer_id_in_nuh[ i ].  layer_present_flag[ i ] equal to 0 specifies that there shall be no NAL units in the target access units with nuh_layer_id equal to layer_id_in_nuh[ i ].

When layer_present_flag[ i ] equal to 1 and i greater than 0, layer_present_flag[ RefLayerId[ i ][ j ] ] shall be equal to 1 for j in 0 .. NumDirectRefLayers[ i ] − 1.

The variable LayerSetPresentFlag[ i ] is derived as follows.

for (i = 0; i <= vps_num_layer_sets_minus1; i++) {

LayerSetPresentFlag[ i ] = 1

for (j = 0; j <= vps_max_layer_id ; j++) {

if ( layer_id_included_flag[ i ][ j ] )

LayerSetPresentFlag[ i ] &= layer_present_flag[ j ]

}

}

Table D-1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| .. |  |
| Layers present | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a layers present SEI message |

# Background

## SVC layers not present SEI message

| layers_not_present( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_layers | 5 | ue(v) |
| for( i = 0; i < num_layers; i++ ) { |  |  |
| layer_id[ i ] | 5 | ue(v) |
| } |  |  |
| } |  |  |

- Layers not present SEI message semantics

The layers not present SEI message provides a mechanism for signalling that NAL units of particular scalable layers indicated by the preceding scalability information SEI message are not present in a particular set of access units.

In the following specification of this subclause, the terms scalable layer and primary coded VCL NAL unit are used as specified in subclause Error: Reference source not found.

A layers not present SEI message shall not be included in a scalable nesting SEI message.

A layers not present SEI message shall not be present in an access unit that does not belong to the target access unit set of any scalability information SEI message. A layers not present SEI message shall not be succeeded, in decoding order, by a scalability information SEI message inside the same access unit. The set of access units consisting of the access unit associated with the layers not present SEI message and all succeeding access units in decoding order until, but excluding, the next access unit that contains a layers not present SEI message or that does not contain any primary coded VCL NAL units with IdrPicFlag equal to 0 (if present), or the end of the bitstream (otherwise) is referred to as the target access unit set. The layers not present SEI message applies to the target access unit set.

A layers not present SEI message refers to the most recent scalability information SEI message in decoding order. Each scalable layer that is referred to in this subclause is a scalable layer indicated in the most recent scalability information SEI message in decoding order. Each layer identifier for a scalable layer that is referred to in this subclause is a layer identifier for a scalable layer indicated in the most recent scalability information SEI message in decoding order.

NOTE 1 – Layers not present SEI messages do not have a cumulative effect.

num_layers specifies the number of syntax elements layer_id[ i ] that are present in the layers not present SEI message. The value of num_layers shall be in the range of 0 to 2047, inclusive.

layer_id[ i ] indicates the layer identifier of a scalable layer for which no VCL NAL units are present in the target access unit set. The value of layer_id[ i ] shall be in the range of 0 to 2047, inclusive. The value of layer_id[ i ] shall be equal to one of the values of layer_id[ i ] in the most recent scalability information SEI message. The target access unit set shall not contain any VCL NAL unit of the scalable layer having a layer identifier equal to layer_id[ i ]. When i is greater than 0, the value of layer_id[ i ] shall not be equal to any of the values of layer_id[ j ] with j = 0..(i − 1).

NOTE 2 – When an application removes NAL units from a scalable bitstream, e.g. in order to adapt the bitstream to a transmission channel or the capabilities of a receiving device, and keeps the present layers not present SEI messages, it might need to modify the content of some of the layers not present SEI messages and remove some other layers not present SEI messages in order to obtain a bitstream conforming to this Recommendation | International Standard.

## MVC operation point not present SEI message

- H.13.1.8 Operation point not present SEI message syntax

| operation_point_not_present( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| num_operation_points | 5 | ue(v) |
| for( k = 0; k < num_operation_points; k++ ) |  |  |
| operation_point_not_present_id[ k ] | 5 | ue(v) |
| } |  |  |

- H.13.2.8 Operation point not present SEI message semantics

This SEI message indicates operation points that are not present in the bitstream starting with the current access unit, and is interpreted with respect to the previous view scalability information SEI message in decoding order. The message remains effective until the next SEI message of the same type or the end of the coded video sequence, whichever is earlier in decoding order.

NOTE 1– Operation point not present SEI messages do not have a cumulative effect.

NOTE 2 – Any operation point identified by a value of operation_point_id[ i ] in the previous view scalability information SEI message, in decoding order, and not identified by a value of operation_point_not_present_id[ k ] must be present in the coded video sequence. Therefore, when an application keeps an operation point not present SEI message in a sub-bitstream extracted according to the process specified in subclause Error: Reference source not found, the application may need to change the content of the operation point not present SEI message according to the semantics.

num_operation_points specifies the number of operation points that are indicated not to be present by the SEI message. num_operation_points equal to 0 indicates that all operation points indicated by the view scalability information SEI message are present. The value of num_operation_points shall be in the range of 0 to the value of num_operation_points_minus1 in the previous view scalability information SEI message in decoding order, inclusive.

operation_point_not_present_id[ k ] identifies an operation point that is not present. operation_point_not_present_id[ k ] shall be equal to the value of one of the operation_point_id[ i ] syntax elements of the previous view scalability information SEI message in decoding order. The value of operation_point_not_present_id[ k ] shall be in the range of 0 to 65535, inclusive.

# Patent rights declaration(s)

Vidyo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).