| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR 18-26 Apr. 2013 | Document: JCTVC-M0044 |
| --- | --- |

| Title: | Layers display info SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang 433 Hackensack Ave Hackensack, NJ 07726 USA | Tel: Email: | {jill,danny,wonkap}@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

A layers display info SEI message is proposed which contains display resolution information about the scalable layers in the bitstream which is useful to a decoder, but which is not present in the VPS.  The coded resolution and cropping parameters already present in the SPS for each layer are sufficient to derive the display resolution, but if a layer is removed by a middle box, the decoder may not be able to access the SPS associated with the removed layer.  The parameters proposed for inclusion in this SEI message are similar to parameters included in SVC’s scalability info SEI message.

# Proposal

The proposed layers display info SEI message contains display resolution information about scalable layers which is useful to a decoder, but which is not included in the VPS extension.  Profile and level information for each layer is included in the VPS extension, but the syntax elements for coded resolution and cropping parameters of each layer are not in the VPS, but rather are located in each layer’s SPS.  If a layer is removed by a middle box, the decoder may not be able to access the SPS associated with that removed layer.

At the initiation of decoding of a bitstream, it is useful for a decoder to know the display resolution of all layers that may be present in the bitstream, even if an individual layer were removed by a middle box for a period of time.   At a future point in time the middle box may stop removing the particular layer, and forward the layer to a decoder.  A decoder may choose to display all decoded pictures at the highest possible display resolution expected in the bitstream, by upsampling decoded pictures of lower resolutions prior to display, in order to avoid changing the display resolution over time.

The proposed SEI message contains parameters for display resolution for each of the layers in the CVS.  The conformance cropping window width and height are restricted to be equal to calculated value determined by SPS parameters for the picture size, i.e., pic_width_in_luma_samples and pic_height_in_luma_samples, and the conformance cropping window parameters, conf_win_left_offset,  conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset.  Sample aspect ratio parameters can also be optionally sent, and when present must match the values in the VUI.

## Proposed syntax and semantics

| layers_display_info( payloadSize ) { |  |
| --- | --- |
| ldi_sei_active_vps_id | u(4) |
| for( i = 0; i <= vps_max_layers_minus1 ; i++ ) { |  |
| conf_win_width_in_luma_samples[ i ] | u(16) |
| conf_win_height_in_luma_samples[ i ] | u(16) |
| ldi_sei_aspect_ratio_info_present_flag[ i ] | u(1) |
| if( ldi_sei_aspect_ratio_info_present_flag[ i ] ) { |  |
| ldi_sei_aspect_ratio_idc[ i ] | u(8) |
| if( ldi_sei_aspect_ratio_idc = = Extended_SAR ) { |  |
| ldi_sei_sar_width[ i ] | u(16) |
| ldi_sei_sar_height[ i ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |

- Layers display info SEI message semantics

The layers display info SEI message provides display information about layers in the CVS and is interpreted with respect to the active video parameter set. When present, the layers display info SEI message applies to the target access unit set that consists of the current access unit and all the subsequent access units, in decoding order, until the next layers display info SEI message or the end of the CVS, whichever is earlier in decoding order.

ldi_sei_active_vps_id identifies an active VPS that contains layer information. The value of ldi_sei_active_vps_id shall be equal to the value of video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

conf_win_width_in_luma_samples[ i ] shall be equal to pic_width_in_luma_samples − conf_win_left_offset − conf_win_right_offset in the active SPS of coded pictures with nuh_layer_id equal to layer_id_in_nuh[ i ].

conf_win_height_in_luma_samples[ i ] shall be equal to pic_ height _in_luma_samples − conf_win_top_offset − conf_win_bottom_offset in the active SPS of coded pictures with nuh_layer_id equal to layer_id_in_nuh[ i ].

ldi_sei_aspect_ratio_info_present_flag[ i ] equal to 1 specifies that ldi_sei_aspect_ratio_idc[ i ] is present. aspect_ratio_info_present_flag equal to 0 specifies that ldi_sei_aspect_ratio_idc[ i ]  is not present.

ldi_sei_aspect_ratio_idc [ i ], when present,  shall be equal to aspect_ratio_idc in the active SPS of coded pictures with nuh_layer_id equal to layer_id_in_nuh[ i ].

ldi_sei_ sar_width [ i ], when present,   shall be equal to sar_width in the active SPS of coded pictures with nuh_layer_id equal to layer_id_in_nuh[ i ].

ldi_sei_ sar_height [ i ], when present,   shall be equal to sar_height in the active SPS of coded pictures with nuh_layer_id equal to layer_id_in_nuh[ i ].

Table D-1 – Persistence scope of SEI messages (informative)

| - SEI message | - Persistence scope |
| --- | --- |
| .. |  |
| Layers display info | - The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains a layers display info SEI message |

# Background - SVC scalability info SEI message

SVC included a scalability info SEI message which optionally contained for each scalable layer the width and height of the coded picture in MBs.  A portion of the syntax table is copied below.

| scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i <= num_layers_minus1; i++ ) { |  |  |
| … |  |  |
| iroi_division_info_present_flag[ i ] | 5 | u(1) |
| … |  |  |
| frm_size_info_present_flag[ i ] | 5 | u(1) |
| … |  |  |
| if( frm_size_info_present_flag[ i ] ¦ ¦ iroi_division_info_present_flag[ i ] ) { |  |  |
| frm_width_in_mbs_minus1[ i ] | 5 | ue(v) |
| frm_height_in_mbs_minus1[ i ] | 5 | ue(v) |
| } |  |  |
| … |  |  |

# Patent rights declaration(s)

Vidyo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).