| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR 18-26 Apr. 2013 | Document: JCTVC-M0045 |
| --- | --- |

| Title: | High-level syntax modifications for HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang 433 Hackensack Ave Hackensack, NJ 07726 USA | Tel: Email: | {jill,danny,wonkap}@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

Several high level syntax design philosophy and/or syntax changes common to extensions to HEVC, including SHVC, MV-HEVC and 3D-HEVC, are proposed, with respect to the SHVC Test Model JCTVC-L1007, SHVC Working Draft JCTVC-L1008, MV-HEVC PDAM JCT3V-C1004, and 3D-HEVC Test Model JCT3V-C1005.

The proposals are:

- Allow different frame rate for different layers

- Define an access unit as containing pictures from multiple layers, as in SVC and MVC

- Conditionally include VPS extension syntax elements based on scalability_mask[ i ]

- Add extension_type_flag[ i ] syntax elements in SPS extension and use for conditional syntax elements

- Conditionally include syntax elements in slice segment header based on ViewId[ nuh_layer_id ], DepthFlag[ nuh_layer_id  ], DependencyId[ nuh_layer_id  ]

- Profile constraints for Stereo main and proposed Scalable main profiles to disallow scalability combinations

# Proposals

## Allow different frame rate for different layers

It is desirable to allow an enhancement layer to have a different frame rate than a reference layer.  In particular, it is useful to allow a higher frame rate for the enhancement layer than for the base layer, e.g. a 1280x720p30 base layer and a 1920x1080p60 enhancement layer.

The proposed access unit definition in the next section 1.2 enables this use case.  The intent in the current SHVC design is unclear on this point.

## Access unit definition

We propose that an access unit be defined as containing coded pictures from multiple layers.

### Background

#### MV-HEVC

In the MV-HEVC PDAM (JCT3V-C1004), there is ambiguity about the appropriate definition of access unit, whether it should contain a single picture from a single layer, or contain pictures from multiple layers, similarly to what was done in MVC, as reflected in this editor’s note.

[Ed. (MH): The present version of this annex uses the same definition for an access unit as in clause 3, which essentially states that an access unit contains one coded picture (with a particular value of nuh_layer_id). One coded picture is defined below essentially identically to a view component in MVC. It is an open issue whether an access unit should instead be defined to contain all view components with the same POC value.]

#### HEVC version 1

In HEVC version 1 (JCTVC-L1003), the scalable nesting SEI message semantics imply that an access unit may contain multiple layers, e.g. contain NAL units with different values of nuh_layer_id, as shown by the semantics for all_layers_flag, which include the words  “consists of all values of nuh_layer_id present in the current access unit…”

all_layers_flag equal to 0 specifies that the list nestingLayerIdList[ 0 ] is specified by nesting_layer_id[ i ] for all i values in the range of 0 to nesting_num_layers_minus1, inclusive. all_layers_flag equal to 1 specifies that the list nestingLayerIdList[ 0 ] consists of all values of nuh_layer_id present in the current access unit that are greater than or equal to nuh_layer_id of the current SEI NAL unit, in increasing order of the values.

#### MVC

MVC defined an access unit as following:

access unit: A set of NAL units that are consecutive in decoding order and contain exactly one primary coded picture consisting of one or more view components. In addition to the primary coded picture, an access unit may also contain one or more redundant coded pictures, one auxiliary coded picture, or other NAL units not containing slices or slice data partitions of a coded picture. The decoding of an access unit always results in one decoded picture consisting of one or more decoded view components.

view component: A coded representation of a view in a single access unit.

MV-HEVC includes the following definition of a coded picture, which differs from MVC.

- coded picture: A coded representation of a picture comprising VCL NAL units with a particular value of nuh_layer_id and containing all coding tree units of the picture.

### Proposed definition for extensions

We propose the following definition for MV-HEVC and SHVC, such that an access unit be defined as containing coded pictures from multiple layers, and allowing different frame rates for the different layers.  Changes are marked with respect to HEVCv1 (JCTVC-L1003).

  - access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain exactly one coded picture contains zero or one coded pictures per layer, and contains a coded picture for at least one layer.

## Conditionally include VPS extension syntax elements based on scalability_mask[ i ]

Through the scalability_mask[ i ] syntax element in the VPS extension, it is known which types of scalability are included in the bitstream.   The mapping table is copied below.

| scalability_mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | multiview | ViewId |
| 1 | spatial/SNR scalability | DependencyId |
| 2-15 | Reserved |  |

Syntax elements directed only at some types of scalability and not others may be conditionally included in the VPS extension based upon which types of scalability are present.  Future types of scalability may still be combined with existing types (view and Spatial/SNR) in a backwards compatible manner.

In particular, the output layer sets syntax elements are used for view scalability but not required for spatial/SNR scalability.  It is proposed to make the presence of those syntax elements conditional upon view scalability being used, corresponding to scalability_mask[ 0 ] equal to 1.  In the future after depth map based scalability is defined, the syntax elements could be present when either scalability_mask[ 0 ] or scalability_mask[ 1 ] is equal to 1.

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( scalability_mask[ 0 ]) { |  |
| num_output_layer_sets | ue(v) |
| for( i = 0; i < num_output_layer_sets; i++ ) { |  |
| output_layer_set_idx[ i ] | ue(v) |
| lsIdx = output_layer_set_idx[ i ] |  |
| for( j = 0 ; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ lsIdx ][ j ] ) |  |
| output_layer_flag[ lsIdx ][ j ] | u(1) |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |

num_output_layer_sets specifies the number of layer sets for which output layers are specified with output_layer_set_index[ i ] and output_layer_flag[ lsIdx ][ j ]. When not present, the value of num_output_layer_sets is inferred to be equal to 0.

When num_output_layer_sets not present, there is a single target output layer with nuh_layer_id equal to layer_id_in_nuh[vps_max_layers_minus1].

## Add extension_type_flag[ i ] syntax elements in SPS extension and use for conditional syntax elements

The SHVC Test Model includes a syntax element in the sequence parameter set extension, sps_inter_layer_mfm_enable_flag, which is not included in the MV-HEVC PDAM, which causes a backwards compatibility problem.

In order to avoid parsing dependency between the SPS and the VPS, the presence of these syntax elements in the SPS extension cannot be conditional based upon scalability_mask.  We propose adding three extension_type_flag syntax elements in the SPS extension, similar to the first three bits of the scalability_mask[ i ] in the VPS extension, for view scalability, spatial/SNR scalablility, and depth map based scalability respectively.  For future extensions, the additional SPS extension flag, sps_extension2_flag, will be required to be used to add syntax elements to the SPS.

The proposed method allows future extensions to avoid the need to parse syntax elements targeted at other extension types not used.

In contribution JCTVC-M0046 we propose specific syntax elements to be added to the SPS extension when extension_type_flag[ 1 ] is set, e.g. when spatial/SNR scalability is used.

| sps_extension( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 3; i++ ) |  |
| extension_type_flag[ i ] | u(1) |
| if( extension_type_flag[ 0 ] ) |  |
| inter_view_mv_vert_constraint_flag | u(1) |
| if( extension_type_flag[ 1 ] ) { |  |
| sps_inter_layer_mfm_enable_flag | u(1) |
| // additional syntax elements |  |
| } |  |
| if( extension_type_flag[ 2 ] ) { |  |
| // additional syntax elements |  |
| } |  |
| } |  |

extension_type_flag[ 0 ] equal to 1 indicates that the inter_view_mv_vert_constraint_flag syntax element is present. extension_type_flag[ 0 ] equal to 0 indicates that the inter_view_mv_vert_constraint_flag syntax element is not present.

extension_type_flag[ 1 ] equal to 1 indicates that the sps_inter_layer_mfm_enable_flag syntax element is present. extension_type_flag[ 1 ] equal to 0 indicates that the sps_inter_layer_mfm_enable_flag syntax element is not present.

extension_type_flag[ 2 ] shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for extension_type_flag[ 2 ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of extension_type_flag[ 2 ].

## Conditionally include syntax elements in slice segment header based on ViewId[ i ], DepthFlag[ i ], DependencyId[ i ]

We propose to add syntax elements in the slice segment header which are present only for enhancement layers associated with some types of scalability, based upon the values of  ViewId[ i ], DepthFlag[ i ], DependencyId[ i ].  For example, by adding a DependencyId[ i ] > 0 condition, the syntax element would be present in a spatial/SNR scalable layer, but not in the base layer or in the second view layer of a Stereo main profile bitstream.  Specific proposed syntax elements are proposed in JCTVC-M0046 for spatial/SNR scalable enhancement layers.  This design philosophy is also compatible with possible future combined scalability extensions.

The new syntax elements would not require use of the slice_segment_header_extension_present_flag.  Conceptually, syntax elements may be conditionally present in the slice header when other types of scalability, e.g. view scalability or depth, without requiring use of the slice_segement_extension_present_flag, by considering the values of nuh_layer_id, and the various scalability dimensions, such as DependencyId[ nuh_layer_id ] or ViewId[ nuh_layer_id ] or DepthFlag[ nuh_layer_id ] sent in the VPS extension.  In the future, slice_segement_extension_present_flag can still be used to associate new syntax elements with the backwards compatible base layer.

Slice segment header:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( nuh_layer_id > 0 ) { |  |
| if( ViewId[ nuh_layer_id ] > 0) { |  |
| // view scalable specific syntax elements |  |
| } |  |
| if( DepthFlag[ nuh_layer_id ] > 0) { |  |
| // view scalable specific syntax elements |  |
| } |  |
| if( DependencyId[ nuh_layer_id ] > 0) { |  |
| // spatial/SNR scalable specific syntax elements |  |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| layer_used_for_inter_layer_prediction | u(v) |
| } |  |
| else |  |
| bit_equal_to_zero | u(1) |
| byte_alignment( ) |  |
| } |  |

## Profile constraints for Stereo main and Scalable main profiles to disallow scalability combinations

When additional profiles are defined for spatial/SNR scalability, the existing constraints for the Stereo main profile in the MV-HEVC PDAM (JCT3V-C1004) will not disallow combined spatial/SNR and view scalability.  For example, there is no existing constraint that would disallow a layer with ViewId = 1 and DependencyId = 1.  It may be useful in the future to allow this combined scalability in a future profile, but this is not the intent of the existing Stereo main profile.

It is proposed to add a constraint to disallow this combination in the Stereo main profile, and in a strawman Scalable main profile, in such a way that a future profile may add additional enhancement layers which reference a layer that is compliant with the Stereo main or Scalable main profile.

### Stereo main profile

Bitstreams conforming to the Stereo Main profile shall obey the following constraints:

- The sub-bitstream resulting from the sub-bitstream extraction process with any value of tIdTarget and a value of 0 in layerIdListTarget as inputs shall conform to the Main profile.

- The bitstream shall contain one layer with nuh_layer_id equal to i for which ViewId[ i ] is greater than 0.

- When ViewId[ i ] is greater than 0, inter_view_mv_vert_constraint_flag shall be equal to 1 in the sps_extension( ) syntax structure of the active layer SPS of any coded pictures with nuh_layer_id equal to i.

- When ViewId[ i ] is greater than 0, ScalabilityId[ i ][ smIdx ] shall be equal to 0 for smIdx in 1..15 for any coded pictures with nuh_layer_id equal to i.

### Scalable main profile

Bitstreams conforming to the Scalable Main profile shall obey the following constraints:

- The sub-bitstream resulting from the sub-bitstream extraction process with any value of tIdTarget and a value of 0 in layerIdListTarget as inputs shall conform to the Main profile.

- When DependencyId[ i ] is greater than 0, ScalabilityId[ i ][ smIdx ] shall be equal to 0 for smIdx in 0..1 or 3..15 for any coded pictures with nuh_layer_id equal to i.

# Patent rights declaration(s)

Vidyo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).