| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR 18-26 Apr. 2013 | Document: JCTVC-M0046 |
| --- | --- |

| Title: | High-level syntax modifications for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce, Danny Hong, Wonkap Jang 433 Hackensack Ave Hackensack, NJ 07726 USA | Tel: Email: | {jill,danny,wonkap}@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

Several high level syntax changes to the SHVC design are proposed, with respect to the combination of JCTVC-L1007 SHVC Test Model and JCTVC-L1008 SHVC Working Draft (which references JCT3V-C1004).  Some of the proposals are based upon proposals general to all extensions included in JCTVC-M0045.

- Add syntax element in slice header to signal inter-layer texture and motion prediction from different direct dependent reference layers, for TextureRL (IntraBL) mode only

- Add syntax elements in the slice segment header for enhancement layer slice skipping

- Add syntax elements in the SPS extension to enable individual inter-layer prediction coding tools

- Add syntax elements in the slice segment header for adaptive and default inter-layer tex ture prediction and enabling motion prediction

# Proposals

## Add syntax elements in slice header to signal inter-layer texture and motion prediction from different direct dependent reference layers, for TextureRL (IntraBL) mode only

The VPS extension includes syntax elements to signal one or more direct dependent reference layers which may be used for inter-layer prediction when coding an enhancement layer, for each possible value of nuh_layer_id.  However, the current SHM software and test model only support the use of one direct dependent reference layer.  SVC uses the ref_layer_dq_id syntax element in the slice header in scalable extension to indicate which of the one or more direct dependent reference layers is used by the current slice for inter-layer prediction.

We propose to add a conditional syntax element in the slice segment header to indicate which of the one or more directly dependent reference layers is used for inter-layer prediction when decoding a slice in a spatial/SNR enhancement layer using the TextureRL mode.  The proposed method is unnecessary when the ref index mode is used, as the existing RPS syntax can be used to signal prediction from different reference layers.

The proposed syntax element, inter_layer_pred_layer_idc, may be present in a spatial/SNR enhancement layer, as indicated by DependencyId[ nuh_layer_id ] > 0.  JCTVC-M0045 proposed that syntax elements related to spatial/SNR enhancement layers could be conditionally included in a spatial/SNR enhancement layer based upon DependencyId[ nuh_layer_id ] > 0.

The proposed syntax element is only present when TextureRL (IntraBL) mode inter-layer prediction is enabled, either for texture prediction or motion prediction, as indicated by the InterLayerTextureRlEnableFlag and InterLayerMotionPredictionEnableFlag variables.

An additional condition for presence of the proposed syntax element is that the VPS indicates more than one possible direct dependent reference layer, using the NumDirectRefLayers[ nuh_layer_id ] variable, the value of which is derived from syntax elements in the VPS.

If NumDirectRefLayers[ nuh_layer_id ] is equal to 0, there are no layers available for inter-layer prediction, and if NumDirectRefLayers[ nuh_layer_id ] is equal to 1, there is only one possible layer available for inter-layer prediction.   For both of those cases, the proposed inter_layer_pred_layer_idc syntax element is not signaled, because it is unnecessary, saving bits in each enhancement layer slice header vs. the method used in SVC which is signaled in all enhancement layer slices.

When NumDirectRefLayers[ nuh_layer_id ] is greater than 1, inter_layer_pred_layer_idc is signaled, using u(v) signaling.  The number of bits used to represent inter_layer_pred_layer_idc is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ]) ).  The related syntax element used in SVC, ref_layer_dq_id is ue(v) coding.

### Proposed slice segment header syntax and semantics modifications

Green indicates changes proposed in JCTVC-M0045, yellow indicates additional changes proposed in the current section.

Slice segment header:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (nuh_layer_id > 0) { |  |
| … |  |
| if( DependencyId [ nuh_layer_id ] > 0) { |  |
| if( NumDirectRefLayers[ nuh_layer_id ] > 1 && ( InterLayerTextureRlEnableFlag ¦ ¦ InterLayerMotionPredictionEnableFlag ) ) |  |
| inter_layer_pred_layer_idc | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

inter_layer_pred_layer_idc specifies that the reference layer picture, rlPic, used for inter layer texture prediction or inter layer motion prediction is the coded picture with nuh_layer_id equal to RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc].  The number of bits used to represent inter_layer_pred_layer_idc is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ]) ).

When not present, inter_layer_pred_layer_idc is inferred to be equal to 0.

When inter_layer_pred_layer_idc present, a coded picture with nuh_layer_id equal to RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc] shall be present in the current access unit.

## Add syntax elements in the slice segment header for enhancement layer slice skipping

SVC contains two syntax elements to indicate if an enhancement layer is skipped, and how many MBs are contained within the skipped slice. In a skipped slice in SVC, the MBs contained within the skipped slice are decoded using inter-layer prediction and residual prediction.  We propose similar slice skip syntax elements for use in SHVC. The proposed syntax elements could be used with either the ref_idx or the textureRL (intra BL) mode, and are described for both modes.

The proposed syntax elements, scalable_slice_skip_flag and num_ctbs_in_skipped_slice, indicate that an enhancement layer slice be skipped, and hence all LCUs use inter-layer prediction from a reference layer.   The proposed syntax elements are only present when DependencyId[ nuh_layer_id ] > 0, e.g., for a spatial/SNR enhancement layer.  There is a presence condition for the new syntax elements, that there is at least one directly dependent layer, as indicated when NumDirectRefLayers[ nuh_layer_id ] is greater than 0, i.e. the proposed syntax elements are not present when there is no direct reference layer for the current layer.  Additionally, these proposed syntax elements are only present, e.g. an enhancement layer slice can only be skipped, when inter-layer texture prediction (either ref index or TextureRL) is enabled in the SPS extension, using the flags proposed in section 1.2.

For improved coding efficiency, u(v) coding rather than ue(v) coding is used for num_ctbs_in_skipped_slice. The size in CTBs of a skipped slice is likely to be large, so ue(v) coding is generally inefficient.

Syntax and semantics for these proposed slice skip syntax elements are provided, but the decoding process for a skipped slice is left undefined, as being out-of-scope of this proposal, because the optimal solution will depend upon which combinations of inter-layer prediction tools are available in the specification.

The proposed syntax elements are similar to those present in SVC’s slice header in scalable extension, adapted for use in SHVC.  The slice skip feature in SVC has been found useful in SVC implementations.  It is useful to an encoder that is CPU starved or bitrate starved, as a low CPU/low bitrate method to encode an enhancement layer slice.  It is also useful to a decoder or middle box for error resiliency, by feeding to the decoder a NAL unit containing the syntax for a skipped slice in place of a missing coded slice, as an error concealment strategy.

### Proposed slice segment header modification

Slice segment header:

Green indicates changes proposed in JCTVC-M0045, yellow indicates changes proposed in current section.

| slice_segment_header( ) | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 ) { |  |
| if( DependencyId[ nuh_layer_id ] > 0 ) { |  |
| if ( NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| if( sps_inter_layer_texture_rl_enable_flag ¦ ¦ sps_inter_layer_ref_idx_enable_flag ) |  |
| scalable_slice_skip_flag | u(1) |
| if( scalable_slice_skip_flag) |  |
| num_ctbs_in_skipped_slice | u(v) |
| } |  |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

scalable_slice_skip_flag equal to 1 specifies that slice_segment_data( ) is not present for the current slice.  scalable_slice_skip_flag equal to 0 specifies that slice_segment_data( ) is present for the current slice.

num_ctbs_in_skipped_slice specifies the number of coding tree blocks for slice with scalable_skip_slice_flag equal to 1. The number of bits used to represent num_ctbs_in_skipped_slice  is Ceil( Log2( PicWidthInCtbsY * PicHeightInCtbsY ) )

[Ed. (JB): Need decoding process description for CUs in skipped slice.]

### Proposed update to slice segment layer to use scalable_slice_skip_flag

When a slice is skipped, the slice_segment_data( ) syntax structure is not present in the coded bitstream.

| slice_segment_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_segment_header( ) |  |
| if (!scalable_slice_skip_flag) |  |
| slice_segment_data( ) |  |
| rbsp_slice_segment_trailing_bits( ) |  |
| } |  |

## Add syntax elements in the SPS extension to enable individual inter-layer prediction coding tools

The SHM1 Test Model includes a syntax element in the sequence parameter set extension, sps_inter_layer_mfm_enable_flag, to indicate if the motion field mapping process for the ref index framework is enabled.  We propose that syntax elements be similarly added to individually signal enabling of the other scalability coding tools included in the Test Model:  ref index texture prediction, texture RL (IntraBL) prediction, and inter layer motion prediction.   The SHM Test Model includes variables to indicate which individual inter-layer prediction coding tools are enabled, without defining how those values of those variables are set, as a place holder until the necessary syntax elements are adopted into the high-level syntax design.  Proposed equations to set the values of those variables are proposed in section 1.4, making use of the syntax elements in the SPS extension proposed in this section and syntax elements in the slice segment header proposed in section 1.4.

In order to avoid parsing dependency between the SPS and the VPS, the presence of these syntax elements cannot be conditional based upon scalability_mask[ i ].  We proposed in JCTVC-M0045 that extension_type_flag[ i ] syntax elements be added in the SPS to enable conditionally present syntax elements in the SPS.  The syntax table proposed here is based upon that proposal.  Alternatively, these syntax elements could be present in the SPS extension unconditionally for all extensions.

### Proposed SPS extension syntax and semantics modifications

SPS extension: Proposal in yellow, previous proposal in JCTVC-M0045 in green.

| sps_extension( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 3; i++ ) |  |
| extension_type_flag[ i ] | u(1) |
| if ( extension_type_flag[ 0 ] ) { |  |
| inter_view_mv_vert_constraint_flag | u(1) |
| // additional syntax elements |  |
| } |  |
| if ( extension_type_flag[ 1 ] ) { |  |
| sps_inter_layer_mfm_enable_flag | u(1) |
| sps_inter_layer_ref_idx_enable_flag | u(1) |
| sps_inter_layer_texture_rl_enable_flag | u(1) |
| sps_inter_layer_motion_prediction_enable_flag | u(1) |
| } |  |
| if ( extension_type_flag[ 2 ] ) { |  |
| // additional syntax elements |  |
| } |  |
| } |  |

sps_inter_layer_ref_idx_enable_flag equal to 1 specifies the inter-layer reference index mapping process is applied as specified in subclause G.8.  sps_inter_layer_ref_idx_enable_flag equal to 0 specifies the inter-layer reference index mapping process is not applied.  When not present, the value of sps_inter_layer_ref_idx_enable_flag is inferred to be equal to 0.

sps_inter_layer_texture_rl_enable_flag equal to 1 specifies that the inter-layer texture_rl process is applied as specified in subclause H.8.  sps_inter_layer_texture_rl_enable_flag equal to 0 specifies the inter-layer texture_rl process is not applied.  When not present, the value of sps_inter_layer_texture_rl_enable_flag is inferred to be equal to 0.

When sps_inter_layer_ref_idx_enable_flag equal to 1, sps_inter_layer_texture_rl_enable_flag shall be equal to 0.

sps_inter_layer_motion_prediction_enable_flag equal to 1 specifies the inter-layer motion prediction process is applied as specified in subclause H.8.5.3.  sps_inter_layer_motion_prediction_enable_flag equal to 0 specifies the inter-layer motion prediction process is not applied.  When not present, the value of sps_inter_layer_motion_prediction_enable_flag is inferred to be equal to 0.

When sps_inter_layer_mfm_enable_flag equal to 1, sps_inter_layer_motion_prediction_enable_flag shall be equal to 0.

## Add syntax elements in the slice segment header for adaptive and default inter-layer texture prediction and enabling motion prediction

JCTVC-L0278 proposed adding a syntax element in the slice segment header to indicate if inter-layer texture prediction is used.  We propose several new syntax elements in the slice segment header for spatial/SNR enhancement layer slices, similar to those present in SVC.  Two syntax elements are proposed to indicate whether the inter-layer texture prediction tool is adaptive for each CU, or has a default value for all the CUs in the slice.  One syntax element is proposed to indicate whether the inter-layer motion prediction tool is enabled for the slice.

The proposed syntax elements could be used with either the ref_idx or the textureRL (intra BL) mode, and are described for both modes.  The proposed syntax elements are used to set the values of the placeholder variables in the SHM Test Model which indicate which inter-layer prediction coding tools are enabled.

The position of the new syntax elements in the proposed slice segment header syntax table is based upon JCTVC-M0045, but could be positioned differently.  The proposed syntax elements are only present when DependencyId[ nuh_layer_id ] > 0, e.g., for a spatial/SNR enhancement layer. There is also a presence condition for the new syntax elements, that there is at least one directly dependent layer, as indicated when NumDirectRefLayers[ nuh_layer_id ] is greater than 0, i.e. the proposed syntax elements are not present when there is no direct reference layer for the current layer.

The proposed syntax elements adaptive_inter_layer_texture_flag and default_inter_layer_texture_flag behave similarly to adaptive and default flags in SVC’s slice header in scalable extension.   These syntax elements are only present when one of the inter-layer texture prediction SPS enable flags is set, for either the ref index or TextureRL (intra BL) mode, using the SPS extension syntax elements proposed in section 1.3.  The values of the placeholder variables in the Test Model are set based upon the values of adaptive_inter_layer_texture_flag and default_inter_layer_texture_flag and the values of the SPS extension syntax elements proposed in section 1.3.

For the TextureRL (intra BL) mode, texture_rl_flag[ x0 ][ y0 ] in the CU is only signaled when adaptive_inter_layer_ texture_flag is equal to 1.  Otherwise, (adaptive_inter_layer_texture_flag is equal to 0), the value of texture_rl_flag[ x0 ][ y0 ] is inferred to be equal to default_inter_layer_texture_flag for all CUs in the slice.

For the ref_idx mode, there are no syntax changes associated with these flags, but the decoding process is impacted.  When adaptive_inter_layer_texture_flag and default_inter_layer_texture_flag are equal to 0, the reference picture list construction process for single layer decoding is used, so that the inter layer reference picture set is not included in the current list.  When default_inter_later_texture_flag is equal to 1, only the inter layer ref pic set is included in the current list.

For the proposed motion prediction syntax element there is a presence condition that one of the inter-layer motion prediction SPS enable flags be set.  enable_inter_layer_motion_flag is present when the inter-layer motion prediction SPS enable flags are set, for either the inter-layer motion prediction associated with the intra_BL mode or motion field mapping associated with the ref_idx mode, to indicate whether the particular inter layer motion prediction mode method is used in the decoding process, or if the single layer motion prediction method is used.   The values of the placeholder variables in the Test Model are set based upon the value of enable_inter_layer_motion_flag and the values of the SPS extension syntax elements proposed in section 1.3.

The proposed syntax elements are similar to those present in SVC’s slice header in scalable extension, adapted for use in SHVC.  These features in SVC have been found useful in SVC implementations.  They provide flexibility to an encoder to always, sometimes, or never use inter-layer prediction tools.  These features are useful to an encoder that is sometimes CPU-starved, and are coding efficient, especially when the bitrate available to encode an enhancement layer is low.

### Proposed slice segment header modification

Green indicates changes proposed in JCTVC-M0045 and previous sections, yellow indicates changes proposed in current section.

| slice_segment_header( ) | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 ) { |  |
| if( DependencyId[ nuh_layer_id ] > 0 ) { |  |
| if ( NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| if( sps_inter_layer_texture_rl_enable_flag ¦ ¦ sps_inter_layer_ref_idx_enable_flag ) |  |
| scalable_slice_skip_flag |  |
| if( scalable_slice_skip_flag) |  |
| num_ctbs_in_skipped_slice | u(v) |
| else { |  |
| if( sps_inter_layer_texture_rl_enable_flag ¦ ¦ sps_inter_layer_ref_idx_enable_flag ) { |  |
| adaptive_inter_layer_ texture_flag | u(1) |
| if( !adaptive_inter_layer_ texture_flag ) |  |
| default_inter_layer_ texture_flag | u(1) |
| } |  |
| if( sps_inter_layer_motion_prediction_enable_flag ¦ ¦ sps_inter_layer_mfm_enable_flag ) |  |
| enable_inter_layer_motion_flag | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

adaptive_inter_layer_texture_flag equal to 1 specifies if the inter-layer texture prediction mode is signaled for coding units in the current slice.  adaptive_inter_layer_texture_flag equal to 0 specifies the inter-layer texture prediction mode is not signaled for coding units in the current slice.  When not present, the value of adaptive_inter_layer_texture_flag is inferred to be equal to 0.

default_inter_layer_texture_flag equal to 1 specifies if inter-layer texture prediction is performed for the current slice.  default_inter_layer_texture_flag equal to 0 specifies inter-layer texture prediction is not performed for the current slice.  When not present, the value of default_inter_layer_texture_flag is inferred to be equal to 0.

The variables InterLayerTextureRlEnableFlag and InterLayerRefIdxEnableFlag are derived as follows.

InterLayerTextureRlEnableFlag =  (adaptive_inter_layer_texture_flag  | | default_inter_layer_texture_flag) && sps_inter_layer_texture_rl_enable_flag

InterLayerRefIdxEnableFlag  =  (adaptive_inter_layer_texture_flag  | | default_inter_layer_texture_flag)  && sps_inter_layer_ref_idx_enable_flag

When InterLayerTextureRlEnableFlag equal to 1, the Decoding process for coding units coded in textureRL prediction mode is applied as specified in subclause H.8.8.  When InterLayerRefIdxEnableFlag equal to 1, the inter-layer ref index prediction process is applied as specified in subclause G.8.3.

enable_inter_layer_motion_flag equal to 1 specifies that inter-layer motion prediction may be performed for the current slice.  enable_inter_layer_motion_flag equal to 0 specifies that inter-layer motion prediction is not performed for the current slice.

The variables InterLayerMotionPredictionEnableFlag and InterLayerMfmEnableFlag are derived as follows.

InterLayerMotionPredictionEnableFlag  =  sps_inter_layer_motion_prediction_enable_flag && enable_inter_layer_motion_flag )

InterLayerMfmEnableFlag  =  sps_inter_layer_mfm_enable_flag && enable_inter_layer_motion_flag

When InterLayerMotionPredictionEnableFlag equal to 1, the derivation process for motion vector components and reference indices process is applied as specified in subclause H.8.5.3.1.  InterLayerMfmEnableFlag equal to 1specifies the motion field mapping process is applied as part of the resampling process for reference pictures specified in subclause Error: Reference source not found

### Proposed update to coding unit syntax to use adaptive and default inter layer texture flags for TextureRL (IntraBL) mode

The texture_rl_flag[ x0 ][ y0 ] syntax element is only present when adaptive_inter_layer_texture_flag is equal to 1.  When adaptive_inter_layer_texture_flag is equal to 0 and default_inter_layer_texture_flag is equal to 1, the texture_rl_flag[ x0 ][ y0 ] syntax element is not present, and textureRL mode is inferred to be in use.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if (nuh_layer_id > 0 && InterLayerTextureRlEnableFlag && adaptive_inter_layer_texture_flag ) |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if( !texture_rl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| … |  |

texture_rl_flag[ x0 ][ y0 ] specifies the prediction mode for the current coding unit. texture_rl_flag[ x0 ][ y0 ] equal to 1 specifies that the prediction samples are derived using reconstructed samples of the reference layer representation as specified in subclause ; texture_rl_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded in either intra prediction or inter prediction mode.

When texture_rl_flag[ x0 ][ y0 ] not present, it is inferred to be equal to default_inter_layer_texture_flag.

When texture_rl_flag is equal to 1, the variable PredMode[ x ][ y ] and PartMode[ x ][ y ] are derived as follows.

for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS – 1,

PredMode[ x ][ y ] = MODE_TEXTURE_RL		(H-xx)

PartMode [ x ][ y ] = PART_2Nx2N		(H-xx)

When texture_rl_flag[ x0 ][ y0 ] not present, it is inferred to be equal to 0.

### Proposed update to reference picture list construction decoding process to use adaptive and default inter layer texture flags for ref_idx mode

The adaptive_inter_layer_texture_flag and default_inter_layer_texture_flag values do not affect the syntax table when the ref_idx mode is used, but affect the reference picture list construction.

When the adaptive_inter_layer_texture_flag is equal to 1, the method of list construction described in the SHM Test Model is used.  When the adaptive_inter_layer_texture_flag is equal to 0 and default_inter_layer_texture_flag is equal to 0, the single layer HEVC list construction method is used and the inter-layer reference picture set is not included in the list.   When the adaptive_inter_layer_texture_flag is equal to 0 and default_inter_layer_texture_flag is equal to 1, a new list construction method is proposed where only the inter-layer reference picture set is included in the list.

G.8.3.4 Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause Error: Reference source not found. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	if ( adaptive_inter_layer_texture_flag | | !default_inter_layer_texture_flag ) {
		for( i = 0; i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(G-xx)
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	}
	if ( adaptive_inter_layer_texture_flag | | default_inter_layer_texture_flag ) {
		for( i = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] 
	}
}

The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			(G-xx)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
		RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	if ( adaptive_inter_layer_texture_flag | | !default_inter_layer_texture_flag ) {
		for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(G-xx)
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ] 
	}
	if ( adaptive_inter_layer_texture_flag | | default_inter_layer_texture_flag ) {
		for( i = NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1; i >= 0; rIdx++, i-- )
			RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ] 
	}
}

When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)	(G-xx)
	RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :RefPicListTemp1[ rIdx ]

## Background: SVC slice header in scalable extension syntax

As background, relevant sections of the SVC slice header in scalable extension are copied below.

| slice_header_in_scalable_extension( ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| if( !no_inter_layer_pred_flag && quality_id = = 0 ) { |  |  |
| ref_layer_dq_id | 2 | ue(v) |
| … |  |  |
| if( !no_inter_layer_pred_flag ) { |  |  |
| slice_skip_flag | 2 | u(1) |
| if( slice_skip_flag ) |  |  |
| num_mbs_in_slice_minus1 | 2 | ue(v) |
| else { |  |  |
| adaptive_base_mode_flag | 2 | u(1) |
| if( !adaptive_base_mode_flag ) |  |  |
| default_base_mode_flag | 2 | u(1) |
| if( !default_base_mode_flag ) { |  |  |
| adaptive_motion_prediction_flag | 2 | u(1) |
| if( !adaptive_motion_prediction_flag ) |  |  |
| default_motion_prediction_flag | 2 | u(1) |
| } |  |  |
| adaptive_residual_prediction_flag | 2 | u(1) |
| if( !adaptive_residual_prediction_flag ) |  |  |
| default_residual_prediction_flag | 2 | u(1) |
| } |  |  |
| if( adaptive_tcoeff_level_prediction_flag ) |  |  |
| tcoeff_level_prediction_flag | 2 | u(1) |
| } |  |  |
| … |  |  |

ref_layer_dq_id specifies the layer representation inside the current coded picture that is used for inter-layer prediction of the current layer representation. When present, the value of ref_layer_dq_id shall be in the range of 0 to DQId − 1, inclusive. When ref_layer_dq_id is not present, it shall be inferred as follows:

–	If quality_id is greater than 0, ref_layer_dq_id is inferred to be equal to (DQId − 1).

–	Otherwise (quality_id is equal to 0), ref_layer_dq_id is inferred to be equal to −1.

slice_skip_flag specifies the presence of the slice data in scalable extension syntax structure. When slice_skip_flag is not present, it shall be inferred to be equal to 0. slice_skip_flag equal to 0 specifies that the slice data in scalable extension syntax structure is present in the NAL unit. slice_skip_flag equal to 1 specifies that the slice data in scalable extension syntax structure is not present in the NAL unit and that the syntax elements for the macroblock layer of the slice are derived by the following process:

- CurrMbAddr is derived by

CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag )	(G-1)

- The variable mbIdx proceeds over the values 0..num_mbs_in_slice_minus1, and for each value of mbIdx, the following ordered steps are specified:

- The bitstream shall not contain data that result in InCropWindow( CurrMbAddr ) equal to 0.

- For the macroblock with address CurrMbAddr, the syntax elements mb_skip_flag (when applicable), mb_skip_run (when applicable), mb_field_decoding_flag, base_mode_flag, residual_prediction_flag and coded_block_pattern shall be inferred as follows:

–	mb_skip_flag (when applicable) and mb_skip_run (when applicable) are inferred to be equal to 0.

–	mb_field_decoding_flag is inferred to be equal to 0.

NOTE 4 – The frame/field mode used for decoding is inferred in subclause Error: Reference source not found.

–	base_mode_flag is inferred to be equal to 1.

–	residual_prediction_flag is inferred to be equal to 1.

–	coded_block_pattern is inferred to be equal to 0.

–	QPY is inferred to be equal to SliceQPY.

–	QP′Y is inferred to be equal to (QPY + QpBdOffsetY).

- When the variable mbIdx is less than num_mbs_in_slice_minus1, CurrMbAddr is set to NextMbAddress( CurrMbAddr ). The bitstream shall not contain data that result in CurrMbAddr being set equal to a value that is not less than PicSizeInMbs.

num_mbs_in_slice_minus1 plus 1specifies the number of macroblocks for a slice with slice_skip_flag equal to 1.

adaptive_base_mode_flag specifies the presence of syntax elements in the slice header and in the macroblock layer in scalable extension. When adaptive_base_mode_flag is not present, it shall be inferred to be equal to 0.

default_base_mode_flag specifies how base_mode_flag is inferred when it is not present in macroblock layer in scalable extension. When default_base_mode_flag is not present, it shall be inferred to be equal to 0.

adaptive_motion_prediction_flag specifies the presence of syntax elements in the macroblock layer in scalable extension. When adaptive_motion_prediction_flag is not present, it shall be inferred to be equal to 0.

default_motion_prediction_flag specifies how motion_prediction_flag_l0[ ] and motion_prediction_flag_l1[ ] are inferred when they are not present in macroblock layer in scalable extension. When default_motion_prediction_flag is not present, it shall be inferred to be equal to 0.

adaptive_residual_prediction_flag specifies the presence of syntax elements in the macroblock layer in scalable extension. When adaptive_residual_prediction_flag is not present, it shall be inferred to be equal to 0.

default_residual_prediction_flag specifies how residual_prediction_flag is inferred when it is not present in the macroblock layer in scalable extension. When default_residual_prediction_flag is not present, it shall be inferred to be equal to 0.

# Patent rights declaration(s)

Vidyo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).