| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Incheon, KR, 18-28 April, 2013 | Document: JCTVC-M0048-r1 WG11 Doc: m28481 |
| --- | --- |

| Title: | RCE1: Adaptive Color Transforms For Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Dai, Madhu Krishnan, Pankaj Topiwala 750 N. Atlantic Ave., Cocoa Beach, FL 32931 | Tel: Email: | +1-410-730-6922 pankajtva@gmail.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

Techniques for adaptive integer color transforms are developed for HEVC Range Extensions, for the case of RGB 4:4:4 input data. Our tools convert the RGB data to one of several (YUV-like) integer color spaces prior to encoding/decoding. The decoded data is converted back to RGB 4:4:4, where performance quality is measured. Example test results, for the YFbFr color space, for the Main and High Tiers respectively, and averaged over RGB color planes, show (AI: -26.5%, -15.9%), (RA: -32.4%, -18.9%), (LDB: -33.1%, -11.0%).  Test results for the YFbFrv2 color space were similar: (AI: -23.7%, -13.4%), (RA: -30.5%, -16.5%), (LDB: -30.5%, -16.1%). Two other color spaces were also studied, YCbCr, and YCgCo.

# Color Transforms

Assuming an RGB 4:4:4 input, our method attempts to optimize the spectral decorrelation available by a fixed, small set of color transforms.  For convenience, we list some key color transforms in a compact linear notation for ease of comparison, where a semicolon (“;”) indicates a new row.

YCbCr  = [.213, .715 .072; -.117, -.394, .511; .511, -.464, -.047]; (Per Rec. 709.)

YUV     = [.299 .587 .114; -.147 -.289 .436; .615 -.515 .1];

DCT      = [.5774 .5774 .5774;.7071 0 -.7071; .4082 -.8165 .4082];

YCgCo  = [.25 .5 .25; 1/2 0 -1/2;-.25 .5 -.25];

YFbFr    = [5/16 3/8 5/16; -1/2 1 -1/2;1 0 -1];

YFbFr2  = [3/16 5/8 3/16; -1/2 1 -1/2;1 0 -1];

In addition, we are introducing a new transform in this contribution:

YFbFr3 = [9/64 23/32 9/64; -1/2 1 -1/2; 1 0 -1];

These last four are actually lossless integer color transforms, and are implemented in a lossless manner with shifts/adds, etc., rather than as matrix multiplies (see below). But this table allows for an easy comparison of their linear, floating-point versions with standard color transforms. We remark that YCgCo is now well established, both in AVC, as well as anticipated in HEVC. The YFbFr - YFbFr3 are related variants.

## Text Description of Transforms

### YCgCo :

The forward conversion:

Cg = R − B

t    = B + (Cg >> 1)

Co = G − t

t   = Round (t + (Co >>1))     :

Cg = Round (Cg >> 1)

Co = Round (Co >> 1)

t = Round (t)

Y = Clip3_FVDO (t)

Co = Clip3_FVDO (Co + (1 << (BitDepthC – 1)))

Cg = Clip3_FVDO (Cg + (1 << (BitDepthC – 1)))

The inverse conversion for the above three equations should be computed as:

t   = Y – (Co – (1 << (BitDepthC – 1)))

G = Clip3_FVDO (t +2* (Co − (1 << ( BitDepthC – 1))))

B = Clip3_FVDO (t − (Cg − (1 << ( BitDepthC – 1))))

R = Clip3_FVDO (t + (Cg − (1 << ( BitDepthC – 1))))

### YFbFr :

The forward conversion:

Fr = R − B

t    = B + (Fr >> 1)

Fb = G − t

t   = t + ((3*Fb) >> 3)    :

Y = Clip3_FVDO (t)

Fr = Clip3_FVDO (Fr + (1 << (BitDepthC – 1)))

Fb = Clip3_FVDO (Fb + (1 << (BitDepthC – 1)))

The inverse conversion for the above three equations should be computed as:

t   = Y – (3*(Fb – (1 << (BitDepthC – 1))) >> 3)

G = Clip3_FVDO (t +2* (Fb − (1 << ( BitDepthC – 1))))

B = Clip3_FVDO (t − (Fr − (1 << ( BitDepthC – 1))))

R = Clip3_FVDO (t + (Fr − (1 << ( BitDepthC – 1))))

### YFbFrv2 :

The forward conversion:

Fr = R − B

t    = B + (Fr >> 1)

Fb = G − t

t   = t + ((5*Fb) >> 3)    :

Y = Clip3_FVDO (t)

Fr = Clip3_FVDO (Fr + (1 << (BitDepthC – 1)))

Fb = Clip3_FVDO (Fb + (1 << (BitDepthC – 1)))

The inverse conversion for the above three equations should be computed as:

t   = Y – (5*(Fb – (1 << (BitDepthC – 1))) >> 3)

G = Clip3_FVDO (t +2* (Fb − (1 << ( BitDepthC – 1))))

B = Clip3_FVDO (t − (Fr − (1 << ( BitDepthC – 1))))

R = Clip3_FVDO (t + (Fr − (1 << ( BitDepthC – 1))))

### YFbFrv3 :

The forward conversion:

Fr = R − B

t    = B + (Fr >> 1)

Fb = G − t

t   = t + ((23*Fb) >> 5)    :

Y = Clip3_FVDO (t)

Fr = Clip3_FVDO (Fr + (1 << (BitDepthC – 1)))

Fb = Clip3_FVDO (Fb + (1 << (BitDepthC – 1)))

The inverse conversion for the above three equations should be computed as:

t   = Y – (23*(Fb – (1 << (BitDepthC – 1))) >> 5)

G = Clip3_FVDO (t +2* (Fb − (1 << ( BitDepthC – 1))))

B = Clip3_FVDO (t − (Fr − (1 << ( BitDepthC – 1))))

R = Clip3_FVDO (t + (Fr − (1 << ( BitDepthC – 1))))

### Macro Functions

Clip3_FVDO( MinVal, MaxVal, a)   ( ((a)<(MinVal)) ? (MinVal) : (((a)>(MaxVal)) ? (MaxVal) :(a)) )

Sign(Val) ((Val < 0) ? -1 : ((Val > 0) ? 1 : 0))

Floor(x) ((Int)(x))

Abs(x) ((x >= 0) ? x : -x)

Round(x) (Sign(x)*Floor(Abs(x) + 0.5))

## Adaptive Color Transforms (ACT)

Our approach to Adaptive Color Transforms (ACT) Range Extensions has scalable granularity.  In the first instance, it is a sequence-adaptive color transform, from our small library L = {YCbCr, YCgCo, YFbFr, YFbFrv2}, which requires only a 2-bit index to indicate it in the bitstream.  For each sequence, the encoder chooses the color transform which provides the best R-D performance, and codes the selected transform in the bitsream.  This method is part of the RCE1 (from our previous proposal L0175).

Natural extensions of this method, not currently part of RCE1, are (a) GOP-adaptive, (b) frame-adaptive, and (c) CU-adaptive color transforms, offering increasingly finer granularity, offering potentially further coding efficiencies, at the cost of correspondingly increasing complexity at the encoder, and slightly increasing overhead in the bitstream. Our preliminary results for frame-adaptive color transforms indicates that it permits 5-7% further coding gain over sequence-adaptive coding.

# Experiments

## RCE1 Results

Experiments were described in the Core Experiment 1 document, L1121. Effectively, for a given sequence, the four color transforms are tested for sequence adaptive use in the current range extensions software.  Encoder tools for selecting the best performing color transform for a sequence, and indicating it in the bitstream, were not part of the RCE1. Test results are provided below, and were cross-checked successfully by MediaTek.

Test Results for RCE1, Test 2.2 (L0175).  In these tests, the data are arranged in the order G-B-R (indicated here as Y-U-V, which conforms to other tests).

YFbFrv :

|  |  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |  |
|  | RGB 4:4:4 | -31.9% | -17.9% | -29.6% | -22.3% | -7.2% | -18.3% | -12.5% | -2.1% | -12.6% |  |
|  |  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |  |
|  | RGB 4:4:4 | -34.9% | -25.1% | -37.1% | -28.5% | -2.4% | -25.9% |  |  |  |  |
|  |  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |  |
|  | RGB 4:4:4 | -38.9% | -21.7% | -38.7% | -32.2% | 1.3% | -27.2% |  |  |  |  |

YFbFrv2 :

|  |  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |  |
|  | RGB 4:4:4 | -44.7% | -8.4% | -18.1% | -37.9% | 2.3% | -4.7% | -28.5% | 8.4% | -1.3% |  |
|  |  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |  |
|  | RGB 4:4:4 | -48.6% | -16.2% | -26.7% | -46.6% | 7.2% | -10.2% |  |  |  |  |
|  |  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |  |
|  | RGB 4:4:4 | -51.5% | -11.7% | -28.3% | -48.5% | 12.1% | -11.8% |  |  |  |  |

YCbCr :

|  |  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |  |
|  | RGB 4:4:4 | -49.3% | -2.4% | -21.0% | -44.0% | 4.7% | -13.6% | -35.7% | 11.4% | -7.7% |  |
|  |  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |  |
|  | RGB 4:4:4 | -53.1% | -6.6% | -29.4% | -52.7% | 10.0% | -18.9% |  |  |  |  |
|  |  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |  |
|  | RGB 4:4:4 | -56.2% | -3.0% | -31.1% | -54.9% | 12.2% | -18.8% |  |  |  |  |

YCoCg :

|  |  |  |  | All Intra Main-tier |  |  |  |  | All Intra High-tier |  |  |  |  | All Intra Super-High-tier |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | U |  | V |  | Y |  | U | V |  | Y |  | U |  | V |  |  |  |  |
|  |  | RGB 4:4:4 |  | -39.4% | -13.8% |  | -25.1% |  | -31.8% |  | -2.8% | -12.6% |  | -22.3% |  | 2.7% |  | -7.7% |  |  |  |  |
|  |  |  |  | Random Access Main-tier |  |  |  |  | Random Access High-tier |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Y | U |  | V |  | Y |  | U | V |  |  |  |  |  |  |  |  |  |  |
|  |  | RGB 4:4:4 |  | -43.1% | -21.6% |  | -33.2% |  | -40.0% |  | 1.6% | -19.8% |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Low delay B Main-tier |  |  |  |  | Low delay B High-tier |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Y | U |  | V |  | Y |  | U | V |  |  |  |  |  |  |  |  |  |  |
|  |  | RGB 4:4:4 |  | -46.2% | -17.7% |  | -34.6% |  | -42.3% |  | 6.0% | -21.1% |  |  |  |  |  |  |  |  |  |  |
|  |  | ## Non-RCE1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | All Intra Main-tier |  |  |  |  |  |  | All Intra High-tier |  |  |  |  |  |  | All Intra Super-High-tier |  |  |  |  |  |
|  |  |  | Y |  |  | U |  | V |  | Y |  |  | U |  | V |  | Y |  |  | U |  | V |
|  | RGB 4:4:4 |  | -48.7% |  |  | -9.3% |  | -25.3% |  | -43.7% |  |  | -1.3% |  | -17.6% |  | -35.2% |  |  | 5.3% |  | -11.9% |
|  | Overall |  | -48.7% |  |  | -9.3% |  | -25.3% |  | -43.7% |  |  | -1.3% |  | -17.6% |  | -35.2% |  |  | 5.3% |  | -11.9% |
|  |  |  | -48.6% |  |  | -9.6% |  | -22.1% |  | -43.5% |  |  | -4.4% |  | -18.3% |  | -35.3% |  |  | 5.2% |  | -12.7% |
|  | Enc Time[%] |  | #VALUE! |  |  |  |  |  |  | #VALUE! |  |  |  |  |  |  | #VALUE! |  |  |  |  |  |
|  | Dec Time[%] |  | #VALUE! |  |  |  |  |  |  | #VALUE! |  |  |  |  |  |  | #VALUE! |  |  |  |  |  |

# Conclusions

Our tests indicate that a substantial and useful coding gain can be obtained by performing adaptive color transforms.  Our tools including the standard YCbCr color transform, which also performs well. An interesting feature of the different color transforms is that although the overall coding gain obtained is similar to that of the well-known YCbCr method, the other transforms distribute the gain differently between the green, red, and blue planes. This offers a kind of color selectivity, at an immaterial 2-bit-per-sequence overhead. Furthermore, on the test sequences of record, the color transform called YFbFrv2 appears to slightly outperform YCbCr in general.

# Draft Text as High-Level Syntax Proposal

We propose the following high level syntax features to support adaptive color transforms:

- We propose a sequence level flag (lite_444_codec_flag), indicating that an adaptive color transforms is to be used.

- If lite_444_codec_flag presents, color_transform_id is used to specify which color space is used. Available color spaces include ycbcr, ycgco, yfbfr and yfbfr version2.

- If lite_444_codec_flag presents, resampling_filter_id is used to specify which down/up filter pair is used. Available down-up filter pair includes FV11d4u or can be user-defined.

# Proposed Syntax and Semantics

The additions in the sequence parameter set syntax (JVTVC-K1007) are indicated in red:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| if (nuh_layer_id == 0) |  |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| lite_444_codec_flag | u(1) |
| if(lite_444_codec_flag) |  |
| color_transform_id | u(2) |
| resampling_filter_id | u(1) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |

lite_444_codec_flag equal to 1 indicates that an adaptive color transform is used.

color_transform_id specifies color format used. Color_transform_id can be 0, 1, 2, or 3.  Color_transform_id equal to 0 indicates yCbCr. Color_transform_id equal to 1 indicates yCgCo. Color_transform_id equal to 2 indicates yFbFr. Color_transform_id equal to 3 indicates yFbFr version 2.

NOTE – color_transform_id present if and only if input video is RGB444 format.

resampling_filter_id specifies down-sampling and up-sampling filters used to convert from CHROMA_444 format to CHROMA_420 format or vice versa. The value of resampling_filter_flag can be 0 or 1. If resampling_filter_id is equal to 0, then downsampling filter is: h11 = [1 0 -3 0 10 16 10 0 -3 0 1] / 32; and upsampling filter is f4 = [-1 5 5 -1] / 8. If resampling_filter_id is equal to 1, then the downsampling and upsampling filters are user-defined, and must be provided with additional input data (via a VUI/SEI).

- Draft Text as VUI/SEI

  - Color Transforms

For the new color transforms, YFbFr, and YFbFr2, we have the following draft text to add to the Draft Spec for HEVC (JCTVC-K1003), in Annex E, currently on page 282:

–	Otherwise, if matrix_coefficients is equal to 11, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-45)

t    = Round( B+(Cr >> 1))                                         	(E-46)

Cb = Round( G − t )        	                                                                                                                (E-47)

Y   = Round( t + (Cb*3) )>>3)		(E-48)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-49)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-50)

NOTE 4 – For purposes of the YFbFr nomenclature used in Table E-1, Cb and Cr of Equations E-49 and E-50 may be referred to as Fb and Fr, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y – (3*( Cb − ( 1 << ( BitDepthC − 1 ) ) )) >> 2	(E-51)

G = Clip1Y( t +2* ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )                                                                                     (E-52)

B = Clip1Y( t − ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-53)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-54)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-55)

t    = Round( B+(Cr >> 1))                                         	(E-56)

Cb = Round( G − t )                                                      	(E-57)

Y   = Round( t + (Cb*3) )>>3)		(E-58)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-59)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-60)

NOTE 5– For purposes of the YFbFr nomenclature used in Table E-1, Cb and Cr of Equations E-57 and E-55 may be referred to as Fb and Fr, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( 3*( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 3 )	(E-61)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-62)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-63)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-64)

–	Otherwise, if matrix_coefficients is equal to 12, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-65)

t    = Round( B+(Cr >> 1))                                         	(E-66)

Cb = Round( G − t )        	                                                                                                                (E-67)

Y   = Round( t + (Cb*5) )>>3)		(E-68)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-69)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-70)

NOTE 6 – For purposes of the YFb2Fr2 nomenclature used in Table E-1, Cb and Cr of Equations E-69 and E-70 may be referred to as Fb and Fr, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y – (5*( Cb − ( 1 << ( BitDepthC − 1 ) ) )) >> 2	(E-71)

G = Clip1Y( t +2* ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )                                                                                     (E-72)

B = Clip1Y( t − ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-73)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-74)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-75)

t    = Round( B+(Cr >> 1))                                         	(E-76)

Cb = Round( G − t )        	                                                                                                                (E-77)

Y   = Round( t + (Cb*5) )>>3)		(E-78)

Cb=  Cb + ( 1 << ( BitDepthC − 1 ) )                                         	(E-79)

Cr = Cr + ( 1 << ( BitDepthC − 1 ) )                                         	(E-80)

NOTE 7 – For purposes of the YFb2Fr2 nomenclature used in Table E-1, Cb and Cr of Equations E-79 and E-80 may be referred to as Fb2 and Fr2, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( 5*( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 3 )	(E-81)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-82)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-83)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-84)

Table E-1 – Matrix coefficients

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | GBR | Typically referred to as RGB; see Equations Error: Reference source not found to Error: Reference source not found IEC 61966-2-1 (sRGB) |
| 1 | KR = 0.2126; KB = 0.0722 | ITU-R Rec. BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-1 (sYCC) IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | KR = 0.30; KB = 0.11 | United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| 5 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.470-6 System B, G (historical) ITU-R Rec. BT.601-6 625 ITU-R Rec. BT.1358 625 ITU-R Rec. BT.1700 625 PAL and 625 SECAM IEC 61966-2-4 xvYCC601 (functionally the same as the value 6) |
| 6 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.601-6 525 ITU-R Rec. BT.1358 525 ITU-R Rec. BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the value 5) |
| 7 | KR = 0.212; KB = 0.087 | Society of Motion Picture and Television Engineers 240M (1999) |
| 8 | YCgCo | See Equations Error: Reference source not found to Error: Reference source not found |
| 9 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020 non-constant luminance system; see Equations Error: Reference source not found to Error: Reference source not found |
| 10 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020 constant luminance system see Equations Error: Reference source not found to Error: Reference source not found |
| 11 | YFbFr | See Equations E-45 to E-64 |
| 12 | YFb2Fr2 | See Equations E-65 to E-84 |
| 13..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

5.2.  Draft Text For Resampling Filters as VUI/SEI

Draft text for our resampling filters, presented as VUI and SEI, is included in the related document, JCTVC-L0077 [13].  Draft text for the use of resampling filters as a normative approach to range extensions will be developed upon request.

References

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “High-performance low-complexity re-sampling filters for SVC”, JVT-T041, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 20th meeting, Klagenfurt, AT, Jul. 2006.

- Thierry Blu, Philippe Thevenaz, and Michael Unser, “MOMS: Maximal-order interpolation of minimal support,” IEEE Trans. On Image Processing, vol. 10, No. 7, pp. 1069-1080, Jul. 2001.

- P. P. Vaidyanathan and Bojan Vrcelj, “Biorthogonal partners and applications”, IEEE Trans. on Signal Processing, vol. 49, No. 5, pp. 1013-1028, May 2001.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Extended Spatial Re-sampling Filters For SVC”, JVT-V030, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Dyadic Spatial Re-sampling Filters For SVC”, JVT-V031, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- Gary Sullivan, “Resampling filters for SVC Upsampling”, JVT-R066, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 18th meeting, Bangkok, Jan. 2006.

- Pankaj Topiwala and Chengjie Tu, “New Invertible Integer Color Transforms Based on Lifting Steps and Coding of 4:4:4 Video”, JVT-I015r7, San Diego, Sept., 2003.

- Thomas Davies, Pankaj Topiwala, “AHG18: Adaptive Resolution Coding (ARC),” JCTVC-G264, Geneva, CH, 21-30 November, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Advanced Resampling Filters for Spatial Scalability and Chroma Sampling Applications in HEVC”, JCTVC-F618, Torino, IT, 14-22 July, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Advanced Resampling Filters for HEVC Applications”, JCTVC-G862, Geneva, CH, 14-22 November, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Integer Color Transforms and Resampling Filters for HEVC Extensions: Scalability, 3DV, and Higher Chroma Coding”, JCTVC-J0127, Stockholm, SE, 10-20 July, 2012.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “AHG7: Colour Spaces and Chroma Sampling Methods For Higher Chroma Format Coding”, JCTVC-K0211, Shanghai, CN, 10-19 October, 2012.

- Takeshi Chujoh, et al, “Additional VUI and SEI for chroma sampling filter,” JCTVC-L0077r2, Geneva, CH, 14-23 January, 2013.

- T. Tran, P. Topiwala, RE40081, US patent, 2008.

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).