| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Incheon, KR, 18-28 April, 2013 | Document: JCTVC-M0049 WG11 Doc: m28482 |
| --- | --- |

| Title: | RCE1: Combined Chroma Tools For Range Extensions, Test 3.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala, FastVDO 750 N. Atlantic Ave., Cocoa Beach, FL 32931 Jungsun Kim, LG Ki Kawamura, KDDI | Tel: Email: | +1-410-730-6922 pankajtva@gmail.com Jungsun.kim@lge.com Ki-kawamura@kddi.com |
| Source: | FastVDO LLC, LG, KDDI |  |  |

_____________________________

# Abstract

Techniques for adaptive chroma processing are developed for HEVC Range Extensions, for the case of RGB 4:4:4 input data, combining a variety of tools previously proposed in Geneva (L0175, L0240, L0370, L0371), for Test 3.3. Test results show for example for the YFbFrv2 color space, for the Main and High Tiers respectively, averaging gains over R, G, and B, we get (AI: -26.4%, -18.2%), (RA: -32.3%, -20.6%), (LDB: -32.8%, -20.4%). Test results for YCbCr color space were (AI: -25.4%, -18.3%), (RA: -31.8%, -20.8%), (LDB: -32.1%, -20.8%).

# Brief Description of Tools

JCTVC-L0240, AHG7: The performance of extended chroma mode for non 4:2:0 format

  - Two methods for chroma prediction which predicts chroma samples using linear combination of luma samples (LM mode)

    - LM as was proposed in HEVC version 1, extended to non-4:2:0

    - LM where parameter calculation is performed on a CU basis rather than per-TU

  - Proponent: LG

JCTVC-L0370, AHG7: Inter-plane intra coding of residual signals for range extensions

  - Intra coding approach based on inter-plane prediction of intra residual signals by using a linear model. Prediction parameter of the model is derived on the encoder. Resulting parameter is predicted from neighbouring parameters.

  - Proponent: KDDI

JCTVC-L0371, AHG7: In-loop color-space transformation of residual signals for range extensions

  - This method transforms prediction error signals into those in a adaptively computed colour space. A transformation matrix is derived from pixel domain for each coding unit. The colour-space transformation is applied to prediction error of both intra and inter mode. No additional signalling is necessary.

  - Proponent: KDDI

JCTVC-L0175, AHG7: A Consumer 4:4:4 Coding Method Based On A 4:2:0 Codec

  - Four integer colour spaces:

    - YCbCr	= [.213, .715 .072; -.117, -.394, .511; .511, -.464, -.047] (Rec. 709)

    - YCoCg	= [.25 .5 .25; 1/2 0 -1/2;-.25 .5 -.25]

    - YFbFr	= [5/16 3/8 5/16; -1/2 1 -1/2;1 0 -1]

    - YFbFr2	= [3/16 5/8 3/16; -1/2 1 -1/2;1 0 -1]

  - Conversion at the input and output of the codec, with SNRs generated relative to the input GBR data

  - Proponents: FastVDO

# Experiments

Experiments were described in the Core Experiment 1 document, L1121, for Test 3.3.

### Test 3.3 Combination of proposals

  - In order to study the interaction of the four proposals, performance of combinations of them will be tested:

JCTVC-L0240 (method 1) + JCTVC-L0370 + JCTVC-L0175 (YCbCr & YFbFr2) + JCTVC-L0371

  - Proponents: FastVDO (for 3.3)

Test Description

This test studied the combined performance of all the tools mentioned in proposals L0175, L0240, L0370 and L0371. The tool selection is made using the macro definitions and configuration parameters mentioned in Tab. 1

| Proposal | Macro & Configuration Parameters |
| --- | --- |
| L0175 | L0175_COLOR_TRANSFORM InputColourSpaceConvert (RGBtoYFbFrv2,RGBtoYCbCr) |
| L0240 | LMCHROMA |
| L0370 | RM_CHROMA |
| L0371 | ADAPTIVE_COLORSPACE |

# Test Results

Tests were conducted by LGE (both YFbFrv2 and YCbCr), and FastVDO (YFbFrv2), and cross-checked by Qualcomm, all with matching results.

YFbFrv2:

|  |  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
|  | RGB 4:4:4 | -21.3% | -45.7% | -12.2% | -10.7% | -38.9% | -5.1% | -4.7% | -29.8% | 1.4% |
|  |  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |
|  | RGB 4:4:4 | -28.7% | -49.4% | -18.8% | -14.1% | -47.1% | -0.7% |  |  |  |
|  |  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |
|  | RGB 4:4:4 | -30.6% | -52.4% | -15.3% | -14.8% | -49.1% | 2.8% |  |  |  |

YCbCr :

|  |  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
|  | RGB 4:4:4 | -23.6% | -51.0% | -1.6% | -14.7% | -45.7% | 5.5% | -7.7% | -36.9% | 11.7% |
|  |  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |
|  | RGB 4:4:4 | -32.3% | -54.4% | -8.7% | -21.1% | -54.3% | 12.9% |  |  |  |
|  |  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |
|  | RGB 4:4:4 | -34.5% | -57.6% | -4.1% | -22.1% | -56.9% | 16.7% |  |  |  |

# Conclusions

Our tests indicate that a substantial and useful coding gain can be obtained by performing adaptive color transforms.  Our tools included the standard YCbCr color transform, which also performed well, with YFbFrv2 slightly outperforming it. An additional feature of the application of different color transforms is that although the overall coding gain obtained averaged over color planes is similar to that of the well-known YCbCr method, the other transforms distribute the gain differently between the green, red, and blue planes. This offers a kind of color selectivity, at an immaterial 2-bit-per-sequence overhead.

# Draft Text

## Draft Text For L0175 Tools

We propose the following high level syntax features so that HEVC can provide a lite version of 444 codec via the baseline 420 codec:

- We propose a sequence level flag (lite_444_codec_flag), indicating that an adaptive color transforms is to be used.

- If lite_444_codec_flag presents, color_transform_id is used to specify which color space is used. Available color spaces include ycbcr, ycgco, yfbfr and yfbfr version2.

- If lite_444_codec_flag presents, resampling_filter_id is used to specify which down/up filter pair is used. Available down-up filter pair includes FV11d4u or can be user-defined.

### Proposed Syntax and Semantics

The additions in the sequence parameter set syntax (JVTVC-K1007) are indicated in red:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| if (nuh_layer_id == 0) |  |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| lite_444_codec_flag | u(1) |
| if(lite_444_codec_flag) |  |
| color_transform_id | u(2) |
| resampling_filter_id | u(1) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |

lite_444_codec_flag equal to 1 indicates coding 444 input via 420 codec. Lite_444_codec_flag equal to 0 indicates either of the following situations: 1) InputChromaFormat is not CHROMA_444; and 2) InputChromaFormat is CHROMA_444, and chroma_format_idc is CHROMA_444.

NOTE – When lite_444_codec_flag  is equal to 1, , chroma_format_idc should be CHROMA_420.

color_transform_id specifies color format used. Color_transform_id can be 0, 1, 2, or 3.  Color_transform_id equal to 0 indicates ycbcr. Color_transform_id equal to 1 indicates ycgco. Color_transform_id equal to 2 indicates yFbFr. Color_transform_id equal to 3 indicates yFbFr version 2.

NOTE – color_transform_id present if and only if input video is RGB444 format.

resampling_filter_id specifies down-sampling and up-sampling filters used to convert from CHROMA_444 format to CHROMA_420 format or vice versa. The value of resampling_filter_flag can be 0 or 1. If resampling_filter_id is equal to 0, then downsampling filter is: h11 = [1 0 -3 0 10 16 10 0 -3 0 1] / 32; and upsampling filter is f4 = [-1 5 5 -1] / 8. If resampling_filter_id is equal to 1, then the downsampling and upsampling filters are user-defined, and must be provided with additional input data (via a VUI/SEI).

### Draft Text as VUI/SEI

  - Color Transforms

For the new color transforms, YFbFr, and YFbFr2, we have the following draft text to add to the Draft Spec for HEVC (JCTVC-K1003), in Annex E, currently on page 282:

–	Otherwise, if matrix_coefficients is equal to 11, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-45)

t    = Round( B+(Cr >> 1))                                         	(E-46)

Cb = Round( G − t )        	                                                                                                                (E-47)

Y   = Round( t + (Cb*3) )>>3)		(E-48)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-49)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-50)

NOTE 4 – For purposes of the YFbFr nomenclature used in Table E-1, Cb and Cr of Equations E-49 and E-50 may be referred to as Fb and Fr, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y – (3*( Cb − ( 1 << ( BitDepthC − 1 ) ) )) >> 2	(E-51)

G = Clip1Y( t +2* ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )                                                                                     (E-52)

B = Clip1Y( t − ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-53)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-54)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-55)

t    = Round( B+(Cr >> 1))                                         	(E-56)

Cb = Round( G − t )                                                      	(E-57)

Y   = Round( t + (Cb*3) )>>3)		(E-58)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-59)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-60)

NOTE 5– For purposes of the YFbFr nomenclature used in Table E-1, Cb and Cr of Equations E-57 and E-55 may be referred to as Fb and Fr, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( 3*( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 3 )	(E-61)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-62)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-63)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-64)

–	Otherwise, if matrix_coefficients is equal to 12, the following applies:

–	If BitDepthC is equal to BitDepthY, the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-65)

t    = Round( B+(Cr >> 1))                                         	(E-66)

Cb = Round( G − t )        	                                                                                                                (E-67)

Y   = Round( t + (Cb*5) )>>3)		(E-68)

Cb= Round( Cb>> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-69)

Cr = Round( Cr >> 1 ) + ( 1 << ( BitDepthC − 1 ) )                                         	(E-70)

NOTE 6 – For purposes of the YFb2Fr2 nomenclature used in Table E-1, Cb and Cr of Equations E-69 and E-70 may be referred to as Fb and Fr, respectively. The inverse conversion for the above three equations should be computed as:

t   = Y – (5*( Cb − ( 1 << ( BitDepthC − 1 ) ) )) >> 2	(E-71)

G = Clip1Y( t +2* ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )                                                                                     (E-72)

B = Clip1Y( t − ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-73)

R = Clip1Y( t + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )		(E-74)

–	Otherwise (BitDepthC is not equal to BitDepthY), the following equations apply:

Cr = Round(  (R − B ) )                                                	(E-75)

t    = Round( B+(Cr >> 1))                                         	(E-76)

Cb = Round( G − t )        	                                                                                                                (E-77)

Y   = Round( t + (Cb*5) )>>3)		(E-78)

Cb=  Cb + ( 1 << ( BitDepthC − 1 ) )                                         	(E-79)

Cr = Cr + ( 1 << ( BitDepthC − 1 ) )                                         	(E-80)

NOTE 7 – For purposes of the YFb2Fr2 nomenclature used in Table E-1, Cb and Cr of Equations E-79 and E-80 may be referred to as Fb2 and Fr2, respectively. The inverse conversion for the above four equations should be computed as.

t   = Y − ( 5*( Cb − ( 1 << ( BitDepthC − 1 ) ) ) >> 3 )	(E-81)

G = Clip1Y( t + ( Cb − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-82)

B = Clip1Y( t − ( ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) >> 1 ) )	(E-83)

R = Clip1Y( B + ( Cr − ( 1 << ( BitDepthC − 1 ) ) ) )	(E-84)

Table E-1 – Matrix coefficients

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | GBR | Typically referred to as RGB; see Equations Error: Reference source not found to Error: Reference source not found IEC 61966-2-1 (sRGB) |
| 1 | KR = 0.2126; KB = 0.0722 | ITU-R Rec. BT.709-5 ITU-R Rec. BT.1361 conventional colour gamut system and extended colour gamut system IEC 61966-2-1 (sYCC) IEC 61966-2-4 xvYCC709 Society of Motion Picture and Television Engineers RP 177 (1993) Annex B |
| 2 | Unspecified | Image characteristics are unknown or are determined by the application. |
| 3 | Reserved | For future use by ITU-T ¦ ISO/IEC |
| 4 | KR = 0.30; KB = 0.11 | United States Federal Communications Commission Title 47 Code of Federal Regulations (2003) 73.682 (a) (20) |
| 5 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.470-6 System B, G (historical) ITU-R Rec. BT.601-6 625 ITU-R Rec. BT.1358 625 ITU-R Rec. BT.1700 625 PAL and 625 SECAM IEC 61966-2-4 xvYCC601 (functionally the same as the value 6) |
| 6 | KR = 0.299; KB = 0.114 | ITU-R Rec. BT.601-6 525 ITU-R Rec. BT.1358 525 ITU-R Rec. BT.1700 NTSC Society of Motion Picture and Television Engineers 170M (2004) (functionally the same as the value 5) |
| 7 | KR = 0.212; KB = 0.087 | Society of Motion Picture and Television Engineers 240M (1999) |
| 8 | YCgCo | See Equations Error: Reference source not found to Error: Reference source not found |
| 9 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020 non-constant luminance system; see Equations Error: Reference source not found to Error: Reference source not found |
| 10 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020 constant luminance system see Equations Error: Reference source not found to Error: Reference source not found |
| 11 | YFbFr | See Equations E-45 to E-64 |
| 12 | YFb2Fr2 | See Equations E-65 to E-84 |
| 13..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LGE may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

KDDI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).