| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Incheon, KR, 18-28 April, 2013 | Document: JCTVC-M0051 WG11 Doc: m28484 |
| --- | --- |

| Title: | AHG12: Low-Complexity Upsampling Filters For SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Dai Madhu Krishnan Pankaj Topiwala, FastVDO 750 N. Atlantic Ave., Cocoa Beach, FL 32931 | Tel: Email: | +1-410-730-6922 pankajtva@gmail.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

In place of the existing 8-tap luma and 4-tap chroma upsampling filters in the current SHVC model, we propose 6-tap luma and 4-tap chroma upsamplers. In this test, the downsampler has not been altered, so that there is no need to compare performance on the low-resolution signal.  We measure the performance variance to the 8/4-tap reference filters, which for the test cases are: (0.7%, 0.8%, 0.9%, AI-2X); (0.4%, 0.5%, 0.6%, RA-2X); (0.1%, 0.6%, 0.7%, LDP-2X).

# Technical Description

This proposal provides new upsampling filters for SHVC of lower computational complexity, while keeping the identical downsampling filter.  This completely bypasses any consideration of the low-resolution signal in the analysis.  In reality, our filters are co-designed with the down and upsampling processes, but their reasonable performance with the existing downsampler, coupled with lower complexity as well as potential enhancements with varying the downsampler, is the value proposition at this stage. We apply a 2D separable 6-tap filter for luma components and a 2D separable 4-tap filter for chroma components. The proposed filter coefficients are provided in the tables below. We remark that all the filter coefficients require only 6-bit accuracy for representation.

Tab.1 Luma up-sampling filters.

| Phase shift | Luma up-sampling filter |
| --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0}/64 |
| 1/3 | {-0, 4, -12, 54, 24, -8, 2, 0}/64 |
| 1/2 | { 0, 2, -8, 38, 38, -8, 2, 0}/64 |
| 2/3 | { 0, 2, -8, 24, 54, -12, 4, 0}/64 |

Tab.2 Chroma up-sampling filters.

| Phase shift | Chroma up-sampling filter |
| --- | --- |
| 0 | { 0, 64, 0, 0}/64 |
| 1/4 | { -8, 58, 16, -2}/64 |
| 1/3 | {-8, 54, 22, -4}/64 |
| 3/8 | {-8, 50, 28, -6}/64 |
| 1/2 | {-8, 40, 40, -8}/64 |
| 7/12 | {-6, 34, 44, -8}/64 |
| 2/3 | {-4, 22, 54, -8}/64 |
| 7/8 | {-2, 10, 58, -2}/64 |
| 11/12 | { -1, 5, 62, -2}/64 |

# Results

These results have been cross-checked by Samsung.

|  |  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |  |  |
|  | Class A | 1.1% | 1.3% | 1.3% |  |  |  |  |  |  |
|  | Class B | 0.5% | 0.7% | 0.7% | 1.1% | 1.3% | 1.3% |  |  |  |
|  | Overall (Test vs Ref) | 0.7% | 0.8% | 0.9% | 1.1% | 1.3% | 1.3% |  |  |  |
|  | Overall (Test vs single layer) | 13.6% | 16.0% | 15.7% | 11.8% | 11.3% | 10.9% |  |  |  |
|  | EL only (Test vs Ref) | 1.2% | 1.4% | 1.4% | 3.1% | 3.4% | 3.5% |  |  |  |
|  | Enc Time[%] |  |  |  |  |  |  |  |  |  |
|  | Dec Time[%] |  |  |  |  |  |  |  |  |  |
|  | Enc Mem[%] |  |  |  |  |  |  |  |  |  |
|  | BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  |  | 0.67% | 0.85% |  | 1.10% | 1.30% |  |  |  |  |
|  |  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |
|  | Class A | 0.7% | 0.9% | 0.9% |  |  |  |  |  |  |
|  | Class B | 0.2% | 0.4% | 0.4% | 1.0% | 0.7% | 0.6% |  |  |  |
|  | Overall (Test vs Ref) | 0.4% | 0.5% | 0.6% | 1.0% | 0.7% | 0.6% |  |  |  |
|  | Overall (Test vs single layer) | 20.6% | 33.9% | 34.9% | 18.6% | 29.0% | 31.4% |  |  |  |
|  | EL only (Test vs Ref) | 0.6% | 0.8% | 0.9% | 2.5% | 2.2% | 2.1% |  |  |  |
|  | Enc Time[%] |  |  |  |  |  |  |  |  |  |
|  | Dec Time[%] |  |  |  |  |  |  |  |  |  |
|  | Enc Mem[%] |  |  |  |  |  |  |  |  |  |
|  | BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  |  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |  |  |  |
|  |  | Y | U | V | Y | U | V |  |  |  |
|  | Class A | 0.4% | 0.9% | 0.9% |  |  |  |  |  |  |
|  | Class B | 0.0% | 0.5% | 0.6% | 1.0% | 1.1% | 1.0% |  |  |  |
|  | Overall (Test vs Ref) | 0.1% | 0.6% | 0.7% | 1.0% | 1.1% | 1.0% |  |  |  |
|  | Overall (Test vs single layer) | 26.7% | 38.7% | 40.0% | 24.1% | 34.2% | 37.0% |  |  |  |
|  | EL only (Test vs Ref) | 0.2% | 0.7% | 0.8% | 2.4% | 2.5% | 2.4% |  |  |  |
|  | Enc Time[%] |  |  |  |  |  |  |  |  |  |
|  | Dec Time[%] |  |  |  |  |  |  |  |  |  |
|  | Enc Mem[%] |  |  |  |  |  |  |  |  |  |
|  | BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  |  | 0.22% | 0.61% |  | 1.01% | 0.87% |  |  |  |  |

# Conclusions

We proposed low-complexity 6-tap upsampling filters for luma, and 4-tap upsampling filters for chroma. These filters are of lower complexity that the 8/4-tap filters in the current draft design, yet provide performance reasonably matching that of the existing filters – even with the existing downsampler.  As this design can benefit from co-designed downsamplers, there is room for improvement in this design alternative, especially if the complexity reduction is itself of value.

# Draft Text

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).