| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0052 |
| --- | --- |

| Title: | RCE2: Sample-based weighted intra prediction for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens Corporate Technology Research and Technology Center Imaging and Computer Vision Otto-Hahn-Ring 6 81739 Munich, Germany Eugen Wige, André Kaup Universität Erlangen-Nürnberg Multimedia Communications and Signal Processing Cauerstr. 7 91058 Erlangen, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This proposal presents an intra-prediction scheme designed for lossless coding. The proposed coding method comprises a pixel-wise prediction based on original samples. It is realized as a separate intra prediction mode, which replaces the PLANAR mode. In order to perform the prediction, a four-sample template around the pixel that is to be predicted is compared to the respective template of a four-pixel neighbourhood. For each reference template, the sum of absolute differences (SAD) is determined. A table look-up of the SAD value gives the respective weighting factor for each neighbourhood pixel. The predictor for the current pixel is calculated as the weighted average of the neighbourhood pixels. In comparison to the unmodified HEVC Test Model HM-10.0_RExt2.0 configured for lossless coding by disabling/bypassing transformation, quantization, and in-loop filters, the proposed method reportedly provides average bitrate savings of 6.8% with SC and 6.4% without SC for AI-Main, 3.8% with SC and 2.4% without SC for RA-Main, and 3.5% with SC and 1.9% without SC for LB-Main configuration settings. The improved version of the proposed predictor provides average bitrate savings of 9.5% with SC and 6.9% without SC for AI-Main, 5.8% with SC and 2.7% without SC for RA-Main, and 5.2% with SC and 2.1% without SC for LB-Main configuration settings.

# Introduction

In some professional applications, image and video content needs to be stored losslessly. This is especially true for medical applications, where dedicated regulations require lossless storage, since coding artefacts might degrade the quality of a medical diagnosis. Due to the large amounts of medical image data that has to be archived in repositories (e.g., PACS – Picture Archiving and Communication System), high coding efficiency is important. Therefore, an efficient lossless coding mode for HEVC is proposed.

# Proposed method

## Overview

In order to improve the intra-frame prediction in lossless image/video compression for HEVC, we propose a backward adaptive sample-based intra prediction scheme. An overview of the proposed scheme is given in the following:

- A dedicated intra prediction mode replacing the PLANAR mode is used to signal the usage of the proposed method.

- A four-sample template around the pixel that is to be predicted is compared to the respective template of a four-pixel neighbourhood. For each reference template, the sum of absolute differences (SAD) is determined.

- A table look-up of the SAD value gives the respective weighting factor for each neighbourhood pixel.

- The predictor for the current pixel is calculated as the weighted average of the neighbourhood pixels.

- The prediction error is calculated and further coded.

- After reconstruction of the respective pixel value, subsequent pixels are processed.

The properties of the proposed prediction algorithm are as follows:

- In contrast to sample-based least-squares methods there is no need of solving linear system of equations for optimal weights derivation. The weights are calculated independently from each other using a simple mathematical relationship.

- As the prediction scheme is backward adaptive there is no need of transmitting of additional side information other than the intra coding mode.

- Furthermore, the algorithm is used for lossless coding by disabling transformation, quantization, in-loop filtering.

The detailed explanation of the proposed scheme is explained in the following subsections.

## Shape of template and neighbourhood

The template used for comparing the similarity between the pixels around the current pixels and the pixels about neighbouring pixels consists of four samples for the regular mode as shown in Figure 1.

Figure 1.  Template shape and neighbourhood shape.

The shape of the neighbourhood is equivalent to the shape of the template.

## Calculation of weights

To calculate the weights, similarity between the four-pixel neighbourhood of the current pixel and the four-pixel neighbourhood of the predictor pixels is calculated, i.e., the SAD is calculated. Figure 2 depicts the template shape of the current pixels, while Figure 3 shows the template shape of the four neighbourhood pixels with the respective shifts.

Figure 2.  Four-pixel template of current pixel.

Figure 3.  Four-pixel templates of neighbourhood pixels.

The SAD between the pixels of the template around the current pixel and the pixels of the template around a pixel of the neighbourhood is calculated as follows:

SADi,j=n∈N0|p[i+n]−p[j+n]|,

where i=(x,y)is the two dimensional coordinate of the predictor pi, jis accordingly the two-dimensional coordinate of the candidate pixel from the neighbourhood, nis the (two-dimensional) offset for each template pixel, and Nois the set of shift values for each template pixel. Specifically in this proposal, only 4 pixels are used, i.e, No=n=(x,y)|(0,−1),(−1,−1),(−1,0),(−1,1) as shown in Figure 1. At boundaries the neighbourhood as well as the template can be shrunk to meet the causal requirements as it is done in the current proposal, i.e., more details in Section 5.

The corresponding weights are calculated by means of a look-up table (LUT) (the look-up table is defined in Section 5 by the array SWP_Table), where the scaled SADvalue is the only relevant parameter. The calculation of the look-up table has been performed by the following formula:

LUT[k]=round(a⋅2(−kh)).

The algorithm is implemented using integer arithmetic with 32 bits precision, i.e, type Int from HM-10.0_RExt2.0 is used. Up to two look-up tables are used in this algorithm, i.e, one table for luma prediction and one table for chorma prediction if the chroma subsampling format differs from 4:4:4. This means that for internal bit depth (BitDepth) higher than 8, the SAD is transformed to 8 bits, which is realized by a right bit shift operation. In order to fully exploit the 32 bits integer arithmetic, the highest possible parameter value a has been chosen for generation of the look-up table with respect to 8 bits internal bit depth. It should be ensured that if choosing 4 times the highest possible weight the summation can still be represented by 31 bits, i.e., a=231−2−8. For other internal bit depth than 8 bits, the resulting value from the table look-up has to be transformed by a right bit shift operation in order to keep the possibility of representing the weighted summation with 31 bits. The parameter hL=4.75 is chosen for the luma channel and hC=2.25 (4:2:0) or hC=3.5 (4:2:2) for the chroma channel, respectively. Using these parameters, the table length is limited to 105 values for luma and 50 (4:2:0) or 78 (4:2:2) values for chroma.

Thus, the corresponding weights for an internal bit depth of 8 bits are calculated as

wi,j=LUTSADi,j

and generally for all possible bit depth higher than 8 bits as

wi,j=(LUTSADi,j>>(BitDepth−8)>>(BitDepth−8)).

## Calculation of predictor for current pixel

The predictor px,y for the current pixel is calculated as a weighted average of the pixels of the four-pixel neighbourhood, i.e.,

pi=(wsum2+j∈i+N0wi,j⋅pj)wsum,

where pj is the (reconstructed) pixel value at position j=(x,y), wi,j are the individual weights calculated as described above, and wsum is the sum of the four weights. Note that the four-pixel neighbourhood is defined by j∈i+N0, as also depicted in Figure 1.

# Simulation results

The proposed method was implemented into HEVC Test Model HM-10.0_RExt2.0. The common test conditions for RCE2 as defined in [2] were used. The simulation platform is a homogenous cluster (consisting of Intel(R) Core(TM)2 Quad CPU Q9400 @ 2.66GHz CPUs).

## Simulation results for the RCE2 test sequences

The cross checked coding simulation results for the RCE2 test sequences as proposed in [2] are depicted in the following table where “Reference” stands for the anchor results using HM-10.0_RExt2.0 and “Tested” stands for the simulation results using the SWP prediction algorithm for lossless coding.

Table 1. Simulation results for test 2 – L0161

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.1 | -7.8% |
| Class B | 2.1 | 2.3 | -6.8% |
| SC(GBR) | 7.5 | 8.2 | -7.2% |
| RangeExt | 2.2 | 2.3 | -4.7% |
| Overall (w/o SC) | 3.2 | 3.4 | -6.4% |
| Overall (w/ SC) | 5.3 | 5.8 | -6.8% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 30.9 | -3.9% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 95.8 | -5.2% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 13.3 | 13.9 | -2.4% |
| Overall (w/ SC) | 51.0 | 54.9 | -3.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 48.9 | -3.0% |
| Class B | 2.6 | 2.6 | -1.5% |
| SC(GBR) | 301.9 | 323.0 | -5.0% |
| RangeExt | 2.5 | 2.6 | -1.1% |
| Overall (w/o SC) | 20.5 | 21.1 | -1.9% |
| Overall (w/ SC) | 161.2 | 172.1 | -3.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2. Simulation results for test 6 – combination of L0117 and L0161

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.3 | -11.4% |
| Class B | 2.1 | 2.3 | -7.2% |
| SC(GBR) | 7.5 | 8.8 | -12.9% |
| RangeExt | 2.2 | 2.3 | -4.9% |
| Overall (w/o SC) | 3.2 | 3.5 | -8.0% |
| Overall (w/ SC) | 5.3 | 6.2 | -10.5% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.8 | -6.6% |
| Class B | 2.6 | 2.6 | -1.9% |
| SC(GBR) | 88.8 | 101.3 | -9.4% |
| RangeExt | 2.5 | 2.6 | -1.3% |
| Overall (w/o SC) | 13.3 | 14.3 | -3.6% |
| Overall (w/ SC) | 51.0 | 57.8 | -6.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 49.8 | -5.4% |
| Class B | 2.6 | 2.6 | -1.6% |
| SC(GBR) | 301.9 | 343.6 | -9.0% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 20.5 | 21.5 | -2.9% |
| Overall (w/ SC) | 161.2 | 182.6 | -6.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 3. Simulation results for test 7 – combination of L0161 and L0176

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.4 | -12.6% |
| Class B | 2.1 | 2.3 | -7.5% |
| SC(GBR) | 7.5 | 8.6 | -11.8% |
| RangeExt | 2.2 | 2.4 | -5.3% |
| Overall (w/o SC) | 3.2 | 3.6 | -8.7% |
| Overall (w/ SC) | 5.3 | 6.1 | -10.3% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.9 | -7.3% |
| Class B | 2.6 | 2.6 | -2.0% |
| SC(GBR) | 88.8 | 99.2 | -8.5% |
| RangeExt | 2.5 | 2.6 | -1.4% |
| Overall (w/o SC) | 13.3 | 14.3 | -3.9% |
| Overall (w/ SC) | 51.0 | 56.8 | -6.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 49.9 | -5.9% |
| Class B | 2.6 | 2.6 | -1.6% |
| SC(GBR) | 301.9 | 335.7 | -7.8% |
| RangeExt | 2.5 | 2.6 | -1.3% |
| Overall (w/o SC) | 20.5 | 21.5 | -3.2% |
| Overall (w/ SC) | 161.2 | 178.6 | -5.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

## Additional coding simulation results for natural videos

Additional lossless coding simulations were performed on Class A to Class E sequences from the standard test set [3]. The coding simulation results are depicted in the following where “Reference” stands for the anchor results using HM-10.0_RExt2.0 and “Tested” stands for the simulation results using the SWP prediction algorithm for lossless coding.

Table 4. Simulation results for SWP used for natural videos

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 2.1 | 2.3 | -8.6% |
| Class B | 2.1 | 2.2 | -4.3% |
| Class C | 2.0 | 2.1 | -5.8% |
| Class D | 1.8 | 2.0 | -6.8% |
| Class E | 2.7 | 2.9 | -7.5% |
| Overall (w/o F) | 2.1 | 2.3 | -6.5% |
| Overall (w/ F) | 2.5 | 2.7 | -6.7% |
| Class F | 4.6 | 5.1 | -7.8% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 2.4 | 2.5 | -3.5% |
| Class B | 2.4 | 2.4 | -1.0% |
| Class C | 2.5 | 2.6 | -1.5% |
| Class D | 2.6 | 2.7 | -1.8% |
| Class E | 3.2 | 3.3 | -1.6% |
| Overall (w/o F) | 2.6 | 2.6 | -1.9% |
| Overall (w/ F) | 7.0 | 7.3 | -2.2% |
| Class F | 29.3 | 30.9 | -3.9% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 2.4 | 2.5 | -3.3% |
| Class B | 2.4 | 2.4 | -0.8% |
| Class C | 2.5 | 2.6 | -1.3% |
| Class D | 2.6 | 2.7 | -1.4% |
| Class E | 3.2 | 3.3 | -1.6% |
| Overall (w/o F) | 2.6 | 2.6 | -1.6% |
| Overall (w/ F) | 10.1 | 10.4 | -1.9% |
| Class F | 47.4 | 48.9 | -3.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |

## Results for SWP with bug fix

If all weights of the available surrounding pixels were determined to be zero an exception algorithm is used for prediction. In the current proposal which was tested in the core experiments an average value has been used for prediction of the current pixel. However, it has been noticed that in the reference implementation of the algorithm only the weights of three surrounding pixels No=n=(x,y)|(0,−1),(−1,−1),(−1,0) have been checked although the upper right pixel was available. In the following the results for the SWP algorithm with bug fix where all available pixels were checked are provided.

Table 5. Simulation results for SWP with bugfix

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.1 | -7.9% |
| Class B | 2.1 | 2.3 | -6.8% |
| SC(GBR) | 7.5 | 8.2 | -7.6% |
| RangeExt | 2.2 | 2.3 | -4.8% |
| Overall (w/o SC) | 3.2 | 3.4 | -6.4% |
| Overall (w/ SC) | 5.3 | 5.8 | -7.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 30.9 | -3.9% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 96.6 | -5.5% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 13.3 | 13.9 | -2.4% |
| Overall (w/ SC) | 51.0 | 55.3 | -4.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 48.9 | -3.0% |
| Class B | 2.6 | 2.6 | -1.5% |
| SC(GBR) | 301.9 | 324.8 | -5.2% |
| RangeExt | 2.5 | 2.6 | -1.1% |
| Overall (w/o SC) | 20.5 | 21.1 | -1.9% |
| Overall (w/ SC) | 161.2 | 173.0 | -3.6% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

## Results for improved version of the proposed scheme

Further improvements in compression results have been achieved by another exception algorithm. In the improved version of the SWP predictor, the pixel with the smallest SAD value is used for prediction within the exception algorithm. In the following the compression results for the improved SWP scheme are presented.

Table 6. Simulation results for improved SWP

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.2 | -9.1% |
| Class B | 2.1 | 2.3 | -6.8% |
| SC(GBR) | 7.5 | 8.8 | -12.1% |
| RangeExt | 2.2 | 2.3 | -4.8% |
| Overall (w/o SC) | 3.2 | 3.5 | -6.9% |
| Overall (w/ SC) | 5.3 | 6.2 | -9.5% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.3 | -4.5% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 103.0 | -8.9% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 13.3 | 14.1 | -2.7% |
| Overall (w/ SC) | 51.0 | 58.5 | -5.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 49.2 | -3.3% |
| Class B | 2.6 | 2.6 | -1.5% |
| SC(GBR) | 301.9 | 349.5 | -8.3% |
| RangeExt | 2.5 | 2.6 | -1.1% |
| Overall (w/o SC) | 20.5 | 21.2 | -2.1% |
| Overall (w/ SC) | 161.2 | 185.4 | -5.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Additionally, the combinations of the improved SWP scheme with other algorithms have been tested.

Table 7. Simulation results for combination of improved SWP with RDPCM (L0117)

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.4 | -12.1% |
| Class B | 2.1 | 2.3 | -7.3% |
| SC(GBR) | 7.5 | 9.2 | -15.4% |
| RangeExt | 2.2 | 2.3 | -5.0% |
| Overall (w/o SC) | 3.2 | 3.6 | -8.3% |
| Overall (w/ SC) | 5.3 | 6.4 | -11.8% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 32.0 | -7.0% |
| Class B | 2.6 | 2.6 | -1.9% |
| SC(GBR) | 88.8 | 105.8 | -11.5% |
| RangeExt | 2.5 | 2.6 | -1.3% |
| Overall (w/o SC) | 13.3 | 14.4 | -3.7% |
| Overall (w/ SC) | 51.0 | 60.1 | -7.6% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 50.0 | -5.6% |
| Class B | 2.6 | 2.6 | -1.6% |
| SC(GBR) | 301.9 | 359.4 | -10.8% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 20.5 | 21.6 | -3.0% |
| Overall (w/ SC) | 161.2 | 190.5 | -6.9% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 8. Simulation results for combination of improved SWP with SAP (L0176)

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.4 | -13.2% |
| Class B | 2.1 | 2.3 | -7.5% |
| SC(GBR) | 7.5 | 9.1 | -14.7% |
| RangeExt | 2.2 | 2.4 | -5.3% |
| Overall (w/o SC) | 3.2 | 3.6 | -8.9% |
| Overall (w/ SC) | 5.3 | 6.3 | -11.8% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 32.1 | -7.6% |
| Class B | 2.6 | 2.6 | -2.0% |
| SC(GBR) | 88.8 | 104.4 | -11.0% |
| RangeExt | 2.5 | 2.6 | -1.4% |
| Overall (w/o SC) | 13.3 | 14.4 | -4.0% |
| Overall (w/ SC) | 51.0 | 59.4 | -7.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 50.1 | -6.1% |
| Class B | 2.6 | 2.6 | -1.6% |
| SC(GBR) | 301.9 | 356.0 | -10.3% |
| RangeExt | 2.5 | 2.6 | -1.3% |
| Overall (w/o SC) | 20.5 | 21.6 | -3.3% |
| Overall (w/ SC) | 161.2 | 188.8 | -6.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 9. Simulation results for combination of improved SWP with SAP-HV

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.4 | -12.6% |
| Class B | 2.1 | 2.3 | -7.2% |
| SC(GBR) | 7.5 | 9.3 | -16.2% |
| RangeExt | 2.2 | 2.3 | -5.0% |
| Overall (w/o SC) | 3.2 | 3.6 | -8.5% |
| Overall (w/ SC) | 5.3 | 6.4 | -12.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 32.2 | -7.3% |
| Class B | 2.6 | 2.6 | -1.9% |
| SC(GBR) | 88.8 | 106.5 | -12.0% |
| RangeExt | 2.5 | 2.6 | -1.3% |
| Overall (w/o SC) | 13.3 | 14.4 | -3.8% |
| Overall (w/ SC) | 51.0 | 60.5 | -7.9% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 50.2 | -5.7% |
| Class B | 2.6 | 2.6 | -1.6% |
| SC(GBR) | 301.9 | 363.2 | -11.3% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 20.5 | 21.6 | -3.1% |
| Overall (w/ SC) | 161.2 | 192.4 | -7.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

# Complexity analysis

The following two metrics are used to measure the decoding implementation complexity:

- Number of operations to decode one sample (worst case):

  - Operation for decision making: three adds, one comparison

  - Operations for weighted prediction: four multiplications, four adds, one shift, one division, four table look-ups

  - Operations for SAD calculation (per weight): four subtractions, three adds

  - Total number of operations per sample: 1x “decision making” + 1x “weighted prediction” + 4x SAD calculation  four multiplications, 19 adds, one shift, one division, four table look-ups, one comparison, 16 subtractions

  - The number of operations at the right block boundary is lower.

  - The number of operations for the exception handling is lower as no weighted prediction is performed

- Throughput (e.g. number of residual samples or pixels which can be decoded in parallel): Pixel rows of a TU can be decoded (and encoded) in parallel. The offset for processing a pixel line to the processing of the previous pixel line is one pixel, since the upper right pixel needs to be available for prediction.

- Comment: Advantage of SIMD optimization, i.e., SAD is calculated using the same reference template

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is given in the following;

- Pixel memory:

  - For the prediction of a single pixel, a maximum of eleven reference pixels is necessary.

  - Pixel accessed from neighbouring TUs accessed during prediction of the current TU:

    - two pixel rows from the upper block,

    - two pixel columns from the left block,

    - four pixels from upper left block (if it exists),

    - four pixels from upper right block (if it exists),

    - no pixels from lower left block.

# Text changes for Proposed Draft Amendment

This chapter is based on document JCTVC-L1005_v1-JCTVC-L1003_v23.

It is suggested to replace subsection 8.4.4.2.1 and additionally add section 8.4.4.2.7.

Replace 8.4.4.2.1 (Changes are marked in yellow)

8.4.4.2.1		General intra sample prediction

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is are the predicted samples predSamples[ x ][ y ], with x, y = 0..nT − 1.

The intra sample prediction process according to predModeIntra and trans_quant_bypass_flag applies as follows:

- If predModeIntra is equal to INTRA_PLANAR and trans_quant_bypass_flag is equal to 1, the process specified in subclause 8.4.4.2.7 is applied.

- Otherwise the following process is applied:

The nT * 4 + 1 neighbouring samples p[ x ][ y ] that are constructed samples prior to the deblocking filter process, with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1, are derived as follows:

–	The neighbouring location ( xN, yN ) is specified by

( xN, yN ) = ( xB + x, yB + y )		(8-1)

- The current luma location ( xBY, yBY ) and the neighbouring luma location ( xNY, yNY ) are derived as follows:

( xBY, yBY ) = ( cIdx  = =  0 ) ? ( xB, yB ) : ( xB * SubWidthC, yB * SubHeightC )	(8-2)

( xNY, yNY ) = ( cIdx  = =  0 ) ? ( xN, yN ) : ( xN * SubWidthC, yN * SubHeightC )	(8-3)

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xBY, yBY ) and the neighbouring luma location ( xNY, yNY ) as inputs, and the output is assigned to availableN.

–	Each sample p[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

  - The variable availableN is equal to FALSE.

  - CuPredMode[ xNY ][ yNY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

- Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xN, yN ) is assigned to p[ x ][ y ].

When at least one sample p[ x ][ y ] with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process for intra sample prediction in subclause Error: Reference source not found is invoked with the samples p[ x ][ y ] with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1, nT, and cIdx as inputs, and the modified samples p[ x ][ y ] with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1 as output.

Depending on the value of predModeIntra, the following ordered steps apply:

- When cIdx is equal to 0 or ChromaArrayType is equal to 3, the filtering process of neighbouring samples specified in Error: Reference source not found is invoked with the sample array p and the transform block size nT as inputs, and the output is reassigned to the sample array p.

- The intra sample prediction process according to predModeIntra applies as follows:

  - If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nT as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p, the transform block size nT, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, (predModeIntra is in the range of INTRA_ANGULAR2..INTRA_ANGULAR34), the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the intra prediction mode predModeIntra, the sample array p, the transform block size nT, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

Add section 8.4.4.2.7

8.4.4.2.7		Specification of intra prediction mode INTRA_SWP

The section describes the process which is invoked for prediction if trans_quant_bypass_flag is equal to 1 and predModeIntra is equal to INTRA_PLANAR.

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

- the residual sample array values r[ x ][ y ], with x, y = 0..nT − 1.

Output of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nT − 1.

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The SWP_Table is given depending on chroma_format_idc and cIdx as follows:

- If chroma_format_idc is equal to 3 or cIdx is equal to 0:

SWP_Table = {2097152, 1812405, 1566320, 1353648, 1169852, 1011012, 873739, 755104, 652577,

563972, 487397, 421219, 364027, 314600, 271884, 234968, 203065, 175493, 151665, 131072,

113275, 97895, 84603, 73116, 63188, 54609, 47194, 40786, 35248, 30462, 26326, 22752, 19662,

16993, 14686, 12692, 10968, 9479, 8192, 7080, 6118, 5288, 4570, 3949, 3413, 2950, 2549, 2203,

1904, 1645, 1422, 1229, 1062, 918, 793, 686, 592, 512, 442, 382, 330, 286, 247, 213, 184, 159,

138, 119, 103, 89, 77, 66, 57, 50, 43, 37, 32, 28, 24, 21, 18, 15, 13, 12, 10, 9, 7, 6, 6, 5, 4, 4, 3, 3,

2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1}

SWP_Max_Argument = 104

- Otherwise the following applies:

- If chroma_format_idc is equal to 2:

SWP_Table = {2097152, 1720368, 1411279, 1157722, 949720, 779089, 639114, 524288, 430092,

352820, 289430, 237430, 194772, 159779, 131072, 107523, 88205, 72358, 59358, 48693,

39945, 32768, 26881, 22051, 18089, 14839, 12173, 9986, 8192, 6720, 5513, 4522, 3710, 3043,

2497, 2048, 1680, 1378, 1131, 927, 761, 624, 512, 420, 345, 283, 232, 190, 156, 128, 105, 86, 71,

58, 48, 39, 32, 26, 22, 18, 14, 12, 10, 8, 7, 5, 4, 4, 3, 2, 2, 2, 1, 1, 1, 1, 1, 1}

SWP_Max_Argument = 77

- Otherwise, chroma_format_idc is equal to 1:

SWP_Table  = {2097152, 1541128, 1132525, 832255, 611597, 449443, 330281, 242713, 178361,

131072, 96321, 70783, 52016, 38225, 28090, 20643, 15170, 11148, 8192, 6020,	4424, 3251,

2389, 1756, 1290, 948, 697, 512, 376, 276, 203, 149, 110, 81, 59, 44, 32, 24, 17, 13, 9, 7, 5, 4, 3,

2, 1, 1, 1, 1}

SWP_Max_Argument = 49

The nT * 4 + 4 neighbouring samples p[ x ][ y ] that are constructed samples, with x = −2..−1, y = −2..nT−1 and x = −2..nT−1, y = −2..−1, are derived as follows:

–	The neighbouring location ( xN, yN ) is specified by

( xN, yN ) = ( xB + x, yB + y )

- The current luma location ( xBY, yBY ) and the neighbouring luma location ( xNY, yNY ) are derived as follows:

( xBY, yBY ) = ( cIdx  = =  0 ) ? ( xB, yB ) : ( xB * SubWidthC, yB * SubHeightC )

( xNY, yNY ) = ( cIdx  = =  0 ) ? ( xN, yN ) : ( xN * SubWidthC, yN * SubHeightC )

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xBY, yBY ) and the neighbouring luma location ( xNY, yNY ) as inputs, and the output is assigned to availableN.

–	Each sample p[ x ][ y ] is derived as follows:

- If the variable availableN is set to FALSE, p[ x ][ y ] is set to ( 1 << ( bitDepth – 1 ) ).

- Otherwise, the sample at the location ( xN, yN ) is assigned to p[ x ][ y ].

- Each sample predSample[ x ][ y ] is derived as follows:

- If x is smaller than ( nT – 1 ), predSample[ x ][ y ] is calculated as follows:

predSample[ x ][ y ] = (p[ x–1 ][ y ] * w-1,0 + p[ x–1 ][ y–1 ] * w-1,-1 + p[ x ][ y–1 ] * w0,-1

+ p[ x+1 ][ y–1 ] * w1,-1 + (w-1,0 + w-1,-1 + w0,-1 + w1,-1)>>1 )  /  ( w-1,0 + w-1,-1 + w0,-1 + w1,-1 )

- Otherwise, predSample[ x ][ y ] is calculated as follows:

predSample[ x ][ y ] = (p[ x–1 ][ y ] * w-1,0 + p[ x–1 ][ y–1 ] * w-1,-1 + p[ x ][ y–1 ] * w0,-1

+ (w-1,0 + w-1,-1 + w0,-1 )>>1 )  /  ( w-1,0 + w-1,-1 + w0,-1 )

- The weighting factors wm,n are looked up in a table SWP_Table depending on x as follows:

  - If ( SADm,n >> ( bitDepth – 8 ) ) is greater than SWP_Max_Argument, wm,n is set to 0.

  - Otherwise wm,n is set to ( SWP_Table[ SADm,n >> ( bitDepth – 8 ) ] >> ( bitDepth – 8 ) ).

  - If x is smaller than ( nT – 2 ), SADm,n is calculated as follows:

SADm,n = | p[ x–1 ][ y ] – p[ x–1+m ][ y+n ] | + | p[ x–1 ][ y–1 ] – p[ x–1+m ][ y–1+n ] | +

| p[ x ][ y–1 ] – p[ x+m ][ y–1+n ] | + | p[ x+1 ][ y–1 ] – p[ x+1+m ][ y–1+n ] |

  - Otherwise:

SADm,n = | p[ x–1 ][ y ] – p[ x–1+m ][ y+n ] | + | p[ x–1 ][ y–1 ] – p[ x–1+m ][ y–1+n ] | +

| p[ x ][ y–1 ] – p[ x+m ][ y–1+n ] |

- If all of the resulting weights from the table look-up are zero, equal averaging process is applied for calculation of predSample[ x ][ y ], i.e., all wm,n are set to one in the previous calculation formulas

- The neighbouring sample p[ x ][ y ] is updated as follows:

p[ x ][ y ] = r[ x ][ y ] + predSample[ x ][ y ]

# Conclusion

Lossless coding is need in several professional applications, e.g., in the medical domain. This contribution presented an intra prediction scheme designed for lossless coding, which shows significant gain over the current HEVC design. Therefore, we propose to include the proposed technique in the Proposed Draft Amendment (PDAM) for Range Extensions of HEVC.

# Acknowledgment

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO).

# References

- P. Amon, A. Hutter, E. Wige, A. Kaup,“ Intra prediction for lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC- L0161/M27497, Geneva, Switzerland, January 2013.

- W. Gao, M. Zhou, P. Amon, S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC- L1122/M28403, Geneva, Switzerland, January 2013.

- F. Bosson, “Common test conditions and software reference configurations”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC- K1100/M27343, Shanghai, China, October 2012.

- M. Zhou, “AHG7: Sample-based angular intra prediction for HEVC range extension”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-L0176, Geneva, Switzerland, January, 2013.

- Sunil Lee, Il-Koo Kim, Chanyul Kim, “AHG7: Residual DPCM for HEVC lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-L0117, Geneva, Switzerland, January 2013.

- Sunil Lee, Il-Koo Kim, Chanyul Kim, “AHG7: Residual DPCM for HEVC lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-L0117, Geneva, Switzerland, January 2013.

- Eugen Wige, Peter Amon, Sunil Lee, Il-Koo Kim, Chanyul Kim, “RCE2: Experimental results for Test 6 – combination of RDPCM and SWP for HEVC lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0053, Incheon, Korea, April 2013.

# Patent rights declaration(s)

Siemens AG and Universität Erlangen-Nürnberg may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).