| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0053 |
| --- | --- |

| Title: | RCE2: Experimental results for Test 6 – combination of RDPCM and SWP for HEVC lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eugen Wige Universität Erlangen-Nürnberg, Germany Peter Amon Siemens, Germany Sunil Lee Il-Koo Kim Chanyul Kim Samsung Electronics Co. Ltd., Korea | Email: Email: Email: Email: Email: | wige@LNT.de p.amon@siemens.com sunil.lee@samsung.com ilkoo.kim@samsung.com dionism@samsung.com |
| Source: | Siemens, Samsung |  |  |

_____________________________

# Abstract

The document reports the experimental results for RCE2 Test 6. In Test 6 the combination of RDPCM (Residual DPCM) and SWP (Sample-based Weighted Prediction) for lossless coding is evaluated. In RDPCM, the sample-based DPCM is applied to the residual samples of intra-coded blocks when the intra prediction mode is either vertical or horizontal mode, and the differentially coded residual samples are signalled, whereas the SWP tool replaces the regular PLANAR mode of the HEVC standard. It is reported that the combined use of SWP and RDPCM achieves bit-rate savings of 10.5% with SC and 8.0% without SC for AI-Main, 6.5% with SC and 3.6% without SC for RA-Main, and 6.0% with SC and 2.9% without SC for LB-Main configuration settings.

# Introduction

The Range Extension Core Experiment 2 (RCE2) on lossless coding has been established for the identification of efficient tools for lossless coding with HEVC [1]. In these experiments, especially screen content coding applications like remote desktop and wireless display are targeted. But, also in professional applications as medical applications, automotive applications, and digital cinema, lossless compression is desired for long-term archiving of image/video data. Two lossless coding tools proposed in the previous meeting in Geneva, Residual DPCM (RDPCM, [2]) and Sample-based Weighted Prediction (SWP, [3]), can be applied simultaneously.

In RDPCM, the sample-based DPCM is applied to the residual samples when lossless coding is enabled for the CU, the prediction mode is intra, and the intra prediction mode is either vertical or horizontal mode. A detailed explanation of the RDPCM algorithm can be found in [2] and [4].

In SWP, the PLANAR mode of the HEVC standard is replaced by a sample-based weighted prediction scheme [3][5]. For prediction using SWP, the similarity of four direct neighbouring pixels to the current pixel to be predicted is determined and based on the estimated similarity a weighted averaging of the neighbouring pixels yields the predicted value. A detailed explanation of the SWP algorithm can be found in [5].

In the next section, the lossless coding simulation results for the combined usage of RDPCM and SWP used in the RCE2 reference software are reported.

# Simulation results

The proposed methods were implemented into HEVC range extension software HM-10.0_RExt2.0. The common test conditions for RCE2 as defined in [1] were used. The simulation platform is a homogenous cluster (consisting of Intel(R) Core(TM)2 Quad CPU Q9400 @ 2.66GHz CPUs). The simulation results are illustrated in the following tables. Compression ratios, relative bit-rate savings, and encoder as well as decoder runtime are shown in these tables for the simulated configurations.

Table 1. Summary of RCE Test 6 for All Intra Main configuration settings

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.3 | -11.4% |
| Class B | 2.1 | 2.3 | -7.2% |
| SC(GBR) | 7.5 | 8.8 | -12.9% |
| RangeExt | 2.2 | 2.3 | -4.9% |
| Overall (w/o SC) | 3.2 | 3.5 | -8.0% |
| Overall (w/ SC) | 5.3 | 6.2 | -10.5% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2. Summary of RCE Test 6 for Random Access Main configuration settings

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.8 | -6.6% |
| Class B | 2.6 | 2.6 | -1.9% |
| SC(GBR) | 88.8 | 101.3 | -9.4% |
| RangeExt | 2.5 | 2.6 | -1.3% |
| Overall (w/o SC) | 13.3 | 14.3 | -3.6% |
| Overall (w/ SC) | 51.0 | 57.8 | -6.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3. Summary of RCE Test 6 for Low delay B Main configuration settings

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 49.8 | -5.4% |
| Class B | 2.6 | 2.6 | -1.6% |
| SC(GBR) | 301.9 | 343.6 | -9.0% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 20.5 | 21.5 | -2.9% |
| Overall (w/ SC) | 161.2 | 182.6 | -6.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

# Complexity analysis and WD text changes

Complexity analysis and the necessary WD text changes are described in the corresponding documents, i.e., RDPCM is described in [4] whereas SWP is described in [5]. The corresponding WD text changes can be applied simultaneously as they do not influence each other.

# Acknowledgement

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO).

# References

- W. Gao, M. Zhou, P. Amon, S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-L1122/M28403, Geneva, Switzerland, January 2013.

- Sunil Lee, Il-Koo Kim, Chanyul Kim, “AHG7: Residual DPCM for HEVC lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-L0117, Geneva, Switzerland, January 2013.

- P. Amon, A. Hutter, E. Wige, A. Kaup, “Intra prediction for lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-L0161, Geneva, Switzerland, January 2013.

- Sunil Lee, Il-Koo Kim, Chanyul Kim, “RCE2: Test 1 – Residual DPCM for HEVC lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0079, Incheon, Korea, April 2013.

- P. Amon, A. Hutter, E. Wige, A. Kaup,“RCE2: Sample-based weighted intra prediction for lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0052, Incheon, Korea, April 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., Siemens AG, and Universität Erlangen-Nürnberg may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).