| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0054 |
| --- | --- |

| Title: | SCE2: Cross-check for Results of test 1.2 and test 1.3 on inter-layer texture prediction (JCTVC-Mxxx) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhan Ma Felix Fernandes 1301 E. Lookout Drive, Richardson, TX 75082, USA | Tel: Email: | +1-972-761-7134 zhan.ma@sta.samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution reports the cross-check for the SCE2 test 1.2 and 1.3 from Texas Instruments. Cross-check results perfectly match the experimental results provided by TI.

# Introduction

Verified inter-layer texture prediction (ILTP) when PU size is not 2Nx2N. To this purpose, proponent adds the ILTP mode into the merge candidate list if a particular condition is met. It doesn’t require additional syntax flag.

In addition, additional results with combined M216 and this proposal are also verified.

# Test Results

Source code for verified test arrived in time enough for study. Performance test was conducted vs SHM1.0 IntraBL framework [3].

Performance test summary is presented in Table 1 and 2.

In average verified tool provide 0,1% Luma BD-rate gain, but leads to 0,1% Chroma BD-rate drop.

We didn’t observe noticeable change of run-time.

Table 1 Results for test 1.2 only

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.3% | -0.3% |  |  |  | -0.2% | -0.1% | 0.0% |
| Class B | -0.2% | 0.1% | 0.3% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | 0.3% |
| Overall (Test vs Ref) | -0.2% | 0.0% | 0.1% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | 0.2% |
| Overall (Test vs single layer) | 19.0% | 32.0% | 33.2% | 16.3% | 28.3% | 30.9% | 14.7% | 28.9% | 33.0% |
| EL only (Test vs Ref) | -0.4% | -0.1% | 0.0% | -0.2% | -0.2% | -0.1% | -0.2% | 0.0% | 0.2% |
| Enc Time[%] | 99.9% |  |  | 99.8% |  |  | 100.0% |  |  |
| Dec Time[%] | 101.8% |  |  | 99.9% |  |  | 100.7% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.2% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.3% | -0.3% |  |  |  | -0.3% | -0.1% | 0.0% |
| Class B | -0.4% | 0.2% | 0.2% | -0.4% | -0.2% | 0.0% | -0.3% | 0.3% | 0.6% |
| Overall (Test vs Ref) | -0.4% | 0.1% | 0.1% | -0.4% | -0.2% | 0.0% | -0.3% | 0.2% | 0.4% |
| Overall (Test vs single layer) | 25.5% | 36.6% | 38.3% | 22.0% | 32.9% | 35.9% | 21.9% | 34.7% | 39.8% |
| EL only (Test vs Ref) | -0.6% | -0.1% | -0.1% | -0.6% | -0.4% | -0.2% | -0.4% | 0.2% | 0.4% |
| Enc Time[%] | 100.1% |  |  | 100.1% |  |  | 100.1% |  |  |
| Dec Time[%] | 102.4% |  |  | 100.7% |  |  | 101.4% |  |  |
| Enc Mem[%] | 100.2% |  |  | 100.2% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2 Test results for test 1.3 (1.1 + 1.2)

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.6% | -0.5% |  |  |  | -0.2% | -0.3% | -0.3% |
| Class B | -0.2% | 0.2% | 0.4% | -0.2% | -0.1% | 0.0% | -0.1% | 0.1% | 0.3% |
| Overall (Test vs Ref) | -0.2% | 0.0% | 0.1% | -0.2% | -0.1% | 0.0% | -0.2% | 0.0% | 0.2% |
| Overall (Test vs single layer) | 19.0% | 32.0% | 33.2% | 16.3% | 28.4% | 30.9% | 14.7% | 28.8% | 32.9% |
| EL only (Test vs Ref) | -0.6% | -0.4% | -0.3% | -1.3% | -1.1% | -1.0% | -0.4% | -0.3% | -0.1% |
| Enc Time[%] | 100.5% |  |  | 98.4% |  |  | 101.4% |  |  |
| Dec Time[%] | 100.8% |  |  | 99.4% |  |  | 101.2% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.3% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.5% | -0.4% |  |  |  | -0.4% | -0.3% | -0.1% |
| Class B | -0.5% | 0.4% | 0.5% | -0.6% | 0.1% | 0.2% | -0.4% | 0.3% | 0.7% |
| Overall (Test vs Ref) | -0.5% | 0.2% | 0.3% | -0.6% | 0.1% | 0.2% | -0.4% | 0.1% | 0.4% |
| Overall (Test vs single layer) | 25.4% | 36.7% | 38.5% | 21.8% | 33.2% | 36.2% | 21.9% | 34.6% | 39.8% |
| EL only (Test vs Ref) | -0.9% | -0.3% | -0.2% | -2.0% | -1.2% | -1.0% | -0.7% | -0.1% | 0.2% |
| Enc Time[%] | 101.0% |  |  | 98.4% |  |  | 101.8% |  |  |
| Dec Time[%] | 101.7% |  |  | 100.7% |  |  | 100.7% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.2% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

More details about test results can be found in excel spreadsheet attached to this contribution.

# Conclusion

Performance of SCE2 test 1.2 and 1.3 are verified in this report [1,2].  The modified merge process improved luma BL+EL BD-rate up to 0.4%. When combined with the skipped ILTP signaling method, luma BL + EL BD-rate was improved up to 0.6%.

# Reference

- D.-K. Kwon, M. Budagavi and M. Zhou, “SCE2: Results of test 1.1 on skipped inter-layer texture prediction (ILTP) block signaling in Inter slice,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M217, Apr. 2013.

- D.-K. Kwon, M. Budagavi and M. Zhou, “Non-TE2: Improved inter-layer texture prediction signaling,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0230, Jan. 2013.

- X. Li, J. Boyce, P. Onno and Y. Ye, “Common test condition and software reference configurations for the scalable test model,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1009, Jan. 2013