| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 April 2013 | Document: JCTVC-M0055 |
| --- | --- |

| Title: | SCE4: Switchable De-ringing Filter for Inter-layer Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhan Ma Felix C. A. Fernandes 1301 E. Lookout Drive, Richardson, TX 75082, USA | Tel: Email: | +1-972-761-7134 zhan.ma@sta.samsung.com felix.f@sta.samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution presents a CU-level, switchable, de-ringing, inter-layer filter (SDRF) designed to improve the inter-layer prediction efficiency. It is reported that the BD-rate (EL+BL) of SDRF with intraBL changes compared to SHM1.0 are -1.0%/-0.3% for AI HEVC 2x/1.5x, -0.9%/-0.2%/-0.5% for RA HEVC 2x/1.5x/SNR, -1.1%/-0.6%/-0.9% for LDP HEVC 2x/1.5x/SNR, -0.8%/-0.2%/-0.6% for LDB HEVC 2x/1.5x/SNR, with direct DIVISION implementation. A low-complexity implementation in which a look-up table (LUT) replaces the division operator is also presented. For this SDRF-using-LUT implementation the reported results are -0.7%/-0.2% for AI HEVC 2x/1.5x, -0.7%/-0.x%/-0.x% for RA HEVC 2x/1.5x/SNR, -0.9%/-0.x%/-0.x% for LDP HEVC 2x/1.5x/SNR, -0.x%/-0.x%/-0.x% for LDB HEVC 2x/1.5x/SNR.

# Introduction

For spatial scalability, the reconstructed picture is one of the most important inter-layer predictions. More specifically, the reconstructed low-resolution picture (referred to as the base-layer picture) is up-sampled to form a prediction for the same picture at a higher resolution level (referred to as the enhancement-layer picture). However, the up-sampled base-layer picture often suffers low quality due to the resolution re-sampling error and the quantization error from base-layer encoding. Consequently, we observe that ringing artifacts are commonly present in the up-sampled base-layer pictures around major edge regions. These artifacts degrade the interlayer prediction performance and reduce the overall coding efficiency of the enhancement layer.

# Proposed De-ringing Filter (DRF)

For each pixel in the up-sampled base layer (BL) reconstruction, the De-Ringing Filter (DRF) is applied to remove noise or to enhance an edge. Currently, it uses a 3x3 template for filtering.

Figure 1 Illustration of 3x3 templates for de-ringing filter

Let I(x) be the pixel intensity at position x to be filtered (as shown in Figure 1), and N(x) be the set consisting of x and its 8 immediate neighbors. Now, for any y in N(x), denote the pixel intensity at y as I(y). After applying the de-ringing filter at position x, the filtered intensity now becomes

|  | (1) |
| --- | --- |

where w(||y-x||) is a spatial kernel (as shown in Figure 1(b) or (c)) which is neighboring-pixel distance dependent, and g(||Iy - Ix||) is an intensity kernel which is dependent on the neighboring pixel-intensity difference. The spatial-distance metric ||y-x|| is defined as shown below for y in N(x) and x located at the center:

In the current implementation, different w() kernels are used for 2x spatial scalability and else 1.5x and SNR scalability as shown in Table 1. Please note that 1.5x and SNR share the same kernels. In [2], different kernels were used for luma and chroma channels. In this contribution, the process is simplified by using the same kernels for luma and chroma.

Table 1: w for de-ringing filter

| 2x | W_luma_2x = {{w(-4), w(-3), w(-2)}, {{ 3, 4, 3 }, {w(-1), w( 0), w( 1)}, = { 4, 5, 4 }, {w( 2), w( 3), w( 4)}} { 3, 4, 3 }} |
| --- | --- |
| 1.5x/SNR | W_luma_15x = {{w(-4), w(-3), w(-2)}, {{ 1, 2, 1 }, {w(-1), w( 0), w( 1)}, = { 2, 8, 2 }, {w( 2), w( 3), w( 4)}} { 1, 2, 1 }} |

The intensity kernel g()  is controlled by the intensity difference between pixel x and its neighboring pixel y. For simplicity, g() is implemented using a look-up table to control the filtering strength, i.e.,

g[65] = { 32, 30, 27, 22, 16, 11, 7, 4, 2, 1, 0, 0 0, 0, … … , 0 };

With the g() and w() kernels presented in this contribution, the maximum value of any product in the numerator of Eq. (1) is max(I())*max(g())*max(w()) = 255*32*8 = 65280 which fits in a 16-bit precision.

Also, please note that filtering operates at the IntraBL CU by padding the left, upper, right and bottom 1 line pixels using the nearest neighbor without requiring the additional memory bandwidth on top of the up-sampling.

## Look-up Table

In Eq. (1), local weighting wd =  is divided by the numerator to derive the filtered results.  In this contribution, this division operator is replaced by a look-up table. Let Isum = , hence, Eq (1) is re-written as

I'(x)=Isumwd=Isum×Awd×1A

where A could be a large number implemented using the shift. In current implementation, A = 1 << 16. Therefore, the division is replaced by approximating Awdas a LUT.  Since it uses different w() for 2x and 1.5x/SNR, different LUT is generated respectively, i.e.,

2x[897] = {410,0	,0	,402	,400	,0	,395	,392	,390	,388	,386	,383	,381	,379	,377	,374	,372	,370	,368	,366	,364	,362	,360	,358	,356	,354	,352	,350	,349	,347	,345	,343	,341    ,340	,338	,336	,334	,333	,331	,329	,328	,326	,324	,323	,321	,320	,318	,317	,315	,314	,312	,311	,309	,308	,306	,305	,303	,302	,301	,299	,298	,297	,295	,294	,293    ,291	,290	,289	,287	,286	,285	,284	,282	,281	,280	,279	,278	,277	,275	,274	,273	,272	,271	,270	,269	,267	,266	,265	,264	,263	,262	,261	,260	,259	,258	,257	,256  ,255	,254	,253	,252	,251	,250	,249	,248	,247	,246	,245	,245	,244	,243	,242	,241	,240	,239	,238	,237	,237	,236	,235	,234	,233	,232	,232	,231	,230	,229	,228	,228    ,227	,226	,225	,224	,224	,223	,222	,221	,221	,220	,219	,218	,218	,217	,216	,216	,215	,214	,213	,213	,212	,211	,211	,210	,209	,209	,208	,207	,207	,206	,205	,205    ,204	,204	,203	,202	,202	,201	,200	,200	,199	,199	,198	,197	,197	,196	,196	,195	,194	,194	,193	,193	,192	,192	,191	,191	,190	,189	,189	,188	,188	,187	,187	,186    ,186	,185	,185	,184	,184	,183	,183	,182	,182	,181	,181	,180	,180	,179	,179	,178	,178	,177	,177	,176	,176	,175	,175	,174	,174	,173	,173	,172	,172	,172	,171	,171    ,170	,170	,169	,169	,168	,168	,168	,167	,167	,166	,166	,165	,165	,165	,164	,164	,163	,163	,163	,162	,162	,161	,161	,161	,160	,160	,159	,159	,159	,158	,158	,158    ,157	,157	,156	,156	,156	,155	,155	,155	,154	,154	,153	,153	,153	,152	,152	,152	,151	,151	,151	,150	,150	,150	,149	,149	,149	,148	,148	,148	,147	,147	,147	,146    ,146	,146	,145	,145	,145	,144	,144	,144	,143	,143	,143	,142	,142	,142	,142	,141	,141	,141	,140	,140	,140	,139	,139	,139	,139	,138	,138	,138	,137	,137	,137	,137    ,136	,136	,136	,135	,135	,135	,135	,134	,134	,134	,133	,133	,133	,133	,132	,132	,132	,132	,131	,131	,131	,131	,130	,130	,130	,130	,129	,129	,129	,129	,128	,128    ,128	,128	,127	,127	,127	,127	,126	,126	,126	,126	,125	,125	,125	,125	,124	,124	,124	,124	,123	,123	,123	,123	,122	,122	,122	,122	,122	,121	,121	,121	,121	,120    ,120	,120	,120	,120	,119	,119	,119	,119	,119	,118	,118	,118	,118	,117	,117	,117	,117	,117	,116	,116	,116	,116	,116	,115	,115	,115	,115	,115	,114	,114	,114	,114    ,114	,113	,113	,113	,113	,113	,112	,112	,112	,112	,112	,111	,111	,111	,111	,111	,111	,110	,110	,110	,110	,110	,109	,109	,109	,109	,109	,109	,108	,108	,108	,108     ,108	,107	,107	,107	,107	,107	,107	,106	,106	,106	,106	,106	,106	,105	,105	,105	,105	,105	,105	,104	,104	,104	,104	,104	,104	,103	,103	,103	,103	,103	,103	,102    ,102	,102	,102	,102	,102	,101	,101	,101	,101	,101	,101	,101	,100	,100	,100	,100	,100	,100	,99	,99	,99	,99	,99	,99	,99	,98	,98	,98	,98	,98	,98	,98      ,97	,97	,97	,97	,97	,97	,97	,96	,96	,96	,96	,96	,96	,96	,95	,95	,95	,95	,95	,95	,95	,94	,94	,94	,94	,94	,94	,94	,93	,93	,93	,93      ,93	,93	,93	,93	,92	,92	,92	,92	,92	,92	,92	,92	,91	,91	,91	,91	,91	,91	,91	,91	,90	,90	,90	,90	,90	,90	,90	,90	,89	,89	,89	,89     ,89	,89	,89	,89	,88	,88	,88	,88	,88	,88	,88	,88	,87	,87	,87	,87	,87	,87	,87	,87	,87	,86	,86	,86	,86	,86	,86	,86	,86	,86	,85	,85     ,85	,85	,85	,85	,85	,85	,85	,84	,84	,84	,84	,84	,84	,84	,84	,84	,83	,83	,83	,83	,83	,83	,83	,83	,83	,83	,82	,82	,82	,82	,82	,82     ,82	,82	,82	,82	,81	,81	,81	,81	,81	,81	,81	,81	,81	,81	,80	,80	,80	,80	,80	,80	,80	,80	,80	,80	,79	,79	,79	,79	,79	,79	,79	,79     ,79	,79	,78	,78	,78	,78	,78	,78	,78	,78	,78	,78	,78	,77	,77	,77	,77	,77	,77	,77	,77	,77	,77	,77	,76	,76	,76	,76	,76	,76	,76	,76      ,76	,76	,76	,76	,75	,75	,75	,75	,75	,75	,75	,75	,75	,75	,75	,74	,74	,74	,74	,74	,74	,74	,74	,74	,74	,74	,74	,73	,73	,73	,73	,73      ,73	,73	,73	,73	,73	,73	,73	,72	,72	,72	,72	,72	,72	,72	,72	,72	,72	,72	,72	,72	,71	,71	,71	,71	,71	,71	,71	,71	,71	,71	,71	,71     ,71	,70	,70	,70	,70	,70	,70	,70	,70	,70	,70	,70	,70	,70	,69	,69	,69	,69	,69	,69	,69	,69	,69	,69	,69	,69	,69	,69	,68	,68	,68	,68     ,68	,68	,68	,68	,68	,68	,68	,68	,68	,68	,67	,67	,67	,67	,67	,67	,67	,67	,67	,67	,67	,67	,67	,67	,67	,66	,66	,66	,66	,66	,66	,66      ,66	,66	,66	,66	,66	,66	,66	,66	,65	,65	,65	,65	,65	,65	,65	,65	,65	,65	,65	,65	,65	,65	,65	,65	,64	,64	,64	,64	,64	,64	,64	,64     ,64	,64	,64	,64	,64	,64	,0	,64	,63	,63	,63	,63	,0	,63	,0	,63	,63	,63	,0	,63	,0	,0	,0	,63	,0	,62	,0	,0	,0	,0	,0	,62}

15x[385]={ 256, 	255		,254		,253		,252		,251		,250	,249	,248	,247	,246	,245	,245	,244	,243	,242	,241	,240		,239	,238	,237	,237	,236	,235	,234	,233	,232	,232	,231	,230	,229	,228	,228	,227	,226	,225	,224	,224	,223	,222	,221	,221		,220	,219	,218	,218	,217	,216	,216	,215	,214	,213	,213	,212		,211	,211	,210	,209	,209	,208	,207	,207	,206	,205	,205	,204	,204	,203	,202	,202	,201	,200	,200	,199	,199	,198	,197	,197		,196	,196	,195	,194	,194	,193	,193	,192	,192	,191	,191	,190	,189	,189	,188	,188	,187	,187	,186	,186	,185	,185	,184	,184		,183	,183	,182	,182	,181	,181	,180	,180	,179	,179	,178	,178		,177	,177	,176	,176	,175	,175	,174	,174	,173	,173	,172	,172	,172	,171	,171	,170	,170	,169	,169	,168	,168	,168	,167	,167	,166	,166	,165	,165	,165	,164	,164	,163	163	,163	,162	,162		,161	,161	,161	,160	,160	,159	,159	,159	,158	,158	,158	,157	,157	,156	,156	,156	,155	,155	,155	,154	,154	,153	,153	,153		,152	,152	,152	,151	,151	,151	,150	,150	,150	,149	,149	,149	,148	,148	,148	,147	,147	,147	,146	,146	,146	,145	,145	,145	,144	,144	,144	,143	,143	,143	,142	,142	,142	,142	,141	,141		,141	,140	,140	,140	,139	,139	,139	,139	,138	,138	,138	,137	,137	,137	,137	,136	,136	,136	,135	,135	,135	,135	,134	,134	,134	,133	,133	,133	,133	,132	,132	,132	,132	,131	,131	,131		,131	,130	,130	,130	,130	,129	,129	,129	,129	,128	,128	,128	,128	,127	,127	,127	,127	,126	,126	,126	,126	,125	,125	,125		,125	,124	,124	,124	,124	,123	,123	,123	,123	,122	,122	,122	,122	,122	,121	,121	,121	,121	,120	,120	,120	,120	,120	,119	,119	,119	,119	,119	,118	,118	,118	,118	,117	,117	,117	,117	,117	,116	,116	,116	,116	,116	,115	,115	,115	,115	,115	,114	,114	,114	,114	,114	,113	,113	,113	,113	,113	,112	,112	,112	,112	,112	,111	,111	,111	,111	,111	,111	,110	,110	,110	,110		,110	,109	,109	,109	,109	,109	,109	,108	,108	,108	,108    ,108	,107	,107	,107	,107	,107	,107	,106	,106	,106	,106	,106	,106	,105	,105	,105	,105	,105	,105	,104	,104	,104	,104	,104	,104	,103	,103	,103	,0	,103	,0	,102 }

According to the structure of presented table, it can be compressed significant to save the storage.

## Proposed CU level Switchable De-ringing Filter (SDRF)

Since the reconstructed BL pictures are used for intraBL only in current SHM1.0, DRF is proposed to be coupled with intraBL mode. In this implementation, if the intraBL flag is true, an additional test is conducted using RDOQ to determine whether DRF is required or not. Therefore, the proposed solution is switchable at coding unit (CU) level.

# Experimental results

Evaluation is done with the test conditions in SCE4 document [1].

## SDRF using direct DIVISION implementation

Table 2 shows the results for each configuration. The proposed method is implemented in SHM1.0.  It is observed that consistent gains over SHM1.0 are achieved with slight increase of encoding and decoding time, especially for RA and LDP cases.

Table 2: Results of SDRF using direct division implementation

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.8% | -2.5% | -2.7% |  |  |  |  |  |  |
| Class B | -0.7% | -1.1% | -1.1% | -0.3% | -0.7% | -0.7% |  |  |  |
| Overall (Test vs Ref) | -1.0% | -1.5% | -1.6% | -0.3% | -0.7% | -0.7% |  |  |  |
| Overall (Test vs single layer) | 11.3% | 12.0% | 11.7% | 9.9% | 9.6% | 8.9% |  |  |  |
| EL only (Test vs Ref) | -1.3% | -1.8% | -1.9% | 0.7% | 0.3% | 0.2% |  |  |  |
| Enc Time[%] | 113.6% |  |  | 111.2% |  |  |  |  |  |
| Dec Time[%] | 118.6% |  |  | 115.3% |  |  |  |  |  |
| Enc Mem[%] | 101.3% |  |  | 102.1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.6% | -0.9% | -1.3% |  |  |  | -0.6% | -0.6% | -0.5% |
| Class B | -0.6% | -0.7% | -0.6% | -0.2% | -0.3% | -0.2% | -0.5% | -0.5% | -0.2% |
| Overall (Test vs Ref) | -0.9% | -0.8% | -0.8% | -0.2% | -0.3% | -0.2% | -0.5% | -0.5% | -0.3% |
| Overall (Test vs single layer) | 18.2% | 31.1% | 32.0% | 16.3% | 28.2% | 30.7% | 14.2% | 28.3% | 32.4% |
| EL only (Test vs Ref) | -1.3% | -1.1% | -1.2% | 0.6% | 0.6% | 0.6% | -0.5% | -0.5% | -0.2% |
| Enc Time[%] | 104.5% |  |  | 103.6% |  |  | 102.7% |  |  |
| Dec Time[%] | 105.3% |  |  | 105.5% |  |  | 106.2% |  |  |
| Enc Mem[%] | 103.0% |  |  | 104.4% |  |  | 112.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.6% | -1.3% | -1.8% |  |  |  | -0.7% | -1.0% | -0.8% |
| Class B | -0.9% | -1.3% | -1.1% | -0.6% | -0.7% | -0.7% | -1.0% | -1.0% | -0.8% |
| Overall (Test vs Ref) | -1.1% | -1.3% | -1.3% | -0.6% | -0.7% | -0.7% | -0.9% | -1.0% | -0.8% |
| Overall (Test vs single layer) | 24.7% | 34.8% | 36.4% | 21.8% | 32.3% | 35.0% | 21.2% | 33.1% | 38.1% |
| EL only (Test vs Ref) | -1.5% | -1.7% | -1.7% | -0.1% | -0.2% | -0.2% | -1.1% | -1.2% | -1.0% |
| Enc Time[%] | 104.5% |  |  | 103.5% |  |  | 102.9% |  |  |
| Dec Time[%] | 108.0% |  |  | 109.1% |  |  | 108.3% |  |  |
| Enc Mem[%] | 103.1% |  |  | 104.3% |  |  | 112.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.5% | -1.1% | -1.5% |  |  |  | -0.7% | -0.7% | -0.7% |
| Class B | -0.5% | -0.9% | -0.9% | -0.2% | -0.3% | -0.3% | -0.5% | -0.7% | -0.4% |
| Overall (Test vs Ref) | -0.8% | -0.9% | -1.1% | -0.2% | -0.3% | -0.3% | -0.6% | -0.7% | -0.5% |
| Overall (Test vs single layer) | 27.2% | 36.1% | 37.1% | 24.4% | 32.6% | 34.8% | 23.0% | 32.6% | 36.4% |
| EL only (Test vs Ref) | -1.1% | -1.2% | -1.4% | 0.4% | 0.4% | 0.4% | -0.6% | -0.7% | -0.6% |
| Enc Time[%] | 105.4% |  |  | 104.8% |  |  | 104.4% |  |  |
| Dec Time[%] | 109.5% |  |  | 113.5% |  |  | 112.2% |  |  |
| Enc Mem[%] | 103.4% |  |  | 104.5% |  |  | 112.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

## SDRF using LUT to replace DIVISION

Table 3 shows the results for each configuration. The proposed method is implemented in SHM1.0.

Table 2: Results of SDRF using LUT for DIVISION (ref. SHM1.0)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.4% | -1.7% | -2.0% |  |  |  |  |  |  |
| Class B | -0.4% | -0.5% | -0.5% | -0.2% | -0.6% | -1.0% |  |  |  |
| Overall (Test vs Ref) | -0.7% | -0.8% | -0.9% | -0.2% | -0.6% | -1.0% |  |  |  |
| Overall (Test vs single layer) | 11.6% | 12.7% | 12.4% | 10.1% | 9.6% | 8.7% |  |  |  |
| EL only (Test vs Ref) | -0.9% | -1.0% | -1.1% | 0.8% | 0.2% | -0.2% |  |  |  |
| Enc Time[%] | 113.8% |  |  | 111.3% |  |  |  |  |  |
| Dec Time[%] | 114.4% |  |  | 109.9% |  |  |  |  |  |
| Enc Mem[%] | 101.6% |  |  | 102.2% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -0.2% | -1.0% |  |  |  | -0.4% | -0.4% | -1.1% |
| Class B | -0.4% | -0.3% | -0.2% | -0.1% | -0.4% | -0.7% | -0.3% | -0.7% | -1.0% |
| Overall (Test vs Ref) | -0.7% | -0.2% | -0.4% | -0.1% | -0.4% | -0.7% | -0.4% | -0.6% | -1.1% |
| Overall (Test vs single layer) | 18.5% | 31.8% | 32.5% | 16.5% | 28.1% | 30.0% | 14.4% | 28.2% | 31.4% |
| EL only (Test vs Ref) | -0.9% | -0.4% | -0.6% | 1.0% | 0.7% | 0.3% | -0.2% | -0.5% | -1.0% |
| Enc Time[%] | 104.4% |  |  | 103.6% |  |  | 102.8% |  |  |
| Dec Time[%] | 104.7% |  |  | 102.9% |  |  | 106.3% |  |  |
| Enc Mem[%] | 103.3% |  |  | 104.1% |  |  | 112.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -0.6% | -1.3% |  |  |  | -0.6% | -0.5% | -1.2% |
| Class B | -0.7% | -0.7% | -0.7% | -0.4% | -0.8% | -0.8% | -0.8% | -1.1% | -1.1% |
| Overall (Test vs Ref) | -0.9% | -0.7% | -0.9% | -0.4% | -0.8% | -0.8% | -0.7% | -0.9% | -1.2% |
| Overall (Test vs single layer) | 25.0% | 35.7% | 37.0% | 22.1% | 32.1% | 35.0% | 21.4% | 33.3% | 37.5% |
| EL only (Test vs Ref) | -1.1% | -0.9% | -1.2% | 0.4% | 0.0% | 0.0% | -0.8% | -1.0% | -1.3% |
| Enc Time[%] | 104.4% |  |  | 103.6% |  |  | 102.5% |  |  |
| Dec Time[%] | 107.8% |  |  | 108.6% |  |  | 106.2% |  |  |
| Enc Mem[%] | 103.2% |  |  | 104.3% |  |  | 112.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -0.4% | -1.2% |  |  |  | -0.5% | -0.3% | -1.3% |
| Class B | -0.3% | -0.5% | -0.5% | -0.1% | -0.5% | -0.5% | -0.4% | -0.9% | -1.0% |
| Overall (Test vs Ref) | -0.6% | -0.5% | -0.7% | -0.1% | -0.5% | -0.5% | -0.4% | -0.7% | -1.0% |
| Overall (Test vs single layer) | 27.5% | 36.8% | 37.7% | 24.6% | 32.3% | 34.5% | 23.2% | 32.5% | 35.7% |
| EL only (Test vs Ref) | -0.8% | -0.6% | -0.8% | 0.7% | 0.2% | 0.2% | -0.4% | -0.7% | -1.1% |
| Enc Time[%] | 105.4% |  |  | 104.7% |  |  | 104.4% |  |  |
| Dec Time[%] | 108.7% |  |  | 110.0% |  |  | 111.8% |  |  |
| Enc Mem[%] | 103.4% |  |  | 104.5% |  |  | 112.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Complexity Measurement

# Conclusion

This contribution presents a CU-level, switchable, de-ringing filter design for inter-layer filtering, so as to improve the inter-layer prediction efficiency. It is reported that the BD-rate (EL+BL) of SDRF with intraBL changes compared to SHM1.0 are -1.0%/-0.3% for AI HEVC 2x/1.5x, -0.9%/-0.2%/-0.5% for RA HEVC 2x/1.5x/SNR,  -1.1%/-0.6%/-0.9% for LDP HEVC 2x/1.5x/SNR, -0.8%/-0.2%/-0.6% for LDB HEVC 2x/1.5x/SNR , with direct DIVISION implementation It is also reported that 0 are -0.7%/-0.2% for AI HEVC 2x/1.5x, -0.7%/-0.x%/-0.x% for RA HEVC 2x/1.5x/SNR,  -0.9%/-0.x%/-0.x% for LDP HEVC 2x/1.5x/SNR, -0.x%/-0.x%/-0.x% for LDB HEVC 2x/1.5x/SNR , with an exemplified LUT implementation for the DIVISION.

It is proposed to adopt this method into SHVC WD.

# Reference

[1] J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park " Description of HEVC Scalable Extensions Core Experiment 4 (SCE4): Inter-layer filtering ", JCT-VC L1104, JCT-VC 12th Meeting: Geneva, Jan. 2013

[2] Z. Ma, F. Fernandes, “Non-TE4: Switchable De-ringing Filter for Inter-layer Prediction” JCT-VC L0195, JCT-VC 12th Meeting: Geneva, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).