| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0056 |
| --- | --- |

| Title: | RCE2: Experimental results on Test 3 and Test 4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-567-0360 madhukar@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

The document reports the experimental results for RCE2 Test 3 and Test 4. Test 3 applies the SAP (Sample Adaptive Angular Intra Prediction) in all 33 angular intra prediction directions; while Test 4 applies the SAP only in the horizontal and vertical angular intra prediction directions and uses the HM10.0 block based angular intra prediction in the rest of directions. The test results revealed that

- Compared to the HM10.0 lossless coding mode, overall (w/SC) the SAP in all directions (Test 3) provides an average gain of  8.9% in AI-Main, 5.1% in RA-Main and 4.6% in LB-Main, and the SAP in horizontal and vertical directions only (Test 4) provides an average gain of 9.2% in AI-Main, 5.8% in RA-Main and 5.3% in LB-Main.

- The SAP in all directions (Test 3) underperforms the SAP in horizontal and vertical directions only (Test 4) by 2.0% in AI-Main, 1.9% in RA-Main and 1.8% in LB-Main for screen content sequences (i.e. SC 4:4:4 RGB only), respectively.

- The SAP in all directions (Test 3) outperforms the SAP in horizontal and vertical directions only (Test 4) by 1.4% in AI-Main, 0.4% in RA-Main and 0.4% in LB-Main for non 4:4:4 screen content sequences (i.e. overall w/o SC) , respectively.

- Overall (w/ SC) the SAP in all directions (Test 3) underperforms the SAP in horizontal and vertical directions only by 0.3% in AI-Main, 0.7 % in RA-Main and 0.7% in LB-Main, respectively.

In terms of decoder throughput, the SAP in all directions (Test 3) can be processed by 4 samples per cycle in the worst case, while the SAP in horizontal and vertical directions only (Test 4) can be processed by 4x4 samples per cycle in the worst case.

# Introduction

In HM10.0 the “lossless coding for free” mode is supported in response to increasing needs of lossless video coding for real-world applications such as remote desktop, online gaming, wireless display, video conferencing, automotive vision, long-distance education and content post-production. In HM10.0 the lossless coding is enabled by bypassing transform, quantization and in-loop filters (de-blocking filter, SAO) and can be turned on/off at CU-level by using lossless coding on/off flag at CU level.

The current HM10.0 lossless coding mode can be improved by introducing new coding tools. One of the tools that has been proposed is so-called sample-based angular intra prediction (SAP) [3][4][5][6]. Just like the block-based angular intra prediction defined in JCTVC-K1003, in the proposed SAP all the samples in a PU still share a same prediction angle, but the prediction sample of the current sample in a PU uses two of its adjacent neighbors (not limited to the boundary reference samples from neighboring PU).  The prediction angle signaling method and the sample interpolation method remain unchanged. Therefore, the SAP introduces changes only in decoding process, but not in syntax and semantics. For the detailed algorithm description of the SAP, please refer to [3][4][5][6][7].

This document reports the SAP performance for RCE2 [1] Test 3 and Test 4 by using the in HM10.0 range extension (r3369). Test 3 applies the SAP in all 33 angular intra prediction directions; while Test 4 applies the SAP only in the horizontal and vertical angular intra prediction directions and uses the HM10.0 block based intra angular prediction in the rest of directions.

No technical changes have been introduced when compared to [3][4][5][6][7]. In both software the SAP is integrated in a way in which it can be turned on/off at CU-level, to align with the current HM10.0 lossless coding signaling method.

# Test Settings and Conditions

HM10.0 range extension software is used and the simulation results are made available for AI-Main, RA-Main and LB-Main configurations.  The simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies.

The test sequences used are all four Class F sequences, Class B sequence 1080p Kimono1 and ParkScene, range extension 1080p 4:2:2 sequence EBUHorse and EBUWaterRocksClose (10-bit, 500 frames), range extension 1080p 4:4:4 sequence EBURainFruits and BirdsInCage (10-bit, 600 frames), and the ten screen content 4:4:4 RGB sequences as listed in Table 1.

| Sequence | Resolution | Number of frames | Bit-depth | Frame-rate |
| --- | --- | --- | --- | --- |
| sc-wordEditing | 1280 x 720 | 600 | 8 | 60 |
| sc-map | 1280 x 720 | 600 | 8 | 60 |
| sc-programming | 1280 x 720 | 600 | 8 | 60 |
| VenueVu | 1920 x 1080 | 300 | 8 | 30 |
| sc-cad-waveform | 1920 x 1080 | 200 | 8 | 20 |
| sc-pcb-layout | 1920 x 1080 | 200 | 8 | 20 |
| sc-ppt-doc | 1920 x 1080 | 200 | 8 | 20 |
| sc-video-conferencing-doc-sharing | 1280 x 720 | 300 | 8 | 30 |
| sc-web-browsing | 1280 x 720 | 300 | 8 | 30 |
| sc-cg-twist-tunnel | 1280 x 720 | 300 | 8 | 30 |

Table 1. List of 4:4:4 screen content sequences (RGB)

# Experimental results

The results for RCE2 Test 3 and Test 4 are provided in Table 3 and Table 4, respectively. The major observations are (see Table 2):

- The SAP in all directions (Test 3) underperforms the SAP in horizontal and vertical directions only (Test 4) by 2.0% in AI-Main, 1.9% in RA-Main and 1.8% in LB-Main for screen content sequences (i.e. SC 4:4:4 RGB only).

- The SAP in all directions (Test 3) outperforms the SAP in horizontal and vertical directions only (Test 4) by 1.4% in AI-Main, 0.4% in RA-Main and 0.4% in LB-Main for non 4:4:4 screen content sequences (i.e. overall w/o SC)

- Compared to the HM10.0 lossless coding mode, overall (w/SC) the SAP in all directions (Test 3) provides an average gain of  8.9% in AI-Main, 5.1% in RA-Main and 4.6% in LB-Main, and the SAP in horizontal and vertical directions only (Test 4) provides an average gain of 9.2% in AI-Main, 5.8% in RA-Main and 4.3% in LB-Main.

- Overall (w/ SC) the SAP in all directions (Test 3) underperforms the SAP in horizontal and vertical directions only by 0.3% in AI-Main, 0.7 % in RA-Main and 0.7% in LB-Main.

|  | AI-Main (%) | RA-Main (%) | LB-Main (%) |
| --- | --- | --- | --- |
|  | SC 4:4:4 RGB only |  |  |
| Test 3 | -10.4 | -6.9 | -6.4 |
| Test 4 | -12.4 | -8.8 | -8.2 |
| Test 3 vs. Test 4 | 2.0 | 1.9 | 1.8 |
|  | Overall (w/o SC) |  |  |
| Test 3 | -7.4 | -3.3 | -2.7 |
| Test 4 | -6.0 | -2.9 | -2.3 |
| Test 3 vs. Test 4 | -1.4 | -0.4 | -0.4 |
|  | Overall (w/ SC) |  |  |
| Test 3 | -8.9 | -5.1 | -4.6 |
| Test 4 | -9.2 | -5.8 | -5.3 |
| Test 3 vs. Test 4 | 0.3 | 0.7 | 0.7 |

Table 2. Summary of RCE2 Test 3 and Test 4 results

Table 3 summarizes the simulation results for RCE2 Test 3 in which the SAP is enabled in all 33 angular intra prediction directions. Detailed results can be found in HM10.0-RangeExt-TI-losslessAnchor_vs_SAP.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.3 | -11.6% |
| Class B | 2.1 | 2.3 | -5.8% |
| SC(GBR) | 7.5 | 8.4 | -10.4% |
| RangeExt | 2.2 | 2.3 | -4.1% |
| Overall (w/o SC) | 3.2 | 3.5 | -7.4% |
| Overall (w/ SC) | 5.3 | 5.9 | -8.9% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.7 | -6.7% |
| Class B | 2.6 | 2.6 | -1.2% |
| SC(GBR) | 88.8 | 96.4 | -6.9% |
| RangeExt | 2.5 | 2.6 | -1.0% |
| Overall (w/o SC) | 13.3 | 14.2 | -3.3% |
| Overall (w/ SC) | 51.0 | 55.3 | -5.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 49.7 | -5.4% |
| Class B | 2.6 | 2.6 | -0.9% |
| SC(GBR) | 301.9 | 328.3 | -6.4% |
| RangeExt | 2.5 | 2.6 | -0.9% |
| Overall (w/o SC) | 20.5 | 21.4 | -2.7% |
| Overall (w/ SC) | 161.2 | 174.9 | -4.6% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

Table 3. Results of Test 3 (SAP in all directions vs. HM10.0 lossless coding mode)

Table 4 summarizes the simulation results for RCE2 Test 4 in which the SAP is enabled only in horizontal and vertical angular intra prediction directions. Detailed results can be found in HM10.0-RangeExt-TI-losslessAnchor_vs_SAP_HVonly.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.2 | -10.1% |
| Class B | 2.1 | 2.2 | -4.4% |
| SC(GBR) | 7.5 | 8.7 | -12.4% |
| RangeExt | 2.2 | 2.3 | -2.9% |
| Overall (w/o SC) | 3.2 | 3.5 | -6.0% |
| Overall (w/ SC) | 5.3 | 6.1 | -9.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.7 | -6.1% |
| Class B | 2.6 | 2.6 | -0.9% |
| SC(GBR) | 88.8 | 99.6 | -8.8% |
| RangeExt | 2.5 | 2.6 | -0.7% |
| Overall (w/o SC) | 13.3 | 14.2 | -2.9% |
| Overall (w/ SC) | 51.0 | 56.9 | -5.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 49.8 | -4.9% |
| Class B | 2.6 | 2.6 | -0.6% |
| SC(GBR) | 301.9 | 340.2 | -8.2% |
| RangeExt | 2.5 | 2.5 | -0.6% |
| Overall (w/o SC) | 20.5 | 21.4 | -2.3% |
| Overall (w/ SC) | 161.2 | 180.8 | -5.3% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |

Table 4. Results of Test 4 (SAP in H&V directions only vs. HM10.0 lossless coding mode)

# Throughput analysis

The SAP in all directions (Test 3) is fully parallel on the encoder side, and can be executed at a speed of one row or one column per cycle on the decoder side. For a 4x4 intra PU, this would mean 4 cycles per 4x4 block (i.e. 4 samples per cycle) on the decoder side, which is far less than a typical cycle budget of 12 per 4x4 block.

The SAP in horizontal and vertical directions only (Test 4) is fully parallel on both the encoder and decoder sides. For a 4x4 intra PU, this would mean 1 cycle per 4x4 block on the decoder side (i.e. 16 samples per cycle).

# Conclusion and recommendation

Efficient lossless coding is required for real-world applications such as such as remote desktop, online gaming, wireless display, video conferencing, automotive vision, long-distance education and content post-production. The proposed lossless coding method with sample-based angular intra prediction provides significant gain over the HM10.0 lossless coding method which bypasses transform, quantization, de-blocking filter, SAO and ALF. It is recommended to adopt this method into test model and working draft of HEVC range extension.

# References

[1]	W. Gao, M. Zhou, P. Amon, S.Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” JCT-VC Document, JCTVC-L1122, Geneva, CH, 14 -23 January, 2013.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent)” JCT-VC Document, JCTVC-L1003, Geneva, CH, 14 -23 January, 2013.

[3] M. Zhou, “AHG22: Sample-based angular prediction (SAP) for HEVC lossless coding”, JCT-VC Document, JCTVC-G093, Geneva, CH, Nov. 2011.

[4] M. Zhou, “AHG19: Method of frame-based lossless coding mode for HEVC”, JCT-VC Document, JCTVC-H083, San Jose, CA, USA, February 2012.

[5] M. Zhou, “AHG13: Sample-based angular intra prediction for HEVC lossless coding”, JCT-VC Document, JCTVC-I0117, Geneva, CH, April 2012.

[6] M. Zhou, “AHG7: Sample-based angular intra prediction for HEVC lossless coding”, JCT-VC Document, JCTVC-K0199, Shanghai, China, October 2012.

[7] M. Zhou, “AHG7: Sample-based angular intra prediction for HEVC range extension”, JCT-VC Document, JCTVC-L0176, Geneva, CH, 14 -23 January, 2013.

# Patent rights declaration(s)

Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation |ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text changes

For Test 3 (the SAP in all directions)

Replace 8.4.4.2.6

##### Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

With (changes are marked in yellow)

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

- the residual sample array values r[ x ][ y ], with x, y = 0..nT−1.

Output of this process are:

–	predicted samples predSamples[ x ][ y ], with x, y = 0..nT−1.

Error: Reference source not found illustrates the total 33 intra angles and Error: Reference source not found specifies the mapping table between intraPredMode and the angle parameter intraPredAngle.

Figure 8-1 – Intra prediction angle definition (informative)

Table 8-1 – Specification of intraPredAngle

| intraPredMode | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Error: Reference source not found further specifies the mapping table between intraPredMode and the inverse angle parameter invAngle.

Table 8-2 – Specification of invAngle

| intraPredMode | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| intraPredMode | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

If cu_transquant_bypass_flag is equal to 0, the values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT, is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nT		(8-1)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x=( nT * intraPredAngle )  >>  5..−1	(8-2)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nT + 1..2 * nT	(8-3)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5		(8-4)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31		(8-5)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-6)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-7)

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0, y = 0..nT − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-8)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nT		(8-9)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = ( nT * intraPredAngle )  >>  5..−1	(8-10)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nT + 1..2 * nT	(8-11)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-12)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-13)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-14)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-15)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0..nT − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-16)

If cu_transquant_bypass_flag is equal to 1, the values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

- If intraPredMode is less than 18,

- if intraPredAngle is less than 0, ref [0] = p[-1 + x][ y-1], ref[1] = p[-1+x][ y];

- if intraPredAngle is larger than or equal to 0,  ref[0] = p[-1 + x][ y]; if y is less than nT -1, ref[1] = p[-1+x][ y+1], otherwise, ref[1] = p[-1+x][ nT-1];

- Otherwise  if intraPredMode is larger than or equal to 18

- if intraPredAngle less than 0, ref[0] = p[-1 + x][ y-1], ref[1] = p[x][ y-1];

- if intraPredAngle large than or equal to  0, ref[0] = p[x][ y-1];  if x is less than nT -1, ref[1] = p[1+x][ y-1], otherwise, ref[1] = p[nT-1, y-1].

–	The multiplication factor iFact are derived by

iFact = ( intraPredAngle ) & 31		(8-62)

–	Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x][y ] is derived by

predSamples[ x][ y ] = ( ( 32 – iFact )*ref[ 0 ] + iFact*ref[ 1] + 16 ) >> 5	(8-63)

- Otherwise, the value of the prediction samples predSamples[ x][ y ] is derived by

predSamples[ x][ y ] = ref[0]		(8-64)

- The neighbouring sample p[x][ y] is reconstructed by

- p[x][y] = r [x][ y] + predSamples[x][ y] 		(8-65)

For Test 4 (the SAP in horizontal and vertical directions)

Replace 8.4.4.2.6

##### Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

With (changes are marked in yellow)

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nT * 2 − 1 and x = 0..nT * 2 − 1, y = −1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

- the residual sample array values r[ x ][ y ], with x, y = 0..nT−1.

Output of this process are:

–	predicted samples predSamples[ x ][ y ], with x, y = 0..nT−1.

Error: Reference source not found illustrates the total 33 intra angles and Error: Reference source not found specifies the mapping table between intraPredMode and the angle parameter intraPredAngle.

Figure 8-2 – Intra prediction angle definition (informative)

Table 8-3 – Specification of intraPredAngle

| intraPredMode | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| intraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Error: Reference source not found further specifies the mapping table between intraPredMode and the inverse angle parameter invAngle.

Table 8-4 – Specification of invAngle

| intraPredMode | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| intraPredMode | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

If cu_transquant_bypass_flag is equal to 0 or intraPredAngle is not equal to 0, the values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT, is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nT		(8-17)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x=( nT * intraPredAngle )  >>  5..−1	(8-18)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nT + 1..2 * nT	(8-19)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5		(8-20)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31		(8-21)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-22)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-23)

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0, y = 0..nT − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-24)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ], with x= −nT..2 * nT is specified as follows: [Ed. (YK): Inconsistent value ranges of x in the sentence and in the equations.]

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nT		(8-25)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nT * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = ( nT * intraPredAngle )  >>  5..−1	(8-26)

[Ed. (YK): Incorrect value range expression y..z with z < y.]

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nT + 1..2 * nT	(8-27)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-28)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-29)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-30)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-31)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0 and nT is less than 32, the following filtering applies with x = 0..nT − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-32)

If cu_transquant_bypass_flag is equal to 1 and intraPredAngle is equal to 0, the values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

- If intraPredMode is less than 18, the value of the prediction samples predSamples[ x][ y ] is derived by

predSamples[ x][ y ] = p[-1 + x][ y]		(8-62)

- Otherwise,  if intraPredMode is larger than or equal to 18 the value of the prediction samples predSamples[ x][ y ] is derived by

predSamples[ x][ y ] = p[x][ y-1]		(8-63)

- The neighbouring sample p[x][ y] is reconstructed by

p[x][y] = r [x][ y] + predSamples[x][ y] 		(8-64)