| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0057 |
| --- | --- |

| Title: | RCE2: Crosscheck of Test 6 (JCTVC-M0053) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi | Tel: Email: | +1-214-567-0630 madhukar@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Introduction

This contribution reports cross-check results for Test 6 of RCE2 (JCTVC-M0053). Test 6 is a combination of residual DPCM for horizontal/vertical intra prediction directions (Test 1, JCTVC-L0117) and sample-based weighted intra prediction (Test 2, JCTVC-L0161). It is reported that there is a match to the bit-rate results generated by the proponents. Summary of bit rate results are provided in Table 1.

# Complexity considerations

In terms of decoder throughput, residual DPCM can be processed by 4x4 samples per cycle in the worst case where as sample-based weighted intra prediction can be processed 1 sample per cycle. The combination will therefore also need to be processed 1 sample/cycle.

Table1: Test6 (L0117+L0161) v/s anchor

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.3 | -11.4% |
| Class B | 2.1 | 2.3 | -7.2% |
| SC(GBR) | 7.5 | 8.8 | -12.9% |
| RangeExt | 2.2 | 2.3 | -4.9% |
| Overall (w/o SC) | 3.2 | 3.5 | -8.0% |
| Overall (w/ SC) | 5.3 | 6.2 | -10.5% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 89% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.8 | -6.6% |
| Class B | 2.6 | 2.6 | -1.9% |
| SC(GBR) | 88.8 | 101.3 | -9.4% |
| RangeExt | 2.5 | 2.6 | -1.3% |
| Overall (w/o SC) | 13.3 | 14.3 | -3.6% |
| Overall (w/ SC) | 51.0 | 57.8 | -6.5% |
| Enc Time[%] | 81% |  |  |
| Dec Time[%] | 83% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 49.8 | -5.4% |
| Class B | 2.6 | 2.6 | -1.6% |
| SC(GBR) | 301.9 | 343.6 | -9.0% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 20.5 | 21.5 | -2.9% |
| Overall (w/ SC) | 161.2 | 182.6 | -6.0% |
| Enc Time[%] | 81% |  |  |
| Dec Time[%] | 83% |  |  |