| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0059 |
| --- | --- |

| Title: | SCE5.1.7: Inter-layer motion data inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng Wenhao Zhang Lidong Xu Yu Han Xiaoxia Cai Yi-Jen Chiu | Email: | zhipin.deng@intel.com wenhao.zhang@intel.com lidong.xu@intel.com yu.han@intel.com xiaoxia.cai@intel.com yi-jen.chiu@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

In this contribution, an inter-layer motion data inheritance (MDI) technique is studied. The proposed MDI is only applied to inter 2Nx2N PU of EL. The EL CU with MDI enabled will derive the motion data from the collocated BL blocks and perform motion compensation at NxN granularity. In addition, the proposed MDI limits the 8x8 PU not being splitted, so that it doesn’t worsen EL’s worst case of memory access during motion compensation. Compared to the MVP hook used in SHM1.0, the proposed MDI is a low complexity solution of inter-layer motion prediction, because it can be implemented in simpler logic and doesn’t change the module for building the merge candidate list. The experimental results show that the proposed method has 0.1% BD-rate loss and the decoding time is decreased by 2.6% on average.

# Proposed algorithm

As shown in Figure 1, the proposed inter-layer motion data inheritance (MDI) method is applied to EL CU to directly copy the motion data from its collocated BL block. Here the motion data consists of motion vector, reference index and inter predictive direction.

Figure 1: The block diagram of the proposed inter-layer motion data inheritance

In this contribution, an inter_layer_MDI_flag is applied to each inter 2Nx2N PU to signal if the inter-layer MDI is used or not. If the inter_layer_MDI_flag is true, the motion data of the collocated BL block are directly inherited for the motion compensation of the EL block. As shown in Figure 2, a 2Nx2N EL block is divided into four NxN sub-blocks, and the center point of each NxN sub-block is used to find the collocated BL block. Since the inherited motion granularity is NxN, each inter 2Nx2N PU has 4 inherited motion vectors. The motion vectors are scaled in the case of spatial 2x and 1.5x scalabilities. Besides, if motion data of the collocated NxN BL block is not available, the collocated NxN block in the up-sampled BL reconstructed picture will be copied as the prediction of the current EL NxN block.

16

16

16

16

BL picture

EL picture

Current 2Nx2N PU

(2N = 16)

Find collocated BL block

Figure 2: The derivation of collocated BL blocks in the case of 2x scalability

Currently, SHM1.0 applies inter-layer motion prediction by using the MVP hook, in which the motion data of collocated BL block is used as an additional merge candidate of EL PU. It has the following issues during the implementation:

- The MVP hook requires more computation operations to perform pruning between the inter-layer motion candidate and the spatial motion candidates.

- The MVP hook cannot reuse the BL’s module of merge candidate list building. It means the additional cost for hardware implementation is required.

Compared to the MVP hook, the proposed MDI method is a simpler solution to achieve effective inter-layer motion prediction for SHVC, for the following reasons,

- The implementation logic of the proposed MDI method is rather simple.

- The merge candidate list building module of BL can directly reused by the EL.

# Experiment and results

## Testing conditions

The proposed MDI method is implemented to SHM1.0 and simulated by using the testing condition specified by the Common Test Conditions in [1]. According to the Description of SCE5 in [2], the RA 2x/1.5x/SNR, LD-P 2x/1.5x/SNR and LD-B 2x/1.5x/SNR cases are tested.

## Experimental results

Table 1-3 give the simulation results of RA, LD-P and LD-B configurations. It’s reported the proposed MDI method has 0.1% BD-rate loss for all the testing cases, and the decoding time is decreased by 2.6% on average. The detailed bit rate and PSNR results are provided in the accompanied spreadsheet.

Table 1: simulation results of RA configuration

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% |  |  |  | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Overall (Test vs single layer) | 19.4% | 32.1% | 33.1% | 16.6% | 28.5% | 30.9% | 15.0% | 28.9% | 32.7% |
| EL only (Test vs Ref) | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 102.8% |  |  | 102.1% |  |  | 102.0% |  |  |
| Dec Time[%] | 97.1% |  |  | 98.3% |  |  | 98.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2: simulation results of LD-P configuration

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% |  |  |  | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Overall (Test vs Ref) | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Overall (Test vs single layer) | 26.2% | 36.6% | 38.2% | 22.6% | 33.1% | 36.0% | 22.5% | 34.5% | 39.1% |
| EL only (Test vs Ref) | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 103.4% |  |  | 102.7% |  |  | 103.4% |  |  |
| Dec Time[%] | 95.2% |  |  | 96.2% |  |  | 99.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3: simulation results of LD-B configuration

|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% |  |  |  | 0.1% | 0.0% | 0.0% |
| Class B | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| Overall (Test vs Ref) | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| Overall (Test vs single layer) | 28.4% | 37.4% | 38.5% | 24.8% | 33.0% | 35.3% | 23.9% | 33.5% | 37.2% |
| EL only (Test vs Ref) | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 102.5% |  |  | 101.9% |  |  | 101.0% |  |  |
| Dec Time[%] | 98.1% |  |  | 96.9% |  |  | 96.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

## Additional results

Table 4 gives the overall results of additional tests versus SHM1.0 without MVP hook. Table 5 gives the results of MDI compared to SHM1.0 without MVP hook. As MVP hook without pruning is the simplest merge mode for SHVC, Table 6 shows the results of MVP hook without pruning compared to SHM1.0 without MVP hook. Table 7 shows the results of SHM1.0 (MVP hook with pruning) compared to SHM1.0 without MVP hook.

It’s reported the proposed MDI method has average 1.1% BD-rate gain as compared to SHM1.0 without MVP hook. The simplest merge mode (MVP hook without pruning) has average 1.0% BD-rate gain as compared to SHM1.0 without MVP hook. And the MVP hook with pruning has average 1.2% BD-rate gain as compared to SHM1.0 without MVP hook. In another word, the pruning process has 0.2% BD-rate gain but with more complexity. And the proposed MDI has 0.1% BD-rate gain compared to the simplest merge mode.

Table 4: overall simulation results of additional tests vs. SHM1.0 without MVP hook

|  | RA 2x | RA 1.5x | RA SNR | LD-P 2x | LD-P 1.5x | LD-P SNR | Avg |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | Y | Y | Y | Y | Y |
| MDI | -1.1% | -1.5% | -1.7% | -0.5% | -0.7% | -1.1% | -1.1% |
| MVP hook without pruning | -0.9% | -1.6% | -1.6% | -0.1% | -0.6% | -1.0% | -1.0% |
| MVP hook with pruning | -1.2% | -1.8% | -2.0% | -0.4% | -0.8% | -1.3% | -1.2% |

Table 5: simulation results of MDI vs. SHM1.0 without MVP hook

Table 6: simulation results of MVP hook without pruning vs. SHM1.0 without MVP hook

Table 7: simulation results of MVP hook with pruning vs. SHM1.0 without MVP hook

Reference

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 12th Meeting, JCTVC-L1009, Geneva, CH, 14–23 Jan. 2013.

- V. Seregin, P. Onno, S. Liu, E. Alshina, C. Kim, H. Yang, “Description of Core Experiment SCE5: Inter-layer syntax prediction using HEVC base layer”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 12th Meeting, JCTVC-L1105, Geneva, CH, 14–23 Jan. 2013.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).