| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0062 |
| --- | --- |

| Title: | Non-SCE3: Inter-layer residual prediction with motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenhao Zhang Zhipin Deng Lidong Xu Yu Han Xiaoxia Cai Yi-Jen Chiu | Email: | wenhao.zhang@intel.com zhipin.deng@intel.com lidong.xu@intel.com yu.han@intel.com xiaoxia.cai@intel.com yi-jen.chiu@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution presents an inter-layer residual prediction technology to improve the coding efficiency of SHVC. It utilizes the inter-coded residual of the collocated BL block to refine the inter prediction of the EL block. For spatial scalability, bilinear interpolation filter is performed to upsample the BL residual block. In addition, the proposed inter-layer residual prediction is enabled under the condition that inter-layer motion prediction is used. Specifically, if a 2Nx2N PU in EL is coded by inter merge mode and its motion candidate is from BL, the residual prediction is applied. No new syntax is added in the proposed implementation, thus no parsing dependent issue is introduced. Compared to the SHM1.0 anchor, the proposed inter-layer residual prediction achieves 0.94% BD-rate saving on average among the RA, LD-P and LD-B testing cases, with marginal increase of coding runtime and memory access bandwidth.

# Introduction

In [1], a SVC-like inter-layer residual prediction method was proposed to SHVC, in which the reconstructed residual signal of BL is utilized to reduce the magnitude of predictive residual of EL. It’s observed that the residual signals across layers are correlated only when the EL block uses same motion with the collocated BL block. Thus, this contribution proposes to apply the inter-layer residual prediction under the condition of inter-layer motion prediction.

# Proposed algorithm

Figure 1 shows the block diagram of the proposed inter-layer residual prediction. At the encoder side of the EL, the residual ResEL is generated by equation (1),

ResEL = OrigEL – PredEL – UP{Res’BL}	(1)

where the OrigEL and PredEL indicate the input signal and predictive signal of EL block, respectively. UP{Res’BL} is reconstructed residual signal of the collocated BL block, which is upsampled by bilinear interpolation filter. Then the ResEL is transformed, quantized and entropy encoded in to bitstream.

At the decoder side of EL, the output reconstructed video is generated by equation (2),

RecEL = Res’EL + PredEL + UP{Res’BL}	(1)

where the Res’EL indicates the reconstructed residual signal of EL after entropy decoding, inverse quantization and inverse transform.

Figure 1: The block diagram of the proposed inter-layer residual prediction

Analysis results show that the cross-layer residual signal redundancy only exists when the EL block has consistent motion with the collocated BL block, the inter-layer residual prediction is proposed to be combined with inter-layer motion prediction. In the intraBL framework of SHM1.0, the motion data of collocated BL is derived and inserted as the first candidate in the merge candidate list of EL PU. For an EL PU, the condition to perform inter-layer residual prediction or not is specified by the following pseudo code,

Bool bInterLayerResidualPred = merge_flag && merge_idx == 0 && bAvailMotionBL;

if ( bInterLayerResidualPred )

{

performInterLayerResidualPred();

}

where the variable bAvailMotionBL specifies if the collocated BL block exists and inter-coded.

It is proposed that the inter-layer residual prediction is always enabled if inter-layer motion prediction is on for current EL PU. It means no new syntax is added and there’s no parsing dependent issue.

Moreover, to achieve the trade-off between the compression performance and the computational complexity, the proposed method is only applied to the EL 2Nx2N PUs.

# Experiments and results

## Testing conditions

The proposed inter-layer residual prediction is implemented to SHM1.0 and simulated by using the testing conditions specified by the Common Test Conditions in [2]. The RA 2x/1.5x/SNR, LD-P 2x/1.5x/SNR and LD-B 2x/1.5x/SNR cases are tested.

## Experimental results

Table 1-3 give the simulation results of RA, LD-P and LD-B configurations. It’s reported the proposed method achieves 0.94% BD-rate reduction on average. It should be noted that current SHM1.0 decoder implementation performs upsampling at whole picture level, which results the large decoding time increment for 2x cases and 1.5x cases. If the residual upsampling is implemented at block level, the decoding time increase will be marginal.

The detailed bit rate and PSNR results are provided in the accompanied spreadsheet.

Table 1: simulation results of RA configuration

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.8% | -0.8% |  |  |  | -0.4% | -1.0% | -1.2% |
| Class B | -0.7% | -1.4% | -1.8% | -0.9% | -1.8% | -2.2% | -0.8% | -1.7% | -2.2% |
| Overall (Test vs Ref) | -0.6% | -1.2% | -1.5% | -0.9% | -1.8% | -2.2% | -0.7% | -1.5% | -1.9% |
| Overall (Test vs single layer) | 18.6% | 30.4% | 31.0% | 15.5% | 26.2% | 28.0% | 14.0% | 27.0% | 30.2% |
| EL only (Test vs Ref) | -1.0% | -1.7% | -2.0% | -1.7% | -2.7% | -3.2% | -1.1% | -2.0% | -2.4% |
| Enc Time[%] | 99.0% |  |  | 98.3% |  |  | 99.1% |  |  |
| Dec Time[%] | 122.0% |  |  | 125.9% |  |  | 108.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2: simulation results of LD-P configuration

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.7% | -0.8% |  |  |  | -0.5% | -0.8% | -0.9% |
| Class B | -0.9% | -1.3% | -1.5% | -1.1% | -1.7% | -2.0% | -1.0% | -1.2% | -1.6% |
| Overall (Test vs Ref) | -0.8% | -1.1% | -1.3% | -1.1% | -1.7% | -2.0% | -0.9% | -1.1% | -1.4% |
| Overall (Test vs single layer) | 25.1% | 35.0% | 36.4% | 21.1% | 30.9% | 33.2% | 21.3% | 33.0% | 37.2% |
| EL only (Test vs Ref) | -1.3% | -1.6% | -1.8% | -2.3% | -2.9% | -3.2% | -1.4% | -1.6% | -1.9% |
| Enc Time[%] | 98.8% |  |  | 97.6% |  |  | 99.3% |  |  |
| Dec Time[%] | 121.0% |  |  | 128.3% |  |  | 107.0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3: simulation results of LD-B configuration

|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.0% | -1.1% |  |  |  | -0.7% | -1.0% | -1.2% |
| Class B | -1.2% | -1.8% | -2.1% | -1.5% | -2.4% | -2.6% | -1.2% | -1.7% | -2.0% |
| Overall (Test vs Ref) | -1.0% | -1.6% | -1.8% | -1.5% | -2.4% | -2.6% | -1.0% | -1.5% | -1.8% |
| Overall (Test vs single layer) | 26.9% | 35.2% | 36.1% | 22.8% | 29.8% | 31.7% | 22.4% | 31.5% | 34.7% |
| EL only (Test vs Ref) | -1.8% | -2.3% | -2.5% | -3.1% | -3.9% | -4.1% | -1.7% | -2.1% | -2.4% |
| Enc Time[%] | 98.4% |  |  | 96.2% |  |  | 99.3% |  |  |
| Dec Time[%] | 122.1% |  |  | 128.7% |  |  | 115.0% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 4 shows the complexity assessment results of the proposed method. According to [3], the memory access bandwidth and mult/add operation number are counted during the decoding tests. Detailed results are also provided in attachment.

Table 4: complexity assessment results

|  | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| --- | --- | --- | --- | --- | --- |
| RA | 115% | 117% | 118% | 107% | 104% |
| LD-P | 122% | 124% | 125% | 109% | 105% |
| LD-B | 116% | 118% | 119% | 107% | 104% |
| All | 118% | 120% | 121% | 108% | 104% |

# Conclusion

This contribution proposes an SVC-like inter-layer residual prediction to improve the inter coding efficiency of SHVC. The residual signal of collocated BL block is stored, and bilinear upsampled if necessary, to refine the inter prediction results of the EL block. It is proposed to enable the inter-layer residual prediction under the condition that inter-layer motion prediction is used. No new syntax is added to avoid the parsing dependent issue and simplify the implementation. It’s reported the average BD-rate reduction of the proposed method is 0.94% compared to the SHM1.0 anchor, and the increase of coding runtime and memory access bandwidth are marginal.

# Reference

- W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu, “TE B3: Inter-layer residual refinement”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 12th Meeting, JCTVC-L0286, Geneva, CH, 14–23 Jan. 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 12th Meeting, JCTVC-L1009, Geneva, CH, 14–23 Jan. 2013.

- E. François, A. Tabatabai, E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 12th Meeting, JCTVC-L0440, Geneva, CH, 14–23 Jan. 2013.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).