| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0063 |
| --- | --- |

| Title: | SCE1: Results of test 3.7 on inter-layer intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhijie Zhao, Joern Ostermann Appelstr. 9A 30165 Hannover | Tel: Email: | 0049 511 7625316 zhao@tnt.uni-hannover.de |
| Source: | Leibniz Universitaet Hannover |  |  |

_____________________________

# Abstract

This document presents the experimental results of SCE1 test 3.7 described in JCTVC-L1101. Inter-layer intra prediction mode coding (ILIPM) has been proposed in JCTVC-L0156. The proposed approaches utilize the base layer intra prediction mode to improve the coding efficiency of the CUs not being coded by Intra_BL in the context of the scalable HEVC coding. The implementations were integrated into SHM-1.0 software. Compared to enhancement layer only of anchor in SCE1, it reports that ILIPM achieves average luma BD-rate gains of 0.4% for All Intra HEVC 2x spatial scalability, 0.1% for All Intra HEVC 1.5x spatial scalability.

# Introduction

During the 12th JCT-VC meeting in Geneva, SCE1 was established to study the proposed methods for inter-layer intra texture and syntax prediction for SHVC [1]. Proposals related to inter-layer intra mode prediction will be evaluated in part 3 of SCE1.

# Proposed method

In the scalable extension of HEVC, since there exists high correlation between base and enhancement layers, intra prediction mode of base layer can be used to improve the coding efficiency of enhancement layer. In JCTVC-L0156 [2], Intra prediction mode of enhancement layer is explicitly signaled whether the intra prediction mode of enhancement layer is equal to that of the co-located unit in base layer. In case intra prediction mode of enhancement layer is derived from that of co-located unit in base layer, MPM process in single layer is bypassed. There is no change to the MPM derivation process. The proposed syntax is shown in Table 1.

Table 1  Proposed syntax changes

coding_unit( x0, y0, log2CbSize ) {

CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ]

if( transquant_bypass_enable_flag ) {

cu_transquant_bypass_flag

}

if( slice_type != I )

cu_skip_flag[ x0 ][ y0 ]

if( cu_skip_flag[ x0 ][ y0 ] )

prediction_unit( x0, y0 , log2CbSize )

else {

if( slice_type != I )

pred_mode_flag

if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize )

part_mode

x1 = x0 + ( ( 1 << log2CbSize ) >> 1 )

y1 = y0 + ( ( 1 << log2CbSize ) >> 1 )

x2 = x1 − ( ( 1 << log2CbSize ) >> 2 )

y2 = y1 − ( ( 1 << log2CbSize ) >> 2 )

x3 = x1 + ( ( 1 << log2CbSize ) >> 2 )

y3 = y1 + ( ( 1 << log2CbSize ) >> 2 )

if( PredMode = = MODE_INTRA ) {

if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIPCMCUSize && log2CbSize <= Log2MaxIPCMCUSize )

pcm_flag

if( pcm_flag ) {

num_subsequent_pcm

NumPCMBlock = num_subsequent_pcm + 1

while( !byte_aligned( ) )

pcm_alignment_zero_bit

for( i = 0; i < 1 << ( log2CbSize << 1 ); i++ )

pcm_sample_luma[ i ]

for( i = 0; i < ( 1 << ( log2CbSize << 1 ) ) >> 1; i++ )

pcm_sample_chroma[ i ]

NumPCMBlock− −

} else {

pbOffset = ( PartMode = = PART_NxN ) ? ( ( 1 << log2CbSize ) >> 2 ) : 0

for( j = 0; j <= pbOffset; j = j + pbOffset )

for( i = 0; i <= pbOffset; i = i + pbOffset ) {

intra_bl_mode_flag[ x0 + i ][ y0+ j ]

if(!intra_bl_mode_flag[ x0 + i ][ y0+ j ])

prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ]

}

for( j = 0; j <= pbOffset; j = j + pbOffset )

for( i = 0; i <= pbOffset; i = i + pbOffset ) {

if(!intra_bl_mode_flag[ x0 + i ][ y0+ j ] ){

if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] )

mpm_idx[ x0 + i ][ y0+ j ]

Else

rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ]

}

}

intra_chroma_pred_mode[ x0 ][ y0 ]

}

} else {

if( PartMode = = PART_2Nx2N )

prediction_unit( x0, y0 , log2CbSize )

else if( PartMode = = PART_2NxN ) {

prediction_unit( x0, y0 , log2CbSize )

prediction_unit( x0, y1 , log2CbSize )

} else if( PartMode = = PART_Nx2N ) {

prediction_unit( x0, y0 , log2CbSize )

prediction_unit( x1, y0 , log2CbSize )

} else if( PartMode = = PART_2NxnU ) {

prediction_unit( x0, y0 , log2CbSize )

prediction_unit( x0, y2 , log2CbSize )

} else if( PartMode = = PART_2NxnD ) {

prediction_unit( x0, y0 , log2CbSize )

prediction_unit( x0, y3 , log2CbSize )

} else if( PartMode = = PART_nLx2N ) {

prediction_unit( x0, y0 , log2CbSize )

prediction_unit( x2, y0 , log2CbSize )

} else if( PartMode = = PART_nRx2N ) {

prediction_unit( x0, y0 , log2CbSize )

prediction_unit( x3, y0 , log2CbSize )

} else { /* PART_NxN */

prediction_unit( x0, y0 , log2CbSize )

prediction_unit( x1, y0 , log2CbSize )

prediction_unit( x0, y1 , log2CbSize )

prediction_unit( x1, y1 , log2CbSize )

}

}

if( !pcm_flag ) {

if( PredMode != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) )

rqt_root_cbf

if( ! rqt_root_cbf ) {

MaxTrafoDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter )

transform_tree( x0, y0, x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 )

}

}

}

}

# Test results

Test sequences and coding condition as shown in [1] were used to evaluate the performance of the proposed approach. SHM-1.0 was used as the anchor. The encoding and decoding test were conducted on a Linux cluster with Intel Xeon CPU X5670, 2.93GHz. The encoder and decoder binaries were compiled with gcc 4.1.2 64-bit. Both the proposed encoder/decoder and anchor encoder/decoder were tested in the same cluster. The average performances of the proposed approaches are shown in the following tables. The detailed results can be found in the enclosed excel sheets.

Table 2 presents the performance of the proposed ILIPM in case Mode Dependent Coefficient Scanning (MDCS) is enabled. The results without MDCS are shown in Table 3.

Table 2 Performance of the proposed ILIPM compared to anchor of SCE1.3, MDCS on

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.1% |  |  |  |
| Class B | -0.3% | -0.3% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.3% | -0.3% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.1% | 13.4% | 13.1% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.4% | -0.4% | -0.4% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101.9% |  |  | 102.5% |  |  |
| Dec Time[%] | 99.9% |  |  | 99.7% |  |  |

Table 3 Performance of the proposed ILIPM compared to anchor of SCE1.3, MDCS off

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | 0.0% |  |  |  |
| Class B | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.2% | 13.5% | 13.3% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.3% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 102.2% |  |  | 102.7% |  |  |
| Dec Time[%] | 100.9% |  |  | 100.8% |  |  |

# References

[1] A. Tabatabai, K. Rapaka, A. Saxena and S. Liu, Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC, JCTVC-L1101, 2013

[2] Z. Zhao, Joern Ostermann, Simplification of TE5.1.5 on inter-layer intra mode prediction, JCTVC-L0156, 2013

# Patent rights declaration(s)

Leibniz Universitaet Hannover may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).