| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0064 |
| --- | --- |

| Title: | SCE1: Crosscheck of Test 3.1 on inter-layer intra mode coding from MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhijie Zhao, Yiqun Liu Appelstr. 9A 30165 Hannover | Tel: Email: | 0049 511 7625316 {zhao,liuyiqun}@tnt.uni-hannover.de |
| Source: | Leibniz Universitaet Hannover |  |  |

_____________________________

# Abstract

The document reports the verification results of SCE1 3.1 described in JCTVC-L1101. It is reported that the BD-rate and PSNR results exactly match those provided by MediaTek.

# Introduction

MediaTek proposed to predict the intra mode at enhancement layer with the intra mode of the collocated block at base layer in order to exploit the correlation of intra modes from different layers. The algorithm is described as follows [1]:

For each of modeBL, modeLeft and modeTop, if it is available and is not DC/Planar, it is put into MPM according to the priority (i.e., modeBL>modeLeft>modeTop) with the repetition removed. When three modes are available in MPM, {modeBL, modeLeft, modeTop} are directly used to predict the current intra mode at enhancement layer; otherwise, MPM is further generated as follows. Denote the number of modes available in MPM as numAvai.

- numAvai = 2, MPM = {MPM[0], MPM[1], DC}

- numAvai = 1, MPM = {MPM[0], MPM[0]-1, MPM[0]+1}

- numAvai = 0, MPM = {DC, VER, HOR}

# Test results

The test is performed under the conditions defined in JCTVC-L1101 [2] with AI 2x and AI 1.5x configurations. The bit rate and PSNR results we got with MediaTek’s software and the results provided by MediaTek are same. The encoding and decoding test were conducted on a Linux cluster with Intel Xeon CPU X5670, 2.93GHz. The encoder and decoder binaries were compiled with gcc 4.1.2 64-bit. Both the proposed encoder/decoder and anchor encoder/decoder were tested in the same cluster.

Table 1 Coding performance, MDCS on

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.2% | -0.1% |  |  |  |
| Class B | -0.4% | -0.3% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.4% | -0.3% | -0.3% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.0% | 13.4% | 13.1% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.5% | -0.4% | -0.4% | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 101.0% |  |  | 100.9% |  |  |
| Dec Time[%] | 100.6% |  |  | 100.5% |  |  |

Table 2 Coding performance, MDCS off

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | 0.0% |  |  |  |
| Class B | -0.3% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs Ref) | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 12.1% | 13.5% | 13.3% | 10.2% | 10.3% | 9.7% |
| EL only (Test vs Ref) | -0.4% | -0.2% | -0.2% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101.6% |  |  | 101.2% |  |  |
| Dec Time[%] | 102.7% |  |  | 103.2% |  |  |

# References

[1] M. Guo, S. Liu, S. Lei (MediaTek), J. Park, J. Kim, B. Jeon, Non-TE5.1: Inter-layer Intra mode prediction, JCTVC-L0260, 2013

[2] A. Tabatabai, K. Rapaka, A. Saxena and S. Liu, Description of Core Experiment SCE 1: Intra Prediction Improvements in SHVC, JCTVC-L1101, 2013

# Patent rights declaration(s)

Leibniz Universitaet Hannover does not have any current or pending patent rights relating to the technology described in this contribution.