| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCT3V-M0065 |
| --- | --- |

| Title: | On collocated picture and low-delay checking for SHVC ref_idx framework |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Xiaozhen Zheng, Xu Chen, Jianhua Zheng No. 156, Rd. Bei-qing Hai-dian district 100095 Beijing, China | Tel: Email: | +86 10 60612372 yblin@huawei.com zhengxiaozhen@huaei.com anderson.chen@huawei.com zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies |  |  |

_____________________________

# Abstract

This contribution proposes two following parts for SHVC ref_idx framework:

1) add constraint on collocated picture signalling for storage reduction prediction, such that the signalled collocated picture shall be an inter-layer reference picture rather than a temporal reference picture for non-base layer coding. A flag in SPS extension is used to indicate the bitstream constraint. As a result, decoder only needs to store motion information in the inter-layer reference picture, not to store that of temporal reference pictures.

2) modification to low-delay checking process for coding efficiency improvement. Low-delay flag is set to true if collocated picture is an inter-layer reference picture. The modification is considered as slice-level change since the low-delay checking is performed at slice-level in SHM1.1 reference software. Test results reportedly show the proposed modification achieves overall BD-rate saving of -0.4%, -0.4% and -0.5% over SHM1.1 for spatial 2x, 1.5x and SNR cases in RA configuration under common test conditions.

# Introduction

In SHVC ref_idx framework, base-layer reference picture is used as an additional reference picture for non-base layer coding [1]. It also means the base-layer reference picture can be enabled as collocated picture in TMVP derivation for non-base layer coding, as done in SHM1.1 due to adoption of JCTVC-L0336 on motion mapping [2].

This contribution firstly proposes to add constraint on collocated picture for reducing the storage of motion information of collocated picture. In addition, modification to low-delay checking process is also proposed for coding efficiency improvement.

# Proposed methods

- Add constraint on collocated picture for storage reduction

It is proposed to add a flag in SPS extension to indicate constraint on collocated picture, such that the collocated picture shall be inter-layer reference picture rather than temporal reference pictures for non-base layer coding.

As a result, only the motion information of the inter-layer reference picture needs to be stored whereas that of temporal reference pictures within current layer are no longer stored. At decoder side, storage requirement for motion information of reference pictures can be reduced to 1/N (N denotes number of reference pictures).

- Set low-delay flag to true if collocated picture is inter-layer reference picture for non-base layer coding.

Low-delay checking is required for TMVP derivation in HEVC. Depending on the result of low-delay checking a collocated MV of collocated block is determined to derive target MV of current block [3]. In HM reference software, a low-delay flag is used to indicate whether all the reference pictures’ POCs are less than or equal to the current picture’s POC. The low-delay flag is determined at slice level and then used in TMVP derivation.

In SHVC ref_idx framework, when inter-layer reference picture is used as collocated picture, the collocated MV in the same reference picture list as that of the target MV is desirable for better TMVP prediction due to more correlation between the collocated MV and the target MV [4]. It is proposed to set the low-delay flag to true in this case.

The modification is considered as slice-level change since the low-delay checking process is performed at slice-level in practical encoder/decoder implementation, e.g. HM implementation.

# Simulation results

SHM1.1 with ref_idx framework configuration is used as anchor [5]. Simulations are carried out under common test conditions [6].

For the proposed method 1, there is no BD-rate change compared to the anchor since the anchor has already used the base-layer reference picture as collocated picture.

For the proposed method 2, the low-delay flag modification only affects RA configuration and no performance change on the other configurations (ALL intra, LP, LB). The test results under RA configuration are shown in Tab. 1. Overall BD-rate savings of -0.4%, -0.4% and -0.5% can be observed for spatial 2x, 1.5x and SNR scalability, respectively.

Table 1: Performance of the proposed method 2 against SHVC ref_idx framework

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.6% | -0.5% |  |  |  | -0.5% | -0.8% | -0.8% |
| Class B | -0.4% | -0.8% | -0.8% | -0.4% | -0.7% | -0.7% | -0.5% | -0.9% | -1.0% |
| Overall (Test vs Ref) | -0.4% | -0.7% | -0.7% | -0.4% | -0.7% | -0.7% | -0.5% | -0.9% | -1.0% |
| Overall (Test vs single layer) | 19.6% | 32.2% | 33.2% | 16.9% | 27.2% | 29.7% | 15.3% | 28.8% | 33.4% |
| EL only (Test vs Ref) | -0.7% | -0.9% | -0.9% | -0.6% | -0.8% | -0.9% | -0.8% | -1.2% | -1.3% |
| Enc Time[%] | 99.5% |  |  | 99.7% |  |  | 99.5% |  |  |
| Dec Time[%] | 100.8% |  |  | 100.7% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

Constraint on collocated picture is proposed for storage reduction, such that the collocated picture shall be inter-layer reference picture. In addition, low-delay checking is modified. The modification to the low-delay checking is carried out at slice level and considered as slice-level change. The proposed modification achieves overall BD-rate saving of -0.4% and 0.5% for spatial and SNR scalability, respectively.

# References

[1] J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, "SHVC Working Draft 1", JCTVC-L1008, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

[2] J. Chen, V. Seregin, L. Guo, M. Karczewicz, “Non-TE5: on motion mapping in SHVC”, JCTVC-L0336, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

[3] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” JCTVC-L1003, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

[4] Y. Lin, X. Zheng, J. Zheng, “Hook on temporal motion vector prediction for MV-HEVC”, JCTVC-L0177, 3rd Meeting: Geneva, CH, 17–23 Jan. 2013

[5] SHVC SHM version 1.1: https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/SHM-1.1/

[6] X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations," JCTVC-L1009, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

# Patent rights declaration(s)

HiSilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix:

Text changes for 1st method (constraint on collocated picture)

- G.7.3.2.2.1  Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| collocated_picture_constraint_flag | u(1) |
| } |  |

- G.7.4.3.2.1  Sequence parameter set extension semantics

inter_view_mv_vert_constraint_flag equal to 1 specifies that vertical component of motion vectors used for inter-layer prediction are constrained in the CVS. When inter_view_mv_vert_constraint_flag is equal to 1, the vertical component of the motion vectors used for inter-layer prediction shall be equal to or less than 56 in units of luma samples. When inter_view_mv_vert_constraint_flag is equal to 0, no constraint for of the vertical component of the motion vectors used for inter-layer prediction is signalled by this flag. When not present, the inter_view_mv_vert_constraint_flag is inferred to be equal to 0.

collocated_picture_constraint_flag equal to 1 specifies that collocated picture used for inter-layer prediction are constrained in the CVS. When collocated_picture_constraint_flag is equal to 1, the collocated picture used for inter-layer prediction shall be the refernce picture with nuh_layer_id not equal to that of current picture. When collocated_picture_constraint_flag is equal to 0, no constraint for the collocated picture used for inter-layer prediction is signalled by this flag. When not present, the collocated_picture_constraint_flag is inferred to be equal to 0.

Text changes for 2nd method (low-delay checking modification)

##### 8.5.3.2.8  Derivation process for collocated motion vectors

Inputs to this process are:

- a variable currPb specifying the current prediction block,

- a variable colPic specifying the collocated picture,

- a variable colPb specifying the collocated prediction block inside the collocated picture specified by colPic,

- a luma location ( xColPb, yColPb ) specifying the top-left sample of the collocated luma prediction block specified by colPb relative to the top-left luma sample of the collocated picture specified by colPic,

- a reference index refIdxLX, with X being 0 or 1.

Outputs of this process are:

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

The variable currPic specifies the current picture.

The arrays predFlagLXCol[ x ][ y ], mvLXCol[ x ][ y ], and refIdxLXCol[ x ][ y ] are set equal to the corresponding arrays of the collocated picture specified by colPic, PredFlagLX[ x ][ y ], MvLX[ x ][ y ], and RefIdxLX[ x ][ y ], respectively, with X being the value of X this process is invoked for.

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If colPb is coded in an intra prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows:

  - If predFlagL0Col[ xColPb ][ yColPb ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL1Col[ xColPb ][ yColPb ], refIdxL1Col[ xColPb ][ yColPb ], and L1, respectively.

  - Otherwise, if predFlagL0Col[ xColPb ][ yColPb ] is equal to 1 and predFlagL1Col[ xColPb ][ yColPb ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL0Col[ xColPb ][ yColPb ], refIdxL0Col[ xColPb ][ yColPb ], and L0, respectively.

  - Otherwise (predFlagL0Col[ xColPb ][ yColPb ] is equal to 1 and predFlagL1Col[ xColPb ][ yColPb ] is equal to 1), the following assignments are made:

      - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice, or DiffPicOrderCnt( colPic, currPic ) is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColPb ][ yColPb ], refIdxLXCol[ xColPb ][ yColPb ] and LX, respectively.

      - Otherwise, mvCol, refIdxCol, and listCol are set equal to mvLNCol[ xColPb ][ yColPb ], refIdxLNCol[ xColPb ][ yColPb ], and LN, respectively, with N being the value of collocated_from_l0_flag.

and mvLXCol and availableFlagLXCol are derived as follows: