| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0067 |
| --- | --- |

| Title: | RCE2: Experimental Results for Test 5 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

The document reports the experimental results for RCE2 Test 5. The proposed method does not modify the current intra prediction scheme and is implemented in a manner that is independent of intra prediction modes and color components as described in JCTVC-K0157. Compared to the HM10.0 lossless coding mode, the tested scheme (Test 5) provides average gains of 4.0% in AI-Main, 2.1% in RA-Main and 2.2% in LB-Main.

# Experimental results

The test sequences used are all four Class F sequences, Class B sequence 1080p Kimono1 and ParkScene, range extension 1080p 4:2:2 sequence EBUHorse and EBUWaterRocksClose (10-bit, 500 frames), range extension 1080p 4:4:4 sequence EBURainFruits and BirdsInCage (10-bit, 600 frames), and ten screen content 4:4:4 RGB sequences.

HM10.0 range extension software is used and the simulation results are presented below for AI-Main, RA-Main and LB-Main configurations:

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.2 | -6.9% |
| Class B | 2.1 | 2.3 | -6.2% |
| SC(GBR) | 7.5 | 7.8 | -3.5% |
| RangeExt | 2.2 | 2.2 | -0.9% |
| Overall (w/o SC) | 3.2 | 3.4 | -4.4% |
| Overall (w/ SC) | 5.3 | 5.6 | -4.0% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 117% |  |  |

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 30.5 | -4.2% |
| Class B | 2.6 | 2.6 | -1.0% |
| SC(GBR) | 88.8 | 91.0 | -2.2% |
| RangeExt | 2.5 | 2.2 | -0.4% |
| Overall (w/o SC) | 13.3 | 13.6 | -2.0% |
| Overall (w/ SC) | 51.0 | 52.3 | -2.1% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 107% |  |  |

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 48.3 | -3.5% |
| Class B | 2.6 | 2.6 | -0.7% |
| SC(GBR) | 301.9 | 310.1 | -2.7% |
| RangeExt | 2.5 | 2.5 | -0.3% |
| Overall (w/o SC) | 20.5 | 20.8 | -1.7% |
| Overall (w/ SC) | 161.2 | 165.5 | -2.2% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 108% |  |  |

# Conclusion

In this contribution, a lossless coding scheme that delays the sample-based prediction till the residue coding stage of the codec and carries out prediction in the residual domain is introduced. In this way, the existing intra prediction scheme in HEVC can be retained and remain unchanged while realizing some of the coding gains associated with sample-based prediction for lossless coding.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text modifications

### 8.6.2 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nT specifying the size of the current transform block.

Output of this process is:

–	the (nT)x(nT) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = QP’C	b		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = QP’C	r		(8-3)

The (nT)x(nT) array of residual samples r is derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

- If cu_transquant_bypass_flag is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT – 1, y = 0..nT – 1 are derived as follows:

  - The variables A, B and C are derived by

A = ( x = = 0 ) ? 0 : r[ x-1 ][ y ]

B = ( y = = 0 ) ? 0 : r[ x ][ y-1 ]

C = ( ( x = = 0 ) | | ( y = = 0 ) ) ? 0 : r[ x-1 ][ y-1 ]

  - The residual sample r[ x ][ y ] is derived by

r[ x ][ y ] = Median( A , B , A + B – C ) + TransCoeffLevel[ x ][ y ][ cIdx ]

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the (nT)x(nT) array of transform coefficients, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a (nT)x(nT) array of scaled transform coefficients d.

- The (nT)x(nT) array of residual samples r is derived as follows.

- If transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are derived as:

r[ x ][ y ] = ( d[ x ][ y ] << 7 )		(8-4)

- Otherwise (transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, and the (nT)x(nT) array of scaled transform coefficients d as the inputs and the output is a (nT)x(nT) array of residual samples r.

- The variable shift is derived as:

shift = (cIdx = = 0) ? 20 – BitDepthY : 20 – BitDepthC	(8-5)

- The residual sample values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are modified as follows.

r[ x ][ y ] = ( r[ x ][ y ] + (1 << ( shift − 1) ) ) >> shift	(8-6)