| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0068 |
| --- | --- |

| Title: | Non-RCE2: TU-level flag for DPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution studies the effectiveness of a TU-level flag when applied to the lossless residue coding method described in JCTVC-K0157. The flag signals, at the TU level, whether the residues in the TU are coded with sample-wise prediction. It is asserted that, with the residue prediction scheme used for both inter and intra coding, the proposed method brings average gains of 9.1%, 6.7% and 7.1% for All-Intra, Random-Access and Low-Delay test configurations.

# Introduction

Lossless coding is achieved in the current HEVC design by bypassing the transform and quantization stage. It has been shown that sample-based prediction can improve coding performance [1][2].

In the proposed scheme in JCTVC-K0157, sample-based prediction is performed on the prediction residues without any modification to the existing intra prediction process. Also, the sample-based prediction process does not depend on the intra prediction mode and color component, i.e., the same prediction process is applied to all intra residual samples.

This contribution studies the effectiveness of a TU-level flag when applied to the lossless residue coding method described in JCTVC-K0157. The flag signals, at the TU level, whether the residues in the TU are coded with sample-wise prediction. With the TU-level flag, the sample-based prediction process can also be applied to inter prediction residues to improve prediction performance.

# Experimental results

The test sequences used are all four Class F sequences, Class B sequence 1080p Kimono1 and ParkScene, range extension 1080p 4:2:2 sequence EBUHorse and EBUWaterRocksClose (10-bit, 500 frames), range extension 1080p 4:4:4 sequence EBURainFruits and BirdsInCage (10-bit, 600 frames), and ten screen content 4:4:4 RGB sequences.

During encoding, the sample-wise prediction is used for a TU if using it results in a smaller sum of absolute values for the residues in the TU.

HM10.0 range extension software is used and the simulation results are presented below for AI-Main, RA-Main and LB-Main configurations:

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.3 | -9.8% |
| Class B | 2.1 | 2.3 | -7.2% |
| SC(GBR) | 7.5 | 8.5 | -11.2% |
| RangeExt | 2.2 | 2.3 | -4.3% |
| Overall (w/o SC) | 3.2 | 3.5 | -7.1% |
| Overall (w/ SC) | 5.3 | 6.0 | -9.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 88% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 32.7 | -8.2% |
| Class B | 2.6 | 2.6 | -2.0% |
| SC(GBR) | 88.8 | 99.8 | -9.2% |
| RangeExt | 2.5 | 2.3 | -1.1% |
| Overall (w/o SC) | 13.3 | 14.5 | -4.1% |
| Overall (w/ SC) | 51.0 | 57.1 | -6.7% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 53.0 | -9.0% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 301.9 | 350.7 | -9.9% |
| RangeExt | 2.5 | 2.6 | -1.0% |
| Overall (w/o SC) | 20.5 | 22.7 | -4.3% |
| Overall (w/ SC) | 161.2 | 186.7 | -7.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

# Conclusion

In this contribution, a lossless coding scheme that delays the sample-based prediction till the residue coding stage of the codec and carries out prediction in the residual domain is introduced. In this way, the existing intra prediction scheme in HEVC can be retained and remain unchanged while realizing the coding gains associated with sample-based prediction for lossless coding. With a TU-level flag, the prediction scheme is also useful for lossless coding of inter prediction residues.

# References

- Y.-L. Lee, K.-H. Han, and G. J. Sullivan, “Improved lossless intra coding for H.264/MPEG-4 AVC,” IEEE Trans. Image Processing, pp. 2610-2615, Sep 2006.

- M. Zhou, “AHG22: Sample-based angular prediction (SAP) for HEVC lossless coding,” JCTVC-G093, Geneva, Nov 2011.

- S. A. Martucci, “Reversible compression of HDTV images using median adaptive prediction and arithmetic coding,” in Proc IEEE Int. Symp. Circuits Syst., pp. 1310-1313, 1990.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).