| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0071 |
| --- | --- |

| Title: | Inferred GRP (IGRP) with reduced motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianghui Wei Jinwen Zan | Tel: Email: | +86-10-82882199 weixianghui@huawei.com jinwen.zan@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents an inferred GRP (IGRP) algorithm based on rqt_root_cbf flag to reduce complexity of generalized residual prediction (GRP). Test results report that average 13.0% of encoding time saving and 0.7% of decoding time saving are achieved. The average BD-rate losses are 0.2% (Y), 0.2% (U) and 0.2% (V). An additional test reports that extra motion compensation introduced by GRP is reduced by 11.7% on average.

# Introduction

GRP reported in JCTVC-L1103 [1] has improved the coding efficiency of SHVC, by combining previously coded base layer and enhancement layer to improve the prediction of the current enhancement layer CU. In JCTVC-L0078 [2] and JCTVC-L0190 [3], four GRP modes (res-0, res-0.5, res-1 and diff-0.5) are provided, which are signalled by grp_weight_idx (0, 1, 2 and 3).

## IGRP based on rqt_root_cbf

On decoder side, IGRP utilizes rqt_root_cbf flag from above and left neighbouring CU to infer grp_weight_idx for inter-predicted CU, as follows

if (AboveCURootCbf == 0 && LeftCURootCbf == 0)

grp_weight_idx = 0;

else

parse grp_weight_idx from bitstream;

where AboveCURootCbf and LeftCURootCbf represent the rqt_root_cbf of upper and left neighbouring CU, respectively.

On encoder side, GRP is disabled for inter-predicted CU when AboveCURootCbf and LeftCURootCbf  are both zeroes, as follows

if (AboveCURootCbf == 0 && LeftCURootCbf == 0)

{

GRP is not applied for current CU;

grp_weight_idx is not written into bitstream;

}

else

{

GRP is applied for current CU;

grp_weight_idx is written into bitstream;

}

# Test Results

IGRP is implemented onto the SCE3.3 software and tested with three configurations: RA, LD-P and LD-B. All tests are based on SHVC common test conditions defined by AHG10. Anchor data is generated by SCE3.3 software. The reported average data is the mean value over three RA cases, three LD-P cases and three LD-B cases.

### Test 1

In this test, anchor is generated by SCE3.3 software with FastGRP=0, meaning all fast GRP options are disabled.

It is reported that on average 13.0% of encoding time and 0.7% of decoding time savings are achieved. The average BD-rate losses are 0.2% (Y), 0.2% (U) and 0.2% (V).

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.3% | 0.4% |  |  |  | 0.3% | -0.2% | 0.0% |
| Class B | 0.2% | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| Overall (Test vs Ref) | 0.2% | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.2% | 0.0% | 0.1% |
| Overall (Test vs single layer) | 17.5% | 26.7% | 26.7% | 15.1% | 23.4% | 24.8% | 12.6% | 22.3% | 23.8% |
| EL only (Test vs Ref) | 0.0% | 0.1% | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| Enc Time[%] | 83.7% |  |  | 85.1% |  |  | 89.3% |  |  |
| Dec Time[%] | 98.7% |  |  | 99.0% |  |  | 98.8% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.5% | 0.4% |  |  |  | 0.4% | -0.4% | -0.3% |
| Class B | 0.3% | 0.2% | 0.0% | 0.4% | 0.4% | 0.3% | 0.4% | -0.1% | 0.1% |
| Overall (Test vs Ref) | 0.3% | 0.3% | 0.1% | 0.4% | 0.4% | 0.3% | 0.4% | -0.2% | 0.0% |
| Overall (Test vs single layer) | 20.6% | 28.2% | 28.6% | 17.9% | 24.2% | 25.6% | 15.0% | 22.9% | 24.4% |
| EL only (Test vs Ref) | 0.1% | 0.1% | 0.0% | -0.2% | -0.2% | -0.3% | 0.2% | -0.3% | -0.1% |
| Enc Time[%] | 82.8% |  |  | 83.5% |  |  | 88.7% |  |  |
| Dec Time[%] | 99.5% |  |  | 99.5% |  |  | 99.1% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.3% | 0.5% |  |  |  | 0.2% | -0.4% | 0.0% |
| Class B | 0.1% | 0.2% | 0.0% | 0.1% | 0.4% | 0.3% | 0.1% | 0.0% | 0.1% |
| Overall (Test vs Ref) | 0.1% | 0.3% | 0.1% | 0.1% | 0.4% | 0.3% | 0.1% | -0.1% | 0.1% |
| Overall (Test vs single layer) | 24.9% | 30.8% | 30.9% | 21.9% | 25.9% | 27.0% | 19.9% | 25.8% | 27.5% |
| EL only (Test vs Ref) | 0.0% | 0.1% | 0.0% | -0.3% | 0.0% | -0.1% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 88.1% |  |  | 88.9% |  |  | 92.6% |  |  |
| Dec Time[%] | 99.7% |  |  | 99.7% |  |  | 99.7% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

### Test 2

In this test, anchor is generated by SCE3.3 software with FastGRP=7, meaning all fast GRP options provided are enabled.

It is reported that average 2.5% of encoding time saving and average 0.2% of decoding time saving are achieved. The average BD-rate gains are 0.0% (Y), 0.3% (U) and 0.3% (V).

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% |  |  |  | 0.1% | -0.8% | -0.6% |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% | 0.0% | -0.3% | -0.3% |
| Overall (Test vs Ref) | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% | 0.0% | -0.5% | -0.4% |
| Overall (Test vs single layer) | 17.5% | 26.7% | 26.7% | 15.1% | 23.4% | 24.8% | 12.6% | 22.3% | 23.8% |
| EL only (Test vs Ref) | -0.2% | -0.5% | -0.5% | -0.3% | -0.4% | -0.4% | -0.1% | -0.6% | -0.6% |
| Enc Time[%] | 95.7% |  |  | 95.6% |  |  | 98.3% |  |  |
| Dec Time[%] | 99.8% |  |  | 99.7% |  |  | 99.8% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% |  |  |  | 0.2% | -1.0% | -0.9% |
| Class B | 0.0% | -0.4% | -0.4% | 0.2% | -0.1% | -0.1% | 0.1% | -0.5% | -0.3% |
| Overall (Test vs Ref) | 0.0% | -0.3% | -0.3% | 0.2% | -0.1% | -0.1% | 0.2% | -0.6% | -0.5% |
| Overall (Test vs single layer) | 20.6% | 28.2% | 28.6% | 17.9% | 24.2% | 25.6% | 15.0% | 22.9% | 24.4% |
| EL only (Test vs Ref) | -0.2% | -0.6% | -0.6% | -0.5% | -0.8% | -0.7% | 0.0% | -0.8% | -0.7% |
| Enc Time[%] | 97.2% |  |  | 96.1% |  |  | 100.2% |  |  |
| Dec Time[%] | 99.7% |  |  | 99.6% |  |  | 99.7% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% |  |  |  | 0.0% | -0.7% | -0.5% |
| Class B | -0.1% | -0.2% | -0.3% | 0.0% | 0.1% | -0.1% | 0.0% | -0.3% | -0.3% |
| Overall (Test vs Ref) | -0.1% | -0.2% | -0.3% | 0.0% | 0.1% | -0.1% | 0.0% | -0.4% | -0.3% |
| Overall (Test vs single layer) | 24.9% | 30.8% | 30.9% | 21.9% | 25.9% | 27.0% | 19.9% | 25.8% | 27.5% |
| EL only (Test vs Ref) | -0.3% | -0.4% | -0.5% | -0.5% | -0.4% | -0.5% | -0.2% | -0.6% | -0.5% |
| Enc Time[%] | 97.5% |  |  | 97.1% |  |  | 99.6% |  |  |
| Dec Time[%] | 99.8% |  |  | 99.8% |  |  | 99.8% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

### Test 3

An additional test reports the calling times of xPredInterUniGRP function in decoder side. xPredInterUniGRP function performs inter prediction for non-zero grp_weight_idx. The calling times reduction of xPredInterUniGRP represents the reduction of extra motion compensation introduced by GRP.

Following table presents results of BasketballDrive with LD-P and 1.5X configuration. The complete results are presented in the attached JCTVC-M0071_test3.xls. The CU4x4 counts are all zeroes because of the GRP constraints on PU no smaller than 8x8.

It is reported that average calling times of xPredInterUniGRP function is reduced by 11.7%.

|  | SCE 3.3 (FastGRP=0) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Total | CU64x64 | CU32x32 | CU16x16 | CU8x8 | CU4x4 |
| LDP_1.5X_BasketballDrive_l0qp22_l1qp22 | 1368457 | 23213 | 200487 | 542564 | 602193 | 0 |
| LDP_1.5X_BasketballDrive_l0qp22_l1qp24 | 669541 | 27865 | 142570 | 281202 | 217904 | 0 |
| LDP_1.5X_BasketballDrive_l0qp26_l1qp26 | 692059 | 35614 | 166184 | 291460 | 198801 | 0 |
| LDP_1.5X_BasketballDrive_l0qp26_l1qp28 | 362762 | 39781 | 110470 | 137130 | 75381 | 0 |
| LDP_1.5X_BasketballDrive_l0qp30_l1qp30 | 441972 | 44405 | 137007 | 170711 | 89849 | 0 |
| LDP_1.5X_BasketballDrive_l0qp30_l1qp32 | 204190 | 41382 | 72769 | 62893 | 27146 | 0 |
| LDP_1.5X_BasketballDrive_l0qp34_l1qp34 | 278521 | 42958 | 102186 | 94189 | 39188 | 0 |
| LDP_1.5X_BasketballDrive_l0qp34_l1qp36 | 152140 | 41569 | 57861 | 39170 | 13540 | 0 |
|  | IGRP |  |  |  |  |  |
|  | Total | CU64x64 | CU32x32 | CU16x16 | CU8x8 | CU4x4 |
| LDP_1.5X_BasketballDrive_l0qp22_l1qp22 | 1267195 | 20667 | 178580 | 495080 | 572868 | 0 |
| LDP_1.5X_BasketballDrive_l0qp22_l1qp24 | 577209 | 23105 | 115166 | 236148 | 202790 | 0 |
| LDP_1.5X_BasketballDrive_l0qp26_l1qp26 | 611356 | 29877 | 141722 | 256503 | 183254 | 0 |
| LDP_1.5X_BasketballDrive_l0qp26_l1qp28 | 290350 | 30457 | 84664 | 107985 | 67244 | 0 |
| LDP_1.5X_BasketballDrive_l0qp30_l1qp30 | 369540 | 35031 | 111133 | 144021 | 79355 | 0 |
| LDP_1.5X_BasketballDrive_l0qp30_l1qp32 | 154023 | 29119 | 53889 | 47447 | 23568 | 0 |
| LDP_1.5X_BasketballDrive_l0qp34_l1qp34 | 221543 | 31720 | 79845 | 76956 | 33022 | 0 |
| LDP_1.5X_BasketballDrive_l0qp34_l1qp36 | 106843 | 28743 | 40666 | 27579 | 9855 | 0 |
|  | Reduction Percentage |  |  |  |  |  |
|  | Total | CU64x64 | CU32x32 | CU16x16 | CU8x8 | CU4x4 |
| LDP_1.5X_BasketballDrive_l0qp22_l1qp22 | -7.40% | -10.97% | -10.93% | -8.75% | -4.87% | #DIV/0! |
| LDP_1.5X_BasketballDrive_l0qp22_l1qp24 | -13.79% | -17.08% | -19.22% | -16.02% | -6.94% | #DIV/0! |
| LDP_1.5X_BasketballDrive_l0qp26_l1qp26 | -11.66% | -16.11% | -14.72% | -11.99% | -7.82% | #DIV/0! |
| LDP_1.5X_BasketballDrive_l0qp26_l1qp28 | -19.96% | -23.44% | -23.36% | -21.25% | -10.79% | #DIV/0! |
| LDP_1.5X_BasketballDrive_l0qp30_l1qp30 | -16.39% | -21.11% | -18.89% | -15.63% | -11.68% | #DIV/0! |
| LDP_1.5X_BasketballDrive_l0qp30_l1qp32 | -24.57% | -29.63% | -25.95% | -24.56% | -13.18% | #DIV/0! |
| LDP_1.5X_BasketballDrive_l0qp34_l1qp34 | -20.46% | -26.16% | -21.86% | -18.30% | -15.73% | #DIV/0! |
| LDP_1.5X_BasketballDrive_l0qp34_l1qp36 | -29.77% | -30.85% | -29.72% | -29.59% | -27.22% | #DIV/0! |

### Complexity assessment

Complexity assessment for the proposed method is done according to [4]. The summary results are as follows

# Proposed syntax and semantics

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0+ j ] ) |  |
| mpm_idx[ x0 + i ][ y0+ j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0+ j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS *3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS *3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS /4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS *3 / 4, nCbS) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS *3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| if( LayerID > 0 && !(LeftCURootCbf==0 && AboveCURootCbf==0) ) |  |
| grp_weight_idx | ae(v) |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

grp_weight_idx signals GRP weighting mode for a coding unit. The value of grp_weight_idx is in the range of 0 to 2, inclusive. When not present, grp_weight_idx is inferred to be equal to 0.

# Conclusions

This contribution presents an inferred GRP (IGRP) algorithm by utilizing rqt_root_cbf flag from neighbouring CU. It is reported that average 13.0% of encoding time saving and 0.7% of decoding time saving are achieved. The average BD-rate losses are 0.2% (Y), 0.2% (U) and 0.2% (V). An additional test reports that extra motion compensation introduced by GRP is reduced by 11.7% on average.

# References

- X. Li, E. François, P. Lai, D. Kwon, A. Saxena, “Description of Core Experiment SCE 3: Combined Inter and Inter-Layer Prediction in SHVC”, JCTVC-L1103, 12th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, 14-23 Jan., 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczenwicz, “TE3: Results of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0078, 12th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, 14-23 Jan., 2013.

- X. Li, J. Chen, K. Rapaka, M. Karczenwicz, “Non-TE3: Extension of Test 4.6.2.1 on Generalized Residual Prediction”, JCTVC-L0190, 12th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, 14-23 Jan., 2013.

- E. François, A. Tabatabai, E. Alshina, “BoG reprot: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440, 12th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, 14-23 Jan., 2013.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).