| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0072 |
| --- | --- |

| Title: | AHG7: Modification of in-loop colour-space transformation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Masaaki Matsumura Seishi Takamura Hiroshi Fujii Atsushi Shimizu Y-517A, 1-1 Hikari-no-oka, Yokosuka-shi, Kanagawa, Japan, 239-0847 | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This document introduces a modification of in-loop colour-space transformation of residual signals. To reduce the redundancy among the Y-Cb-Cr/R-G-B components, a colour-space transformation method was proposed in the contribution of JCTVC-L0371. In this contribution, to prevent the deterioration of the coding performance of L0371 and to speed up the run-times, the transformation is adaptively applied to residual signals using an eigenvalue derived by the calculation process of transformation matrices. The overall average coding gains against the anchor (HM10.0 RExt2.0) were 9.8% / 8.1% / 11.5% (AI High-tier), 11.2% / 7.4% / 12.7% (RA High-tier), and 11.1% / 5.8% / 12.7% (LD High-tier). The average run-times of encoding and decoding were 103.2% and 106.2%, respectively.

# Introduction

An in-loop colour-space transformation method was proposed in the contribution JCTVC-L0371 [1] at the previous Geneva meeting. In the method, the transformation matrices are calculated from locally-decoded reference samples with the simplified singular-value-decomposition. The colour-space transformation is always applied to the residual signals except the case that the prediction error energy is equal to zero. However, the colour-space transformation is not always effective for the improvement of coding efficiency. For example, from the experimental results of the sequence “DucksAndLegs”, the 22.6% coding loss was observed in B-component of the Random access / High-tier / RGB4:4:4 [1]. In this contribution, an adaptive colour-space transformation is proposed to prevent the deterioration.

# Modification of in-loop colour-space transformation

## Overview

This proposal employs the same matrix derivation algorithm as L0371. The derived transformation matrices are used for the colour-space transformation and the inverse transformation. The block diagram is depicted in Figure 1. The difference is that a transformation decision unit is newly introduced. The unit decides whether the colour-space transformation is applied to residual signals or not.

Figure 1: Block diagram of the modified in-loop colour-space transformation.

## Decision algorithm of colour-space transformation

When the transformation matrices are derived, an eigenvalue which represents a contribution ratio to each component is also derived. Three eigenvalues are obtained for each matrix derivation process. The eigenvalue of the prime component, which is the largest value of the three, is picked up and compared to the threshold value. If the eigenvalue is greater than or equal to the threshold, the colour-space transformation is applied. Otherwise, it is not applied. The eigenvalue is derived in the method of L0371. Therefore, the additional process is minimal; the comparison of the largest eigenvalue and the threshold.

## Syntax related to the proposal

As described in JCTVC-L0371 [1], coding structure is required to be identical among all components because of the colour-space transformation. There is no modification for the syntax of range extensions. In this contribution, the transformation matrices are also derived from the locally-decoded samples and not coded. Furthermore, in the current implementation, the threshold value is shared between the encoder and the decoder. Therefore, no modification is required in terms of the syntax.

# Experimental results

The proposal was implemented to HM10.0 Range Extensions 2.0. The test conditions are subject to the common test conditions for HEVC range extensions [2]. In this experiment, the threshold for the eigenvalue was 3891 which is magnified 4096 times (3891 is equivalent to about 0.95). The results of the average BD-rate and the runtimes are shown in Tables 1 to 3. The detailed results of comparison between the proposal and L0371 (RCE1’s results) in the RGB4:4:4 are shown in Tables 4 to 6. It was confirmed that the coding losses were alleviated. In addition, compared to L0371, the proposal decreased the encoding and decoding run-times since some of the colour-space transformations are skipped. The detailed comparison results of YUV4:4:4/4:2:2 are described in the appendix.

Figs. 2 to 4 visualize the relationships between the BD-rate and the decoding times in the case of All intra / Super-high-tier / RGB4:4:4.

Table 1: Results of All Intra HE conditions (Main-tier / High-tier / Super-high-tier).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |  |
| RGB 4:4:4 | -21.8% | -15.1% | -20.9% | -16.9% | -11.5% | -16.7% | -12.3% | -8.4% | -12.4% |  |
| YCbCr 4:4:4 | -1.6% | -7.1% | -6.8% | -1.6% | -4.1% | -5.6% | -1.2% | -2.3% | -4.1% |  |
| Overall | -12.4% | -11.4% | -14.3% | -9.8% | -8.1% | -11.5% | -7.1% | -5.6% | -8.5% |  |
|  | -12.1% | -11.8% | -14.5% | -9.9% | -8.2% | -11.6% | -7.4% | -5.7% | -8.7% |  |
| Enc Time[%] | 103% |  |  | 103% |  |  | 102% |  |  |  |
| Dec Time[%] | 107% |  |  | 105% |  |  | 103% |  |  |  |

Table 2: Results of Random Access HE conditions (Main-tier / High-tier).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -24.4% | -13.4% | -20.7% | -19.8% | -7.9% | -17.1% |
| YCbCr 4:4:4 | -1.2% | -10.0% | -8.1% | -1.3% | -6.9% | -7.6% |
| Overall | -13.6% | -11.8% | -14.8% | -11.2% | -7.4% | -12.7% |
|  | -13.2% | -13.6% | -13.8% | -13.0% | -9.6% | -13.9% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |

Table 3: Results of Low Delay HE conditions (Main-tier / High-tier).

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -25.2% | -10.3% | -19.7% | -19.4% | -5.3% | -16.5% |
| YCbCr 4:4:4 | -1.5% | -10.1% | -9.1% | -1.7% | -6.3% | -8.3% |
| Overall | -14.2% | -10.2% | -14.8% | -11.1% | -5.8% | -12.7% |
|  | -13.3% | -7.3% | -12.4% | -12.9% | -6.2% | -12.9% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 108% |  |  | 108% |  |  |

Table 4: Comparison of the proposal and L0371 in All Intra case of RGB4:4:4.

Table 5: Comparison of the proposal and L0371 in Random Access case of RGB4:4:4.

Table 6: Comparison of the proposal and L0371 in Low Delay case of RGB4:4:4.

Figure 2: Relationship of the BD-rate and the decoding times for Y component (All intra / Super-high-tier / RGB4:4:4).

Figure 3: Relationship of the BD-rate and the decoding times for U component (All intra / Super-high-tier / RGB4:4:4).

Figure 4: Relationship of the BD-rate and the decoding times for V component (All intra / Super-high-tier / RGB4:4:4).

# Conclusion

A modified in-loop colour-space transformation was proposed. The proposal decides whether the colour-space transformation is applied or not by using an eigenvalue derived from the calculation process of the transformation matrices. Experimental results showed that the proposal alleviated the coding losses and decreased the run-times.

It is recommended that the proposal is investigated in RCE and adopted in HEVC range extensions.

# References

[1] K. Kawamura, T. Yoshino, and S.Naito, “AHG7: In-loop colour-space transformation of residual signals for range extensions,” JCTVC-L0371, 12th meeting, Geneva, CH, 14-23 January, 2013.

[2] D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-L1006, 12th JCT-VC meeting, Geneva, CH, 14-23 January, 2013.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

The overall coding gains including YUV4:2:2 are shown in Tables 7 to 9.

Table 7: Results of All Intra HE conditions (Main-tier / High-tier / Super-high-tier).

|  |  | All Intra HE Main-tier |  |  |  |  |  | All Intra HE High-tier |  |  |  |  | All Intra HE Super-High-tier |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |  | Y | U |  | V |  | Y |  | U |  | V |  |  |
| RGB 4:4:4 |  | -21.8% |  | -15.1% |  | -20.9% |  | -16.9% | -11.5% |  | -16.7% |  | -12.3% |  | -8.4% |  | -12.4% |  |  |
| YCbCr 4:4:4 |  | -1.6% |  | -7.1% |  | -6.8% |  | -1.6% | -4.1% |  | -5.6% |  | -1.2% |  | -2.3% |  | -4.1% |  |  |
| YCbCr 4:2:2 |  | 0.7% |  | -5.9% |  | -5.7% |  | 1.5% | -2.1% |  | -3.1% |  | 2.4% |  | -0.5% |  | -1.1% |  |  |
| Overall |  | -8.2% |  | -9.6% |  | -11.6% |  | -6.2% | -6.2% |  | -8.8% |  | -4.1% |  | -4.0% |  | -6.2% |  |  |
|  |  | -8.0% |  | -9.9% |  | -11.7% |  | -6.3% | -6.4% |  | -8.9% |  | -4.3% |  | -4.1% |  | -6.3% |  |  |
| Enc Time[%] |  | 103% |  |  |  |  |  | 103% |  |  |  |  | 102% |  |  |  |  |  |  |
| Dec Time[%] |  | 108% |  |  |  |  |  | 106% |  |  |  |  | 105% |  |  |  |  |  |  |

Table 8: Results of Random Access HE conditions (Main-tier / High-tier).

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -24.4% | -13.4% | -20.7% | -19.8% | -7.9% | -17.1% |
| YCbCr 4:4:4 | -1.2% | -10.0% | -8.1% | -1.3% | -6.9% | -7.6% |
| YCbCr 4:2:2 | 0.6% | -9.3% | -7.5% | 1.3% | -3.6% | -4.9% |
| Overall | -9.1% | -11.0% | -12.5% | -7.2% | -6.2% | -10.2% |
|  | -8.8% | -12.2% | -11.8% | -8.5% | -7.6% | -11.0% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |

Table 9: Results of Low Delay HE conditions (Main-tier / High-tier).

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -25.2% | -10.3% | -19.7% | -19.4% | -5.3% | -16.5% |
| YCbCr 4:4:4 | -1.5% | -10.1% | -9.1% | -1.7% | -6.3% | -8.3% |
| YCbCr 4:2:2 | 0.4% | -9.5% | -8.6% | 1.0% | -3.2% | -5.6% |
| Overall | -9.5% | -10.0% | -12.8% | -7.3% | -5.0% | -10.4% |
|  | -9.0% | -8.0% | -11.2% | -8.5% | -5.0% | -10.6% |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 108% |  |  | 108% |  |  |

The comparison results between the proposal and L0371 (RCE1’s results) of the YUV4:4:4 are shown in Tables 10 to 12. The comparison results between the proposal and L0371 (RCE1’s results) of the YUV4:2:2 are shown in Tables 13 to 15.

Table 10: Comparison of the proposal and L0371 in All Intra case of YUV4:4:4.

Table 11: Comparison of the proposal and L0371 in Random Access case of YUV4:4:4.

Table 12: Comparison of the proposal and L0371 in Low Delay case of YUV4:4:4.

Table 13: Comparison of the proposal and L0371 in All Intra case of YUV4:2:2.

Table 14: Comparison of the proposal and L0371 in Random Access case of YUV4:2:2.

Table 15: Comparison of the proposal and L0371 in Low Delay case of YUV4:2:2.