| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0079 |
| --- | --- |

| Title: | RCE2: Test 1 – Residual DPCM for HEVC lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunil Lee Il-Koo Kim Chanyul Kim | Email: | sunil.lee@samsung.com ilkoo.kim@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the test results of RCE2 Test 1 on the residual DPCM (RDPCM) for HEVC lossless coding. It is reported that the overall coding gain of the RDPCM is 8.4% with screen content sequences (SC) and 5.7% without SC in AI-Main, 5.2% with SC and 2.7% without SC in RA-Main, and 4.6% with SC and 2.2% without SC in LB-Main configurations, respectively. It is also reported that the overall coding gain of RDPCM with non-normative encoder-only improvement is 9.0% with SC and 6.1% without SC in AI-Main, 5.6% with SC and 2.9% without SC in RA-Main, and 5.0% with SC and 2.3% without SC in LB-Main configurations, respectively. Other additional test results are also provided to analyze the various aspects of the RDPCM. In terms of the throughput, the RPCM can be fully parallelized at both encoder and decoder. The RDPCM is exactly the same as the intra residual transform-bypass decoding process of H.264/AVC standard.

# Introduction

At the last JCT-VC meeting in Geneva, it was agreed to set up a core experiment on intra prediction for lossless coding (RCE2) [1]. In Test 1 of RCE2, the residual DPCM (RDPCM) for HEVC lossless coding is evaluated in terms of both coding efficiency and complexity. In the RDPCM, the sample-based DPCM is applied to the residual samples when the lossless coding is enabled for the corresponding CU, the prediction mode is intra, and the intra prediction mode is either vertical or horizontal mode [2]. Even though RDPCM is a sample-based differential coding method, it does not suffer from the throughput problem, since each pixel value can be reconstructed instantaneously and independently from the parsed residual samples. Various test results are provided in this contribution to show the effectiveness of the RDPCM. The RDPCM is also exactly the same as the intra residual transform-bypass decoding process of H.264/AVC standard, which is specified in Section 8.5.15 of the specification document [3].

# Algorithm description

## Residual DPCM

The algorithm of the RDPCM is exactly the same as proposed in [2]. The RDPCM process is invoked when cu_transquant_bypass_flag is equal to 1 (lossless coding is enabled in the current CU), the prediction mode is intra, and the applicable intra prediction mode is equal to either vertical or horizontal mode. The process for the chroma components is applied in the same way as for the luma component.

Figure 1. The RDPCM when intra prediction mode is (a) vertical and (b) horizontal mode.

Let the MxN array R with elements ri,j be either an array including the residual samples of luma or chroma component obtained as a result of intra prediction. The RDPCM is applied to the residual samples to obtain the modified MxN array R with elements ri,j as follows when the intra prediction mode is vertical mode

ri,j=ri,j,i=0,0≤j≤(N−1)ri,j−r(i−1),j,1≤i≤(M−1),0≤j≤(N−1),

or when the intra prediction mode is horizontal mode

ri,j=ri,j,0≤i≤(M−1),j=0ri,j−ri,(j−1),0≤i≤(M−1),1≤j≤(N−1).

When the RDPCM is invoked, the modified residual samples of R are signalled to the decoder, instead of the original residual samples of R.

At the decoder side, the original residual samples can be reconstructed after the modified residual samples are parsed as follows when the intra prediction mode is vertical mode

ri,j=k=0irk,j   ,0≤i≤(M−1),0≤j≤(N−1),

or when the intra prediction mode is horizontal mode

ri,j=k=0jri,k   ,0≤i≤(M−1),0≤j≤(N−1).

Note that the current residual sample can be directly reconstructed from the sum of the modified residual samples without reconstructing the previous (left in horizontal mode and above in vertical mode) residual sample. Thus the throughput problem of the conventional sample-based differential coding method can be avoided in the RDPCM. As explained in Section 1, the RDPCM is exactly the same as the intra residual transform-bypass decoding process specified in Section 8.5.15 of H.264/AVC specification [3].

## Non-normative encoder-only improvement

In the current version of the HEVC range extension software, a fast encoding method is employed for the intra prediction, and only a subset of the intra prediction modes is fully evaluated in the rate-distortion (RD) optimization process at the encoder. Since only the distortion between the original samples and the predicted samples is considered to choose the intra prediction modes for the full RD optimization, the benefit of the RDPCM is not taken into account at all in the current implementation. To alleviate this problem, the fast encoding method in the software is modified so that the horizontal and vertical modes are added to the list of intra prediction modes for the full RD optimization, when they are not included in the list. The test results in Table 2 show that this non-normative encoder-only change provides the additional coding gain.

## Differences between RDPCM and SAP_HV

In Test 4 of RCE2, the sample-based angular prediction (SAP) in only vertical and horizontal directions (SAP_HV) is evaluated [1]. The SAP_HV is a variant of the SAP proposed in [4], where the sample-based prediction is applied only to horizontal and vertical directions, while the conventional block-based prediction is employed for the other intra prediction modes [5]. Even though the SAP_HV is mathematically identical to the RDPCM, the bit-streams generated by both methods were different due to the following reasons.

- The edge filtering specified in Section 8.4.4.2.6 of HEVC range extension draft text is disabled in the SAP [6]. In the RDPCM, it is still enabled, since all the intra prediction process remains unchanged in the RDPCM. When the edge filtering is manually disabled in the RDPCM, the bit-streams generated by the RDPCM encoder can be decoded by the SAP_HV decoder, vice versa. It is also tested how the edge filtering affects the coding gain of the RDPCM, and the test results are provided in Table 3 and 4. Please note that what we propose in this contribution is to enable the edge filtering, since it is a part of the existing intra prediction process and test results show that its effect to the coding efficiency is sequence-dependent.

- In the SAP_HV, the predicted samples are generated by the sample-based prediction, while the sample-based prediction is applied to the residual samples in the RDPCM. This difference leads to the selection of different intra prediction modes in the fast encoding process of the intra prediction, and eventually the different coded results.

Even though the decoding process described in the draft text of the RDPCM (Section 6) and the SAP_HV [5] look different, both processes are mathematically identical to each other. However, to implement the fully parallel decoder, the decoding process has to be formulated as proposed in the draft text of the RDPCM. Please note that the decoding process of the RDPCM is basically identical to the intra residual transform-bypass decoding process specified in Section 8.5.15 of H.264/AVC specification [3].

# Test results

The RDPCM is implemented on top of HEVC range extension software HM10.0_RExt2.0, and its coding gain is evaluated under the common test condition for RCE2 specified in [1].

Table 1. Performance of the RDPCM

| Config. Class | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class F | -9.3% | -5.7% | -4.6% |
| Class B | -4.5% | -0.9% | -0.6% |
| SC (GBR) | -11.1% | -7.7% | -7.1% |
| RangeExt | -2.8% | -0.7% | -0.6% |
| Overall (w/o SC) | -5.7% | -2.7% | -2.2% |
| Overall (w/ SC) | -8.4% | -5.2% | -4.6% |
| Enc Time[%] | 101% | 100% | 100% |
| Dec Time[%] | 98% | 102% | 98% |

Table 1 shows that the overall coding gain of the RDPCM is 8.4% with screen content sequences (SC) and 5.7% without SC in AI-Main, 5.2% with SC and 2.7% without SC in RA-Main, and 4.6% with SC and 2.2% without SC in LB-Main configurations, respectively. The RDPCM is more effective for SC than natural video sequences, e.g. those in Class B.

Table 2. Performance of the RDPCM with non-normative encoder-only change

| Config. Class | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class F | -9.9% | -6.0% | -4.8% |
| Class B | -4.8% | -0.9% | -0.7% |
| SC (GBR) | -12.0% | -8.3% | -7.7% |
| RangeExt | -3.0% | -0.7% | -0.6% |
| Overall (w/o SC) | -6.1% | -2.9% | -2.3% |
| Overall (w/ SC) | -9.0% | -5.6% | -5.0% |
| Enc Time[%] | 105% | 101% | 101% |
| Dec Time[%] | 96% | 101% | 101% |

Table 2 shows that the overall coding gain of RDPCM with non-normative encoder-only improvement is 9.0% with SC and 6.1% without SC in AI-Main, 5.6% with SC and 2.9% without SC in RA-Main, and 5.0% with SC and 2.3% without SC in LB-Main configurations, respectively. The additional coding gain up to 0.6% (in AI-Main with SC) could be achieved by the non-normative encoder-only change described in Section 2.2. The encoding time increases especially in AI-Main configuration, since the average number of intra prediction modes to be evaluated in the full RD increases in this encoder change. The encoding time increase in RA and LDB is marginal.

Table 3. Performance of the RDPCM without edge filtering

| Config. Class | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class F | -9.4% | -5.8% | -4.7% |
| Class B | -4.0% | -0.8% | -0.6% |
| SC (GBR) | -11.6% | -8.0% | -7.6% |
| RangeExt | -2.6% | -0.6% | -0.6% |
| Overall (w/o SC) | -5.6% | -2.7% | -2.2% |
| Overall (w/ SC) | -8.6% | -5.4% | -4.9% |
| Enc Time[%] | 100% | 100% | 100% |
| Dec Time[%] | 95% | 100% | 99% |

The only normative difference between the RDPCM and SAP_HV is the edge filtering. In this test, the edge filtering is manually disabled in the RDPCM implementation. Table 3 shows that the overall coding gain of RDPCM without edge filtering is 8.6% with SC and 5.6% without SC in AI-Main, 5.4% with SC and 2.7% without SC in RA-Main, and 4.9% with SC and 2.2% without SC in LB-Main configurations, respectively. As shown in the table, the coding efficiency of the RDPCM is improved for SC when the edge filtering is disabled, while there was almost no performance change or even loss for non-SC sequences. This means that the effect of the edge filtering to the coding efficiency of HEVC lossless coding is highly sequence-dependent.

Table 4. Performance of the RDPCM with non-normative encoder-only change and without edge filtering

| Config. Class | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class F | -10.2% | -6.2% | -4.9% |
| Class B | -4.4% | -0.9% | -0.6% |
| SC (GBR) | -12.6% | -9.0% | -8.3% |
| RangeExt | -2.9% | -0.7% | -0.6% |
| Overall (w/o SC) | -6.1% | -2.9% | -2.4% |
| Overall (w/ SC) | -9.4% | -6.0% | -5.3% |
| Enc Time[%] | 105% | 101% | 101% |
| Dec Time[%] | 95% | 102% | 101% |

In this test, the edge filter is manually disabled and the encoder improvement is applied to the RDPCM. Table 4 shows that the overall coding gain of RDPCM without edge filtering and with the encoder improvement is 9.4% with SC and 6.1% without SC in AI-Main, 6.0% with SC and 2.9% without SC in RA-Main, and 5.3% with SC and 2.4% without SC in LB-Main configurations, respectively.

# Complexity analysis

## Throughput

In the RDPCM, a pixel value p[x][y] of an (nT)x(nT) intra-coded block can be reconstructed from the reference pixel and the parsed residual samples as follows when the intra prediction mode is vertical

p[x][y]=p[x][-1]+k=0yr[x][k],   with x=0..nT-1 and y=0..nT-1,

or when the intra prediction mode is horizontal

p[x][y]=p[-1][y]+k=0xr[k][y],   with x=0..nT-1 and y=0..nT-1.

Since the reconstruction of each pixel is not dependent on the availability of the adjacent pixel, the RDPCM can be fully parallelized at both encoder and decoder, which means that all the samples in an intra PU can be encoded and decoded in parallel.

## Number of operations

The equations given in Section 4.1 also show that when the block size is (nT)x(nT), all the residual samples are available, and all the pixels need to be decoded in parallel at once, (nT+1)/2 additions are required on average to reconstruct one pixel value. On the other hand, when only each column or row needs to be decoded in parallel, only 1 addition is required to reconstruct one pixel value, since the reconstructed pixel of the previous column or row is already available, and then the reconstruction process can be reformulated as follows when the intra prediction mode is vertical

p[x][y]=p[x][y-1]+r[x][y],   with x=0..nT-1 and y=0..nT-1,

or when the intra prediction mode is horizontal

p[x][y]=p[x-1][y]+r[x][y],   with x=0..nT-1 and y=0..nT-1.

# Conclusion

In this contribution, the test results of RCE2 Test 1 on the RDPCM are provided. Under the common test conditions defined for RCE2, the RDPCM provides the significant coding gain while the intra prediction process and the syntax elements remain unchanged. Furthermore, the RDPCM is not a newly introduced coding tool, but exactly the same as the intra residual transform-bypass decoding process of H.264/AVC standard. It is recommended to adopt the RDPCM into the next version of the test model and the draft text for HEVC range extension.

# Proposed text changes

The proposed text changes are based on the document JCTVC-L1005-v2-JCTVC-L1003_v23.doc. It is suggested to modify “8.6.2 Scaling and transformation process” of the text and add “8.6.6 Intra residual transform-bypass decoding process” into the text. The changes are marked in yellow.

### 8.6.2 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nT specifying the size of the current transform block.

Output of this process is the (nT)x(nT) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = QP′Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = QP′Cb		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = QP′Cr			(8-3)

The (nT)x(nT) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1, the following ordered steps apply:

- The (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

- If CuPredMode[ xT ][ yT ] is equal to MODE_INTRA and predModeIntra (predModeIntra is set equal to IntraPredModeY[ xT ][ yT ] if cIdx is equal to 0, otherwise, predModeIntra is set equal to IntraPredModeC) is equal to either 26 (Intra_Vertical) or 10 (Intra_Horizontal), the intra residual transform-bypass decoding process as specified in subclause 8.6.6 is invoked with the size of the transform block nT, the intra prediction mode predModeIntra, and the (nT)x(nT) array r as the inputs, and the output is a modified version of the (nT)x(nT) array r.

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nT)x(nT) array of scaled transform coefficients d.

- The (nT)x(nT) array of residual samples r is derived as follows:

- If transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  7 )	(8-4)

- Otherwise (transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the colour component variable cIdx, and the (nT)x(nT) array of scaled transform coefficients d as inputs, and the output is an (nT)x(nT) array of residual samples r.

- The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-5)

- The residual sample values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-6)

8.6.6 Intra residual transform-bypass decoding process

This process is invoked when cu_transquant_bypass_flag is equal to 1, pred_mode_flag is equal to 1, and the applicable intra prediction mode is equal to either vertical or horizontal mode. The process for the Cb and Cr components is applied in the same way as for the luma component.

Inputs to this process are:

- a variable nT specifying the transform block size

- a variable predModeIntra specifying the intra prediction mode

- an (nT)x(nT) array r with elements r[ x ][ y ] which is either an array relating to a residual transform-bypass block of the luma component or an array relating to a residual transform-bypass block of the Cb and Cr component.

Output of this process is a modified version of the (nT)x(nT) array r with elements r[ x ][ y ] containing the result of the intra residual transform-bypass decoding process.

Let f be a temporary (nT)x(nT) array with element f[ x ][ y ], which are derived by;

f[ x ][ y ] = r[ x ][ y ]   with x=0..nT-1 and y=0..nT-1

Depending on predModeIntra, the following applies:

- If predModeIntra is equal to 26 (Intra_Vertical), the modified array r is derived by:

r[x][y]=k=0yf[x][k]  with x=0..nT-1 and y=0..nT-1

Otherwise (predModeIntra is equal to 10 (Intra_Horizontal)), the modified array r is derived by:

r[x][y]=k=0xf[k][y]  with x=0..nT-1 and y=0..nT-1

# References

- W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1122, Jan. 2013.

- Sunil Lee, Il-Koo Kim, and Chanyul Kim, “AHG7: Residual DPCM for HEVC lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0117, Jan. 2013.

- “ITU-T Recommendation H.264 and ISO/IEC 14496-10 AVC,” Joint Video Team of ISO/IEC JCT1/SC29/WG11 & ITU-T SG16/Q.6, Jan. 2012.

- M. Zhou, “AHG7: Sample-based angular intra prediction for HEVC range extension,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0176, Jan. 2013.

- M. Zhou and M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0056, Apr. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 2 (for PDAM),” Document of Joint Collaborative Team on Video Coding, JCTVC-L1005, Jan. 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).