| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0080 |
| --- | --- |

| Title: | AHG7: Post filter for colour-space transformation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Matsumura Shohei Matsuo Seishi Takamura Hiroshi Fujii Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2070 matuo.shohei@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

To reduce the redundancy among Y-Cb-Cr/R-G-B components, an in-loop color-space transformation method was proposed in JCTVC-L0371. In this contribution, a post filter which replaces the in-loop colour-space transformation with post processing is proposed. The overall average coding gains based on GBR BD-rate compared to the anchor (HM10.0 RExt. 2.0) of  23.8% / 13.5% / 7.5% (AI main-tier / high-tier / super-high-tier), 33.2% / 12.7% (RA main-tier / high-tier), and 33.4% / 18.5% (LD-B main-tier / high-tier) were confirmed. The average run-times of encoding and decoding (including pre/post filtering) were 86.0% and 90.3%, respectively.

# Introduction

An in-loop color-space transformation method was proposed in the contribution JCTVC-L0371 [1] at the previous Geneva meeting. JCTVC-L0371 reported remarkable coding gains in RGB444 format, the average coding gains based on GBR BD-rate ware 23.8% / 17.6% (AI main-tier / high-tier), 23.4% / 13.6% (RA main-tier / high-tier), and 20.2% / 12.3% (LD-B main-tier / high-tier), respectively.

However, the method proposed in JCTVC-L0371 requires some sample references to all components around the target Coding Unit or reference pictures for deriving a colour-space transformation matrix. Therefore, the independency between colour components will be sacrificed, and the transformation might cause the delay for in-loop process. Figure 1 shows the block diagram for decoder proposed in JCTVC-L0371. In this figure, the differences between the anchor (HM10.0 RExt. 2.0) and JCTVC-L0371 are set coloured in red.

Inter prediction

Output frame

Bit-stream

Entropy decoder

IQ

IDCT/IDST

+

Local decoded picture buffer

Intra prediction

Frame buffer

Colour-space inverse transform

In-loop filters

Matrix derivation

Figure 1: Block diagram for decoder proposed in JCVTV-L0371.

# Proposed post filter

In the proposed post filter, information for colour-space transformation is derived by encoder from original sequence, and is signaled into SEI message described in section 6. Therefore, there is no change in in-loop process for encoder and decoder. The independency among colour-components is maintained. Figure 2 shows the block diagram for decoder with proposed post filter. In this figure, the differences between the anchor (HM10.0 RExt. 2.0) and proposed one are set coloured in red.

To get the post filtered result, additional operations such as offset, scaling, and matrix multiplication will be required to each sample. However, parallel processors such as GPUs will reduce the additional computational complexity.

Inter prediction

Output frame

Bit-stream

Entropy decoder

IQ

IDCT/IDST

+

Local decoded picture buffer

Intra prediction

Frame buffer

In-loop filters

Post filter

Figure 2: Block diagram for proposed decoder.

# Experimental results

The source code was compiled by Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7, Intel Core i5-2500K@4.2GHz and 16GB RAM. Although experimental conditions are subject to the common test conditions [2], in the case of RA condition, closed GOP structure (DecodingRefreshType is set to IDR) is employed. The obtained results in Table 1 and Table 2 present coding efficiencies calculated by BD-rate [3] and computational complexities compared to HM10.0 range extensions 2.0 anchors. The results for Table 1 are cross-checked by KDDI [4].

# Post filter with proposed colour-space transformation

In the experiment for proposed post filter, in the case of AI condition, the information for colour-space transformation is derived in each frame, and signaled into SEI message of each frame. In the case of RA condition, the information is swapped to suitable information calculated within each interval of I-slices. In the case of LD-B condition, the information is derived in whole sequence.

| Table 1: Results of proposed post filter. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  | All Intra Super-High-tier |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR | G | B | R | GBR |
| Traffic | -34.8% | -16.0% | -37.9% | -26.8% | -26.2% | -8.5% | -31.3% | -18.6% | -17.2% | -2.6% | -24.4% | -11.6% |
| Kimono | -27.1% | -11.2% | -27.8% | -20.7% | -10.1% | 18.0% | -16.3% | 4.8% | 9.1% | 19.3% | 0.3% | 11.6% |
| EBULupoCandlelight | -42.9% | -39.3% | -40.3% | -40.7% | -42.4% | -33.7% | -41.7% | -38.6% | -39.8% | -22.3% | -41.8% | -32.6% |
| EBURainFruits | -49.1% | -34.5% | -45.5% | -42.6% | -46.0% | -30.2% | -41.7% | -38.5% | -41.5% | -23.3% | -37.1% | -32.6% |
| VenueVu | -27.0% | -24.0% | -25.8% | -25.4% | -23.3% | -20.7% | -22.1% | -21.9% | -18.8% | -16.8% | -18.1% | -17.9% |
| DucksAndLegs | -14.2% | 10.2% | -13.1% | 0.2% | 13.7% | 14.7% | 11.1% | 14.2% | 28.5% | 17.5% | 22.5% | 22.3% |
| OldTownCross | -28.3% | -2.2% | -25.8% | -15.2% | -3.8% | 3.9% | -8.1% | -1.5% | 3.1% | 1.4% | -0.8% | 1.2% |
| ParkScene | -23.6% | -13.5% | -21.6% | -19.2% | -15.8% | 0.8% | -14.8% | -7.8% | -2.4% | 3.9% | -6.0% | -0.7% |
| Overall | -30.9% | -16.3% | -29.7% | -23.8% | -19.2% | -7.0% | -20.6% | -13.5% | -9.9% | -2.9% | -13.2% | -7.5% |
|  | -30.2% | -16.5% | -28.0% | -22.1% | -18.4% | -10.4% | -20.8% | -13.6% | -11.2% | -3.1% | -14.1% | -8.0% |
| Enc Time[%] | 89.9% |  |  |  | 89.7% |  |  |  | 90.4% |  |  |  |
| Dec Time[%] | 90.4% |  |  |  | 87.0% |  |  |  | 85.0% |  |  |  |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |  |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR |  |  |  |  |
| Traffic | -40.4% | -13.9% | -42.7% | -29.2% | -34.8% | 0.0% | -38.7% | -18.8% |  |  |  |  |
| Kimono | -32.9% | -35.9% | -35.2% | -35.7% | -33.6% | 5.0% | -37.7% | -20.0% |  |  |  |  |
| EBULupoCandlelight | -37.5% | -35.1% | -36.3% | -36.2% | -42.9% | -36.9% | -43.3% | -40.9% |  |  |  |  |
| EBURainFruits | -46.0% | -30.0% | -46.8% | -40.5% | -47.9% | -29.3% | -47.9% | -40.9% |  |  |  |  |
| VenueVu | -22.8% | -22.3% | -25.2% | -24.3% | -21.8% | -21.2% | -23.7% | -23.0% |  |  |  |  |
| DucksAndLegs | -36.2% | 2.4% | -35.1% | -26.7% | -8.2% | 42.8% | -5.2% | 15.1% |  |  |  |  |
| OldTownCross | -40.3% | -28.8% | -52.7% | -46.7% | -9.3% | 16.2% | -15.8% | -4.1% |  |  |  |  |
| ParkScene | -25.2% | -24.5% | -27.4% | -26.4% | -25.5% | 1.9% | -28.6% | -18.2% |  |  |  |  |
| Overall | -35.2% | -23.5% | -37.7% | -33.2% | -28.0% | -2.7% | -30.1% | -18.8% |  |  |  |  |
|  | -34.8% | -15.2% | -38.1% | -32.8% | -20.4% | -15.4% | -25.5% | -11.7% |  |  |  |  |
| Enc Time[%] | 84.4% |  |  |  | 80.3% |  |  |  |  |  |  |  |
| Dec Time[%] | 99.4% |  |  |  | 90.6% |  |  |  |  |  |  |  |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |  |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR |  |  |  |  |
| Traffic | -45.2% | -11.2% | -48.7% | -31.5% | -39.0% | 5.1% | -45.4% | -19.8% |  |  |  |  |
| Kimono | -33.8% | -32.2% | -33.7% | -33.7% | -40.1% | 15.3% | -42.0% | -15.9% |  |  |  |  |
| EBULupoCandlelight | -34.7% | -31.5% | -32.6% | -32.4% | -46.5% | -32.6% | -45.0% | -40.4% |  |  |  |  |
| EBURainFruits | -46.9% | -26.9% | -44.7% | -39.2% | -50.0% | -25.1% | -46.5% | -40.0% |  |  |  |  |
| VenueVu | -23.8% | -20.8% | -24.3% | -23.7% | -22.6% | -20.0% | -23.0% | -22.6% |  |  |  |  |
| DucksAndLegs | -46.9% | 8.5% | -44.7% | -27.8% | -12.7% | 42.8% | -6.3% | 16.2% |  |  |  |  |
| OldTownCross | -55.0% | -29.3% | -58.0% | -51.8% | -21.2% | 14.8% | -16.4% | -7.6% |  |  |  |  |
| ParkScene | -27.9% | -22.4% | -27.8% | -26.8% | -32.5% | 8.1% | -31.6% | -17.9% |  |  |  |  |
| Overall | -39.3% | -20.7% | -39.3% | -33.4% | -33.1% | 1.1% | -32.0% | -18.5% |  |  |  |  |
|  | -39.0% | -5.1% | -39.7% | -32.3% | -21.6% | -6.3% | -28.1% | -15.8% |  |  |  |  |
| Enc Time[%] | 85.2% |  |  |  | 82.3% |  |  |  |  |  |  |  |
| Dec Time[%] | 93.8% |  |  |  | 85.9% |  |  |  |  |  |  |  |

# Post filter with YCbCr colour-space transformation

As the additional experiment, results of post filter with YCbCr colour-space transformation are observed (Table 2). In this experiment, additional overhead for SEI message is only description for payloadType (8-bit), no other signals are coded into the payload of SEI message.

| Table 2: Results of post filter with YCbCr colour-space transformation. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  | All Intra Super-High-tier |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR | G | B | R | GBR |
| Traffic | -48.4% | -5.6% | -39.7% | -26.6% | -44.7% | 0.4% | -34.2% | -19.9% | -39.6% | 5.7% | -28.0% | -13.7% |
| Kimono | -47.8% | -3.9% | -24.0% | -21.0% | -47.3% | 14.3% | -15.8% | -3.6% | -28.5% | 29.7% | 2.7% | 7.7% |
| EBULupoCandlelight | -52.5% | -32.4% | -45.6% | -43.5% | -54.0% | -26.3% | -45.1% | -40.7% | -54.0% | -5.4% | -40.5% | -29.8% |
| EBURainFruits | -53.4% | -36.4% | -48.9% | -45.7% | -52.0% | -31.4% | -44.9% | -41.9% | -49.8% | -22.2% | -38.0% | -35.2% |
| VenueVu | -34.6% | -1.9% | -16.4% | -17.3% | -31.6% | 4.6% | -10.5% | -12.0% | -28.4% | 18.3% | 0.1% | -3.3% |
| DucksAndLegs | -37.6% | 11.2% | -15.8% | -7.5% | -16.0% | 15.6% | 0.8% | 3.8% | 4.0% | 24.2% | 13.5% | 15.9% |
| OldTownCross | -55.9% | 11.2% | -29.3% | -20.6% | -39.4% | 25.9% | 2.0% | 1.6% | -24.0% | 23.2% | 6.4% | 4.5% |
| ParkScene | -40.1% | 6.6% | -15.3% | -14.8% | -36.1% | 15.1% | -10.5% | -5.3% | -24.6% | 21.9% | -0.9% | 2.6% |
| Overall | -46.3% | -6.4% | -29.4% | -24.6% | -40.1% | 2.3% | -19.8% | -14.8% | -30.6% | 11.9% | -10.6% | -6.4% |
|  | -46.2% | -5.4% | -28.8% | -23.8% | -39.8% | -0.5% | -19.0% | -15.0% | -30.3% | 11.2% | -10.4% | -6.8% |
| Enc Time[%] | 85.3% |  |  |  | 84.3% |  |  |  | 85.0% |  |  |  |
| Dec Time[%] | 94.1% |  |  |  | 88.3% |  |  |  | 84.6% |  |  |  |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |  |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR |  |  |  |  |
| Traffic | -51.7% | -2.1% | -42.7% | -27.8% | -51.5% | 10.6% | -39.0% | -18.9% |  |  |  |  |
| Kimono | -49.0% | -16.5% | -32.4% | -32.8% | -56.3% | 9.8% | -31.6% | -22.9% |  |  |  |  |
| EBULupoCandlelight | -48.8% | -29.9% | -41.5% | -40.3% | -57.2% | -30.7% | -46.2% | -44.8% |  |  |  |  |
| EBURainFruits | -49.9% | -34.6% | -48.3% | -43.9% | -52.6% | -32.9% | -48.5% | -44.1% |  |  |  |  |
| VenueVu | -32.3% | -1.4% | -16.7% | -17.8% | -32.4% | 4.0% | -13.0% | -15.1% |  |  |  |  |
| DucksAndLegs | -51.9% | 4.6% | -31.3% | -27.4% | -38.8% | 25.8% | -12.1% | -3.7% |  |  |  |  |
| OldTownCross | -66.6% | -30.1% | -53.6% | -52.8% | -57.3% | 32.7% | -5.1% | -7.7% |  |  |  |  |
| ParkScene | -42.3% | 4.5% | -20.7% | -21.7% | -47.2% | 16.3% | -21.3% | -17.1% |  |  |  |  |
| Overall | -49.1% | -13.2% | -35.9% | -33.1% | -49.2% | 4.5% | -27.1% | -21.8% |  |  |  |  |
|  | -49.0% | -10.7% | -35.6% | -32.3% | -46.0% | 12.5% | -12.5% | -8.6% |  |  |  |  |
| Enc Time[%] | 80.3% |  |  |  | 74.9% |  |  |  |  |  |  |  |
| Dec Time[%] | 109.5% |  |  |  | 95.8% |  |  |  |  |  |  |  |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |  |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR |  |  |  |  |
| Traffic | -54.4% | 3.7% | -46.9% | -28.3% | -53.2% | 19.6% | -42.3% | -17.2% |  |  |  |  |
| Kimono | -49.7% | -10.9% | -31.8% | -30.8% | -59.3% | 14.9% | -33.4% | -19.7% |  |  |  |  |
| EBULupoCandlelight | -46.2% | -22.4% | -39.0% | -35.9% | -59.6% | -20.5% | -48.1% | -42.6% |  |  |  |  |
| EBURainFruits | -51.2% | -31.9% | -46.5% | -43.0% | -55.1% | -29.8% | -47.3% | -43.8% |  |  |  |  |
| VenueVu | -35.2% | 1.4% | -16.1% | -17.6% | -35.1% | 6.7% | -12.6% | -14.8% |  |  |  |  |
| DucksAndLegs | -57.1% | 9.8% | -37.4% | -27.3% | -37.4% | 23.6% | -9.5% | -3.0% |  |  |  |  |
| OldTownCross | -75.5% | -44.7% | -62.0% | -62.8% | -61.0% | 27.1% | -1.2% | -11.0% |  |  |  |  |
| ParkScene | -45.1% | 9.5% | -19.5% | -21.3% | -52.4% | 22.1% | -22.0% | -16.8% |  |  |  |  |
| Overall | -51.8% | -10.7% | -37.4% | -33.4% | -51.6% | 7.9% | -27.0% | -21.1% |  |  |  |  |
|  | -51.8% | -5.9% | -37.1% | -32.2% | -44.7% | 23.9% | 51.3% | 5.3% |  |  |  |  |
| Enc Time[%] | 80.7% |  |  |  | 76.9% |  |  |  |  |  |  |  |
| Dec Time[%] | 102.4% |  |  |  | 89.9% |  |  |  |  |  |  |  |

# Comparison of YCbCr colour-space transformation and proposed one

Table 3 shows comparison results of post filter with YCbCr colour-space transformation and proposed post filter. In this table, the anchor is set to the result of post filter with YCbCr colour-space transformation.

| Table 3: Comparison of YCbCr colour-space transformation (as the anchor) and proposed one. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  | All Intra Super-High-tier |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR | G | B | R | GBR |
| Traffic | 25.6% | -14.1% | 1.1% | -2.7% | 32.9% | -12.0% | 2.1% | -1.1% | 37.1% | -9.6% | 3.3% | 0.9% |
| Kimono | 35.9% | -16.6% | -4.9% | -3.3% | 61.3% | -4.6% | -5.8% | 3.2% | 57.7% | -4.8% | -5.1% | 4.1% |
| EBULupoCandlelight | 19.2% | -10.9% | 10.5% | 4.9% | 24.2% | -13.9% | 6.9% | 2.2% | 30.4% | -19.7% | -1.0% | -3.8% |
| EBURainFruits | 8.4% | 1.6% | 5.5% | 4.6% | 11.9% | 0.8% | 4.8% | 4.7% | 15.7% | -2.2% | 0.7% | 2.7% |
| VenueVu | 11.8% | -23.4% | -11.7% | -10.4% | 12.4% | -24.8% | -13.3% | -11.5% | 13.5% | -28.5% | -17.1% | -14.4% |
| DucksAndLegs | 39.1% | -5.7% | -1.4% | 3.9% | 39.5% | -2.9% | 5.4% | 6.8% | 29.9% | -3.6% | 7.5% | 6.8% |
| OldTownCross | 61.2% | -18.8% | -5.9% | 0.3% | 65.1% | -17.9% | -12.3% | -4.0% | 44.3% | -16.9% | -9.2% | -3.3% |
| ParkScene | 27.2% | -24.8% | -8.3% | -7.2% | 31.9% | -17.3% | -7.2% | -5.2% | 31.5% | -14.3% | -6.2% | -3.7% |
| Overall | 28.5% | -14.1% | -1.9% | -1.2% | 34.9% | -11.6% | -2.4% | -0.6% | 32.5% | -12.5% | -3.4% | -1.3% |
|  | 29.2% | -14.7% | -1.3% | -0.8% | 35.0% | -10.8% | -3.3% | -0.4% | 30.3% | -11.9% | -3.9% | -1.1% |
| Enc Time[%] | 105.4% |  |  |  | 106.3% |  |  |  | 106.4% |  |  |  |
| Dec Time[%] | 96.1% |  |  |  | 98.4% |  |  |  | 100.5% |  |  |  |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |  |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR |  |  |  |  |
| Traffic | 22.3% | -15.4% | -0.9% | -4.2% | 32.7% | -13.5% | -1.8% | -3.1% |  |  |  |  |
| Kimono | 28.6% | -26.0% | -2.8% | -4.8% | 45.9% | -13.9% | -9.3% | -1.9% |  |  |  |  |
| EBULupoCandlelight | 20.5% | -6.8% | 10.5% | 7.4% | 30.1% | -10.2% | 7.7% | 6.7% |  |  |  |  |
| EBURainFruits | 7.8% | 6.7% | 3.0% | 6.0% | 9.9% | 5.6% | 1.8% | 5.8% |  |  |  |  |
| VenueVu | 13.8% | -20.9% | -10.1% | -7.7% | 15.2% | -23.7% | -12.1% | -9.2% |  |  |  |  |
| DucksAndLegs | 30.2% | -9.1% | -7.7% | -2.4% | 44.8% | 7.6% | -0.3% | 13.6% |  |  |  |  |
| OldTownCross | 66.8% | -22.4% | -7.4% | 1.4% | 114.6% | -18.6% | -21.9% | -2.6% |  |  |  |  |
| ParkScene | 27.7% | -29.9% | -7.8% | -6.6% | 37.4% | -20.5% | -9.8% | -4.8% |  |  |  |  |
| Overall | 27.2% | -15.5% | -2.9% | -1.4% | 41.3% | -10.9% | -5.7% | 0.6% |  |  |  |  |
|  | 26.9% | -13.4% | -3.7% | -1.7% | 43.2% | -19.5% | -10.4% | -3.0% |  |  |  |  |
| Enc Time[%] | 105.2% |  |  |  | 107.2% |  |  |  |  |  |  |  |
| Dec Time[%] | 90.8% |  |  |  | 94.5% |  |  |  |  |  |  |  |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |  |  |  |  |
|  | G | B | R | GBR | G | B | R | GBR |  |  |  |  |
| Traffic | 18.9% | -17.4% | -3.9% | -6.7% | 28.6% | -16.2% | -8.0% | -6.7% |  |  |  |  |
| Kimono | 28.9% | -25.7% | -0.1% | -3.8% | 43.7% | -8.0% | -10.5% | -0.9% |  |  |  |  |
| EBULupoCandlelight | 20.6% | -11.2% | 13.4% | 7.0% | 29.7% | -15.8% | 9.9% | 4.8% |  |  |  |  |
| EBURainFruits | 9.1% | 6.1% | 3.8% | 6.4% | 11.4% | 6.5% | 2.3% | 6.7% |  |  |  |  |
| VenueVu | 17.1% | -21.2% | -9.6% | -7.3% | 18.9% | -24.3% | -11.8% | -9.0% |  |  |  |  |
| DucksAndLegs | 24.0% | -5.1% | -13.3% | -1.8% | 36.1% | 9.6% | -4.3% | 13.0% |  |  |  |  |
| OldTownCross | 74.5% | -7.0% | -3.9% | 8.1% | 112.6% | -15.2% | -23.1% | -1.5% |  |  |  |  |
| ParkScene | 29.4% | -30.0% | -8.5% | -6.6% | 39.4% | -20.1% | -12.5% | -5.3% |  |  |  |  |
| Overall | 27.8% | -14.0% | -2.8% | -0.6% | 40.0% | -10.4% | -7.2% | 0.1% |  |  |  |  |
|  | 27.4% | -9.3% | -3.5% | -0.6% | 39.1% | -17.8% | -14.3% | -6.5% |  |  |  |  |
| Enc Time[%] | 105.5% |  |  |  | 107.0% |  |  |  |  |  |  |  |
| Dec Time[%] | 91.6% |  |  |  | 95.6% |  |  |  |  |  |  |  |

# Conclusion

In this contribution, a post filter which replaces the in-loop colour-space transformation proposed in JCTVC-L0371 into post processing was proposed. As the result, the overall average coding gains based on GBR BD-rate compared to the anchor (HM10.0 RExt 2.0) of 23.8% / 13.5% / 7.5% (AI main-tier / high-tier / super-high-tier), 33.2% / 12.7% (RA main-tier / high-tier), and 33.4% / 18.5% (LD-B main-tier / high-tier) were confirmed. The average run-times of encoding and decoding (including pre/post filtering) were 86.0% and 90.3%, respectively.

It is recommended that the proposal is adopted in HEVC range extensions.

# References

- K. Kawamura, T. Yoshino, S. Naito, “AHG7: In-loop color-space transformation of residual signals for range extensions,” JCTVC-L0371, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013

- D. Flynn, C. Rosewarnem, “Common test conditions and software reference configurations for HEVC range extensions, ” JCTVC-L1006, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013

- G. Bjøntegaard, “Improvements of the BD-PSNR model,” VCEG-AI11, ITU-T SG16 Q.6, 35th VCEG meeting, Berlin, Germany, July 2008.

- K. Kawamura, S. Naito, “Cross-check of post filter for colour-space transformation (JCTVC-M0080), ” JCTVC-M0408, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013.

# Patent rights declaration(s)

NTT may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

# Syntax and semantics

In this section, syntax and semantics for proposed post filter are described. The modified syntax set highlighted in yellow are specified in Table 4. And additional syntax is specified in Table 5.

| Table 4: General SEI message syntax |  |
| --- | --- |
| sei_payload( payloadType, payloadSize ) { | Descriptor |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| … |  |
| else if( payloadType == 24 ) |  |
| post_filter_colour_transformation( payloadSize ) |  |
| … |  |
| } |  |

| Table 5: Post filter colour-space transformation SEI message syntax |  |
| --- | --- |
| post_filter_colour_transformation( payloadSize ){ | Descriptor |
| ct_segment_mode | ue(v) |
| if(ct_segment_mode == 1){ |  |
| ct_matrix_rotation_index | f(9) |
| RotationIndex[0] = ct_matrix_rotation_index |  |
| RotationIndex[1] = 50 |  |
| RotationIndex[2] = 64 |  |
| for(cIdx = 1; cIdx < 3; cIdx++){ |  |
| ct_matrix_rotation_zero_flag[cIdx] | u(1) |
| if(!ct_matrix_rotation_zero_flag[cIdx]){ |  |
| ct_matrix_rotation_sign_minus_flag[cIdx] | u(1) |
| ct_matrix_rotation_absolute_index_minus1[cIdx] | ue(v) |
| Sign = ct_matrix_rotation_sign_minus_flag[cIdx] ? −1 : 1 |  |
| RotationIndex[cIdx] += Sign * (ct_matrix_rotation_absolute_index_minus1[cIdx] + 1) |  |
| } |  |
| } |  |
| RotationIndex[1] = RotationIndex[1] < 0 ? 512 + RotationIndex[1] : RotationIndex[1] |  |
| RotationIndex[2] = RotationIndex[2] < 0 ? 512 + RotationIndex[2] : RotationIndex[2] |  |
| ScaleIndex[0] = ScaleIndex[1] = ScaleIndex[2] = 255 |  |
| for(cIdx = 0; cIdx < 3; cIdx++){ |  |
| ct_matrix_scale_index[cIdx] | ue(v) |
| ScaleIndex[cIdx] –= ct_matrix_scale_index[cIdx] |  |
| } |  |
| OffsetForOriginalCoord[0] = OffsetForOriginalCoord[1] = OffsetForOriginalCoord[2] = 1 << ( BitDepthY − 1) |  |
| for(cIdx = 0; cIdx < 3; cIdx++){ |  |
| ct_matrix_offset_for_original_zero_flag[cIdx] | u(1) |
| if(!ct_matrix_offset_for_original_zero_flag[cIdx]){ |  |
| ct_matrix_offset_for_original_sign_minus_flag[cIdx] | u(1) |
| ct_matrix_offset_for_original_absolute_index_minus1[cIdx] | ue(v) |
| Sign = ct_matrix_offset_for_original_sign_minus_flag[cIdx] ? −1 : 1 |  |
| OffsetForOriginalCoord[cIdx] += Sign * (ct_matrix_offset_for_original_absolute_index_minus1[cIdx] + 1) |  |
| } |  |
| } |  |
| OffsetForTransCoord[0] = OffsetForTransCoord[1] = OffsetForTransCoord[2] = 0 |  |
| for(cIdx = 0; cIdx < 3; cIdx++){ |  |
| ct_matrix_offset_for_trans_zero_flag[cIdx] | u(1) |
| if(!ct_matrix_offset_for_trans_zero_flag[cIdx]){ |  |
| ct_matrix_offset_for_trans_sign_minus_flag[cIdx] | u(1) |
| ct_matrix_offset_for_trans_absolute_index_minus1[cIdx] | ue(v) |
| Sign = ct_matrix_offset_for_trans_sign_minus_flag[cIdx] ? −1 : 1 |  |
| OffsetForTransCoord[cIdx] = Sign * (ct_matrix_offset_for_trans_absolute_index_minus1[cIdx] + 1) |  |
| } |  |
| } |  |
| } |  |
| } |  |

ct_segment_mode equal to 1 specifies that the information for a colour-space transformation is present in the payload. ct_segment_mode equal to 2 specifies that the information is not applied to the segment, and the post filter is skipped in the segment. ct_segment_mode equal to 0 specifies that all the settings for colour-space transformation are referred from previous frame. Generally, ct_segment_mode equal to 1 or 2 is utilized only to intra frame.

RotationIndex[cIdx] specifies the angle for rotation matrix in transformed colour-space coordinate.

ct_matrix_rotation_index specifies the angle for rotation matrix in transformed colour-space coordinate of the 0th-axis.

ct_matrix_rotation_zero_flag[cIdx] equal to 1 specifies that the delta angle for rotation matrix in transformed colour-space coordinate of the cIdx-axis is equal to 0. ct_matrix_rotation_zero_flag[cIdx] equal to 0 specifies that the delta angle is present.

ct_matrix_rotation_sign_minus_flag[cIdx] equal to 1 specifies that the delta angle is negative value. ct_matrix_rotation_sign_minus_flag[cIdx] equal to 0 specifies that the delta angle is positive value.

ct_matrix_rotation_absolute_index_minus1[cIdx] specifies the absolute index of the delta angle. ct_matrix_rotation_absolute_index_minus1[cIdx] shall be in the range of 0 to 255.

ScaleIndex[cIdx] specifies the scaling factor for rotation matrix in the transformed colour-space coordinate.

ct_matrix_scale_index[cIdx] specifies the scale index for rotation matrix in the transformed colour-space coordinate of cIdx-axis. ct_matrix_scale_index[cIdx] shall be in the range of 0 to 255.

OffsetForOriginalCoord[cIdx] specifies the offset for original colour-space coordinate of cIdx-axis.

ct_matrix_offset_for_original_zero_flag[cIdx] equal to 1 specifies that the delta offset in original colour-space coordinate of the cIdx-axis is equal to 0. ct_matrix_offset_for_original_zero_flag[cIdx] equal to 0 specifies that the delta offset is present.

ct_matrix_offset_for_original_sign_minus_flag[cIdx] equal to 1 specifies that the delta offset is negative value. ct_matrix_offset_for_original_sign_minus_flag[cIdx] equal to 0 specifies that the delta offset is positive value.

ct_matrix_offset_for_original_absolute_index_minus1[cIdx] specifies the absolute index of the delta offset.

OffsetForTransCoord[cIdx] specifies the offset for transformed colour-space coordinate of cIdx-axis.

ct_matrix_offset_for_trans_zero_flag[cIdx] equal to 1 specifies that the delta offset in transformed colour-space coordinate of the cIdx-axis is equal to 0. ct_matrix_offset_for_trans_zero_flag[cIdx] equal to 0 specifies that the delta offset is present.

ct_matrix_offset_for_trans_sign_minus_flag[cIdx] equal to 1 specifies that the delta offset is negative value. ct_matrix_offset_for_trans_sign_minus_flag[cIdx] equal to 0 specifies that the delta offset is positive value.

ct_matrix_offset_for_trans_absolute_index_minus1[cIdx] specifies the absolute index of the delta offset.

# Decoder description

In this section, the decoding process of proposed post filter of colour-space transformation is described. Where, variables used in this section are described below:

Inputs of this process are the reconstructed picture buffers prior to post filter recPictureL, recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture buffers after post filter filteredPictureL, filteredPictureCb and filteredPictureCr.

For the derivation of filteredPictureL, filteredPictureCb and filteredPictureCr, following ordered steps apply:

- If ct_segment is equal to 2, all samples of filteredPictureL, filteredPictureCb and filteredPictureCr are set equal to recPictureL, recPictureCb and recPictureCr, respectively.

- Otherwise, if ct_segment is equal to 1, all the settings for colour-space transformation RotationIndex, ScaleIndex, OffsetForOriginalCoord and OffsetForTransCoord are referred from previous coded picture.

- For all the sample of recPicture with location (x, y), following ordered steps apply:

  - The derivation process for colour-transformation matrix as specified in 7.1 is involed with angles for rotation matrix in transformed coordinate RotationIndex as the inputs and the colour-space transformation matrix Mtx is the output.

  - Filtered results are derived by:

PelMaxL = (1 << BitDepthY) − 1							(7-1)
PelMaxC = (1 << BitDepthC) − 1							(7-2)

OffsetL = 1 << (BitDepthY − 1) 							(7-3)
OffsetC = 1 << (BitDepthC − 1) 							(7-4)

RecL  = recPictureL[x][y] 								(7-5)
RecCb = recPictureCb[x][y] 								(7-6)
RecCr = recPictureCr[x][y] 								(7-7)

SL  = (3 * ScaleIndex[0] + 1024) * (RecL  − OffsetL) + OffsetForTransCoord[0] 	(7-8)
SCb = (3 * ScaleIndex[1] + 1024) * (RecCb − OffsetC) + OffsetForTransCoord[1] 	(7-9)
SCr  = (3 * ScaleIndex[2] + 1024) * (RecCr − OffsetC) + OffsetForTransCoord[2] 	(7-10)

TL  = (Mtx[0][0] * SL + Mtx[1][0] * SCb + Mtx[2][0] * SCr + 8388608) >> 24		(7-11)
TCb = (Mtx[0][1] * SL + Mtx[1][1] * SCb + Mtx[2][1] * SCr + 8388608) >> 24		(7-12)
TCr  = (Mtx[0][2] * SL + Mtx[1][2] * SCb + Mtx[2][2] * SCr + 8388608) >> 24		(7-13)

filteredPictureL [x][y] = Clop3(0, PelMaxL, TL  + OffsetForOriginalCoord[0]) 		(7-14)
filteredPictureCb[x][y] = Clop3(0, PelMaxC, TCb + OffsetForOriginalCoord[1]) 		(7-15)
filteredPictureCr[x][y] = Clop3(0, PelMaxC, TCr + OffsetForOriginalCoord[2]) 		(7-16)

# Derivation process for colour-transformation matrix

Inputs to this process are angles for rotation matrix in transformed coordinate RotationIndex[cIdx].

Output of this process is 3x3 colour-space transformation matrix Mtx.

Mtx is derived by:

Sx = IntSin[RotationIndex[0]] 								(7-17)
Sy = IntSin[RotationIndex[1] == 0 ? 0 : 512 – RotationIndex[1]] 				(7-18)
Sz = IntSin[RotationIndex[2]] 								(7-19)

Cx = IntCos[RotationIndex[0]] 								(7-20)
Cy = IntCos[RotationIndex[1] == 0 ? 0 : 512 – RotationIndex[1]] 				(7-21)
Cz = IntCos[RotationIndex[2]] 								(7-22)

Mx=1024000Cx−Sx0SxCx									(7-23)
My=Cy0Sy010240−Sy0Cy									(7-24)
Mz=Cz−Sz0SzCz0001024									(7-25)
Mtx = MzMyMx / 65536									(7-26)

NOTE – IntSin[n] and IntCos[n] ( n = 0..511 ) are assigned by

IntSin[ n ]= IntCosTable[128−n];n<129IntCosTable[n−128];n<257IntCosTable[384−n];n<385IntCosTable[n−384];otherwise.					(7-27)

IntCos[ n ]= IntCosTable[n];n<129IntCosTable[256−n];n<257IntCosTable[n−256];n<385IntCosTable[512−n];otherwise.					(7-28)

NOTE – Pre-computed look-up-table IntCosTable[n] ( n = 0..128 ) is assigned by

IntCosTable[ n ]=round1024*cosnπ256.							(7-29)

Here, function cos denotes mathematical cosine function.