| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0083 |
| --- | --- |

| Title: | RCE2: Cross-verification of Test 3 – Sample-based Angular Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunil Lee Chanyul Kim | Email: | sunil.lee@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-verification results of RCE2 Test 3 on the sample-based angular intra prediction (SAP). The source code provided by the proponent was reviewed and it was confirmed that the implementation was consistent with the algorithm description of the contribution document. The coding performance is independently evaluated by Samsung and the results match those provided by the proponent. The throughput analysis provided in the contribution is also reviewed and discussed in this cross-verification report.

# Introduction

This contribution reports the cross-verification results of RCE2 Test 3 on the sample-based angular intra prediction (SAP) [1]. In the SAP, the sample-based prediction is applied to all 33 angular intra prediction directions in HEVC lossless coding [2]. According to the implementation schedule agreed in the RCE2 [1], the proponent implemented the SAP on top of HEVC range extension software HM10.0_RExt2.0, and then provided the source code and test results to the CE participants. Samsung reviewed the source code and confirmed that the implementation was consistent with the algorithm described in the contribution document. Samsung compiled the source code and did the simulation on a linux cluster under the common test condition defined for RCE2 [1].

# Test results

The cross-verification results of the SAP are summarized in Table 1, and they match those provided by the proponent [3].

Table 1. Cross-verification results of the SAP

| Config. Class | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class F | -11.6% | -6.7% | -5.4% |
| Class B | -5.8% | -1.2% | -0.9% |
| SC (GBR) | -10.4% | -6.9% | -6.4% |
| RangeExt | -4.1% | -1.0% | -0.9% |
| Overall (w/o SC) | -7.4% | -3.3% | -2.7% |
| Overall (w/ SC) | -8.9% | -5.1% | -4.6% |
| Enc Time[%] | 103% | 101% | 101% |
| Dec Time[%] | 95% | 102% | 101% |

# Complexity analysis

In terms of the throughput, the SAP can be fully parallelized at the encoder, while it can be executed only at a speed of one row or one column at the decoder, since the pixel values of the previous row or column have to be available to reconstruct the pixel values of the current row or column. In terms of the number of operations, when the reference pixel value at the fractional-pel position is used for the prediction, 4 additions, 2 multiplications, and 1 shift operations are required to reconstruct one pixel value, while when the reference pixel value at the integer-pel position is used, e.g. when the intra prediction mode is either horizontal or vertical, only one addition is required.

# Conclusion

This contribution reports the cross-verification results of RCE2 Test 3 on the SAP. It was confirmed that the implementation was consistent with the algorithm description and the test results matched those provided by the proponent. The complexity of the SAP was also analyzed in terms of throughput and number of operations.

# References

- W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1122, Jan. 2013.

- M. Zhou, “AHG7: Sample-based angular intra prediction for HEVC range extension,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0176, Jan. 2013.

- M. Zhou and M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0056, Apr. 2013.